3GPP TSG-RAN WG4 Meeting #92-bis											  	     		    R4-1911644

Chongqing, China, 14 – 18 October, 2019

Source: 	Nokia, Nokia Shanghai Bell

Title: 					TP to TR 38.883: FR2 DL 256QAM link level simulation results

Agenda item:			8.12.2

Document for:			Approval

# 1	Introduction

A work item on 256QAM DL support for frequency range 2 was agreed in [1]. The work was started in RAN4#91 where parameters for link level simulations were agreed and initial simulation results were provided in RAN4#92 in [3]. In this contribution further simulation results are provided.

# 2	Discussion

The work on FR2 DL 256QAM aims at enhancing the network throughput especially for scenarios with very high data rate demand and for UEs in favourable channel conditions. Being able to serve these UEs in less time also frees resources and therefore enhances also the overall network capacity.

Work item objectives in [1] include two phases, first a feasibility and performance benefit study to confirm that FR2 DL 256QAM provides gains over 64QAM and then the actual setting of requirements. In this contribution link level simulation results on performance gains of 256QAM over 64QAM are presented. Used simulation parameters have been gathered in Table 1.

Table 1: Link level simulation parameters

| Parameter ​ | Value ​ |
| --- | --- |
| Carrier frequency​ | 29 GHz ​ |
| CBW​ | 100MHz​ |
| SCS​ | 60kHz​ |
| Allocated RBs​ | Full allocation​ |
| Propagation​ | TDL-D 30ns delay spread, 35Hz Doppler frequency |
| MCS​ | Rank-1; 64QAM: MCS 26, 28 in TS 38.214 Table 5.1.3.1-1. 256QAM: MCS 21, 23, 25, 27 in TS 38.214 Table 5.1.3.1-2. Rank-2; Link adaptation up to 64 or 256QAM enabled by CSI-RS as in 38.101-4 with OLLA correction factor up to +/- 3 dB.. |
| Precoding​ | Rank-1: Fixed precoder applying direct spatial domain multiplexing Rank-2: Based on maximum SINR codebook |
| Symbol type ​ | CP-OFDM ​ |
| HARQ ​ | Rank-1: None ​ Rank-2: With 3 re-transmissions |
| Antenna configuration​ | Fading channel: 2x2 for Rank-1 and Rank-2 |
| Channel estimation ​ | Practical ​ |
| Receiver type​ | MMSE​ |
| PDSCH configuration​ | Type A mapping, Start symbol 1, Duration 13 (for D slots)​ |
| DMRS configuration​ | Type 1, Single symbol, 1 additional DMRS​ |
| PTRS configuration​ | KPTRS : 2 (every 2 RBs), LPTRS : 1 (every 1 symbol)​ |
| Phase noise model​ | Phase noise profile from [2], scaled to match the frequency difference modelled Phase noise for TX and RX​ |
| TX EVM / RX EVM excluding phase noise | TX EVM: 2%, 3% RX EVM: 2..5% |

Results in [1] were for rank-1 transmissions using the phase noise model from 38.803, section 6.1.10. These results have now been updated with the phase noise model from [2]. The updated results have been reported in Figures 1 and 2. Figure 1 shows rank-1 results with 2.0% EVM for Tx and Rx excluding the impact of phase noise. When the phase noise is calculated in, the total Tx EVM equals 3.0%. Figure 2 shows rank-1 results with 3.0% EVM for Tx and Rx excluding the impact of phase noise. When the phase noise is calculated in, the total Tx EVM equals 3.7%.

Figure 1: Link level simulation results for rank-1 with 2% EVM for Tx and Rx excluding EVM from phase noise

Figure 2: Link level simulation results for rank-1 with 3% EVM for Tx and Rx excluding EVM from phase noise

It can be seen that throughput with 256QAM exceeds throughput with 64 QAM at 23 dB SNR in Figure 1 and 24 dB SNR in Figure 2. It can also be seen that the very highest coding rate which theoretically should achieve highest throughput fails to improve the performance in Figure 2. This means that in the field link adaption needs to work properly to maximise the throughput.

Observation 1: With Rank-1 transmissions 256QAM provides performance gain over 64 QAM when SNR is better than 23..24 dB depending on Tx and Rx EVM.

To complete the evaluation using simulations were done also using rank-2 transmissions. It was also decided to use link adaptation instead of fixed MCS to observe whether Tx and Rx EVM impact link adaptation. The results are reported in Figures 3 and 4. Figure 3 shows rank-2 results with 2.0% EVM for Tx and Rx excluding the impact of phase noise. When the phase noise is calculated in, the total Tx EVM equals 3.0%. Figure 4 shows rank-2 results with 3.0% EVM for Tx and Rx excluding the impact of phase noise. When the phase noise is calculated in, the total Tx EVM equals 3.7%.

Figure 3: Throughput for rank-2 transmissions with 2% EVM for Tx and Rx excluding EVM from phase noise

Figure 4: Throughput for rank-2 transmissions with 3% EVM for Tx and Rx excluding EVM from phase noise

It can be seen that throughput with 256QAM exceeds throughput with 64 QAM at 25 dB SNR in Figure 3 and 28 dB SNR in Figure 4. It can also be seen that link adaptation works properly and similar effect as with fixed MCS is not observed.

Observation 2: With Rank-2 transmissions 256QAM provides performance gain over 64 QAM when SNR is better than 25..28 dB when Tx and Rx EVM vary from 2% to 3% excluding the impact of phase noise.

Finally we did also simulations to see the impact of varying Rx EVM when the Tx EVM is kept constant. The Tx EVM was kept at 2.0% excluding phase noise and total Tx EVM was therefore 3.0%. The results are shown in Figure 5.

Figure 5: Throughput for rank-2 transmissions with total Tx EVM of 3% and varying Rx EVM.

It can be seen in Figure 5 that when Rx EVM is 5%, throughput gains are obtained only at SNRs above 30 dB, and overall Rx EVM has significant impact on the SNR level where 256QAM starts to provide gain over 64QAM. For this reason it is important that sufficiently strict Rx EVM needs to be assumed when setting the requirements.

Observation 3: Rx EVM has significant impact on SNR level where 256QAM start to provide gain over 64QAM, preferably 2% Rx EVM excluding phase noise needs to be assumed when setting the requirements.

# 3	Conclusion

In this contribution link level simulation results were provided for both rank-1 and rank-2 transmissions. Following observation was made

Observation 1: With Rank-1 transmissions 256QAM provides performance gain over 64 QAM when SNR is better than 23..24 dB when Tx and Rx EVM vary from 2% to 3% excluding the impact of phase noise.

Observation 2: With Rank-2 transmissions 256QAM provides performance gain over 64 QAM when SNR is better than 25..28 dB when Tx and Rx EVM vary from 2% to 3% excluding the impact of phase noise.

Observation 3: Rx EVM has significant impact on SNR level where 256QAM start to provide gain over 64QAM, preferably 2% Rx EVM excluding phase noise needs to be assumed when setting the requirements.

# 4	References

[1]	RP-190925 New WID: Add support of NR DL 256QAM for FR2, China Telecom

[2] 	Staffan Ek et al., A 28-nm FD-SOI 115-fs Jitter PLL-Based LO System for 24-30-GHz Sliding-IF 5G Transceivers,  IEEE Journal of Solid-State Circuits ( Volume: 53 , Issue: 7 , July 2018 )

[3] 	R4-1909269 Initial FR2 DL 256QAM link level simulation results, Nokia, Nokia Shanghai Bell

< Start of text proposal >

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	RP-190760, “Add support of NR DL 256QAM for FR2”, China Telecom

[3]	3GPP TR 38.803: "Study on new radio access technology: RF and co-existence aspects".

[4]	3GPP TS 38.306: "User Equipment (UE) radio access capabilities".

[5]	R4-1909091, “Link level simulation results for the feasibility study of FR2 DL 256QAM”, China Telecom

[6]	R4-1909269, “Initial FR2 DL 256QAM link level simulation results”, Nokia

[7]	R4-1908660, “Feasibility evaluation for NR FR2 DL256QAM”, NTT DOCOMO

[8]	R4-1909186, “Simulation results on DL 256QAM”, Huawei

[9]	R4-1909401, “Initial Link Simulation Results on DL 256 QAM FR2”, Ericsson

[10]	R4-1908391, “Discussion on 256QAM for FR2”, CATT

[11] 	R4-1908211, “Discussion on feasibility of DL 256QAM in FR2 scenarios”, Intel

[12] 	R4-1908144, “Views on feasibility of 256QAM for FR2”, Qualcomm

[13]	Staffan Ek et al., A 28-nm FD-SOI 115-fs Jitter PLL-Based LO System for 24-30-GHz Sliding-IF 5G Transceivers,  IEEE Journal of Solid-State Circuits ( Volume: 53 , Issue: 7 , July 2018 )

[14]	R4-1909403, “Feasibility of UE demodulation testing”, Ericsson

[15]	R4-190XXXX “TP to TR: 38.883 FR2 DL 256QAM link level simulation results”, Nokia, Nokia Shanghai Bell

[16]	Staffan Ek et al., A 28-nm FD-SOI 115-fs Jitter PLL-Based LO System for 24-30-GHz Sliding-IF 5G Transceivers,  IEEE Journal of Solid-State Circuits ( Volume: 53 , Issue: 7 , July 2018 )

[5.2.2.1-1] 	R4-1713125, “Evaluation on BS Tx EVM for mmWave”, Huawei, HiSilicon

[5.2.2.2-2] 	R4-1906014, “System level simulation for 256QAM DL in FR2”, Nokia, Nokia Shanghai Bell

[5.2.2.2-3]	T. Tuominen, N. Tervo, A.Pärssinen; Analyzing  5G  RF  System  Performance  and Relation to Link Budget for Directive MIMO,  IEEE Transactions on Antennas and Propagation ( Volume: 65 , Issue: 12 , Dec. 2017 )

[5.2.2.3-4]	R4-1908211, “Discussion on feasibility of DL 256QAM in FR2 scenarios”, Intel

[A-1]	R4-1711777, WF on simulation assumption for NR BS EVM requirement, Huawei, HiSilicon

[A-2]	R4-1709666, Simulation assumptions on UE Tx EVM for mmWave, Huawei, HiSilicon

[A-3]	3GPP Radio Access Network Working Group. 2016. “E-UTRA and UTRA Radio Frequency (RF) requirement background for Active Antenna System (AAS) Base Station (BS) (Release 13)”. 3GPP TR 37.842 V13.0.0.

[A-4]	Du Jinfeng, Chizhik Dmitry, Rodriguez Rodolfo Feick, Mauricio, Castro Guillermo, Valenzuela Reinaldo. A. 2018. “Suburban Fixed Wireless Access Channel Measurements and Models at 28 GHz for 90% Outdoor Coverage”, arXiv: 1807:03763.

[A-5]	3GPP Radio Access Network Working Group. 2018. “Study on channel model for frequencies from 0.5 to 100 GHz (Release 15)”. 3GPP TR 38.901 V15.0.0.

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

# 3	Definitions of terms, symbols and abbreviations

< Unchanged sections omitted >

### 5.2.1	Link level simulation

Link level simulation is targeted as mainstream way to evaluate if FR2 256QAM can achieve benefit by comparing to 64QAM. The simulation results from companies are listed as below.

#### 5.2.1.1	Simulation assumptions

The link level simulation assumptions are listed as in table 5.2.1.1-1, based on which, to evaluate the throughput difference between 64QAM and 256QAM. The study aims to identify conditions where DL 256QAM provides performance benefits.

Table 5.2.1.1-1 link level simulation assumptions

| Parameter | Value |
| --- | --- |
| Carrier frequency | 29 GHz (n257) and 39 GHz (n260) |
| CBW | 50 MHz, 100MHz |
| SCS | 60kHz, 120 kHz; |
| Allocated RBs | Full allocation |
| Propagation | TDL-A 30ns delay spread, 35Hz Doppler frequency TDL-D 30ns delay spread, 35Hz Doppler frequency Static (AWGN) |
| MCS | 64QAM: MCS 23, 24, 26, 28 in TS 38.214 Table 5.1.3.1-1, and other MCSs are not precluded 256QAM: MCS 21, 23, 25, 27 in TS 38.214 Table 5.1.3.1-2, and other MCSs are not precluded Baseline: fixed MCSs |
| Precoding | Precoding configuration defined in 38.101-4 Section 7.2 for fading channels and Section 7.5 for static channel; follow PMI |
| Symbol type | CP-OFDM |
| HARQ | 8, None |
| Antenna configuration | Fading channel: 2x2 for Rank1 and Rank2, Low correlation Static channel: 1x2 for Rank1, 2x2 for Rank2 |
| Channel estimation | Practical |
| Receiver type | MMSE |
| PDSCH configuration | Type A mapping, Start symbol 1, Duration 13 (for D slots) |
| DMRS configuration | Type 1, Single symbol, 1 additional DMRS |
| PTRS configuration | KPTRS : 2 (every 2 RBs), LPTRS : 1 (every 1 symbol) |
| Phase noise compensation | Practical based on PTRS |
| Phase noise model | TR 38.803 model (in section 6.1.10 and section 6.1.11) modelled Phase noise for TX and RX Option a): example1 (BS) + example1(UE) Option b): example2 (BS) + example2(UE) Option c): example2 (BS) + example2(BS) Option d):example2 (BS) + PN model config1: example1(UE) Option e): Other phase noise models, e.g. ones extracted from commercially available components or published results, are not excluded |
| txEVM + rxEVM excluding phase noise for 256QAM | txEVM: [1.0%-5.0%], rxEVM: [1.0%-5.0%] Option 1: txEVM <= rxEVM; Option2: no restriction |
| Other parameters | follow assumptions in 38.101-4 Section 7.2 for fading channels (e.g., case 2-6) and Section 7.5 for static channels |

The assumptions adopted by each company are shown as following table 5.2.1.1-2 which are down-selected based on the table 5.2.1.1-1.

Table 5.2.1.1-2 link level simulation assumptions down-selected by companies

| Parameter |  |  |  | CTC[5] | Nokia[6][15] | Docomo[7] | Huawei[8] | Ericsson[9] | CATT[10] | Intel[11] | Qualcomm[12] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Carrier frequency |  |  |  | 29 GHz | 29 GHz | 29 GHz | 29 GHz |  | 29 GHz |  |  |
| CBW |  |  |  | 50MHz | 100MHz | 100MHz | 50MHz |  | 50MHz | 50MHz | 100MHz |
| SCS |  |  |  | 120kHz | 60kHz | 120kHz | 120kHz |  | 60kHz | 60kHz | 120kHz |
| Allocated RBs |  |  |  |  |  |  |  |  |  |  |  |
| Propagation |  |  | TDL-A |  |  |  |  |  |  |  |  |
|  |  |  | TDL-D |  |  |  |  |  |  |  |  |
|  |  |  | Static |  |  |  |  |  |  |  |  |
| MCS |  |  | 64QAM | 28 | 26,28 | 24,25,26,28 | 23,24,26,28 | 23,24,26,28 | 23 | 23,24,26,28 | 26,27,28 |
|  |  |  | 256QAM | 27 | 21,23,25,27 | 21,23,25,27 | 21,23,25,27 | 21,23,25,27 | 21 | 21,23,25,27 | 20,21,22 |
| Precoding |  |  |  |  |  |  |  |  |  |  |  |
| Symbol type |  |  |  |  |  |  |  |  |  |  |  |
| HARQ |  |  |  | None | None | 8 |  | None | 8 | 8 | 8 |
| Antenna configuration |  | Fading |  | 2x2 for Rank1 | 2x2 for Rank1 |  |  | 1x2 for Rank1 | 2x2 for Rank1 |  | 2x2 for Rank2 |
|  |  | Static |  |  |  | 2x2 for Rank2 |  | 1x2 for Rank1 | 1x2 for Rank1 |  | 2x2 for Rank2 |
| Channel estimation |  |  |  |  |  |  |  |  |  |  |  |
| Receiver type |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH configuration |  |  |  |  |  |  |  |  |  |  |  |
| DMRS configuration |  |  |  |  |  |  |  | No additional |  |  |  |
| PTRS configuration |  |  |  | None |  |  |  |  |  |  |  |
| Phase noise compensation |  |  |  | None |  | Ideal |  |  |  |  |  |
| Phase noise model | Option a) |  |  |  |  |  |  |  |  |  |  |
|  | Option b) |  |  |  |  |  |  |  |  |  |  |
|  | Option c) |  |  |  |  |  |  |  |  |  |  |
|  | Option d) |  |  |  |  |  |  |  |  |  |  |
|  | Option e) |  |  |  |  |  |  |  |  |  | example1BS+example2UE internal PN model |
| txEVM + rxEVM excluding phase noise for 256QAM |  |  |  | Tx+Rx: 3%, 4% | txEVM: 2%-3%, rxEVM: 2%-5% | txEVM: 0%, 3%, rxEVM: 0%, 3% | txEVM: 1%-3%, rxEVM: 1%-3% |  |  | txEVM: 1%-3%, rxEVM: 1%-3% | Tx:3% Rx:internal |
| Other parameters |  |  |  |  |  |  |  |  |  |  |  |
| Note: The symbol of means selecting hhhhhhhhhhhhhhhhhhhhhhhhhhhhhhhhhhhhhhhhhhhhhhhthe parameters corresponding to table 5.2.1.1-1. |  |  |  |  |  |  |  |  |  |  |  |

#### 5.2.1.2	Results from China Telecom [5]

< Unchanged sections omitted >

#### 5.2.1.3	Results from Nokia [6][15]

Simulation results obtained with parameters in table 5.2.1.1-2 are shown in Figure 5.2.1.3-1.

Figure 5.2.1.3-1: Link level simulation results

In figure 5.2.1.3-1 it can be seen that the throughput with 256QAM exceeds throughput with 64QAM approximately at 28 dB SNR.

One important aspect to be taken into account when analysing the results is the used phase noise model. As shown in Figure 5.2.1.3-2, the phase noise performance is significantly worse that what is actually achievable with reasonable silicon area and power consumption [13]. On average the phase noise model is 6.5 dB worse than performance in [13]. Therefore with realistic phase noise assumptions more gains are expected.

One should also note that it may be overly optimistic to compare 64QAM and 256QAM with the same baseline EVM on Tx side, as 64QAM EVM budget is more relaxed and this can be utilized by heavier crest factor reduction.

Figure 5.2.1.3-2: Comparison of the used phase noise model and published results

In addition, the results in Figure 5.2.1.3-1 are obtained using 3% Tx EVM, excluding the EVM impact from the phase noise. It should be noted the gain is observed with total EVM of the Tx chain of approximately 5.3 %, excluding the benefits from PT-RS based equalization. Further gains would be observed using EVM contribution which keeps the total EVM similar to FR1 requirement.

Based on the results and analysis the following observation is made.

Observation 1: Even with the used pessimistic phase noise and EVM assumptions throughput gains over 64QAM can be observed with 256QAM.

Observation 2: TR 38.803 config 1 PN model is too pessimistic compared to currently achievable performance. Therefore a [6.5] dB downscaling of the corresponding PN model should be considered.

After providing the results above new simulations were done and reported in [15].

Results in [6] were for rank-1 transmissions using the phase noise model from 38.803, section 6.1.10. These results have now been updated with the phase noise model from [16]. The updated results have been reported in Figures 5.2.1.3-3 and 5.2.1.3-4. Figure 5.2.1.3-3 shows rank-1 results with 2.0% EVM for Tx and Rx excluding the impact of phase noise. When the phase noise is calculated in, the total Tx EVM equals 3.0%. Figure 5.2.1.3-4 shows rank-1 results with 3.0% EVM for Tx and Rx excluding the impact of phase noise. When the phase noise is calculated in, the total Tx EVM equals 3.7%.

Figure 5.2.1.3-3: Link level simulation results for rank-1 with 2% EVM for Tx and Rx excluding EVM from phase noise

Figure 5.2.1.3-4: Link level simulation results for rank-1 with 3% EVM for Tx and Rx excluding EVM from phase noise

It can be seen that throughput with 256QAM exceeds throughput with 64 QAM at 23 dB SNR in Figure 5.2.1.3-3 and 24 dB SNR in Figure 5.2.1.3-4. It can also be seen that the very highest coding rate which theoretically should achieve highest throughput fails to improve the performance in Figure 2. This means that in the field link adaption needs to work properly to maximise the throughput.

Observation 3: With Rank-1 transmissions 256QAM provides performance gain over 64 QAM when SNR is better than 23..24 dB depending on Tx and Rx EVM.

To complete the evaluation using simulations were done also using rank-2 transmissions. It was also decided to use link adaptation instead of fixed MCS to observe whether Tx and Rx EVM impact link adaptation. The results are reported in Figures 5.2.1.3-5 and 5.2.1.3-6. Figure 5.2.1.3-5 shows rank-2 results with 2.0% EVM for Tx and Rx excluding the impact of phase noise. When the phase noise is calculated in, the total Tx EVM equals 3.0%. Figure 5.2.1.3-6 shows rank-2 results with 3.0% EVM for Tx and Rx excluding the impact of phase noise. When the phase noise is calculated in, the total Tx EVM equals 3.7%.

Figure 5.2.1.3-5: Throughput for rank-2 transmissions with 2% EVM for Tx and Rx excluding EVM from phase noise

Figure 5.2.1.3-6: Throughput for rank-2 transmissions with 3% EVM for Tx and Rx excluding EVM from phase noise

It can be seen that throughput with 256QAM exceeds throughput with 64 QAM at 25 dB SNR in Figure 5.2.1.3-5 and 28 dB SNR in Figure 5.2.1.3-5. It can also be seen that link adaptation works properly and similar effect as with fixed MCS is not observed.

Observation 4: With Rank-2 transmissions 256QAM provides performance gain over 64 QAM when SNR is better than 25..28 dB when Tx and Rx EVM vary from 2% to 3% excluding the impact of phase noise.

Finally we did also simulations to see the impact of varying Rx EVM when the Tx EVM is kept constant. The Tx EVM was kept at 2.0% excluding phase noise and total Tx EVM was therefore 3.0%. The results are shown in Figure 5.2.1.3-7.

Figure 5.2.1.3-7: Throughput for rank-2 transmissions with total Tx EVM of 3% and varying Rx EVM.

It can be seen in Figure 5.2.1.3-7 that when Rx EVM is 5%, throughput gains are obtained only at SNRs above 30 dB, and overall Rx EVM has significant impact on the SNR level where 256QAM starts to provide gain over 64QAM. For this reason it is important that sufficiently strict Rx EVM needs to be assumed when setting the requirements.

Observation 5: Rx EVM has significant impact on SNR level where 256QAM start to provide gain over 64QAM, preferably 2% Rx EVM excluding phase noise needs to be assumed when setting the requirements.

#### 5.2.1.4	Results from DoCoMo [7]

< Unchanged sections omitted >

#### 5.2.1.10	Conclusion

Editor Note: this conclusion is an initial conclusion based on the initial results provided in RAN4#91meeting.Further update will be made based on the results updated in the next meeting.

Based on the simulation results and observations provided above, the following table summarizes the SNR ranges in which 256QAM shows benefit by comparing to 64QAM below in table 5.2.1.10-1.

Table 5.2.1.10-1: SNR required to achieve gains for 256QAM

| Contributor | AWGN SNR (dB) | TDL-D SNR (dB) | TDL-A SNR (dB) |
| --- | --- | --- | --- |
| China Telecom |  |  | > 25dB |
| Nokia |  | > 23dB |  |
| DoCoMo | > 21dB |  |  |
| Huawei |  | > 24dB |  |
| Ericsson |  |  | No benefit |
| CATT | > 21dB | > 27dB | > 31dB |
| Intel | > 25dB | > 30dB | > 35dB |
| Qualcomm | > 21dB | > 23dB |  |
| Average | > 21dB | > 25.6dB |  |

< End of text proposal >