3GPP TSG-RAN WG4 Meeting #92bis	R4-1911721

# Chongqing, China, October 14th – 18th 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.133: introduction of Cross Link Interference measurement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CLI_RIM-Core |  |  |  |  |  | Date: |  |  | 2019-05-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | CR to add a new section 9.x for Cross Link Interference measurement |  |  |  |  |  |  |  |  |
| X |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR defines CLI measurement requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The CLI measurement requirement is not defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Section 9.x and section 10.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Next Modified Subclause

# 9	Measurement Procedure

### 9.x	NR Cross Link Interference (CLI) measurements

### 9.x.1 	Introduction

When configured by the network, the UE shall be able to perform CLI measurements of configured resources for CLI-RSSI, CLI SRS-RSRP or CLI-RSSI and CLI SRS-RSRP.  The measurements shall be performed for a serving cell, including PCell, PSCell, or SCell, on the resources configured for CLI measurements within the active BWP. The measurements shall be performed for TDD mode only.

The UE shall be able to measure all CLI RSSI resources of the RSSI-Measurement-ResourceConfig-CLI configured for CLI-RSSI measurement for the active BWP, provided the number of resources does not exceed 64.

The UE shall be able to measure all CLI SRS resources of the SRS-RSRP-Measurement-ResourceConfig configured for CLI-SRS-RSRP measurement for the active BWP, provided the number of resources within a slot does not exceed 8 and the total number of resources does not exceed 32.

### 9.x.2	Requirement applicability

The requirements in clause 9.x apply, provided:

-	The resources configured for CLI-RSSI, CLI SRS-RSRP or CLI-RSSI and CLI SRS-RSRP measurements are measurable.

A CLI SRS resource configured for CLI SRS-RSRP shall be considered measurable when for each relevant SRS the following conditions are met:

-	CLI SRS-RSRP related side conditions given in clauses 10.1.x.2, for a corresponding band,

-	SRS_RP and SRS Ês/Iot according to Annex B.2.4.x for a corresponding band.

A CLI-RSSI resource configured for CLI RSSI shall be considered measurable when for each relevant RSSI resource the following conditions are met:

-	CLI RSSI related side conditions given in clauses 10.1.x.1, for a corresponding band,

-	All symbols duing each RSSI measurement resource are available for RSSI sampling.

### 9.x.3	Measurement reporting requirements

The UE shall report the measurement quantity (reportQuantity) and send periodic, semi-persistent or aperiodic reports, according to the reportConfigType according to the CLI reporting configuration(s) (CLI-ReportConfig) for the active BWP.

#### 9.x.3.1	Periodic Reporting

Reported CLI-RSSI and CLI SRS-RSRP measurements contained in periodically triggered measurement reports shall meet the requirements in clauses 10.1.x.1 and 10.1.x.2, respectively.

#### 9.x.3.2	Event-triggered Periodic Reporting

Reported CLI-RSSI and CLI SRS-RSRP measurements contained in periodically triggered measurement reports shall meet the requirements in clauses 10.1.x.1 and 10.1.x.2, respectively.

The first report in event triggered periodic measurement reporting shall meet the requirements specified in clause 9.x.3.3.

#### 9.x.3.3	Event Triggered Reporting

Reported CLI-RSSI and CLI SRS-RSRP measurements contained in event triggered measurement reports shall meet the requirements in clauses 10.1.x.1 and 10.1.x.2, respectively.

The UE shall not send any event triggered measurement reports as long as no reporting criteria are fulfilled.

The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH. This measurement reporting delay excludes a delay which caused by no UL resoureces for UE to send the measurement report.

### 9.x.4	CLI measurement requirements

### 9.x.4.1 	CLI-RSSI measurement

FFS.

### 9.x.4.2	CLI SRS-RSRP measurement

The UE shall be capable of performing SRS-RSRP measurements based on the configured SRS resource for SRS-RSRP computation, and the UE physical layer shall be capable of reporting SRS-RSRP measured over K samples, where the value of K depends on the CLI SRS-RSRP measurement periodicity as in below table.

Table 1. Measurement period for CLI SRS-RSRP measurement

| CLI SRS-Measurement-Periodicity (number of slots) | Number of measurement samples |
| --- | --- |
| Sl1, Sl2, Sl4…..Sl160 | 3 samples |
| Sl320, Sl640 | 1 sample |

### 9.x.5	Scheduling availability of UE during CLI measurement

UE are required to be capable of measuring CLI without measurement gaps when the CLI-RSSI symbols or SRS symbol are completely contained in the active bandwidth part of the UE. The SRS symbol or CLI-RSSI symbols to be measured in the following clauses are the symbols indicated by SRS-RSRP-Measurement-ResourceConfig-CLI and RSSI-Measurement-ResourceConfig-CLI respectively [2], The scheduling availability restriction in this clause applies to TDD bands only.  UE is not required to measure SRS using different SCS compared to the downlink active BWP SCS of the same carrier.

##### 9.x.5.x	Scheduling availability of UE performing measurements with a same subcarrier spacing as PDSCH/PDCCH on FR1

When UE performs CLI SRS-RSRP measurements, the following restrictions apply:

-	UE is not expected to transmit PUCCH/PUSCH/SRS on SRS symbols to be measured, and on Y data symbols before each SRS symbol to be measured.

-	Depending on the UE capability, UE is not expected to receive PDCCH/PDSCH on SRS symbols to be measured, and on Y data symbol before each SRS symbol to be measured in intra-band CA.

When UE performs CLI RSSI measurements, the following restrictions apply:

-	UE is not expected to transmit PUCCH/PUSCH/SRS on CLI-RSSI symbols to be measured, and on X data symbol before each consecutive RSSI symbols to be measured.

-	Depending on the UE capability, UE is not expected to receive PDCCH/PDSCH on CLI-RSSI symbols to be measured in intra-band CA.

- FFS for scheduling restriction on X data symbol before CLI-RSSI measured symbol

When TDD intra-band carrier aggregation is performed, the scheduling restrictions due to one serving cell should also apply to all other serving cells in the same band on the symbols that fully or partially overlap with aforementioned restricted symbols.

### 10.1	NR measurements

### 10.1.1	Introduction

### 10.1.x.1 	CLI-RSSI accuracy requirements

Unless otherwise specified, the requirements for absolute accuracy of CLI-RSSI in this clause apply to all measurement resources of the serving cell configured for CLI-RSSI measurement.

The accuracy requirements in Table 10.1.19.1.1-1 are valid under the following conditions:

-	Conditions for CLI-RSSI measurements are fulfilled according to Annex B.x.x.x for a corresponding Band.

### 10.1.x.2 	CLI-SRS-RSRP accuracy requirements

The accuracy of CLI SRS-RSRP is defined as the SRS-RSRP measured from one SRS compared to the ideal value of SRS-RSRP.

The accuracy requirements in Table 10.x.x.x.x are valid under the following conditions:

-	Conditions for SRS-RSRP measurements are fulfilled according to Annex B.x.x.x for a corresponding Band for each relevant SRS.

-	The bandwidth of SRS is 48 PRBs and the transmission comb is n2.

- The performance with larger bandwidth of SRS is equal to or better than the accuracy requirements in Table 10.x.x.x.x.

| Accuracy |  |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition |  | TCLI | SRS Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  |  |  | NR operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB |  |  | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| ±1.9 | X |  | ≤ 2*CP | -3 dB | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -70 |
|  |  |  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -70 |
|  |  |  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -70 |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -70 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -70 |
|  |  |  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -70 |
|  |  |  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -70 |
|  |  | NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

Table 10.x.x.x.x: CLI SRS-RSRP accuracy proposal for TCLI ≤ 2*CP

Next Modified Subclause