3GPP TSG-RAN WG4 Meeting #92Bis	R4-1911744

Chongqing, China,  14th Oct - 18th Oct, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.105 | CR |  | rev |  | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS37.105 Corrections on NBB requirement (section 7.4 and 10.5) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-10-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Similar as R4-1910421 approved in the RAN4#92 meeting, the Note 7 should only applied for NR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Change the applicability of Note 7 in NBB blocking requirement; - Remove the duplicated note for wanted signal. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Note 7 might also apply for E-UTRA or UTRA which is not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Section 7.4.2.2 and 10.5.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS37.145-1, 37.145-2 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Start of CR>

7.4.2.2	General narrowband blocking minimum requirement

For the general narrowband blocking requirement, the interfering signal shall be an E-UTRA 1RB signal as specified in 3GPP TS 37.104 [9], annex A.3.

The requirement is applicable outside the Base Station RF Bandwidth or Radio Bandwidth. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Radio Bandwidth edges.

For TAB connectors supporting operation in non-contiguous spectrum, the requirement applies in addition inside any sub-block gap, in case the sub-block gap size is at least 3 MHz. The interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.

For multi-band TAB connectors, the requirement applies in addition inside any Inter RF Bandwidth gap at those connectors, in case the gap size is at least 3 MHz. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap.

For the wanted and interfering signal coupled to the TAB connector using the parameters in table 7.4.2.2-1, the following requirements shall be met:

-	For any E-UTRA carrier, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channel defined in 3GPP TS 36.104 [8], subclause 7.2.1.

-	For any UTRA FDD carrier, the BER shall not exceed 0,001 for the reference measurement channel defined in 3GPP TS 25.104 [6], subclause 7.2.1.

-	For any UTRA TDD carrier, the BER shall not exceed 0,001 for the reference measurement channel defined in 3GPP TS 25.105 [7], subclause 7.2.1.2.

-	For any NR carrier, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel defined for BS type 1-H in TS 38.104 [28], subclause 7.2.2.

Table 7.4.2.2-1: Narrowband blocking requirement

| Base Station Type | RAT of the carrier | Wanted signal mean power [dBm] (NOTE 1,2,6) | Interfering signal mean power [dBm] | Interfering RB (NOTE 3) centre frequency offset from the AAS Base Station RF Bandwidth edge or edge of sub-block inside a gap [kHz] |
| --- | --- | --- | --- | --- |
| Wide Area BS | E-UTRA, UTRA, NR | PREFSENS + x dB | -49 | ±(240 +m*180), m=0, 1, 2, 3, 4, 9, 14 (Note 4) ±(550 +m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 (Note 5,7) |
| Medium Range BS |  |  | -44 |  |
| Local Area BS |  |  | -41 |  |
| NOTE 1: PREFSENS depends on the RAT, the AAS BS class and on the channel bandwidth, see subclause 7.2.2. NOTE 2: "x" is equal to 6 dB in case of E-UTRA or UTRA or NR wanted signals. NOTE 3: Interfering signal (E-UTRA 3 MHz) consisting of one resource block positioned at the stated offset, the channel bandwidth of the interfering signal is located adjacently to the AAS Base Station RF Bandwidth edge. NOTE 4: Applicable for channel bandwidths equal to or below 20 MHz. NOTE 5: Applicable for channel bandwidths above 20MHz. NOTE 6: 7.5 kHz shift is not applied to the wanted signal of NR. NOTE 7: The centre of the interfering RB refers to the frequency location between the two central subcarriers. |  |  |  |  |

< Next of Change>

10.5.2.2	General narrowband blocking minimum requirement

For the general narrowband blocking requirement, the interfering signal shall be an E-UTRA 1RB signal as specified in 3GPP TS 37.104 [9], annex A.

The requirement is applicable outside the Base Station RF Bandwidth or Radio Bandwidth. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Radio Bandwidth edges.

For RIBs supporting operation in non-contiguous spectrum, the requirement applies in addition inside any sub-block gap, in case the sub-block gap size is at least 3 MHz. The interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.

For multi-band RIBs, the requirement applies in addition inside any Inter RF Bandwidth gap, in case the gap size is at least 3 MHz. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap.

For the wanted and interfering signal at the RIB using the parameters in table 10.5.2.2-1, the following requirements shall be met:

-	For any E-UTRA carrier, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channel defined in 3GPP TS 36.104 [8], subclause 7.2.1.

-	For any UTRA FDD carrier, the BER shall not exceed 0,001 for the reference measurement channel defined in 3GPP TS 25.104 [6], subclause 7.2.1.

-	For any NR carrier, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel defined for BS type 1-O in TS 38.104 [28], subclause 10.3.2

Table 10.5.2.2-1: Narrowband blocking requirement

| Base Station Type | RAT of the carrier | Wanted signal mean power [dBm] (NOTE 1,2,6) | Interfering signal mean power [dBm] | Interfering RB (NOTE 3) centre frequency offset from the AAS Base Station RF Bandwidth edge or edge of sub-block inside a gap [kHz] |
| --- | --- | --- | --- | --- |
| Wide Area BS | E-UTRA, UTRA, NR | EISREFSENS + x dB | -49 - ΔOTAREFSENS | ±(240 +m*180), m=0, 1, 2, 3, 4, 9, 14 (Note 4) ±(550 +m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 (Note 5,7) |
|  |  | EISminSENS + x dB | -49 – ΔminSENS |  |
| Medium Range BS |  | EISREFSENS + x dB | -44 - ΔOTAREFSENS |  |
|  |  | EISminSENS + x dB | -44 – ΔminSENS |  |
| Local Area BS |  | EISREFSENS + x dB | -41 - ΔOTAREFSENS |  |
|  |  | EISminSENS + x dB | -41 – ΔminSENS |  |
| NOTE 1: EISREFSENS and EISminSENS depend on the RAT, the BS class and on the channel bandwidth, see subclauses 10.3 and 10.2. NOTE 2: "x" is equal to 6 dB in case of E-UTRA or UTRA or NR wanted signals. NOTE 3: Interfering signal (E-UTRA 3 MHz) consisting of one resource block positioned at the stated offset, the channel bandwidth of the interfering signal is located adjacently to the AAS Base Station RF Bandwidth edge. NOTE 4: Applicable for channel bandwidths equal to or below 20 MHz. NOTE 5: Applicable for channel bandwidths above 20 MHz. NOTE 6: 7.5 kHz shift is not applied to the wanted signal of NR. NOTE 7: The centre of the interfering RB refers to the frequency location between the two central subcarriers. |  |  |  |  |

< End of Change>