3GPP TSG-RAN WG4 Meeting#92bis                               	R4-1911759

Chongqing, China, October 14th – 18th 2019

Source:	Nokia, Nokia Shanghai Bell

Title:	Simulation results for CSI-RS based measurements

Agenda item:	8.16.1.6

Document for:	Discussion

Introduction

In this paper, we provide our simulation results for CSI-RS based measurements.

Simulation assumptions

In this contribution, a single link noise-only scenario is considered with no other interference sources. The general parameters of the scenario are listed in Table 1.

Table 1: General parameters

| Simulation parameters | Comments/values |  |
| --- | --- | --- |
| Carrier frequency | 4 GHz | 30 GHz |
| BS transmit antennas | 1 tx |  |
| UE receive antennas | 2 rx (uncorrelated with equal gain, no rx beamforming) |  |
| Measurement period (in number of samples) | 1, 3, 5, 10 |  |
| Data Modulation | QPSK |  |
| Slot length | 14 symbols |  |
| CP length | Normal |  |
| SNR range | -9, -6, -3, 0 dB |  |
| Propagation channel | AWGN |  |

Table 3: CSI-RS configuration parameters

| Parameter | 4 GHz | 30 GHz |
| --- | --- | --- |
| CSI-RS bandwidth | 24, 48, 96 PRBs |  |
| CSI-RS SCS | 15 kHz, 30 kHz, 60 kHz | 60 kHz, 120 kHz |
| Periodicity | 40 ms |  |
| CSI-RS configuration: <X,D,N,(Y,Z),CDM>, where: X=number of CSI-RS ports, D=density [RE/RB/port], N=number of OFDM symbols in the same slot, Y=adjacent in frequency REs in PRB, Z=adjacent in time REs in PRB | <1,1,1,N/A,noCDM> |  |

Simulation results

Simulation results are provided for CSI-RSRP, CSI-RSRQ and CSI-SNR. The 5th, 50th and 95th percentiles of the CDF curves of the delta measurement are presented below.

CSI-RSRP

CSI-RSRP - 4 GHz, 24 PRBs, 15 kHz SCS, D=1

| Number of samples | SNR (dB) | 5 % | 50 % | 95 % | Absolute accuracy |
| --- | --- | --- | --- | --- | --- |
| 1 | -9 | -0,23 | 3,62 | 6,41 | 6,41 |
| 1 | -6 | -1,51 | 2,03 | 4,61 | 4,61 |
| 1 | -3 | -1,56 | 1,18 | 3,23 | 3,23 |
| 1 | 0 | -1,16 | 0,76 | 2,31 | 2,31 |
| 3 | -9 | -1,18 | 2,31 | 4,69 | 4,69 |
| 3 | -6 | -1,44 | 1,25 | 3,16 | 3,16 |
| 3 | -3 | -0,97 | 0,74 | 2,17 | 2,17 |
| 3 | 0 | -0,63 | 0,49 | 1,5 | 1,5 |
| 5 | -9 | -1,21 | 1,83 | 3,9 | 3,9 |
| 5 | -6 | -1,01 | 1,04 | 2,65 | 2,65 |
| 5 | -3 | -0,72 | 0,61 | 1,8 | 1,8 |
| 5 | 0 | -0,49 | 0,39 | 1,23 | 1,23 |
| 10 | -9 | -0,93 | 1,4 | 3,11 | 3,11 |
| 10 | -6 | -0,65 | 0,75 | 2,01 | 2,01 |
| 10 | -3 | -0,45 | 0,45 | 1,32 | 1,32 |
| 10 | 0 | -0,31 | 0,28 | 0,9 | 0,9 |

CSI-RSRP - 4 GHz, 24 PRBs, 30 kHz SCS, D=1

| Number of samples | SNR (dB) | 5 % | 50 % | 95 % | Absolute accuracy |
| --- | --- | --- | --- | --- | --- |
| 1 | -9 | -0,07 | 3,59 | 6,32 | 6,32 |
| 1 | -6 | -1,66 | 1,93 | 4,57 | 4,57 |
| 1 | -3 | -1,79 | 1,08 | 3,23 | 3,23 |
| 1 | 0 | -1,27 | 0,69 | 2,31 | 2,31 |
| 3 | -9 | -1,28 | 2,23 | 4,56 | 4,56 |
| 3 | -6 | -1,43 | 1,19 | 3,09 | 3,09 |
| 3 | -3 | -0,97 | 0,7 | 2,08 | 2,08 |
| 3 | 0 | -0,64 | 0,43 | 1,41 | 1,41 |
| 5 | -9 | -1,18 | 1,77 | 3,74 | 3,74 |
| 5 | -6 | -1,02 | 0,96 | 2,42 | 2,42 |
| 5 | -3 | -0,7 | 0,54 | 1,61 | 1,61 |
| 5 | 0 | -0,48 | 0,34 | 1,08 | 1,08 |
| 10 | -9 | -0,95 | 1,26 | 2,94 | 2,94 |
| 10 | -6 | -0,75 | 0,69 | 1,83 | 1,83 |
| 10 | -3 | -0,5 | 0,41 | 1,18 | 1,18 |
| 10 | 0 | -0,34 | 0,26 | 0,77 | 0,77 |

CSI-RSRP - 4 GHz, 24 PRBs, 60 kHz SCS, D=1

| Number of samples | SNR (dB) | 5 % | 50 % | 95 % | Absolute accuracy |
| --- | --- | --- | --- | --- | --- |
| 1 | -9 | -0,12 | 3,54 | 6,36 | 6,36 |
| 1 | -6 | -1,63 | 1,93 | 4,6 | 4,6 |
| 1 | -3 | -1,65 | 1,09 | 3,24 | 3,24 |
| 1 | 0 | -1,22 | 0,67 | 2,27 | 2,27 |
| 3 | -9 | -1,29 | 2,12 | 4,42 | 4,42 |
| 3 | -6 | -1,41 | 1,1 | 2,98 | 2,98 |
| 3 | -3 | -1,03 | 0,66 | 2,01 | 2,01 |
| 3 | 0 | -0,71 | 0,4 | 1,39 | 1,39 |
| 5 | -9 | -1,55 | 1,66 | 3,64 | 3,64 |
| 5 | -6 | -1,33 | 0,87 | 2,43 | 2,43 |
| 5 | -3 | -0,92 | 0,52 | 1,62 | 1,62 |
| 5 | 0 | -0,57 | 0,34 | 1,11 | 1,11 |
| 10 | -9 | -1,05 | 1,11 | 2,79 | 2,79 |
| 10 | -6 | -0,77 | 0,58 | 1,76 | 1,76 |
| 10 | -3 | -0,53 | 0,34 | 1,14 | 1,14 |
| 10 | 0 | -0,37 | 0,21 | 0,77 | 0,77 |

CSI-RSRP - 4 GHz, 48 PRBs, 15 kHz SCS, D=1

| Number of samples | SNR (dB) | 5 % | 50 % | 95 % | Absolute accuracy |
| --- | --- | --- | --- | --- | --- |
| 1 | -9 | -1,04 | 2,45 | 5,1 | 5,1 |
| 1 | -6 | -1,8 | 1,26 | 3,53 | 3,53 |
| 1 | -3 | -1,26 | 0,72 | 2,4 | 2,4 |
| 1 | 0 | -0,85 | 0,45 | 1,67 | 1,67 |
| 3 | -9 | -1,57 | 1,37 | 3,48 | 3,48 |
| 3 | -6 | -1,26 | 0,71 | 2,27 | 2,27 |
| 3 | -3 | -0,82 | 0,43 | 1,47 | 1,47 |
| 3 | 0 | -0,51 | 0,27 | 0,99 | 0,99 |
| 5 | -9 | -1,46 | 1,1 | 2,74 | 2,74 |
| 5 | -6 | -1,04 | 0,6 | 1,74 | 1,74 |
| 5 | -3 | -0,64 | 0,34 | 1,12 | 1,12 |
| 5 | 0 | -0,4 | 0,21 | 0,76 | 0,76 |
| 10 | -9 | -1,34 | 0,73 | 1,97 | 1,97 |
| 10 | -6 | -0,8 | 0,41 | 1,22 | 1,22 |
| 10 | -3 | -0,48 | 0,25 | 0,8 | 0,8 |
| 10 | 0 | -0,31 | 0,15 | 0,54 | 0,54 |

CSI-RSRP - 4 GHz, 48 PRBs, 30 kHz SCS, D=1

| Number of samples | SNR (dB) | 5 % | 50 % | 95 % | Absolute accuracy |
| --- | --- | --- | --- | --- | --- |
| 1 | -9 | -0,83 | 2,58 | 5,06 | 5,06 |
| 1 | -6 | -1,64 | 1,41 | 3,51 | 3,51 |
| 1 | -3 | -1,25 | 0,81 | 2,44 | 2,44 |
| 1 | 0 | -0,85 | 0,51 | 1,69 | 1,69 |
| 3 | -9 | -1,36 | 1,48 | 3,46 | 3,46 |
| 3 | -6 | -1,07 | 0,8 | 2,29 | 2,29 |
| 3 | -3 | -0,69 | 0,48 | 1,54 | 1,54 |
| 3 | 0 | -0,45 | 0,3 | 1,03 | 1,03 |
| 5 | -9 | -1,28 | 1,1 | 2,79 | 2,79 |
| 5 | -6 | -0,82 | 0,61 | 1,83 | 1,83 |
| 5 | -3 | -0,47 | 0,37 | 1,22 | 1,22 |
| 5 | 0 | -0,31 | 0,23 | 0,82 | 0,82 |
| 10 | -9 | -1,03 | 0,84 | 2,12 | 2,12 |
| 10 | -6 | -0,61 | 0,47 | 1,37 | 1,37 |
| 10 | -3 | -0,38 | 0,28 | 0,88 | 0,88 |
| 10 | 0 | -0,23 | 0,18 | 0,59 | 0,59 |

CSI-RSRP - 4 GHz, 48 PRBs, 60 kHz SCS, D=1

| Number of samples | SNR (dB) | 5 % | 50 % | 95 % | Absolute accuracy |
| --- | --- | --- | --- | --- | --- |
| 1 | -9 | -0,95 | 2,52 | 5,05 | 5,05 |
| 1 | -6 | -1,63 | 1,39 | 3,49 | 3,49 |
| 1 | -3 | -1,2 | 0,81 | 2,45 | 2,45 |
| 1 | 0 | -0,8 | 0,5 | 1,71 | 1,71 |
| 3 | -9 | -1,55 | 1,51 | 3,48 | 3,48 |
| 3 | -6 | -1,12 | 0,83 | 2,31 | 2,31 |
| 3 | -3 | -0,72 | 0,5 | 1,54 | 1,54 |
| 3 | 0 | -0,47 | 0,32 | 1,06 | 1,06 |
| 5 | -9 | -1,4 | 1,17 | 2,87 | 2,87 |
| 5 | -6 | -0,88 | 0,65 | 1,84 | 1,84 |
| 5 | -3 | -0,54 | 0,4 | 1,22 | 1,22 |
| 5 | 0 | -0,34 | 0,25 | 0,84 | 0,84 |
| 10 | -9 | -0,93 | 0,8 | 2,13 | 2,13 |
| 10 | -6 | -0,6 | 0,46 | 1,37 | 1,37 |
| 10 | -3 | -0,38 | 0,29 | 0,91 | 0,91 |
| 10 | 0 | -0,24 | 0,19 | 0,6 | 0,6 |

CSI-RSRP - 4 GHz, 96 PRBs, 15 kHz SCS, D=1

| Number of samples | SNR (dB) | 5 % | 50 % | 95 % | Absolute accuracy |
| --- | --- | --- | --- | --- | --- |
| 1 | -9 | -1,32 | 1,88 | 3,97 | 3,97 |
| 1 | -6 | -1,23 | 1,04 | 2,63 | 2,63 |
| 1 | -3 | -0,85 | 0,61 | 1,79 | 1,79 |
| 1 | 0 | -0,57 | 0,38 | 1,24 | 1,24 |
| 3 | -9 | -1,07 | 1,11 | 2,6 | 2,6 |
| 3 | -6 | -0,73 | 0,62 | 1,68 | 1,68 |
| 3 | -3 | -0,47 | 0,38 | 1,1 | 1,1 |
| 3 | 0 | -0,33 | 0,24 | 0,74 | 0,74 |
| 5 | -9 | -0,82 | 0,89 | 2,16 | 2,16 |
| 5 | -6 | -0,54 | 0,49 | 1,38 | 1,38 |
| 5 | -3 | -0,35 | 0,29 | 0,89 | 0,89 |
| 5 | 0 | -0,23 | 0,18 | 0,61 | 0,61 |
| 10 | -9 | -0,48 | 0,65 | 1,58 | 1,58 |
| 10 | -6 | -0,29 | 0,36 | 0,98 | 0,98 |
| 10 | -3 | -0,2 | 0,23 | 0,64 | 0,64 |
| 10 | 0 | -0,14 | 0,15 | 0,43 | 0,43 |

CSI-RSRP - 4 GHz, 96 PRBs, 30 kHz SCS, D=1

| Number of samples | SNR (dB) | 5 % | 50 % | 95 % | Absolute accuracy |
| --- | --- | --- | --- | --- | --- |
| 1 | -9 | -1,43 | 1,9 | 4,06 | 4,06 |
| 1 | -6 | -1,43 | 1,05 | 2,74 | 2,74 |
| 1 | -3 | -0,92 | 0,61 | 1,85 | 1,85 |
| 1 | 0 | -0,61 | 0,38 | 1,26 | 1,26 |
| 3 | -9 | -1,05 | 1,16 | 2,74 | 2,74 |
| 3 | -6 | -0,71 | 0,63 | 1,73 | 1,73 |
| 3 | -3 | -0,47 | 0,37 | 1,11 | 1,11 |
| 3 | 0 | -0,3 | 0,23 | 0,74 | 0,74 |
| 5 | -9 | -0,79 | 0,91 | 2,27 | 2,27 |
| 5 | -6 | -0,51 | 0,48 | 1,42 | 1,42 |
| 5 | -3 | -0,33 | 0,27 | 0,9 | 0,9 |
| 5 | 0 | -0,22 | 0,17 | 0,61 | 0,61 |
| 10 | -9 | -0,6 | 0,66 | 1,73 | 1,73 |
| 10 | -6 | -0,36 | 0,35 | 1,06 | 1,06 |
| 10 | -3 | -0,23 | 0,2 | 0,66 | 0,66 |
| 10 | 0 | -0,15 | 0,12 | 0,43 | 0,43 |

CSI-RSRP - 4 GHz, 96 PRBs, 60 kHz SCS, D=1

| Number of samples | SNR (dB) | 5 % | 50 % | 95 % | Absolute accuracy |
| --- | --- | --- | --- | --- | --- |
| 1 | -9 | -1,35 | 1,83 | 4,06 | 4,06 |
| 1 | -6 | -1,23 | 1,01 | 2,68 | 2,68 |
| 1 | -3 | -0,81 | 0,62 | 1,8 | 1,8 |
| 1 | 0 | -0,52 | 0,4 | 1,23 | 1,23 |
| 3 | -9 | -1,01 | 1,07 | 2,68 | 2,68 |
| 3 | -6 | -0,7 | 0,58 | 1,7 | 1,7 |
| 3 | -3 | -0,44 | 0,36 | 1,08 | 1,08 |
| 3 | 0 | -0,28 | 0,24 | 0,73 | 0,73 |
| 5 | -9 | -0,9 | 0,85 | 2,21 | 2,21 |
| 5 | -6 | -0,59 | 0,47 | 1,38 | 1,38 |
| 5 | -3 | -0,36 | 0,29 | 0,89 | 0,89 |
| 5 | 0 | -0,23 | 0,19 | 0,59 | 0,59 |
| 10 | -9 | -0,71 | 0,64 | 1,56 | 1,56 |
| 10 | -6 | -0,39 | 0,37 | 0,96 | 0,96 |
| 10 | -3 | -0,23 | 0,21 | 0,62 | 0,62 |
| 10 | 0 | -0,15 | 0,14 | 0,42 | 0,42 |

CSI-RSRP - 30 GHz, 24 PRBs, 60 kHz SCS, D=1

| Number of samples | SNR (dB) | 5 % | 50 % | 95 % | Absolute accuracy |
| --- | --- | --- | --- | --- | --- |
| 1 | -9 | -0,12 | 3,54 | 6,36 | 6,36 |
| 1 | -6 | -1,63 | 1,93 | 4,6 | 4,6 |
| 1 | -3 | -1,65 | 1,09 | 3,24 | 3,24 |
| 1 | 0 | -1,22 | 0,67 | 2,27 | 2,27 |
| 3 | -9 | -1,29 | 2,12 | 4,42 | 4,42 |
| 3 | -6 | -1,41 | 1,1 | 2,98 | 2,98 |
| 3 | -3 | -1,03 | 0,66 | 2,01 | 2,01 |
| 3 | 0 | -0,71 | 0,4 | 1,39 | 1,39 |
| 5 | -9 | -1,55 | 1,66 | 3,64 | 3,64 |
| 5 | -6 | -1,33 | 0,87 | 2,43 | 2,43 |
| 5 | -3 | -0,92 | 0,52 | 1,62 | 1,62 |
| 5 | 0 | -0,57 | 0,34 | 1,11 | 1,11 |
| 10 | -9 | -1,05 | 1,11 | 2,79 | 2,79 |
| 10 | -6 | -0,77 | 0,58 | 1,76 | 1,76 |
| 10 | -3 | -0,53 | 0,34 | 1,14 | 1,14 |
| 10 | 0 | -0,37 | 0,21 | 0,77 | 0,77 |

CSI-RSRP - 30 GHz, 24 PRBs, 120 kHz SCS, D=1

| Number of samples | SNR (dB) | 5 % | 50 % | 95 % | Absolute accuracy |
| --- | --- | --- | --- | --- | --- |
| 1 | -9 | -0,24 | 3,52 | 6,27 | 6,27 |
| 1 | -6 | -1,67 | 1,89 | 4,49 | 4,49 |
| 1 | -3 | -1,63 | 1,08 | 3,22 | 3,22 |
| 1 | 0 | -1,13 | 0,71 | 2,3 | 2,3 |
| 3 | -9 | -1,22 | 2,11 | 4,41 | 4,41 |
| 3 | -6 | -1,4 | 1,17 | 3 | 3 |
| 3 | -3 | -0,97 | 0,69 | 2,07 | 2,07 |
| 3 | 0 | -0,63 | 0,43 | 1,43 | 1,43 |
| 5 | -9 | -1,47 | 1,64 | 3,7 | 3,7 |
| 5 | -6 | -1,05 | 0,93 | 2,48 | 2,48 |
| 5 | -3 | -0,71 | 0,55 | 1,65 | 1,65 |
| 5 | 0 | -0,48 | 0,35 | 1,12 | 1,12 |
| 10 | -9 | -1,13 | 1,18 | 2,85 | 2,85 |
| 10 | -6 | -0,77 | 0,68 | 1,86 | 1,86 |
| 10 | -3 | -0,48 | 0,39 | 1,25 | 1,25 |
| 10 | 0 | -0,31 | 0,24 | 0,85 | 0,85 |

CSI-RSRP - 30 GHz, 48 PRBs, 60 kHz SCS, D=1

| Number of samples | SNR (dB) | 5 % | 50 % | 95 % | Absolute accuracy |
| --- | --- | --- | --- | --- | --- |
| 1 | -9 | -0,95 | 2,52 | 5,05 | 5,05 |
| 1 | -6 | -1,63 | 1,39 | 3,49 | 3,49 |
| 1 | -3 | -1,2 | 0,81 | 2,45 | 2,45 |
| 1 | 0 | -0,8 | 0,5 | 1,71 | 1,71 |
| 3 | -9 | -1,55 | 1,51 | 3,48 | 3,48 |
| 3 | -6 | -1,12 | 0,83 | 2,31 | 2,31 |
| 3 | -3 | -0,72 | 0,5 | 1,54 | 1,54 |
| 3 | 0 | -0,47 | 0,32 | 1,06 | 1,06 |
| 5 | -9 | -1,4 | 1,17 | 2,87 | 2,87 |
| 5 | -6 | -0,88 | 0,65 | 1,84 | 1,84 |
| 5 | -3 | -0,54 | 0,4 | 1,22 | 1,22 |
| 5 | 0 | -0,34 | 0,25 | 0,84 | 0,84 |
| 10 | -9 | -0,93 | 0,8 | 2,13 | 2,13 |
| 10 | -6 | -0,6 | 0,46 | 1,37 | 1,37 |
| 10 | -3 | -0,38 | 0,29 | 0,91 | 0,91 |
| 10 | 0 | -0,24 | 0,19 | 0,6 | 0,6 |

CSI-RSRP - 30 GHz, 48 PRBs, 120 kHz SCS, D=1

| Number of samples | SNR (dB) | 5 % | 50 % | 95 % | Absolute accuracy |
| --- | --- | --- | --- | --- | --- |
| 1 | -9 | -1,03 | 2,48 | 4,93 | 4,93 |
| 1 | -6 | -1,73 | 1,28 | 3,47 | 3,47 |
| 1 | -3 | -1,3 | 0,75 | 2,38 | 2,38 |
| 1 | 0 | -0,86 | 0,49 | 1,62 | 1,62 |
| 3 | -9 | -1,55 | 1,39 | 3,39 | 3,39 |
| 3 | -6 | -1,2 | 0,75 | 2,22 | 2,22 |
| 3 | -3 | -0,78 | 0,44 | 1,47 | 1,47 |
| 3 | 0 | -0,51 | 0,28 | 1,01 | 1,01 |
| 5 | -9 | -1,55 | 1,1 | 2,78 | 2,78 |
| 5 | -6 | -1,02 | 0,59 | 1,79 | 1,79 |
| 5 | -3 | -0,67 | 0,34 | 1,19 | 1,19 |
| 5 | 0 | -0,44 | 0,22 | 0,8 | 0,8 |
| 10 | -9 | -1,28 | 0,73 | 2,16 | 2,16 |
| 10 | -6 | -0,78 | 0,39 | 1,37 | 1,37 |
| 10 | -3 | -0,49 | 0,24 | 0,89 | 0,89 |
| 10 | 0 | -0,31 | 0,15 | 0,59 | 0,59 |

CSI-RSRP - 30 GHz, 96 PRBs, 60 kHz SCS, D=1

| Number of samples | SNR (dB) | 5 % | 50 % | 95 % | Absolute accuracy |
| --- | --- | --- | --- | --- | --- |
| 1 | -9 | -1,35 | 1,83 | 4,06 | 4,06 |
| 1 | -6 | -1,23 | 1,01 | 2,68 | 2,68 |
| 1 | -3 | -0,81 | 0,62 | 1,8 | 1,8 |
| 1 | 0 | -0,52 | 0,4 | 1,23 | 1,23 |
| 3 | -9 | -1,01 | 1,07 | 2,68 | 2,68 |
| 3 | -6 | -0,7 | 0,58 | 1,7 | 1,7 |
| 3 | -3 | -0,44 | 0,36 | 1,08 | 1,08 |
| 3 | 0 | -0,28 | 0,24 | 0,73 | 0,73 |
| 5 | -9 | -0,9 | 0,85 | 2,21 | 2,21 |
| 5 | -6 | -0,59 | 0,47 | 1,38 | 1,38 |
| 5 | -3 | -0,36 | 0,29 | 0,89 | 0,89 |
| 5 | 0 | -0,23 | 0,19 | 0,59 | 0,59 |
| 10 | -9 | -0,71 | 0,64 | 1,56 | 1,56 |
| 10 | -6 | -0,39 | 0,37 | 0,96 | 0,96 |
| 10 | -3 | -0,23 | 0,21 | 0,62 | 0,62 |
| 10 | 0 | -0,15 | 0,14 | 0,42 | 0,42 |

CSI-RSRP - 30 GHz, 96 PRBs, 120 kHz SCS, D=1

| Number of samples | SNR (dB) | 5 % | 50 % | 95 % | Absolute accuracy |
| --- | --- | --- | --- | --- | --- |
| 1 | -9 | -1,49 | 1,86 | 3,96 | 3,96 |
| 1 | -6 | -1,4 | 1,01 | 2,66 | 2,66 |
| 1 | -3 | -0,92 | 0,59 | 1,78 | 1,78 |
| 1 | 0 | -0,58 | 0,37 | 1,21 | 1,21 |
| 3 | -9 | -1,16 | 1,04 | 2,71 | 2,71 |
| 3 | -6 | -0,77 | 0,58 | 1,73 | 1,73 |
| 3 | -3 | -0,48 | 0,35 | 1,14 | 1,14 |
| 3 | 0 | -0,3 | 0,22 | 0,76 | 0,76 |
| 5 | -9 | -0,91 | 0,82 | 2,26 | 2,26 |
| 5 | -6 | -0,58 | 0,44 | 1,45 | 1,45 |
| 5 | -3 | -0,37 | 0,26 | 0,92 | 0,92 |
| 5 | 0 | -0,24 | 0,17 | 0,6 | 0,6 |
| 10 | -9 | -0,75 | 0,57 | 1,73 | 1,73 |
| 10 | -6 | -0,44 | 0,31 | 1,07 | 1,07 |
| 10 | -3 | -0,28 | 0,19 | 0,67 | 0,67 |
| 10 | 0 | -0,18 | 0,12 | 0,44 | 0,44 |

CSI-RSRQ

CSI-RSRQ - 4 GHz, 24 PRBs, 15 kHz SCS, D=1

| Number of samples | SNR (dB) | 5 % | 50 % | 95 % | Absolute accuracy |
| --- | --- | --- | --- | --- | --- |
| 1 | -9 | -0,23 | 3,65 | 6,3 | 6,3 |
| 1 | -6 | -1,49 | 2,02 | 4,51 | 4,51 |
| 1 | -3 | -1,59 | 1,18 | 3,23 | 3,23 |
| 1 | 0 | -1,13 | 0,75 | 2,28 | 2,28 |
| 3 | -9 | -1,14 | 2,3 | 4,62 | 4,62 |
| 3 | -6 | -1,41 | 1,27 | 3,14 | 3,14 |
| 3 | -3 | -0,93 | 0,76 | 2,13 | 2,13 |
| 3 | 0 | -0,61 | 0,49 | 1,48 | 1,48 |
| 5 | -9 | -1,16 | 1,83 | 3,92 | 3,92 |
| 5 | -6 | -1 | 1,04 | 2,65 | 2,65 |
| 5 | -3 | -0,7 | 0,62 | 1,78 | 1,78 |
| 5 | 0 | -0,46 | 0,38 | 1,21 | 1,21 |
| 10 | -9 | -0,9 | 1,38 | 3,09 | 3,09 |
| 10 | -6 | -0,65 | 0,76 | 2,01 | 2,01 |
| 10 | -3 | -0,45 | 0,45 | 1,31 | 1,31 |
| 10 | 0 | -0,31 | 0,28 | 0,87 | 0,87 |

CSI-RSRQ - 4 GHz, 24 PRBs, 30 kHz SCS, D=1

| Number of samples | SNR (dB) | 5 % | 50 % | 95 % | Absolute accuracy |
| --- | --- | --- | --- | --- | --- |
| 1 | -9 | -0,12 | 3,6 | 6,32 | 6,32 |
| 1 | -6 | -1,67 | 1,92 | 4,52 | 4,52 |
| 1 | -3 | -1,76 | 1,09 | 3,17 | 3,17 |
| 1 | 0 | -1,26 | 0,69 | 2,26 | 2,26 |
| 3 | -9 | -1,25 | 2,26 | 4,54 | 4,54 |
| 3 | -6 | -1,38 | 1,2 | 3,05 | 3,05 |
| 3 | -3 | -0,96 | 0,7 | 2,04 | 2,04 |
| 3 | 0 | -0,62 | 0,42 | 1,38 | 1,38 |
| 5 | -9 | -1,2 | 1,8 | 3,71 | 3,71 |
| 5 | -6 | -0,98 | 0,98 | 2,43 | 2,43 |
| 5 | -3 | -0,69 | 0,55 | 1,6 | 1,6 |
| 5 | 0 | -0,44 | 0,34 | 1,07 | 1,07 |
| 10 | -9 | -0,91 | 1,27 | 2,91 | 2,91 |
| 10 | -6 | -0,75 | 0,69 | 1,85 | 1,85 |
| 10 | -3 | -0,5 | 0,42 | 1,18 | 1,18 |
| 10 | 0 | -0,34 | 0,26 | 0,76 | 0,76 |

CSI-RSRQ - 4 GHz, 24 PRBs, 60 kHz SCS, D=1

| Number of samples | SNR (dB) | 5 % | 50 % | 95 % | Absolute accuracy |
| --- | --- | --- | --- | --- | --- |
| 1 | -9 | -0,18 | 3,57 | 6,31 | 6,31 |
| 1 | -6 | -1,65 | 1,9 | 4,49 | 4,49 |
| 1 | -3 | -1,61 | 1,09 | 3,11 | 3,11 |
| 1 | 0 | -1,15 | 0,65 | 2,2 | 2,2 |
| 3 | -9 | -1,25 | 2,12 | 4,37 | 4,37 |
| 3 | -6 | -1,41 | 1,11 | 2,93 | 2,93 |
| 3 | -3 | -1,08 | 0,66 | 1,96 | 1,96 |
| 3 | 0 | -0,71 | 0,41 | 1,36 | 1,36 |
| 5 | -9 | -1,53 | 1,65 | 3,64 | 3,64 |
| 5 | -6 | -1,33 | 0,87 | 2,4 | 2,4 |
| 5 | -3 | -0,9 | 0,52 | 1,57 | 1,57 |
| 5 | 0 | -0,58 | 0,33 | 1,06 | 1,06 |
| 10 | -9 | -1,1 | 1,11 | 2,75 | 2,75 |
| 10 | -6 | -0,81 | 0,57 | 1,73 | 1,73 |
| 10 | -3 | -0,54 | 0,32 | 1,11 | 1,11 |
| 10 | 0 | -0,38 | 0,2 | 0,76 | 0,76 |

CSI-RSRQ - 4 GHz, 48 PRBs, 15 kHz SCS, D=1

| Number of samples | SNR (dB) | 5 % | 50 % | 95 % | Absolute accuracy |
| --- | --- | --- | --- | --- | --- |
| 1 | -9 | -0,96 | 2,45 | 5,04 | 5,04 |
| 1 | -6 | -1,79 | 1,26 | 3,47 | 3,47 |
| 1 | -3 | -1,25 | 0,73 | 2,35 | 2,35 |
| 1 | 0 | -0,83 | 0,46 | 1,64 | 1,64 |
| 3 | -9 | -1,54 | 1,38 | 3,46 | 3,46 |
| 3 | -6 | -1,28 | 0,71 | 2,25 | 2,25 |
| 3 | -3 | -0,79 | 0,41 | 1,46 | 1,46 |
| 3 | 0 | -0,51 | 0,26 | 0,97 | 0,97 |
| 5 | -9 | -1,45 | 1,08 | 2,74 | 2,74 |
| 5 | -6 | -1,05 | 0,59 | 1,75 | 1,75 |
| 5 | -3 | -0,66 | 0,34 | 1,13 | 1,13 |
| 5 | 0 | -0,4 | 0,2 | 0,75 | 0,75 |
| 10 | -9 | -1,34 | 0,72 | 1,99 | 1,99 |
| 10 | -6 | -0,8 | 0,4 | 1,23 | 1,23 |
| 10 | -3 | -0,48 | 0,24 | 0,78 | 0,78 |
| 10 | 0 | -0,3 | 0,15 | 0,53 | 0,53 |

CSI-RSRQ - 4 GHz, 48 PRBs, 30 kHz SCS, D=1

| Number of samples | SNR (dB) | 5 % | 50 % | 95 % | Absolute accuracy |
| --- | --- | --- | --- | --- | --- |
| 1 | -9 | -0,8 | 2,59 | 5,07 | 5,07 |
| 1 | -6 | -1,66 | 1,4 | 3,48 | 3,48 |
| 1 | -3 | -1,22 | 0,81 | 2,39 | 2,39 |
| 1 | 0 | -0,83 | 0,49 | 1,66 | 1,66 |
| 3 | -9 | -1,38 | 1,47 | 3,45 | 3,45 |
| 3 | -6 | -1,04 | 0,8 | 2,28 | 2,28 |
| 3 | -3 | -0,69 | 0,48 | 1,51 | 1,51 |
| 3 | 0 | -0,43 | 0,3 | 1 | 1 |
| 5 | -9 | -1,26 | 1,12 | 2,81 | 2,81 |
| 5 | -6 | -0,77 | 0,61 | 1,82 | 1,82 |
| 5 | -3 | -0,47 | 0,36 | 1,21 | 1,21 |
| 5 | 0 | -0,3 | 0,23 | 0,81 | 0,81 |
| 10 | -9 | -1,04 | 0,84 | 2,12 | 2,12 |
| 10 | -6 | -0,6 | 0,46 | 1,33 | 1,33 |
| 10 | -3 | -0,36 | 0,28 | 0,87 | 0,87 |
| 10 | 0 | -0,22 | 0,18 | 0,58 | 0,58 |

CSI-RSRQ - 4 GHz, 48 PRBs, 60 kHz SCS, D=1

| Number of samples | SNR (dB) | 5 % | 50 % | 95 % | Absolute accuracy |
| --- | --- | --- | --- | --- | --- |
| 1 | -9 | -0,97 | 2,55 | 5,03 | 5,03 |
| 1 | -6 | -1,62 | 1,38 | 3,45 | 3,45 |
| 1 | -3 | -1,18 | 0,82 | 2,38 | 2,38 |
| 1 | 0 | -0,77 | 0,51 | 1,65 | 1,65 |
| 3 | -9 | -1,54 | 1,51 | 3,48 | 3,48 |
| 3 | -6 | -1,16 | 0,84 | 2,28 | 2,28 |
| 3 | -3 | -0,68 | 0,49 | 1,51 | 1,51 |
| 3 | 0 | -0,46 | 0,32 | 1,03 | 1,03 |
| 5 | -9 | -1,4 | 1,16 | 2,87 | 2,87 |
| 5 | -6 | -0,9 | 0,64 | 1,82 | 1,82 |
| 5 | -3 | -0,56 | 0,38 | 1,2 | 1,2 |
| 5 | 0 | -0,35 | 0,24 | 0,81 | 0,81 |
| 10 | -9 | -0,93 | 0,77 | 2,15 | 2,15 |
| 10 | -6 | -0,58 | 0,45 | 1,37 | 1,37 |
| 10 | -3 | -0,37 | 0,28 | 0,88 | 0,88 |
| 10 | 0 | -0,24 | 0,18 | 0,59 | 0,59 |

CSI-RSRQ - 4 GHz, 96 PRBs, 15 kHz SCS, D=1

| Number of samples | SNR (dB) | 5 % | 50 % | 95 % | Absolute accuracy |
| --- | --- | --- | --- | --- | --- |
| 1 | -9 | -1,33 | 1,88 | 3,97 | 3,97 |
| 1 | -6 | -1,22 | 1,02 | 2,59 | 2,59 |
| 1 | -3 | -0,81 | 0,6 | 1,77 | 1,77 |
| 1 | 0 | -0,57 | 0,38 | 1,21 | 1,21 |
| 3 | -9 | -1,04 | 1,11 | 2,62 | 2,62 |
| 3 | -6 | -0,69 | 0,62 | 1,66 | 1,66 |
| 3 | -3 | -0,47 | 0,37 | 1,09 | 1,09 |
| 3 | 0 | -0,3 | 0,24 | 0,74 | 0,74 |
| 5 | -9 | -0,83 | 0,89 | 2,15 | 2,15 |
| 5 | -6 | -0,53 | 0,49 | 1,37 | 1,37 |
| 5 | -3 | -0,34 | 0,29 | 0,88 | 0,88 |
| 5 | 0 | -0,23 | 0,18 | 0,58 | 0,58 |
| 10 | -9 | -0,47 | 0,65 | 1,57 | 1,57 |
| 10 | -6 | -0,29 | 0,36 | 0,97 | 0,97 |
| 10 | -3 | -0,19 | 0,22 | 0,63 | 0,63 |
| 10 | 0 | -0,13 | 0,14 | 0,42 | 0,42 |

CSI-RSRQ - 4 GHz, 96 PRBs, 30 kHz SCS, D=1

| Number of samples | SNR (dB) | 5 % | 50 % | 95 % | Absolute accuracy |
| --- | --- | --- | --- | --- | --- |
| 1 | -9 | -1,44 | 1,88 | 4,04 | 4,04 |
| 1 | -6 | -1,48 | 1,07 | 2,72 | 2,72 |
| 1 | -3 | -0,9 | 0,6 | 1,81 | 1,81 |
| 1 | 0 | -0,57 | 0,36 | 1,23 | 1,23 |
| 3 | -9 | -1,03 | 1,15 | 2,71 | 2,71 |
| 3 | -6 | -0,71 | 0,63 | 1,71 | 1,71 |
| 3 | -3 | -0,47 | 0,35 | 1,1 | 1,1 |
| 3 | 0 | -0,3 | 0,22 | 0,73 | 0,73 |
| 5 | -9 | -0,79 | 0,9 | 2,27 | 2,27 |
| 5 | -6 | -0,49 | 0,48 | 1,41 | 1,41 |
| 5 | -3 | -0,33 | 0,26 | 0,9 | 0,9 |
| 5 | 0 | -0,22 | 0,16 | 0,59 | 0,59 |
| 10 | -9 | -0,59 | 0,65 | 1,72 | 1,72 |
| 10 | -6 | -0,36 | 0,35 | 1,05 | 1,05 |
| 10 | -3 | -0,23 | 0,2 | 0,65 | 0,65 |
| 10 | 0 | -0,15 | 0,12 | 0,41 | 0,41 |

CSI-RSRQ - 4 GHz, 96 PRBs, 60 kHz SCS, D=1

| Number of samples | SNR (dB) | 5 % | 50 % | 95 % | Absolute accuracy |
| --- | --- | --- | --- | --- | --- |
| 1 | -9 | -1,33 | 1,83 | 4 | 4 |
| 1 | -6 | -1,23 | 1 | 2,68 | 2,68 |
| 1 | -3 | -0,8 | 0,6 | 1,77 | 1,77 |
| 1 | 0 | -0,5 | 0,38 | 1,2 | 1,2 |
| 3 | -9 | -1 | 1,06 | 2,66 | 2,66 |
| 3 | -6 | -0,68 | 0,58 | 1,66 | 1,66 |
| 3 | -3 | -0,42 | 0,36 | 1,08 | 1,08 |
| 3 | 0 | -0,29 | 0,23 | 0,73 | 0,73 |
| 5 | -9 | -0,9 | 0,84 | 2,19 | 2,19 |
| 5 | -6 | -0,58 | 0,47 | 1,37 | 1,37 |
| 5 | -3 | -0,36 | 0,28 | 0,87 | 0,87 |
| 5 | 0 | -0,23 | 0,18 | 0,57 | 0,57 |
| 10 | -9 | -0,7 | 0,64 | 1,56 | 1,56 |
| 10 | -6 | -0,41 | 0,36 | 0,96 | 0,96 |
| 10 | -3 | -0,23 | 0,21 | 0,62 | 0,62 |
| 10 | 0 | -0,15 | 0,13 | 0,41 | 0,41 |

CSI-RSRQ - 30 GHz, 24 PRBs, 60 kHz SCS, D=1

| Number of samples | SNR (dB) | 5 % | 50 % | 95 % | Absolute accuracy |
| --- | --- | --- | --- | --- | --- |
| 1 | -9 | -0,18 | 3,57 | 6,31 | 6,31 |
| 1 | -6 | -1,65 | 1,9 | 4,49 | 4,49 |
| 1 | -3 | -1,61 | 1,09 | 3,11 | 3,11 |
| 1 | 0 | -1,15 | 0,65 | 2,2 | 2,2 |
| 3 | -9 | -1,25 | 2,12 | 4,37 | 4,37 |
| 3 | -6 | -1,41 | 1,11 | 2,93 | 2,93 |
| 3 | -3 | -1,08 | 0,66 | 1,96 | 1,96 |
| 3 | 0 | -0,71 | 0,41 | 1,36 | 1,36 |
| 5 | -9 | -1,53 | 1,65 | 3,64 | 3,64 |
| 5 | -6 | -1,33 | 0,87 | 2,4 | 2,4 |
| 5 | -3 | -0,9 | 0,52 | 1,57 | 1,57 |
| 5 | 0 | -0,58 | 0,33 | 1,06 | 1,06 |
| 10 | -9 | -1,1 | 1,11 | 2,75 | 2,75 |
| 10 | -6 | -0,81 | 0,57 | 1,73 | 1,73 |
| 10 | -3 | -0,54 | 0,32 | 1,11 | 1,11 |
| 10 | 0 | -0,38 | 0,2 | 0,76 | 0,76 |

CSI-RSRQ - 30 GHz, 24 PRBs, 120 kHz SCS, D=1

| Number of samples | SNR (dB) | 5 % | 50 % | 95 % | Absolute accuracy |
| --- | --- | --- | --- | --- | --- |
| 1 | -9 | -0,14 | 3,55 | 6,24 | 6,24 |
| 1 | -6 | -1,61 | 1,9 | 4,47 | 4,47 |
| 1 | -3 | -1,66 | 1,08 | 3,14 | 3,14 |
| 1 | 0 | -1,13 | 0,69 | 2,22 | 2,22 |
| 3 | -9 | -1,14 | 2,14 | 4,37 | 4,37 |
| 3 | -6 | -1,33 | 1,19 | 2,95 | 2,95 |
| 3 | -3 | -0,95 | 0,68 | 2,01 | 2,01 |
| 3 | 0 | -0,58 | 0,42 | 1,38 | 1,38 |
| 5 | -9 | -1,46 | 1,65 | 3,7 | 3,7 |
| 5 | -6 | -1,02 | 0,93 | 2,45 | 2,45 |
| 5 | -3 | -0,67 | 0,55 | 1,63 | 1,63 |
| 5 | 0 | -0,45 | 0,34 | 1,11 | 1,11 |
| 10 | -9 | -1,13 | 1,19 | 2,83 | 2,83 |
| 10 | -6 | -0,74 | 0,67 | 1,86 | 1,86 |
| 10 | -3 | -0,46 | 0,39 | 1,25 | 1,25 |
| 10 | 0 | -0,28 | 0,25 | 0,84 | 0,84 |

CSI-RSRQ - 30 GHz, 48 PRBs, 60 kHz SCS, D=1

| Number of samples | SNR (dB) | 5 % | 50 % | 95 % | Absolute accuracy |
| --- | --- | --- | --- | --- | --- |
| 1 | -9 | -0,97 | 2,55 | 5,03 | 5,03 |
| 1 | -6 | -1,62 | 1,38 | 3,45 | 3,45 |
| 1 | -3 | -1,18 | 0,82 | 2,38 | 2,38 |
| 1 | 0 | -0,77 | 0,51 | 1,65 | 1,65 |
| 3 | -9 | -1,54 | 1,51 | 3,48 | 3,48 |
| 3 | -6 | -1,16 | 0,84 | 2,28 | 2,28 |
| 3 | -3 | -0,68 | 0,49 | 1,51 | 1,51 |
| 3 | 0 | -0,46 | 0,32 | 1,03 | 1,03 |
| 5 | -9 | -1,4 | 1,16 | 2,87 | 2,87 |
| 5 | -6 | -0,9 | 0,64 | 1,82 | 1,82 |
| 5 | -3 | -0,56 | 0,38 | 1,2 | 1,2 |
| 5 | 0 | -0,35 | 0,24 | 0,81 | 0,81 |
| 10 | -9 | -0,93 | 0,77 | 2,15 | 2,15 |
| 10 | -6 | -0,58 | 0,45 | 1,37 | 1,37 |
| 10 | -3 | -0,37 | 0,28 | 0,88 | 0,88 |
| 10 | 0 | -0,24 | 0,18 | 0,59 | 0,59 |

CSI-RSRQ - 30 GHz, 48 PRBs, 120 kHz SCS, D=1

| Number of samples | SNR (dB) | 5 % | 50 % | 95 % | Absolute accuracy |
| --- | --- | --- | --- | --- | --- |
| 1 | -9 | -0,96 | 2,46 | 4,93 | 4,93 |
| 1 | -6 | -1,71 | 1,27 | 3,42 | 3,42 |
| 1 | -3 | -1,26 | 0,74 | 2,35 | 2,35 |
| 1 | 0 | -0,86 | 0,46 | 1,62 | 1,62 |
| 3 | -9 | -1,53 | 1,38 | 3,39 | 3,39 |
| 3 | -6 | -1,2 | 0,73 | 2,21 | 2,21 |
| 3 | -3 | -0,8 | 0,44 | 1,44 | 1,44 |
| 3 | 0 | -0,51 | 0,28 | 0,98 | 0,98 |
| 5 | -9 | -1,54 | 1,1 | 2,76 | 2,76 |
| 5 | -6 | -1,02 | 0,58 | 1,77 | 1,77 |
| 5 | -3 | -0,67 | 0,34 | 1,15 | 1,15 |
| 5 | 0 | -0,43 | 0,23 | 0,77 | 0,77 |
| 10 | -9 | -1,27 | 0,73 | 2,15 | 2,15 |
| 10 | -6 | -0,81 | 0,41 | 1,35 | 1,35 |
| 10 | -3 | -0,48 | 0,23 | 0,86 | 0,86 |
| 10 | 0 | -0,31 | 0,14 | 0,56 | 0,56 |

CSI-RSRQ - 30 GHz, 96 PRBs, 60 kHz SCS, D=1

| Number of samples | SNR (dB) | 5 % | 50 % | 95 % | Absolute accuracy |
| --- | --- | --- | --- | --- | --- |
| 1 | -9 | -1,33 | 1,83 | 4 | 4 |
| 1 | -6 | -1,23 | 1 | 2,68 | 2,68 |
| 1 | -3 | -0,8 | 0,6 | 1,77 | 1,77 |
| 1 | 0 | -0,5 | 0,38 | 1,2 | 1,2 |
| 3 | -9 | -1 | 1,06 | 2,66 | 2,66 |
| 3 | -6 | -0,68 | 0,58 | 1,66 | 1,66 |
| 3 | -3 | -0,42 | 0,36 | 1,08 | 1,08 |
| 3 | 0 | -0,29 | 0,23 | 0,73 | 0,73 |
| 5 | -9 | -0,9 | 0,84 | 2,19 | 2,19 |
| 5 | -6 | -0,58 | 0,47 | 1,37 | 1,37 |
| 5 | -3 | -0,36 | 0,28 | 0,87 | 0,87 |
| 5 | 0 | -0,23 | 0,18 | 0,57 | 0,57 |
| 10 | -9 | -0,7 | 0,64 | 1,56 | 1,56 |
| 10 | -6 | -0,41 | 0,36 | 0,96 | 0,96 |
| 10 | -3 | -0,23 | 0,21 | 0,62 | 0,62 |
| 10 | 0 | -0,15 | 0,13 | 0,41 | 0,41 |

CSI-RSRQ - 30 GHz, 96 PRBs, 120 kHz SCS, D=1

| Number of samples | SNR (dB) | 5 % | 50 % | 95 % | Absolute accuracy |
| --- | --- | --- | --- | --- | --- |
| 1 | -9 | -1,51 | 1,86 | 3,97 | 3,97 |
| 1 | -6 | -1,38 | 1,01 | 2,63 | 2,63 |
| 1 | -3 | -0,88 | 0,59 | 1,75 | 1,75 |
| 1 | 0 | -0,57 | 0,37 | 1,18 | 1,18 |
| 3 | -9 | -1,16 | 1,04 | 2,69 | 2,69 |
| 3 | -6 | -0,77 | 0,58 | 1,71 | 1,71 |
| 3 | -3 | -0,48 | 0,36 | 1,12 | 1,12 |
| 3 | 0 | -0,3 | 0,22 | 0,74 | 0,74 |
| 5 | -9 | -0,91 | 0,81 | 2,26 | 2,26 |
| 5 | -6 | -0,56 | 0,44 | 1,42 | 1,42 |
| 5 | -3 | -0,36 | 0,26 | 0,91 | 0,91 |
| 5 | 0 | -0,22 | 0,17 | 0,59 | 0,59 |
| 10 | -9 | -0,73 | 0,57 | 1,72 | 1,72 |
| 10 | -6 | -0,42 | 0,31 | 1,05 | 1,05 |
| 10 | -3 | -0,27 | 0,18 | 0,67 | 0,67 |
| 10 | 0 | -0,18 | 0,11 | 0,44 | 0,44 |

CSI-SNR

CSI-SNR - 4 GHz, 24 PRBs, 15 kHz SCS, D=1

| Number of samples | SNR (dB) | 5 % | 50 % | 95 % | Absolute accuracy |
| --- | --- | --- | --- | --- | --- |
| 1 | -9 | -0,37 | 3,87 | 7,45 | 7,45 |
| 1 | -6 | -1,83 | 2,41 | 5,75 | 5,75 |
| 1 | -3 | -1,73 | 1,62 | 4,56 | 4,56 |
| 1 | 0 | -1,32 | 1,19 | 3,68 | 3,68 |
| 3 | -9 | -1,3 | 2,54 | 5,34 | 5,34 |
| 3 | -6 | -1,53 | 1,51 | 3,89 | 3,89 |
| 3 | -3 | -1,15 | 0,98 | 2,9 | 2,9 |
| 3 | 0 | -0,8 | 0,7 | 2,27 | 2,27 |
| 5 | -9 | -1,31 | 2,02 | 4,51 | 4,51 |
| 5 | -6 | -1,09 | 1,25 | 3,22 | 3,22 |
| 5 | -3 | -0,82 | 0,8 | 2,32 | 2,32 |
| 5 | 0 | -0,6 | 0,57 | 1,8 | 1,8 |
| 10 | -9 | -1,01 | 1,53 | 3,44 | 3,44 |
| 10 | -6 | -0,74 | 0,92 | 2,33 | 2,33 |
| 10 | -3 | -0,5 | 0,6 | 1,64 | 1,64 |
| 10 | 0 | -0,35 | 0,43 | 1,22 | 1,22 |

CSI-SNR - 4 GHz, 24 PRBs, 30 kHz SCS, D=1

| Number of samples | SNR (dB) | 5 % | 50 % | 95 % | Absolute accuracy |
| --- | --- | --- | --- | --- | --- |
| 1 | -9 | -0,2 | 3,8 | 7,52 | 7,52 |
| 1 | -6 | -1,86 | 2,3 | 5,7 | 5,7 |
| 1 | -3 | -2,05 | 1,51 | 4,44 | 4,44 |
| 1 | 0 | -1,5 | 1,15 | 3,61 | 3,61 |
| 3 | -9 | -1,44 | 2,39 | 5,32 | 5,32 |
| 3 | -6 | -1,66 | 1,41 | 3,83 | 3,83 |
| 3 | -3 | -1,21 | 0,91 | 2,82 | 2,82 |
| 3 | 0 | -0,86 | 0,65 | 2,16 | 2,16 |
| 5 | -9 | -1,24 | 1,92 | 4,23 | 4,23 |
| 5 | -6 | -1,21 | 1,11 | 3,02 | 3,02 |
| 5 | -3 | -0,87 | 0,73 | 2,2 | 2,2 |
| 5 | 0 | -0,62 | 0,52 | 1,69 | 1,69 |
| 10 | -9 | -1,02 | 1,43 | 3,24 | 3,24 |
| 10 | -6 | -0,86 | 0,85 | 2,17 | 2,17 |
| 10 | -3 | -0,61 | 0,54 | 1,5 | 1,5 |
| 10 | 0 | -0,43 | 0,37 | 1,13 | 1,13 |

CSI-SNR - 4 GHz, 24 PRBs, 60 kHz SCS, D=1

| Number of samples | SNR (dB) | 5 % | 50 % | 95 % | Absolute accuracy |
| --- | --- | --- | --- | --- | --- |
| 1 | -9 | -0,04 | 3,75 | 7,46 | 7,46 |
| 1 | -6 | -1,83 | 2,31 | 5,69 | 5,69 |
| 1 | -3 | -2,03 | 1,48 | 4,39 | 4,39 |
| 1 | 0 | -1,5 | 1,14 | 3,64 | 3,64 |
| 3 | -9 | -1,49 | 2,25 | 5,02 | 5,02 |
| 3 | -6 | -1,55 | 1,28 | 3,62 | 3,62 |
| 3 | -3 | -1,19 | 0,82 | 2,67 | 2,67 |
| 3 | 0 | -0,92 | 0,62 | 2,08 | 2,08 |
| 5 | -9 | -1,67 | 1,79 | 4,19 | 4,19 |
| 5 | -6 | -1,52 | 1,04 | 2,93 | 2,93 |
| 5 | -3 | -1,02 | 0,67 | 2,13 | 2,13 |
| 5 | 0 | -0,71 | 0,47 | 1,65 | 1,65 |
| 10 | -9 | -1,25 | 1,21 | 3,09 | 3,09 |
| 10 | -6 | -0,92 | 0,68 | 2,13 | 2,13 |
| 10 | -3 | -0,62 | 0,44 | 1,52 | 1,52 |
| 10 | 0 | -0,46 | 0,32 | 1,16 | 1,16 |

CSI-SNR - 4 GHz, 48 PRBs, 15 kHz SCS, D=1

| Number of samples | SNR (dB) | 5 % | 50 % | 95 % | Absolute accuracy |
| --- | --- | --- | --- | --- | --- |
| 1 | -9 | -1,2 | 2,65 | 5,84 | 5,84 |
| 1 | -6 | -2,01 | 1,57 | 4,31 | 4,31 |
| 1 | -3 | -1,43 | 1,05 | 3,28 | 3,28 |
| 1 | 0 | -1,05 | 0,78 | 2,58 | 2,58 |
| 3 | -9 | -1,68 | 1,52 | 3,93 | 3,93 |
| 3 | -6 | -1,4 | 0,89 | 2,75 | 2,75 |
| 3 | -3 | -0,97 | 0,59 | 1,97 | 1,97 |
| 3 | 0 | -0,68 | 0,42 | 1,51 | 1,51 |
| 5 | -9 | -1,66 | 1,18 | 3,02 | 3,02 |
| 5 | -6 | -1,21 | 0,68 | 2,05 | 2,05 |
| 5 | -3 | -0,8 | 0,46 | 1,48 | 1,48 |
| 5 | 0 | -0,56 | 0,34 | 1,14 | 1,14 |
| 10 | -9 | -1,43 | 0,82 | 2,22 | 2,22 |
| 10 | -6 | -0,9 | 0,47 | 1,46 | 1,46 |
| 10 | -3 | -0,6 | 0,31 | 1,02 | 1,02 |
| 10 | 0 | -0,42 | 0,23 | 0,78 | 0,78 |

CSI-SNR - 4 GHz, 48 PRBs, 30 kHz SCS, D=1

| Number of samples | SNR (dB) | 5 % | 50 % | 95 % | Absolute accuracy |
| --- | --- | --- | --- | --- | --- |
| 1 | -9 | -1,13 | 2,8 | 5,8 | 5,8 |
| 1 | -6 | -2,02 | 1,68 | 4,28 | 4,28 |
| 1 | -3 | -1,49 | 1,1 | 3,22 | 3,22 |
| 1 | 0 | -1,08 | 0,82 | 2,53 | 2,53 |
| 3 | -9 | -1,54 | 1,62 | 3,88 | 3,88 |
| 3 | -6 | -1,26 | 0,99 | 2,69 | 2,69 |
| 3 | -3 | -0,83 | 0,66 | 1,94 | 1,94 |
| 3 | 0 | -0,61 | 0,47 | 1,48 | 1,48 |
| 5 | -9 | -1,41 | 1,25 | 3,17 | 3,17 |
| 5 | -6 | -0,97 | 0,74 | 2,2 | 2,2 |
| 5 | -3 | -0,65 | 0,48 | 1,57 | 1,57 |
| 5 | 0 | -0,47 | 0,36 | 1,17 | 1,17 |
| 10 | -9 | -1,1 | 0,93 | 2,44 | 2,44 |
| 10 | -6 | -0,66 | 0,54 | 1,65 | 1,65 |
| 10 | -3 | -0,44 | 0,35 | 1,17 | 1,17 |
| 10 | 0 | -0,31 | 0,25 | 0,88 | 0,88 |

CSI-SNR - 4 GHz, 48 PRBs, 60 kHz SCS, D=1

| Number of samples | SNR (dB) | 5 % | 50 % | 95 % | Absolute accuracy |
| --- | --- | --- | --- | --- | --- |
| 1 | -9 | -1,14 | 2,8 | 5,79 | 5,79 |
| 1 | -6 | -1,82 | 1,7 | 4,28 | 4,28 |
| 1 | -3 | -1,36 | 1,11 | 3,29 | 3,29 |
| 1 | 0 | -0,96 | 0,8 | 2,61 | 2,61 |
| 3 | -9 | -1,75 | 1,67 | 4,05 | 4,05 |
| 3 | -6 | -1,33 | 1 | 2,84 | 2,84 |
| 3 | -3 | -0,9 | 0,64 | 2,08 | 2,08 |
| 3 | 0 | -0,63 | 0,47 | 1,6 | 1,6 |
| 5 | -9 | -1,58 | 1,27 | 3,34 | 3,34 |
| 5 | -6 | -1,04 | 0,76 | 2,3 | 2,3 |
| 5 | -3 | -0,71 | 0,5 | 1,66 | 1,66 |
| 5 | 0 | -0,48 | 0,37 | 1,28 | 1,28 |
| 10 | -9 | -0,97 | 0,87 | 2,48 | 2,48 |
| 10 | -6 | -0,65 | 0,54 | 1,71 | 1,71 |
| 10 | -3 | -0,45 | 0,37 | 1,19 | 1,19 |
| 10 | 0 | -0,32 | 0,28 | 0,9 | 0,9 |

CSI-SNR - 4 GHz, 96 PRBs, 15 kHz SCS, D=1

| Number of samples | SNR (dB) | 5 % | 50 % | 95 % | Absolute accuracy |
| --- | --- | --- | --- | --- | --- |
| 1 | -9 | -1,57 | 2,06 | 4,51 | 4,51 |
| 1 | -6 | -1,37 | 1,22 | 3,2 | 3,2 |
| 1 | -3 | -0,98 | 0,8 | 2,37 | 2,37 |
| 1 | 0 | -0,69 | 0,58 | 1,83 | 1,83 |
| 3 | -9 | -1,12 | 1,21 | 2,95 | 2,95 |
| 3 | -6 | -0,81 | 0,72 | 2,01 | 2,01 |
| 3 | -3 | -0,57 | 0,48 | 1,4 | 1,4 |
| 3 | 0 | -0,42 | 0,35 | 1,08 | 1,08 |
| 5 | -9 | -0,88 | 0,98 | 2,37 | 2,37 |
| 5 | -6 | -0,6 | 0,58 | 1,59 | 1,59 |
| 5 | -3 | -0,43 | 0,38 | 1,12 | 1,12 |
| 5 | 0 | -0,32 | 0,28 | 0,85 | 0,85 |
| 10 | -9 | -0,53 | 0,71 | 1,77 | 1,77 |
| 10 | -6 | -0,32 | 0,42 | 1,15 | 1,15 |
| 10 | -3 | -0,23 | 0,28 | 0,81 | 0,81 |
| 10 | 0 | -0,19 | 0,2 | 0,61 | 0,61 |

CSI-SNR - 4 GHz, 96 PRBs, 30 kHz SCS, D=1

| Number of samples | SNR (dB) | 5 % | 50 % | 95 % | Absolute accuracy |
| --- | --- | --- | --- | --- | --- |
| 1 | -9 | -1,67 | 2,08 | 4,56 | 4,56 |
| 1 | -6 | -1,57 | 1,27 | 3,26 | 3,26 |
| 1 | -3 | -1,1 | 0,82 | 2,42 | 2,42 |
| 1 | 0 | -0,77 | 0,58 | 1,87 | 1,87 |
| 3 | -9 | -1,15 | 1,25 | 3,04 | 3,04 |
| 3 | -6 | -0,8 | 0,74 | 2,04 | 2,04 |
| 3 | -3 | -0,57 | 0,47 | 1,45 | 1,45 |
| 3 | 0 | -0,41 | 0,33 | 1,1 | 1,1 |
| 5 | -9 | -0,89 | 1,01 | 2,54 | 2,54 |
| 5 | -6 | -0,63 | 0,58 | 1,7 | 1,7 |
| 5 | -3 | -0,44 | 0,37 | 1,16 | 1,16 |
| 5 | 0 | -0,31 | 0,27 | 0,87 | 0,87 |
| 10 | -9 | -0,62 | 0,75 | 1,95 | 1,95 |
| 10 | -6 | -0,42 | 0,43 | 1,26 | 1,26 |
| 10 | -3 | -0,29 | 0,27 | 0,85 | 0,85 |
| 10 | 0 | -0,21 | 0,19 | 0,62 | 0,62 |

CSI-SNR - 4 GHz, 96 PRBs, 60 kHz SCS, D=1

| Number of samples | SNR (dB) | 5 % | 50 % | 95 % | Absolute accuracy |
| --- | --- | --- | --- | --- | --- |
| 1 | -9 | -1,54 | 1,99 | 4,55 | 4,55 |
| 1 | -6 | -1,44 | 1,22 | 3,25 | 3,25 |
| 1 | -3 | -1,05 | 0,78 | 2,4 | 2,4 |
| 1 | 0 | -0,74 | 0,59 | 1,86 | 1,86 |
| 3 | -9 | -1,08 | 1,21 | 3,03 | 3,03 |
| 3 | -6 | -0,8 | 0,72 | 2,02 | 2,02 |
| 3 | -3 | -0,54 | 0,47 | 1,42 | 1,42 |
| 3 | 0 | -0,4 | 0,34 | 1,07 | 1,07 |
| 5 | -9 | -0,96 | 0,93 | 2,44 | 2,44 |
| 5 | -6 | -0,65 | 0,54 | 1,58 | 1,58 |
| 5 | -3 | -0,41 | 0,36 | 1,13 | 1,13 |
| 5 | 0 | -0,28 | 0,27 | 0,85 | 0,85 |
| 10 | -9 | -0,74 | 0,7 | 1,75 | 1,75 |
| 10 | -6 | -0,44 | 0,42 | 1,13 | 1,13 |
| 10 | -3 | -0,26 | 0,27 | 0,77 | 0,77 |
| 10 | 0 | -0,18 | 0,19 | 0,58 | 0,58 |

CSI-SNR - 30 GHz, 24 PRBs, 60 kHz SCS, D=1

| Number of samples | SNR (dB) | 5 % | 50 % | 95 % | Absolute accuracy |
| --- | --- | --- | --- | --- | --- |
| 1 | -9 | -0,04 | 3,75 | 7,46 | 7,46 |
| 1 | -6 | -1,83 | 2,31 | 5,69 | 5,69 |
| 1 | -3 | -2,03 | 1,48 | 4,39 | 4,39 |
| 1 | 0 | -1,5 | 1,14 | 3,64 | 3,64 |
| 3 | -9 | -1,49 | 2,25 | 5,02 | 5,02 |
| 3 | -6 | -1,55 | 1,28 | 3,62 | 3,62 |
| 3 | -3 | -1,19 | 0,82 | 2,67 | 2,67 |
| 3 | 0 | -0,92 | 0,62 | 2,08 | 2,08 |
| 5 | -9 | -1,67 | 1,79 | 4,19 | 4,19 |
| 5 | -6 | -1,52 | 1,04 | 2,93 | 2,93 |
| 5 | -3 | -1,02 | 0,67 | 2,13 | 2,13 |
| 5 | 0 | -0,71 | 0,47 | 1,65 | 1,65 |
| 10 | -9 | -1,25 | 1,21 | 3,09 | 3,09 |
| 10 | -6 | -0,92 | 0,68 | 2,13 | 2,13 |
| 10 | -3 | -0,62 | 0,44 | 1,52 | 1,52 |
| 10 | 0 | -0,46 | 0,32 | 1,16 | 1,16 |

CSI-SNR - 30 GHz, 24 PRBs, 120 kHz SCS, D=1

| Number of samples | SNR (dB) | 5 % | 50 % | 95 % | Absolute accuracy |
| --- | --- | --- | --- | --- | --- |
| 1 | -9 | -0,19 | 3,82 | 7,36 | 7,36 |
| 1 | -6 | -1,86 | 2,37 | 5,66 | 5,66 |
| 1 | -3 | -1,86 | 1,58 | 4,45 | 4,45 |
| 1 | 0 | -1,36 | 1,21 | 3,64 | 3,64 |
| 3 | -9 | -1,26 | 2,29 | 4,95 | 4,95 |
| 3 | -6 | -1,47 | 1,36 | 3,62 | 3,62 |
| 3 | -3 | -1,09 | 0,88 | 2,69 | 2,69 |
| 3 | 0 | -0,76 | 0,63 | 2,09 | 2,09 |
| 5 | -9 | -1,57 | 1,8 | 4,1 | 4,1 |
| 5 | -6 | -1,22 | 1,08 | 2,87 | 2,87 |
| 5 | -3 | -0,88 | 0,7 | 2,07 | 2,07 |
| 5 | 0 | -0,61 | 0,5 | 1,61 | 1,61 |
| 10 | -9 | -1,24 | 1,33 | 3,14 | 3,14 |
| 10 | -6 | -0,87 | 0,78 | 2,17 | 2,17 |
| 10 | -3 | -0,59 | 0,5 | 1,55 | 1,55 |
| 10 | 0 | -0,43 | 0,35 | 1,18 | 1,18 |

CSI-SNR - 30 GHz, 48 PRBs, 60 kHz SCS, D=1

| Number of samples | SNR (dB) | 5 % | 50 % | 95 % | Absolute accuracy |
| --- | --- | --- | --- | --- | --- |
| 1 | -9 | -1,14 | 2,8 | 5,79 | 5,79 |
| 1 | -6 | -1,82 | 1,7 | 4,28 | 4,28 |
| 1 | -3 | -1,36 | 1,11 | 3,29 | 3,29 |
| 1 | 0 | -0,96 | 0,8 | 2,61 | 2,61 |
| 3 | -9 | -1,75 | 1,67 | 4,05 | 4,05 |
| 3 | -6 | -1,33 | 1 | 2,84 | 2,84 |
| 3 | -3 | -0,9 | 0,64 | 2,08 | 2,08 |
| 3 | 0 | -0,63 | 0,47 | 1,6 | 1,6 |
| 5 | -9 | -1,58 | 1,27 | 3,34 | 3,34 |
| 5 | -6 | -1,04 | 0,76 | 2,3 | 2,3 |
| 5 | -3 | -0,71 | 0,5 | 1,66 | 1,66 |
| 5 | 0 | -0,48 | 0,37 | 1,28 | 1,28 |
| 10 | -9 | -0,97 | 0,87 | 2,48 | 2,48 |
| 10 | -6 | -0,65 | 0,54 | 1,71 | 1,71 |
| 10 | -3 | -0,45 | 0,37 | 1,19 | 1,19 |
| 10 | 0 | -0,32 | 0,28 | 0,9 | 0,9 |

CSI-SNR - 30 GHz, 48 PRBs, 120 kHz SCS, D=1

| Number of samples | SNR (dB) | 5 % | 50 % | 95 % | Absolute accuracy |
| --- | --- | --- | --- | --- | --- |
| 1 | -9 | -1,1 | 2,66 | 5,71 | 5,71 |
| 1 | -6 | -2,01 | 1,57 | 4,21 | 4,21 |
| 1 | -3 | -1,5 | 1,06 | 3,23 | 3,23 |
| 1 | 0 | -1,09 | 0,77 | 2,59 | 2,59 |
| 3 | -9 | -1,8 | 1,56 | 3,84 | 3,84 |
| 3 | -6 | -1,41 | 0,86 | 2,71 | 2,71 |
| 3 | -3 | -0,97 | 0,58 | 1,98 | 1,98 |
| 3 | 0 | -0,7 | 0,43 | 1,52 | 1,52 |
| 5 | -9 | -1,7 | 1,18 | 3,22 | 3,22 |
| 5 | -6 | -1,12 | 0,69 | 2,18 | 2,18 |
| 5 | -3 | -0,77 | 0,46 | 1,57 | 1,57 |
| 5 | 0 | -0,57 | 0,33 | 1,22 | 1,22 |
| 10 | -9 | -1,41 | 0,83 | 2,4 | 2,4 |
| 10 | -6 | -0,88 | 0,49 | 1,61 | 1,61 |
| 10 | -3 | -0,59 | 0,33 | 1,17 | 1,17 |
| 10 | 0 | -0,43 | 0,24 | 0,86 | 0,86 |

CSI-SNR - 30 GHz, 96 PRBs, 60 kHz SCS, D=1

| Number of samples | SNR (dB) | 5 % | 50 % | 95 % | Absolute accuracy |
| --- | --- | --- | --- | --- | --- |
| 1 | -9 | -1,54 | 1,99 | 4,55 | 4,55 |
| 1 | -6 | -1,44 | 1,22 | 3,25 | 3,25 |
| 1 | -3 | -1,05 | 0,78 | 2,4 | 2,4 |
| 1 | 0 | -0,74 | 0,59 | 1,86 | 1,86 |
| 3 | -9 | -1,08 | 1,21 | 3,03 | 3,03 |
| 3 | -6 | -0,8 | 0,72 | 2,02 | 2,02 |
| 3 | -3 | -0,54 | 0,47 | 1,42 | 1,42 |
| 3 | 0 | -0,4 | 0,34 | 1,07 | 1,07 |
| 5 | -9 | -0,96 | 0,93 | 2,44 | 2,44 |
| 5 | -6 | -0,65 | 0,54 | 1,58 | 1,58 |
| 5 | -3 | -0,41 | 0,36 | 1,13 | 1,13 |
| 5 | 0 | -0,28 | 0,27 | 0,85 | 0,85 |
| 10 | -9 | -0,74 | 0,7 | 1,75 | 1,75 |
| 10 | -6 | -0,44 | 0,42 | 1,13 | 1,13 |
| 10 | -3 | -0,26 | 0,27 | 0,77 | 0,77 |
| 10 | 0 | -0,18 | 0,19 | 0,58 | 0,58 |

CSI-SNR - 30 GHz, 96 PRBs, 120 kHz SCS, D=1

| Number of samples | SNR (dB) | 5 % | 50 % | 95 % | Absolute accuracy |
| --- | --- | --- | --- | --- | --- |
| 1 | -9 | -1,75 | 2,05 | 4,6 | 4,6 |
| 1 | -6 | -1,62 | 1,24 | 3,27 | 3,27 |
| 1 | -3 | -1,08 | 0,82 | 2,41 | 2,41 |
| 1 | 0 | -0,79 | 0,6 | 1,92 | 1,92 |
| 3 | -9 | -1,22 | 1,14 | 3,12 | 3,12 |
| 3 | -6 | -0,84 | 0,68 | 2,1 | 2,1 |
| 3 | -3 | -0,58 | 0,46 | 1,49 | 1,49 |
| 3 | 0 | -0,44 | 0,35 | 1,13 | 1,13 |
| 5 | -9 | -1 | 0,91 | 2,51 | 2,51 |
| 5 | -6 | -0,7 | 0,54 | 1,71 | 1,71 |
| 5 | -3 | -0,46 | 0,35 | 1,2 | 1,2 |
| 5 | 0 | -0,33 | 0,26 | 0,92 | 0,92 |
| 10 | -9 | -0,78 | 0,66 | 1,95 | 1,95 |
| 10 | -6 | -0,49 | 0,39 | 1,29 | 1,29 |
| 10 | -3 | -0,34 | 0,27 | 0,9 | 0,9 |
| 10 | 0 | -0,24 | 0,2 | 0,68 | 0,68 |

Conclusion

In this contribution, CSI-RSRP, CSI-RSRQ and CSI-SNR simulation results are provided for scenarios in line with the simulation assumptions presented in Section 2.

- References