3GPP TSG-RAN WG4 Meeting #92bis	R4-1911763

Chongqing, China, 14 – 18 October 2019

Agenda Item:	8.17.2.2

Source:	Ericsson

Title:	Discussion on NR PDSCH demodulation requirements for HST

Document for:	Discussion

# 1	Introduction

RAN4#92 agreed with the way forward on high speed train scenario for NR UE demodulation requirements [1]. We discuss some open issues from way forward.

# 2	Discussion

## 2.1	Transmission scheme

- Transmission scheme - HST-SFN with joint transmission. - Same transmission scheme as LTE HST-SFN - Interested companies are encouraged to study the feasibility and performance benefits for HST-SFN with DPS transmission - Focus on Rel-15 RAN1 physical layer design in this WI first. FFS for Rel-16 RAN1 physical layer design.

One of the open issues is whether RAN4 considers the Dynamic Point Selection (DPS) for transmission scheme in HST-SFN. In our understanding, DRS in the proposal is to transmit the PDCCH/PDSCH from one out of four transmission points (e.g., RRH) and the transmission point can be changed every X slot(s).

In LTE, RAN4 has introduced the UE demodulation requirements assuming DPS as in TS36.101 8.3.1.3.2 and 8.3.1.3.5. However, in the specification, it is noted that:

PDSCH transmission from TP 1 and TP 2 shall be randomly determined independently for each subframe. Probabilities of occurrence of PDSCH transmission from TPs are specified.

For example, in TS36.101 8.3.1.3.2, TP1 transmits PDSCH with 30% of transmit time and TP2 transmits with 70%, and the transmission is randomly selected by TE.

Observation 1: LTE UE PDSCH demodulation requirements with DPS is specified with assumption of randomly selected TPs.

If RAN4 tests without random transmission as we discussed in RAN4#92, the test setup becomes very complex. Firstly TE needs choose the best TP, and it is known be L1-RSRP reporting from UE. This requires the test setup where each RRH configures the different CSI-RS resources for beam measurements. Another concern is the UE performance depends not only on PDSCH demodulation performance but also on the L1-RSRP measurement performance. RAN4 UE demodulation requirements usually does not enable CSI feedback in order to focus on the UE demodulation performance except for PMI feedback.

Observation 2: In HST-SFN scenario, TE needs to configure CSI-RS resource for L1-RSRP measurement per TP, and TE schedules the best TP according to the L1-RSRP reporting from UE.

Proposal 1: Considering the test setup complexity of DPS for HST-SFN scenario, RAN4 should assume the joint transmission only for HST-SFN scenario.

## 2.2	Multi-path fading channel

- Deployment scenario - HST-SFN with bidirectional coverage - HST single tap - FFS for multi-path fading channel - Applicability rule will be discussed

It was agreed to consider HST-SFN with bidirectional model and HST single tap in HST scenario, however it was FFS for multi-path fading channel.

According to Rel-15 TS38.101-4, the maximum Doppler frequency for FR1 UE demodulation requirement was 400Hz (TDLB100-400), which corresponds to 120km/h at carrier frequency of 3.6GHz.

According to WID [2], the objective of this WI is to show NR UE demodulation requirements for high speed train scenario with up to 500km/h. There is also ongoing WI for LTE high speed train [3] and whose objective and target are like NR WI, i.e., target speed is 500km/h although the carrier frequency is different. In LTE HST enhancement WI, RAN4 has already agreed to consider only HST-SFN and HST single tap for 500km/h scenario, that is, LTE WI does not introduce a new multi-path fading channel test cases assuming e.g., 500kmh/h.

On the other hand, if we check the Rel-15 TS36.101, the maximum Doppler frequency without HST scenario is set to ETU600. Since the maximum Doppler frequency for Rel-15 NR UE demodulation requirements is 400Hz, one possible compromise is to specify the UE demodulation requirements with the maximum Doppler frequency of 600Hz, which corresponds to 180km/h at the carrier frequency of 3.6GHz and 308km/h at the carrier frequency of 2.1GHz. If we consider lower carrier frequency such as 1GHz, the corresponding UE velocity is more than 600km/h.

Observation 3: The maximum Doppler frequency used in LTE is 600Hz.

Proposal 2: If RAN4 considers the multi-path fading channel also in Rel-16 NR UE demodulation requirements, the maximum Doppler frequency should be at most 600Hz.

# 3	Summary

Observation 1: LTE UE PDSCH demodulation requirements with DPS is specified with assumption of randomly selected TPs.

Observation 2: In HST-SFN scenario, TE needs to configure CSI-RS resource for L1-RSRP measurement per TP, and TE schedules the best TP according to the L1-RSRP reporting from UE.

Proposal 1: Considering the test setup complexity of DPS for HST-SFN scenario, RAN4 should assume the joint transmission only for HST-SFN scenario.

Observation 3: The maximum Doppler frequency used in LTE is 600Hz.

Proposal 2: If RAN4 considers the multi-path fading channel also in Rel-16 NR UE demodulation requirements, the maximum Doppler frequency should be at most 600Hz.

# References

- R4-1910050, “WF on UE demodulation for NR HST”, CMCC.

- RP-191788, “New WID on NR support for high speed train scenario”, CMCC.

- RP-181482, “Further performance enhancement for LTE in high speed scenario”, NTT Docomo.