3GPP TSG-RAN WG4 Meeting #92bis	R4-1911764

Chongqing, China, 14 – 18 October 2019

Agenda Item:	8.17.1

Source:	Ericsson

Title:	RLM for NR HST

Document for:	Discussion

# 1	Introduction

RAN4#92 agreed with the work plan for RRM part of NR support for high speed train scenario [1], according to the plan,

RAN4#92bis - Discuss and identify which RRM requirements need to be enhanced to support up to 500km/h, both SA single carrier scenario and EN-DC scenario are considered. - Idle and inactive mode: cell reselection including cell identification and measurement requirements - Connected mode: cell identification, measurement delay, beam management related requirements, e.g. L1-RSRP measurement, RLM, UL timing. - If needed other requirements are not precluded. - Discuss the candidate solution for the RRM enhancement - Align simulation assumption for RRM enhancement - Discuss the possible signaling impact (e.g. UE capabilities, network flag)

In this contribution we discuss the impact of RLM under high speed trains scenario.

# 2	Discussion

Radio link monitoring (RLM) is a procedure where UE measures the channel quality based on SSB or CSI-RS. If the measured download channel quality is below the threshold Qout, UE indicates out-of-synch to the higher layer. On the other hand, if the measured channel quality exceeds the threshold during the out-of-synch, UE indicates in-synch to the higher layer.

According to TS38.133 8.1.2/8.1.3, the threshold Qout is defined as the level at which the channel quality corresponds to BLERout (%) BLER of a hypothetical PDCCH. On the other hand, the threshold Qin is defined as the level at which the channel quality can be significantly more reliably received than at Qout and shall correspond to BLERin (%) BLER of a hypothetical PDCCH. The assumed PDCCH parameters are shown in Table 1. According to TS38.133 8.1.1, configuration 0 assumes BLERout=10% and BLERin=2%, which is the same assumption as RLM used in LTE.

When RAN4 introduces the RLM requirements, RAN4 usually performs PDCCH link level simulations based on the assumption parameters and verifies the thresholds Qin (i.e., SNR corresponding to 2% of PDCCH BLER) is significantly high compared with Qout (i.e., SNR corresponding to 10% of PDCCH BLER).

In order to investigate the RLM impact due to the high speed scenario, we performed the PDCCH link simulation with the parameters listed in Table 1.

Table 1	Simulation parameters for DPCCH for RLM

| Parameters | Values |
| --- | --- |
| Frequency range | FR1 |
| DCI format | 1-0 |
| Number of OFDM symbols | 2 |
| Aggregation level | Qin: 4 Qout: 8 |
| Bandwidth (RB) | 24 |
| SCS | 15kHz, 30kHz for FR1 |
| DMRS precoder granularity | REG bundle size, and bundle size = 6 |
| CP length | Normal |
| Mapping from REG to CCE | Distributed |
| Antenna configuration | 1x2 |
| Propagation channel | TDLA30, TDLC300 |
| Maximum Doppler frequency (Hz) | 1000, 1666 |

Table 2	SNR (dB) to achieve 2% (Qin) and 10% (Qout) of PDCCH BLER and the difference between Qin and Qout (PDCCH power boosting not applied).

| Channel model | SCS | Qin | Qout | Diff |
| --- | --- | --- | --- | --- |
| TDLA30-1000 | 15kHz | -0.3 | -6.5 | 6.2 |
| TDLA30-1666 | 15kHz | -0.4 | -6.4 | 6.0 |
| TDLA30-1000 | 30kHz | -1.0 | -6.5 | 5.5 |
| TDLA30-16666 | 30kHz | -1.0 | -6.5 | 5.5 |
| TDLC300-1000 | 15kHz | -0.3 | -6.5 | 6.2 |
| TDLC300-1666 | 15kHz | -0.4 | -6.4 | 6.0 |
| TDLC300-1000 | 30kHz | -1.0 | -6.5 | 5.5 |
| TDLC300-1666 | 30kHz | -1.0 | -6.5 | 5.5 |

Table 2 summarizes our simulation results that show the required SNR to achieve 2% (Qin) and 10% (Qout) of PDCCH BLER. Note we don’t assume 4dB power boosting for Qout. From the simulation results, it is observed that the SNR separation between Qin and Our is more than 5.5dB. If we consider 4dB power boosting assumption for Qout, the difference is close to 10dB.

Observation: SNR difference between Qin and Qout are separated enough even with the maximum Doppler frequency of 1666Hz.

We therefore don’t think any RLM enhancement is needed due to NR HST scenario.

Proposal: RAN4 does not need to introduce any RLM enhancement for NR HST WI.

# 3	Conclusion

Observation: SNR difference between Qin and Qout are separated enough even with the maximum Doppler frequency of 1666Hz.

Proposal: RAN4 does not need to introduce any RLM enhancement for NR HST WI.

# References

- R4-1908677, “Work plan for RRM part of NR support for high speed train scenario”, CMCC.

- RP-191788, “New WID on NR support for high speed train scenario”, CMCC.