3GPP TSG-RAN WG4 Meeting #92bis	R4-1911765

Chongqing, China, 14 – 18 October 2019

Agenda Item:	8.17.1

Source:	Ericsson

Title:	L1-RSRP measurements for high speed train scenario

Document for:	Discussion

# 1	Introduction

RAN4#92 agreed with the work plan for RRM part of NR support for high speed train scenario [1], according to the plan,

RAN4#92bis - Discuss and identify which RRM requirements need to be enhanced to support up to 500km/h, both SA single carrier scenario and EN-DC scenario are considered. - Idle and inactive mode: cell reselection including cell identification and measurement requirements - Connected mode: cell identification, measurement delay, beam management related requirements, e.g. L1-RSRP measurement, RLM, UL timing. - If needed other requirements are not precluded. - Discuss the candidate solution for the RRM enhancement - Align simulation assumption for RRM enhancement - Discuss the possible signaling impact (e.g. UE capabilities, network flag)

In this contribution we discuss the impact of L1-RSRP measurements for reporting under high speed trains scenario.

# 2	L1-RSRP measurement for reporting

UE shall report the L1-RSRP measurement results for the configured SSB and/or CSI-RS resources as a part of CSI reporting. The network configures the L1-RSRP reporting with periodic reporting with PUCCH, semi-persistent reporting with PUCCH/PUSCH, or aperiodic reporting with PUSCH. For the beam reporting, RAN4 specified the measurement period.

Table 1 shows the measurement period of SSB-based L1-RSRP in FR1 used for the beam reporting, specified in TS38.133 9.5.4. For SSB-based L1-RSRP measurement, the measurement period is based on the measurements of M SSB samples, and therefore the evaluation period depends on the P (measurement gap), TSSB (SSB periodicity), TDRX (DRX cycle length), and the minimum measurement period of TReport (configured periodicity of L1-RSRP reporting). The number of SSB samples, M, depends on the configuration; it is set to 1 if timeRestrictionForChannelMeasurement is configured and set to 3 otherwise.

In the case of CSI-RS based L1-RSRP reporting, the number of CSI-RS samples also depends on the network configuration; it is set to 1 if aperiodic CSI-RS resource is configured. For the periodic or semi-persistent CSI-RS resources, it is set to 1 if timeRestrictionForChannelMeasurement is configured and set to 3 otherwise.

Table 1	SSB based L1-RSRP measurement period for FR1.

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(M*P)*TDRX |
| Note: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

RAN4 also specified the L1-RSRP measurement accuracy requirements based on the measurement period above. Table 2 shows the SSB-based L1-RSRP absolute accuracy in FR1 specified in TS38.133 10.1.19 as example.

Table 2	SSB based L1-RSRP absolute accuracy in FR1.

Table 10.1.19.1.1-1: SSB based L1-RSRP absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| ±5.0 | ±9.5 | -3dB | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -70 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -70 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -70 |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -70 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -70 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -70 |
| ±8.5 | ±11.5 | -3dB | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_G, NR_FDD_FR1_H, | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

# 3	L1-RSRP measurement impact under high speed train scenario

According to WID for NR HST [2],

- Feasibility of supporting speeds of up to a maximum of 500km/h will be investigated. The actual maximum supported velocity at 3.6GHz will be decided in this WI.

If we consider 500km/h at the carrier frequency of 3.6GHz, the maximum Doppler frequency becomes 1666Hz. We studied the L1-RSRP estimation accuracy based on the parameters used to set the L1-RSRP measurement and accuracy requirements in Rel-15 [3].

## 3.1	SSB based L1-RSRP measurements

Table 3 shows the simulation parameters for SSB based L1-RSRP measurements in HST scenario. We also investigated more SSB samples than [3], for HST condition.

Table 3	Simulation parameters for SSB based L1-RSRP measurements in HST scenario.

| Parameters | Values |
| --- | --- |
| SCS | 15kHz, 30kHz |
| Side condition (SNR) | -3dB |
| Number of samples (M) | 1 (Baseline), 3, 5 |
| Propagation condition | TDLA30, TDLC300 |
| Maximum Doppler frequency | 1000Hz (=300km/h at 3.6GHz), 1666Hz |

Table 4	SSB based L1-RSRP estimation simulation results.

| SCS | Samples | AWGN [4] | TDLA30-1000 | TDLA30-1666 | TDLC300-1000 | TDLC300-1666 |
| --- | --- | --- | --- | --- | --- | --- |
| 15kHz | 1 | 1.2 (Rel-15) | 1.39 | 1.53 | 1.44 | 1.48 |
|  | 3 |  | 0.71 | 0.71 | 0.79 | 0.78 |
|  | 5 |  | 0.54 | 0.54 | 0.55 | 0.54 |
| 30kHz | 1 | 0.8 (Rel-15) | 0.88 | 0.9 | 0.97 | 0.99 |
|  | 3 |  | 0.46 | 0.49 | 0.58 | 0.58 |
|  | 5 |  | 0.36 | 0.4 | 0.42 | 0.43 |

Table 4 shows our simulation results of the SSB based L1-RSRP estimation. We put the maximum of absolute value of 5%-ile error and 95%-ile error values as the same metric as [4]. For reference we put our results in the static condition, which is used to specify the L1-RSRP measurement accuracy requirements in Rel-15.

If we compare the fading case (Doppler=1000Hz or 1666Hz) with AWGN case, we don’t observe the accuracy degradation due to the fading channel and/or high Doppler frequency.

Observation 1: SSB based L1-RSRP accuracy is not impacted with the high speed scenario.

Proposal 1: RAN4 does not introduce new measurement/accuracy requirements for SSB-based L1-RSRP reporting in NR HST WI.

## 3.2	CSI-RS based L1-RSRP measurements

Table 5 shows the simulation parameters for CSI-RS based L1-RSRP measurements in HST scenario. Like SSB-based L1-RSRP, we investigated more CSI-RS samples and larger BW for HST scenario.

Table 5	Simulation parameters for CSI-RS based L1-RSRP measurements in HST scenario.

| Parameters | Values |
| --- | --- |
| SCS | 15kHz, 30kHz |
| Side condition (SNR) | -3dB |
| Density (D) | 3 |
| Number of samples (M) | 1 (Baseline), 3, 5 |
| Number of PRBs | 48 (Baseline), 96 |
| Propagation condition | TDLA30, TDLC300 |
| Maximum Doppler frequency | 1000Hz, 1666Hz |

Table 6	CSI-RS based L1-RSRP estimation simulation results.

| SCS | Samples | PRB | AWGN [4] | TDLA30-1000 | TDLA30-1666 | TDLC300-1000 | TDLC300-1666 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 15kHz | 1 | 48 | 0.77 (Rel15) | 1.78 | 1.77 | 1.77 | 1.67 |
|  |  | 96 |  | 0.98 | 0.97 | 0.91 | 0.87 |
|  | 3 | 48 |  | 0.83 | 0.83 | 0.9 | 0.93 |
|  |  | 96 |  | 0.48 | 0.52 | 0.48 | 0.5 |
|  | 5 | 48 |  | 0.62 | 0.64 | 0.79 | 0.73 |
|  |  | 96 |  | 0.41 | 0.37 | 0.37 | 0.38 |
| 30kHz | 1 | 48 | 0.55 (Rel15) | 1.01 | 1.16 | 1.18 | 1.26 |
|  |  | 96 |  | 0.59 | 0.65 | 0.59 | 0.62 |
|  | 3 | 48 |  | 0.54 | 0.57 | 0.66 | 0.7 |
|  |  | 96 |  | 0.34 | 0.35 | 0.35 | 0.38 |
|  | 5 | 48 |  | 0.42 | 0.46 | 0.52 | 0.54 |
|  |  | 96 |  | 0.29 | 0.3 | 0.27 | 0.27 |

Table 6 shows our simulation results of the CSI-RS based L1-RSRP estimation. We set the same metric as SSB based L1-RSRP. Unlike SSB-based L1-RSRP results, it is observed the measurement accuracy degradation due to fading and/or high Doppler frequency, i.e., from 0.77dB to more than 1.67dB in the case of SCS=15kHz.

Observation 2: CSI-RS based L1-RSRP accuracy is degraded significantly due to the high speed scenario.

## 3.3	Discussion

From our simulation results it is observed that the CSI-RS based L1-RSRP accuracy is degraded with the high speed scenario. It is also observed that the increase of CSI-RS samples in time domain and/or frequency domain can maintain the same accuracy. However, we don’t think more CSI-RS samples in time domain is good solution because UE is moving very fast and the network needs the latest beam information to select the best beam. To address this issue, there are two options:

Option 1: Keep the same CSI-RS based L1-RSRP accuracy as normal speed scenario but wider CSI-RS bandwidth (e.g., 96RB) should be assumed as side condition. The drawback is to limit the network configuration because we need 20MHz if we allocate 96RBs for CSI-RS with SCS=15kHz.

Option 2: Relax L1-RSRP accuracy requirements (e.g., by 1dB) but the network keeps the same CSI-RS bandwidth assumption (i.e., 48RB) as normal speed scenario.

Proposal 2: RAN4 study the impact of CSI-RS based L1-RSRP measurement requirements due to the HST scenario.

# 4	Summary

Observation 1: SSB based L1-RSRP accuracy is not impacted with the high speed scenario.

Proposal 1: RAN4 does not introduce new measurement/accuracy requirements for SSB-based L1-RSRP reporting in NR HST WI.

Observation 2: CSI-RS based L1-RSRP accuracy is degraded significantly due to the high speed scenario.

Proposal 2: RAN4 study the impact of CSI-RS based L1-RSRP measurement requirements due to the HST scenario.

# References

- R4-1908677, “Work plan for RRM part of NR support for high speed train scenario”, CMCC.

- RP-191788, “New WID on NR support for high speed train scenario”, CMCC.

- R4-1904760, “Simulation summary of L1-RSRP accuracy”, Ericsson.

- R4-1904820, “Way forward on L1-RSRP accuracy requirements”, Ericsson, Nokia, CMCC.