3GPP TSG-RAN WG4 Meeting #92bis	R4-1911770

Chongqing, China, 14 – 18 October 2019

Agenda Item:	8.11.2.2

Source:	Ericsson

Title:	Discussion on link recovery procedure for SCell

Document for:	Discussion

# 1	Introduction

In Rel-16 Enhancements on MIMO for NR [1], it was agreed to support SCell also for beam failure recovery.

- Extend specification support in the following areas [RAN1] […] - Enhancements on multi-beam operation, primarily targeting FR2 operation: - Perform study and, if needed, specify enhancement(s) on UL and/or DL transmit beam selection specified in Rel-15 to reduce latency and overhead - Specify beam failure recovery for SCell with DL/UL as well as DL-only, where PCell can be operating in FR1 as well as FR2 - Specify measurement and reporting of either L1-RSRQ or L1-SINR […] - Specify core requirements associated with the items specified by RAN1 [RAN4] - Specify RRM requirements - Identify impact on RF requirements for the reduced PAPR pi/2-BPSK DMRS and, if needed, specify RF requirements - Identify impact on RF requirements for the uplink full power transmission and, if needed, specify RF requirements

We discuss the beam failure recovery requirements on Scell based RAN1 agreements.

# 2	Discussion

One of the WI objective on NR eMIMO is to specify the beam failure recovery for SCell. The link recovery procedure requirements specified in Rel-15 TS38.133 8.5.1 is however applicable only for PCell and PScell show below.

The UE shall assess the downlink link quality of a serving cell based on the reference signal in the set as specified in TS 38.213 [3] in order to detect beam failure instance for any of: - PCell in SA, NR-DC, or NE-DC operation mode, - PSCell in NR-DC and EN-DC operation mode,

In our understanding, RAN1 agreements so far is to apply the same procedure to Scell-based link recovery as exiting Pcell/PScell based link recovery procedure. We propose to include the SCell-based link recovery procedure also as the purpose of requirements in TS38.133 8.5.

Proposal: RAN4 includes SCell-based link recovery procedures in TS38.133 8.5, i.e.,

- SCell in SA, NR-DC, or NE-DC operation mode

If RAN1 considers some limitation for SCell-based link recovery, RAN4 needs update the requirements accordingly.

# 3	Summary

Proposal: RAN4 includes SCell-based link recovery procedures in TS38.133 8.5, i.e.,

- SCell in SA, NR-DC, or NE-DC operation mode

# References

- RP-192271, “Revised WID: Enhancements on MIMO for NR”, Samsung.

# Appendix:	RAN1 agreements on BFR in SCell

RAN1#94bis (1810 Chengdu) Agreement RAN1 has identified the following scenarios to be important for SCell BFR - Scenario 1: SCell with both uplink and downlink - Scenario 2: SCell with downlink only Agreement RAN1#95AH (1901 Taipei) Specification support will be provided for gNB to derive at least the failed CC index during SCell BFR procedure - FFS: Whether the information is implicitly derived or explicitly conveyed by the UE - FFS: Whether new beam information should be included - FFS: Details on triggering for transmitting BFRQ Agreement - SCell BFD is based on periodic 1-port CSI-RS, which can be configured explicitly by RRC or implicitly by TCI state. - Down-select one of the following alternatives in RAN1#96: - Alt 1: SCell BFD RS is in current CC - Alt 2: SCell BFD RS is in current CC for explicit configuration and can be in current CC or another CC for implicit configuration - Alt 3: SCell BFD RS can be in current CC or another CC for both explicit and implicit configuration Agreement Down-select at least one of the following alternatives: - Alt 1: For SCell BFR, BFRQ can be transmitted if UE declares beam failure and identifies a new candidate beam. - UE reports new beam information by or after BFRQ - Alt 2: For SCell BFR, BFRQ can be transmitted if UE declares beam failure. - UE only indicates beam failure happens by BFRQ - Note: new beam identification can be done by using DL BM procedure - Alt 3: For SCell BFR, BFRQ can be transmitted if UE declares beam failure - UE may report new beam information during BFR procedure - FFS: impact of new beam identification threshold - Note: It is up to UE whether to do beam failure detection and new beam identification in parallel or not - For Alt1 and Alt3, reference signals for new candidate DL beam(s) are configured, which are based on CSI-RS and/or SSB. - FFS: whether the CSI-RS and/or SSB can be in another CC - FFS: signaling details, e.g. RRC and/or MAC CE Agreement For SCell BFR - Decide BFRQ solution for BFR on SCell with DL only first, PCell in FR1+FR2 Above is to facilitate RAN1 discussion but not to prioritize certain scenarios RAN #96 (1902 Athens) Agreement - For SCell BFR, BFRQ shall be conveyed if UE declares beam failure - UE shall convey new beam information during BFR procedure if new candidate beam RS and corresponding threshold is configured and at least if channel quality of new beam is above or equal to threshold - FFS: whether no new beam identified could be included as a state of new beam information - FFS: details if no new beam is above or equal to threshold Agreement - For SCell with downlink only, UE reports failed CC index(es) and new beam information (if present) by PUSCH or PUCCH - FFS: whether it is carried by MAC CE or UCI-like PUSCH or PUCCH - Down-select at least one options for BFRQ procedure in RAN1 #97: - Option 1: Failed CC index(es), new beam information (if present) and beam failure event to be reported by a single report by MAC CE - FFS: whether or not to have spec impact on resource for MAC CE - Resource for MAC CE is not triggered by dedicated PUCCH/PRACH for BFR - Option 2: step 1: UE conveys beam failure event, and step 2: UE reports new beam information (if present) and failed CC index(es) - Step 1 is carried by dedicated PUCCH/PRACH resource - Step 2 is carried by MAC CE or UCI - Option 3: step 1: UE conveys beam failure event and failed CC index(es), and step 2: UE reports new beam information (if present) - Step 2 is carried by MAC CE or UCI, e.g. AP-CSI - PUCCH/PRACH is used for step 1 to carry failed CC index(es) implicitly - FFS: whether it is single-bit PUCCH or multi-bit PUCCH - The failed CC index(es) should be selected from up to N_max CCs for SCell BFR - FFS: N_max RAN1#97 (1905 Reno) Agreement During a BFRQ procedure, UE reports only 1 beam with corresponding beam index only per SCell Agreement On BFRQ procedure for SCell - Step 1 can be carried by at least a dedicated SR-like PUCCH resource for BFR over PCell or PSCell - FFS: Details including whether or not it is precluded that MAC CE in step 2 is multiplexed in a PUSCH not triggered by step 1 - (Working Assumption) Step 2 is carried by MAC CE Above applies at least for SCell with downlink only Send an LS to RAN2 to ask their input with reference to this agreement from their specification work point of view considering their workload. The draft LS in R1-1907850 is modified and endorsed in R1-1907870. Agreement When SCell BFD RS is configured in an implicit manner, BFD RS can be transmitted in active BWP of either current CC or another CC. Agreement A UE can be configured to perform BFR for any configured SCells - The maximum number of SCells for which the UE performs BFR is a UE capability Agreement - When SCell BFR is configured and RS for new beam identification is configured, the threshold for new beam identification should be always configured - If a SCell has failed, when there is no new beam with L1-RSRP higher than configured threshold for SCell BFR, for new beam information reporting, UE reports that there is no new beam identified for the SCell RAN1#98 (1908 Prague) Agreement Support that the BLER threshold for SCell BFD is the same as the default value of rlmInSyncOutOfSyncThreshold. Agreement - For SCell BFR, support maximum number of RS for new beam identification per BWP to be 64. - The range of threshold for SCell new beam identification is based on range specified in RSRP-Range. Agreement For SCell with both UL and DL, at least reuse the same BFRQ procedure as SCell with DL only. - Note: Whether to support CBRA/CFRA based BFRQ for both scenarios is a separate issue. - Note: At least from RAN1 perspective, there is no need for introducing restrictions on MAC CE transmission for BFR in Rel-16 - FFS: Whether PUCCH-BFR can be configured on SCells Agreement Support PUCCH-BFR to be configured by either one of PUCCH format 0 and PUCCH format 1 - FFS: details when PUCCH-BFR transmission is to be made in the same slot with other uplink signal(s). Conclusion - The details on MAC CE for BFR, and whether to transmit a MAC CE to carry BFRQ information for 1 SCell or multiple SCells is up to RAN2 - RAN1 identified that beam failure on multiple SCells can occur simultaneously but have not reached consensus on how often this occurs Agreement Include the above conclusion as part of LS to RAN2. Conclusion From RAN1 perspective, the response after UE transmits PUCCH-BFR can be a normal uplink grant with C-RNTI/MCS-C-RNTI - No RAN1 spec impact Agreement Send an LS to RAN2 suggesting higher priority for SCell BFR MAC CE than at least UL data, and to have a higher priority for SCell step 1 PUCCH than normal SR - To be included as part of LS to RAN2 Agreement RAN1 will conclude on the following issue in RAN1#98bis Q3: Is there a case where the SR-like dedicated PUCCH resource for SCell BFR is not configured? If the SR-like dedicated PUCCH resource is not configured, one possible option being considered by RAN2 is that the UE follows the existing framework for requesting uplink resources when no uplink resources are available (i.e. performs CBRA on SpCell). Agreement The BFRR to step 2 is a normal uplink grant to schedule a new transmission for the same HARQ process as PUSCH carrying the step 2 MAC CE - The procedure is the same as normal “ACK” for PUSCH - When UE receives BFRR to step 2, UE can consider BFR procedure is finished - No RAN1 spec impact - Included as part of LS to RAN2