3GPP TSG-RAN WG4 Meeting #92bis	R4-1911771

Chongqing, China, 14 – 18 October 2019

Agenda Item:	7.11.3.3

Source:	Ericsson

Title:	Discussion on channel quality reporting for NB-IoT

Document for:	Discussion

# 1	Introduction

RAN plenary #83 revised the WID for Rel-16 NB-IoT [1] and added to support DL channel quality support in CONNECTED as well as MSG3-based quality report for non-anchor carrier.

Improved multi-carrier operation: - Specify support of Msg3 quality reporting for non-anchor access [RAN1, RAN2, RAN4] - Specify support for quality reporting in connected mode for anchor and non-anchor carriers. The quality report is not carried in the physical layer. [RAN1, RAN2, RAN4]. - Specify signalling to indicate on a non-anchor carrier for paging a set of subframes which will contain NRS even when no paging NPDCCH is transmitted [RAN1, RAN2, RAN4]

RAN4#92 discussed the channel quality reporting in MSG3 and agreed with the way forward as follows [2]:

- Msg3 based channel quality reporting - Use evaluation period T2 if quality report is associated with non-anchor carrier.

- Connected channel quality report - Use the same hypothetical NPDCCH parameters, report mapping and accuracy requirements as MSG3-based report. - UE can use the NPDCCH period which carries the UL grant of channel quality report for measurement of DL channel quality of the configured carrier. Per TS 36.213, the NPDCCH repetition level can be from the set - R ∈ {1,2,4,8,16,32,64,128,256,512,1024,2048}

We discuss further on the channel quality reporting for NB-IoT based on the latest agreements by RAN1/RAN2 as listed in Appendix.

# 2	Discussion

## 2.1	MSG3-based channel quality report associated with non-anchor carrier

Figure 1	DL channel quality reporting on non-anchor for NB-IoT

Figure 1 summarizes the DL channel quality measurement resource and period for MSG3-based quality reporting according to the agreements in RAN1/RAN2/RAN4 so far. For the DL channel quality report on non-anchor carrier, it was agreed that UE measures the DL channel quality at least on the non-anchor carrier for RAR during the RAR transmission period, i.e., T2 in Figure 1. In our understanding, RAN1 also concluded they does not further discuss any other carrier than the one UE monitors to receive Msg2 (i.e. RAR) in Rel-16.

Based on the agreements, RAN4 can specify the RRM core requirements for MSG3-based DL channel quality reporting for non-anchor carrier as follows:

#### MSG3-based channel quality report for non-anchor for UE Category NB1 The DL channel quality provides the serving eNB with information about the minimum NPDCCH repetition level to satisfy the hypothetical NPDCCH block error rate of 1% with the parameters specified in Table X. Table X: NPDCCH transmission parameters for downlink quality reporting. Parameters; Values. DCI format; Format N1. Number of information bits (excluding CRC); 23bits. System bandwidth; 200kHz. Aggregation level; 2. DRX; OFF. The reported NPDCCH repetition level shall be derived from the channel quality measured in the period the carrier where the random access response is transmitted. The NPDCCH repetition level for CQI-NPDCCH-NB and CQI-NPDCCH-Short-NB is chosen from the supported NPDCCH repetition levels [3]. The report mapping is defined in 9.1.22.15. The UE shall satisfy the downlink channel quality measurement accuracy requirements as specified in 9.1.22.16.

## 2.2	DL quality reporting in CONNECTED mode for anchor/non-anchor carriers

For the channel quality report in connected mode, RAN1 agreed to reuse the same definition as Rel-14 NB-IoT MSG3-based channel quality report, i.e., the number of repetitions that the UE needs to decode hypothetical NPDCCH with BLER of 1%.

Regarding the measurement resources, RAN1 also agreed to use at least the carrier assigned to monitor in USS for NPDCCH and the associated NPDSCH. In our understanding it is still FFS whether other carrier(s) are used for the DL channel quality measurement.

According to RAN2 agreements, the DL quality report in connected mode is not periodic or on-demand. They also agreed to use MAC CE to report the DL channel quality of the configured carrier in RRC connected mode, and to introduce a one-byte MAC CE DL channel quality report with a maximum of 8 bits.

In our view RAN4 should wait for the RAN1/RAN2 conclusion for DL quality reporting in CONNECTED mode.

# 3	Summary

Proposal 1: RAN4 specifies the RRM core requirements for MSG3-based channel quality reporting for non-anchor carrier in IDLE as shown in Section 2.1.

Proposal 2: For DL quality reporting in CONNECTED mode, RAN4 waits for RAN1/RAN2 decision.

# References

- RP-190757, “WID revision: Additional enhancements for NB-IoT”, Huawei, HiSilicon.

- R4-1910576, “Way forward on RRM impact of the R16 enhancement for NB-IoT ​”, Huawei, HiSilicon.

# Appendix

## RAN1 agreements

RAN1#94 Agreement For channel quality report in Msg3 on non-anchor access, the channel quality definition is denoted by the number of repetitions that the UE needs to decode hypothetical NPDCCH with BLER of 1% - FFS: Whether the details on the hypothetical NPDCCH are specified or not Working Assumption For channel quality report in Msg3 on non-anchor access, UE performs the channel quality measurement on the carrier it monitors to receive Msg2 (i.e. RAR) - FFS: Whether the UE performs measurement on other carriers Agreement For non-anchor access, RAN1 further studies how UEs report the measured channel quality. RAN1#94bis Agreement RAN1 does not define search space for hypothetical NPDCCH for channel quality report in Msg3 on non-anchor access. Agreement From RAN1 point of view, specification support for measurement period for non-anchor access in RAN1 specifications is not needed Agreement RAN1 does not define measurement reference resource for non-anchor access. For further study: The following scenarios with regards to downlink channel quality reporting in msg3 for non-anchor carrier access. - For EDT/non-EDT, msg3 associated with PDCCH order PRACH, IDLE - PUR RAN4#95 Agreement In case 4 bits is used for a non-anchor carrier, all repetition i.e. 12 candidate values {1,2,4,8,16,32,64,128,256,512,1024,2048} can be reported in Msg3. Agreement In case of 2 bits is used for a non-anchor carrier, 3 candidate values can be reported in Msg3. Select one of the following alternatives for determining the 3 values: - Depending on Rmax, the maximum number of repetitions for NPDCCH Type 2 CSS. - Depending on R, "DCI subframe repetition number" indicated in DCI format N1 for Msg2 scheduling. Depending on Rdecoded, based on the number of repetitions for NPDCCH scheduling Msg2 where UE decodes successfully. RAN1#96 For further study in future meetings: For channel quality report in Msg3 on non-anchor access, RAN1 identified the following scenarios in which UE may monitor a carrier that is different from the carrier in which the UE receives Msg2 (i.e. RAR): - Idle mode - Non-EDT random access procedure - EDT procedure - Connected mode - NPDCCH ordered NPRACH procedure - Random access procedure to apply for UL-SCH resource Agreement For the measurement on carrier(s) other than the one UE is receiving RAR for non-anchor access, if supported, RAN1 to select one or more among the following candidates: - Paging carrier - Anchor carrier - Carrier(s) configured by CarrierConfigDedicated-NB in connected mode - Other carriers configured by network with implicit or explicit signaling The following issues are identified for RAN1 further study - Whether amount of time or gap is needed for the measurement - Which carrier(s) UE reported if more than one carrier is measured - What kind of NRS UE can use for the measurement and whether needs indication Agreement In case of 2 bits are used for a non-anchor carrier, one of 3 candidate values can be reported in Msg3 - Which depends on Rmax, the maximum number of repetitions for NPDCCH Type 2 CSS. For further study in future meetings: If supported, RAN1 further discuss how to derive the candidate values reported in Msg3 if no NPDCCH Type2 CSS on the carrier RAN1#96bis Agreement For channel quality report in connected mode, the channel quality definition is denoted by the number of repetitions that the UE needs to decode hypothetical NPDCCH with BLER of 1%. Agreement For channel quality report in connected mode other than Msg3, UE performs measurement on the carrier it is assigned to monitor in USS for NPDCCH and the associated NPDSCH. - FFS other carrier(s) Agreement For channel quality report in Msg3 for non-anchor access, RAN1 send LS to RAN2 requesting the following information: - Number(s) of available bits for the quality reporting in Msg3 - Whether the quality report in Msg3 is transmitted in RRC or MAC CE Send LS to RAN2 and CC RAN4. Ask, from RAN2, perspective, whether the above might be different for different cases Agreement For channel quality measurement in Msg3 for non-anchor access, the maximum number of carriers reported in Msg3 is no larger than 2. - FFS detail of the maximum number (1 or 2) Agreement eNB can enable/disable channel quality report in Msg3 for non-anchor access via SIB - FFS details - FFS whether the field in Rel-14 for anchor access is reused RAN1#97 Agreement For channel quality measurement in Msg3 for non-anchor access, the maximum number of carriers reported in Msg3 is 1. Agreement For channel quality report in Msg3 on non-anchor access in idle mode, channel quality measurement on the carrier UE monitors to receive Msg2 (i.e. RAR) is supported. - FFS: Support of network indication of which carrier to use to report Msg3 (e.g. explicit signaling in SI) Agreement For channel quality report in connected mode other than Msg3, RAN1 assumes the following aspects are decided by RAN2 and RAN4. - number of candidate values for channel quality reports - mapping value of reported channel quality - report in RRC message or MAC CE LS to RAN2 and RAN4 to convey this agreement is endorsed in R1-1907624 (Zhe, Huawei) RAN1#98 Conclusion For channel quality report in Msg3 on non-anchor access in idle mode, RAN1 does not further discuss any other carrier than the one UE monitors to receive Msg2 (i.e. RAR) in Rel-16. Conclusion No consensus on the support of physical layer triggering for channel quality report in connected mode (other than Msg3) in Rel-16

## RAN2 agreements

RAN2#103bis agreements: None. RAN2#104 agreements: Re-use the code points defined in Rel-14. Study the impact of re-using the Rel-14 RRC reporting mechanism and consider whether a MAC mechanism should be used instead. RAN2 further study how to support the use case of enabling measurements in non-anchor carrier while reducing measurement on anchor carrier. RAN2#105 agreements: None. RAN2#105bis agreements: - Channel Quality Reporting: Existing Msg3 signalling is used for reporting downlink channel quality when UE makes access on non-anchor carrier. Existing SIB2 signalling is used to enable UE to report downlink channel quality on non-anchor carrier. The DL channel quality reported in connected mode corresponds to the carrier used for the unicast transmission (i.e. configured by MSG4 or by a subsequent reconfiguration procedure). eNB enables the reporting of the DL Channel quality in connected mode. - FFS whether dedicated or broadcast signaling is used for enabling. Periodic reporting or on-demand reporting are not supported. DL channel quality reported in connected mode is optional for the UE. Working assumption that the same code points as Msg3 reporting is used. FFS whether MAC or RRC is used for reporting. R2-1905263 LS on non-anchor carrier CQI reporting in MSG3 Qualcomm Incorporated LS out Rel-16 NB_IOTenh3-Core To:RAN1 Cc: RAN4 - Non-anchor carrier measurements for RRM: If RAN4 confirms feasibility, and NRS is present in non-anchor carrier, RAN2 can make use of that for RRM measurements in non-anchor carrier. R2-1905264 LS on non-anchor carrier idle mode measurements for RRM Qualcomm Incorporated LS out Rel-16 NB_IOTenh3-Core To:RAN4 Cc: RAN1 RAN2#106 agreements: Channel Quality Reporting: The DL channel quality of the configured carrier in RRC connected mode is reported by MAC CE. Use the same LCID as agreed for eMTC. We need to wait for RAN4 before deciding when to report. FFS whether broadcast or dedicated signalling is used for NW to enable the reporting. RAN2#107 agreements: Agreements: - Quality report in connected mode is transmitted on demand. - No implicit trigger is supported. - The trigger command is defined as a MAC CE with empty payload and one of the reserved LCID is used. - No feature enable indication is defined in RRC signalling. - UE capability signalling for quality reporting in connected mode is defined. Agreements: - RAN2 confirm that DL Channel quality in MSG3 applies to initial access, i.e. when MAC MSG3 contain a CCCH SDU, and that the report corresponds to the carrier where the UE performs the random access procedure. - From RAN2 point of view, there is no need for a physical layer trigger for DL channel quality in connected mode. - From RAN2 point of view, there is no need for DL channel quality reporting (for RAR or unicast carrier) in MSG3 in connected mode. - CCCH SDU in the MSG3 buffer needs to be updated after receiving RAR - Support of DL channel quality in MSG3 for non-anchor carrier is optional without capability reporting and is a separate capability from support of DL channel quality in MSG3 for the anchor carrier. - Use the codepoint/index of “10001” for the MAC CE DL channel quality report. - Introduce a one-byte MAC CE DL channel quality report to support a maximum of 8 bits report. - FFS: The new MAC CE DL channel quality report has the next priority after MAC CE for BSR. - When the measurement is available, the MAC CE DL channel quality report is sent at the first opportunity according to the MAC logical channel prioritisation rules. - Support of DL channel quality in MSG3 for non-anchor carrier and support of DL channel quality in connected mode also apply when the UE is accessing via 5GC.