3GPP TSG-RAN WG4 Meeting #92bis	R4-1911780

Chongqing, China, 14 – 18 October 2019

Agenda Item:	5.2.5

Source:	Ericsson

Title:	Simulation results on NB-IoT NPRACH demodulation performance for TDD

Document for:	Discussion

# 1	Introduction

Addressing the issue on the huge TDD NPRACH performance gap between companies, RAN4 agreed with the way forward [1], where it asks the interested companies to provide the simulation results in RAN4#92 to identify the reason of performance gap. Based on the simulation results, RAN4#92bis also agreed with the way forward:

Agreement: Companies are encouraged to provide the simulation results for alignment and with impairment according to the agreed simulation assumptions in the next meeting to target at concluding this topic. - Companies should provide the information about the simulation length in terms of number of detections.

This contribution provides our simulation results.

# 2	Discussion

## 2.1	Simulation parameters

Table 1 and Table 2 show the simulation parameters specified in TS36.104.

Table 1	Test parameters for TDD NPRACH.

| Parameter | Value |
| --- | --- |
| Narrowband physical layer cell identity | 0 |
| Initial subcarrier index | 0 |
| Uplink-downlink configuration for TDD | 1 |
| Special subframe configuration for TDD | 7 |

Table 2	Simulation parameters for TDD NPRCH demodulation requirements.

| Number of TX antennas | Number of RX antennas | Repetition number | Propagation conditions and correlation matrix (Annex B) | Frequency offset | SNR[dB] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Preamble format 0 | Preamble format 1 | Preamble format 0-a | Preamble format 1-a |
| 1 | 2 | 8 | AWGN | 0 |  |  |  |  |
|  |  |  | EPA1 Low | 200 Hz |  |  |  |  |
|  |  | 32 | AWGN | 0 |  |  |  |  |
|  |  |  | EPA1 Low | 200 Hz |  |  |  |  |

## 2.2	Simulation results

Table 3 shows our alignment simulation result, where we show the SNR of the probability of preamble detection success rate of 99% or more with the false alarm probability < 0.1% and timing estimation error of the strongest path is within 3.6546us.

Table 3	Alignment simulation results of NPRACH for TDD formats.

| Number of TX antennas | Number of RX antennas | Repetition number | Propagation conditions and correlation matrix (Annex B) | Frequency offset | SNR[dB] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Preamble format 0 | Preamble format 1 | Preamble format 0-a | Preamble format 1-a |
| 1 | 2 | 8 | AWGN | 0 | -0.6 | -1.0 | To be updated | To be updated |
|  |  |  | EPA1 Low | 200 Hz | To be updated | To be updated | To be updated | To be updated |
|  |  | 32 | AWGN | 0 | -5.3 | -5.5 | To be updated | To be updated |
|  |  |  | EPA1 Low | 200 Hz | To be updated | To be updated | To be updated | To be updated |

Note we ran 1000 NPRACH preamble transmissions to derive SNR values above.

# 3	References

- R4-1907388, “Way forward on Rel-15 NB-IOT NPRACH demodulation requirements”, Ericsson, Huawei, Nokia.