3GPP TSG-RAN WG4 Meeting#92bis                	R4-1911786

Chongqing, China, October 14th – 18th 2019

Source:	Nokia, Nokia Shanghai Bell

Title:	Clarification on MRTD and MTTD requirements in LTE NR dual connectivity

Agenda item:	6.10.6.2

Document for:	Discussion

Introduction

RAN4 has specified the MRTD and MTTD for E-UTRA NR dual connectivity, and related requirements are defined in section 7.5 and 7.6 of 38.133 [1]. However, the requirements are not very clear for inter-band EN-DC and inter-band NE-DC.

In this paper, we will provide our views on synchronous or asynchronous requirements in MRTD and MTTD requirements.

Discussion

In 38.133 [1] section 7.5 and section 7.6, the requirements for MTTD and MRTD are defined for

- Inter-band synchronous EN-DC

- Inter-band asynchronous EN-DC

- Intra-band synchronous EN-DC

- Intra-band asynchronous EN-DC

The UE capability in supporting above EN-DC type are agreed in [2], and the clarification on EN-DC deployment scenarios are agreed in [3] as below:

For inter-band EN-DC, RAN2 and RAN4 has common agreement that all UEs support asynchronous DC between LTE and NR for inter-band LTE-NR in Rel-15, and no UE capability signaling is needed for this.

This information is stated in [2], and clarified in [3]:

- All UEs support asynchronous DC between LTE and NR for inter-band LTE-NR in Rel-15. - Asynchronous here refers to subframe/slot timing boundary alignment

Observation 1: RAN2 and RAN4 have clear agreement that “for inter-band EN-DC, all UEs support asynchronous DC between LTE and NR for inter-band LTE-NR in Rel-15”, and no UE capability signaling is needed for this. Proposal 2: RAN4 reconfirm the agreement that “for inter-band EN-DC, all UEs support asynchronous DC between LTE and NR for inter-band LTE-NR in Rel-15”. Proposal 3: No need to introduce UE capability signaling to discriminate UE support of “synchronous EN-DC only” or “asynchronous EN-DC (i.e., support both synchronous and asynchronous EN-DC)” for inter-band TDD-FDD and inter-band TDD-TDD EN-DC combinations.

In 38.133 [1], MTTD requirements for inter-band EN-DC defined as blow:

### 7.5.2 Minimum Requirements for inter-band EN-DC The UE shall be capable of handling a maximum uplink transmission timing difference between E-UTRA PCell and PSCell as shown in Table 7.5.2-1. Table 7.5.2-1 Maximum uplink transmission timing difference requirement for asynchronous EN-DC Sub-carrier spacing in E-UTRA PCell (kHz); UL Sub-carrier spacing for data in PSCell (kHz); Maximum uplink transmission timing difference (µs). 15; 15; 500. 15; 30; 250. 15; 60; 125. 15; 120Note1; 62.5. NOTE 1: For E-UTRA FDD-NR FDD and E-UTRA TDD-NR TDD intra-band EN-DC, for which the requirement is defined in clause 7.5.3 and this Table 7.5.2-1 is also applicable, the scenario with 120kHz PSCell does not exist.. The UE shall be capable of handling a maximum uplink transmission timing difference between E-UTRA PCell and PSCell as shown in Table 7.5.2-2. The requirements for synchronous EN-DC are applicable for E-UTRA TDD-NR TDD, E-UTRA FDD-NR FDD, E-UTRA TDD-NR FDD and E-UTRA FDD-NR TDD inter-band EN-DC. Table 7.5.2-2 Maximum uplink transmission timing difference requirement for inter-band synchronous EN-DC Sub-carrier spacing in E-UTRA PCell (kHz); UL Sub-carrier spacing for data in PSCell (kHz); Maximum uplink transmission timing difference (µs). 15; 15; 35.21. 15; 30; 35.21. 15; 60; 35.21. 15; 120; 35.21. NOTE 1: Void.

Looking at the first sentence in this requirement, it is said the MTTD requirement is showed in Table 7.5.2-1, but Table 7.5.2-1 is for asynchronous inter-band EN-DC. And then specified the requirement for synchronous inter-band EN-DC in Table 7.5.2-2. This requirement may cause a little confusing for the others outside RAN4 who do not know the agreement that UE is mandatory to support asynchronous inter-band EN-DC for all cases of FDD-FDD, TDD-TDD and TDD-FDD.

Add Clarification in specification to indicate that for inter-band, UE is mandatory to support async EN-DC and async EN-DC for all cases of FDD-FDD, TDD-TDD and TDD-FDD

This proposal will be captured in CR [4]

Conclusion

- In this paper, we provide our views on synchronous or asynchronous requirements in MRTD and MTTD requirements, and have the proposal as below:

- Add Clarification in specification to indicate that for inter-band, UE is mandatory to support async EN-DC for all cases of FDD-FDD, TDD-TDD and TDD-FDD

References

- 38.133, V15.7.0

- R4-1711964, WF on synchronous and asynchronous definitions for LTE-NR DC in Rel-15, Ericsson, MediaTek

- R4-1809935, Clarification on Sync/Async EN-DC scenarios, Samsung

- R4-1911787, CR on MRTD and MTTD requirements, Nokia, Nokia Shanghai Bell

# A