3GPP TSG-WG RAN4 Meeting #92bis	R4-1911806

# Chongqing, Chia, 14 – 18 October, 2019

Source: 	Orange

Title: 					      TP to TR 36.716-02-01: CA_7-7-20

Agenda Item: 			7.2.3

Document for:	Approval

# Introduction

This TP introduces the following LTE inter-band CA for 2 bands DL with 1 band UL for the CA bands CA_7-7-20.

- CA_7A-7A-20A-BCS0

**************************** Start of changes ********************************************

## 5.X	CA_7-7-20

### 5.X.1	Channel bandwidths per operating band for CA

Table 5.X.1-1: Supported E-UTRA bandwidths per CA configuration for 2-band DL inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| CA_7A-7A-20A | 7 | See CA_7A-7A Bandwidth Combination Set 3 in Table 5.6A.1-3 |  |  |  |  |  | 60 | 0 |
|  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |

### 5.X.2	∆TIB and ∆RIB values

The ∆TIB and ∆RIB values of CA_7-7-20 is proposed to be the same as CA_7-20 already specified in TS 36.101.

Table 5.X.2-1: ΔTIB,c for 3DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_7-7-20 | 7 | 0.3 |
|  | 20 | 0.3 |

Table 5.X.2-2: ΔRIB,c for 3DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_7-7-20 | 7 | 0 |
|  | 20 | 0 |

### 5.X.3	REFSENS requirements

There is no REFSENS exception for CA_7A-7A-20A.

**************************** End of changes ********************************************