TSG-RAN Working Group 4 (Radio) meeting #92-bis	R4-1911828

Chongqing, China, 14th – 18th October 2019

Source:	Ericsson

Title:	TP to TR 38.820: PA indicative ACLR values in subclause 5.5.2

Agenda item:	10.4.6

Document for:	Approval

# Introduction

At last meeting in Ljubljana (RAN4#92), technical background information about essential PA characteristics was include in TR 38.820.

In this contribution we propose some conclusions based on measurement results and PA trends for indicative values for BS ACLR. At the end of this contribution a text proposal is attached for approval.

# Discussion

In [1], the complex dependencies between PA output power, ACLR and PAE was discussed in detail which is already captured in 7-24 GHz frequency range technical report. Some indicative and feasible values were also presented in [1]. Measurement results for three different commercially available power amplifiers around 10 GHz both with and without linearization for bandwidths of 200 MHz and 400 MHz was presented in [2] which confirmed the analysis and indicative values in [1].

The intention with this paper is to propose text to capture the indicative and feasible BS ACLR values for example frequencies for 7-24 GHz range.

# Conclusion

It is proposed that the attached text proposal is included in TR 38.820 [3].

# References

[1]	R4-1906180, “Further elaboration on 7-24 GHz PA dependencies for different example frequencies”, Ericsson

[2] 	R4-1908761, “Further elaboration on PA aspects for 10 GHz example frequency”, Ericsson

[3]	R4-1909280, “Draft TR 38.820”, Huawei

TEXT PROPOSAL:

### 5.5.2	Power amplifiers

In subclauses 5.4.1.1, 5.4.1.2, 5.4.2 and 5.4.3, the PA trends and saturated output power, PAE trends, AAS power scaling principles and PA dependencies respectively are described. The analysis resulted in indicative ACLR values without linearization of ~33 dB, ~31 dB and ~30 dB for example frequencies of 10, 15 and 20 GHz respectively. The indicative ACLR values for does not fully cater for the larger bandwidth likely for spectrum within 7-24 GHz frequency range.

For 10 GHz example frequency, due to propagation condition where AAS BS can possibly re-use the existing foot print and thus would benefit from higher PA power and high PAE. Three different PA:s based on GaN technology from 3 different vendors (vendor A, vendor B and vendor C) were investigated without and with linearization. The measurements were performed at ~8 GHz for practical reasons indicating that the measured results will be slightly worse for ~10 GHz.

The PA:s have been linearized through Digital Pre-Distortion (DPD). The block diagram below represents the test system employed.

Figure 5.5.2-1	test system simplified block diagram

The pre-distorter is based on a Generalized Memory Polynomial (GMP) which includes a number of memory terms and can be formulated as follows:

As the intended application is a multi-branch AAS radio for this frequency range, the maximum number of memory terms  has been set to 4 in order to limit the total amount of signal processing and computational complexity in the radio.

Due to possible size of spectrum at higher frequencies, the bandwidth of 200 MHz and 400 MHz corresponding to 2 or 4 carriers, each 100 MHz was assumed.

The measurement results of the important characteristics are summarized in table 5.5.2-1.

Table 5.5.2-1: Measurement results

|  | Power (dBm) 200MHz/400MHz | PAE (%) 200MHz/400MHz | ACLR (dB) Without linearization 200MHz/400MHz | ACLR (dB) With linearization 200MHz/400MHz |
| --- | --- | --- | --- | --- |
| Vendor A | 35 | 11.7 / 11.5 | 36.2 / 35.5 | 42.1 / 39.8 |
| Vendor B | 37 / 36 (1 dB back off to reach EVM) | 12.7 / 12.3 | 26.4/ 21.6 | 38.9 / 29.8 |
| Vendor C | 33 | 16.8 / 17 | 32.7 / 23.2 | 42.8 / 24.3 |

The measured PAE for different PA:s are quite aligned with the PAE analysis in sub-clause 5.4.1.2. Note that the PAE needs to be further improved in the AAS BS context given the need for fairly high power at these frequencies.

The result also shows the impact of bandwidth where the achievable ACLR is quite limited for 400 MHz bandwidth which is quite a reasonable bandwidth at these frequencies.

The empirical analysis above indicates that the achievable ACLR values even with linearization will worsen with bandwidth which need to be further considered in future work item for specific frequency band.

Considering the need for a fairly high power and efficiency for these frequency ranges and the empirical analysis and measurement results in this paper the indicative ACLR value for 10 GHz example frequency considering a reasonable linearization should be around 33 to 35 dB.

Thus, indicative values for feasible ACLR and reasonable achievable output power for different PA technologies for example frequencies can be summarized as in table 5.5.2-2.

Table 5.5.2-2: BS ACLR as function of frequency

| Example frequency | ACLR | Output power CMOS | Output power GaN |
| --- | --- | --- | --- |
| 10 GHz | ~33 – ~35 dB | ~24 dBm | ~36 dBm |
| 15 GHz | ~31 dB | ~21 dBm | ~33 dBm |
| 20 GHz | ~30 dB | ~19 dBm | ~31 dBm |