TSG-RAN Working Group 4 (Radio) meeting #92-bis	R4-1911844

Chongqing, China, 14th – 18th October 2019

Source:	Ericsson

Title:	TP to TR 38.820: Requirement approach in subclause 7.4.1 and subclause 7.4.2

Agenda item:	10.4.8

Document for:	Approval

# Introduction

At the last RAN4 meeting (RAN4#92 in Ljubljana) two tables to capture relevant information about RX and TX requirements was included in TR 38.820.

In this contribution we provide a text proposal with input relevant for the transmitter table in subclause 7.4.1 and the receiver table in subclause 7.4.2. The text proposal is attached at the end of this contribution.

# Discussion

As a part of SI objectives, the requirement approaches for different requirements should be studied. For some example frequencies the FR2 approach can be used while for others an approach based on FR1 or a combination of FR1 and FR2 could be useful. As all technical and trend analysis during SI was based on example frequencies, the discussion around requirement approach in this paper also relates to example frequencies.

In this paper considering the logical requirement sets for 10 GHz example frequency requirement sets based on -C, -H and -O requirement sets as well as using -O for example frequencies of 15 and 20 GHz, the requirement approaches, and some indicative values concluded during SI is presented.

# Conclusion

It is proposed that the attached text proposal is included in TR 38.820 [1].

# References

[1]	R4-1909280, “Draft TR 38.820“, Huawei

TEXT PROPOSAL:

************ Start of change *************

### 7.4.1	Transmitter requirements

#### 7.4.1.1	Tx requirements overview

Summary of the conducted and radiated Tx requirements specified in Rel-15 for the NR BS is presented in this subclause. More detailed elaboration on the motivation on selected requirements is provided in dedicated subclauses below. The analysis on requirement approach for different requirement is based on example frequencies since the specific bands in 7-24 GHz range will be handled in the future WI.

Table 7.4.1.1-1: Overview of conducted and radiated Tx requirements for NR BS in 7 – 24 GHz range

| Tx requirement |  |  | Conclusions from SI | Items to be completed in related WI |
| --- | --- | --- | --- | --- |
| Base station output power |  |  | Some indicative PA output power levels are discussed but BS output power depending on the array size is not investigated as no specific frequency band is handled. | To be completed in the WI when specific frequency bands are specified. |
| Output power dynamics |  | RE power control dynamic range | Not discussed in SI | For 10 GHz possible FR1 approach can be used while for 15 and 20 GHz, the FR2 approach should be adopted. |
|  |  | Total power dynamic range | Not discussed in SI | For 10 GHz possible FR1 approach can be used while for 15 and 20 GHz, the FR2 approach should be adopted. |
| Transmit ON/OFF power |  | Transmitter OFF power | Not discussed in SI | For 10 GHz possible FR1 approach can be used while for 15 and 20 GHz, the FR2 approach should be adopted. |
|  |  | Transmitter transient period | Not discussed in SI | For 10 GHz possible FR1 approach can be used while for 15 and 20 GHz, the FR2 approach should be adopted. This would depend on chosen SCS. |
| Transmitted signal quality |  | Frequency error | Not discussed in SI | For 10 GHz possible FR1 approach can be used while for 15 and 20 GHz, the FR2 approach should be adopted. This would depend on chosen SCS. |
|  |  | Modulation quality | Not discussed in SI but we foresee that same EVM requirement levels which is common between FR1 and FR2 should apply. Phase noise performance and parameterized phase noise model was discussed in SI. | For 10 GHz possible FR1 approach can be used while for 15 and 20 GHz, the FR2 approach should be adopted. |
|  |  | Time alignment error | Not discussed in SI. | For 10 GHz possible FR1 approach can be used while for 15 and 20 GHz, the FR2 approach should be adopted. |
| Occupied bandwidth |  |  | Not discussed in SI. | For 10 GHz possible FR1 approach can be used while for 15 and 20 GHz, the FR2 approach should be adopted. |
| ACLR |  |  | Indicative ACLR values of 33-35 dB, 31 dB and 30 dB for example frequencies of 10, 15 and 20 GHz was concluded. | The indicative ACLR values should be re-used during WI. |
| Operating band unwanted emissions |  |  | Discussed in SI but no conclusion around the OBUE domain and possible mask shape. Higher ΔfOBUE, than in FR1 needs to be considered, probably with an approach similar to FR2. Existing mask shapes could also be a starting point. | To be completed in the WI when specific frequency bands are specified. |
| Transmitter spurious emissions | General transmitter spurious emissions |  | Regulation defines Category A and B limits, but Category B limits for AAS BS operating in 6 to 24.25 GHz are presently not defined. A starting point for those limits could be the present limits above 24.25 GHz, but the work defining them will take place in ECC. Feasibility of filters based on LTCC technology was discussed during WI where based on attenuation assumptions, insertion loss and needed guard has been investigated. | To be completed in the WI. |
|  | Protection of the BS receiver of own or different BS |  | No discussion during SI. | To be completed in the WI. |
|  | Additional spurious emissions requirements |  | No discussion during SI. Feasibility of filters based on LTCC technology was discussed during WI where based on attenuation assumptions, insertion loss and needed guard has been investigated. | To be completed in the WI. |
|  | Co-location with other base stations |  | Discussed during SI but no consensus on need for all example frequencies nor what approach to adopt. Alternative solutions based on measurements of DUT based on field strength instead of using measurement antennas. | To be completed in the WI. |
| Transmitter intermodulation |  |  | No discussion during SI. | To be completed in the WI. |

************ End of change *************

************ Next changed subclause *************

### 7.4.2	Receiver requirements

#### 7.4.2.1	Rx requirements overview

Summary of the conducted and radiated Rx requirements specified in Rel-15 for the NR BS is presented in this subclause. More detailed elaboration on the motivation on selected requirements is provided in dedicated subclauses below. The analysis on requirement approach for different requirement is based on example frequencies since the specific bands in 7-24 GHz range will be handled in the future WI.

Table 7.4.2.1-1: Overview of conducted Rx requirements for NR BS in 7 – 24 GHz range

| Rx requirement |  | Conclusions from SI | Items to be completed in related WI |
| --- | --- | --- | --- |
| Reference sensitivity level |  | Indicative Noise figure values was concluded in the SI where for 10, 15 and 20 GHz example frequencies, the NF value of 7, 8 and 9 dB respectively. | Beside -C and -H for 10 GHz example frequency, the FR2 approach can be adopted for the 15 and 20 GHz example frequencies during WI. |
| Dynamic range |  | Not discussed within SI. Possibly 15 and 20 GHz example frequencies would not require Dynamic range requirements | To be concluded in WI. |
| In-band selectivity and blocking | ACS | Not discussed during SI. Beside the -C, the FR2 approach should be adopted. | To be concluded in WI when specific frequency bands are defined, and proper co-existence studies are performed. |
|  | In-band blocking | Not discussed during SI. Beside the -C, the FR2 approach should be adopted. | To be concluded in WI when specific frequency bands are defined, and proper co-existence studies are performed. |
| Out-of-band blocking | General out-of-band blocking | Discussed in SI where possible changes for transition point of 12.75 GHz might be needed when specific frequency bands during WI are defined. FR2 approach should be used for -O requirement sets | To be concluded in WI. FR2 approach should be used for -O requirement sets |
|  | Co-location | Discussed in SI but no conclusion | To be concluded in the WI |
| Receiver spurious emissions |  | Not discussed but possible FR1 for -C /-H and FR2 approached for -O can be used. | To be concluded in the WI |
| Receiver intermodulation |  | Discussed in SI but no conclusion | To be concluded in the WI |
| In-channel selectivity |  | Not discussed but possible FR1 for -C /-H and FR2 approached for -O can be used. | To be concluded in the WI |

************ End of change *************