3GPP TSG-RAN WG4 Meeting #92bis	 R4-1911850

Chongqing, China, 14-18 Oct, 2019

Source: 	Charter Communications, Cable Labs

Title: 	Further discussions on Emission Mask Considerations for NR-U DL single wideband carrier operation modes

Agenda item:	8.1.1.2

Document for:	Approval

Introduction

In RAN4 #92, we continued discussing emission mask considerations in the 5 GHZ band for NR-U wideband carrier operation modes. At the conclusion of the meeting, a WF [1] was approved outlining 9 alternatives that will insure fair co-existence between NR-U wideband operation and current incumbents in the 5 GHz band.  These alternatives are:

  - Alt. 1- Baseline Emission Mask based on the NR Spectrum Emission Mask (SEM) as given in section 6.5.2.2 of 38.101-1

  - Alt. 2 - ETSI EN 301 893 (v2.1.1) mask (Failed LBT sub-bands see slide 7)

  - Alt. 3 - IEEE 802.11ac (No requirements for failed LBT sub-bands)

  - Alt. 4 - IEEE 802.11ax

  - Alt. 5 - Modified ETSI mask (with -28 dB floor across failed LBT sub-bands)

  - Alt. 6 - Modified ETSI mask
with -25 dB floor across failed LBT sub-bands

  - Alt. 7 - Modified IEEE 802.11ac/ax
with -25 dB floor across failed LBT sub-bands

  - Alt. 8 – Modified ETSI/ IEEE 802.11ac/ax mask
with -25 dB floor across failed LBT sub-bands

  - Alt. 9 – Modified ETSI/ IEEE 802.11ac/ax mask
with -28 dB floor across failed LBT sub-bands

In this paper we are planning to propose the alternative that we feel best insures that the NR-U design enables fair coexistence between already deployed Wi-Fi generations and NR-U, between NR-U and LTE-LAA, and between different NR-U systems in the 5 GHz band.  NR-U should not impact already deployed Wi-Fi generations more than an additional Wi-Fi network of the same generation on the same carrier

Discussion

- In this discussion section, we will highlight our proposals for NR-U baseline emission mask and the cases were single or multiple punctured occur.  We will also describe our reasoning for such proposals.

  - NR-U – Baseline Spectral Emission Mask

Figure 1: General spectral emission mask

In Figure 1, the ETSI EN 301 893 (v2.1.1) and IEEE 802.11ac have been considered for the general out-of-carrier spectral emission mask. This mask addresses the scaling factor in the current ETSI EN 301 893 (v2.1.1) under wider band operations and limits the nominal bandwidth to -28 dbr to insure fair co-existence with Wi-Fi and other technologies in the 5 GHz band

Proposal 1: Consider the emission mask proposal in Figure 1 and the corresponding values for the interception points as the NR-U general out of carrier spectral emission mask.

2.2 NR-U Spectral Emission Mask with punctured channels

In addition to the baseline emission mask, we must consider the scenarios for the case of one punctured channel and multiple punctured channels.

Figure 2. Wide-band Spectral power mask for N=20 MHz (BW=80 MHz) with 2 and 1 punctured channels (RBW=1MHz).

Figure 2 represent the addition of 2 masks (with 2 and 1 punctured channels) for a total BW=80 MHz.

For the case of one punctured channel, the composite EN 301 893 mask (also the IEEE 802.11ac mask) level at the punctured area is -25 dBr level. For the case of two punctured channels, the composite EN 301 893 mask (also the IEEE 802.11ac mask) level in the punctured area is -28dbr

Proposal 2: We propose for the case of one punctured channel, the composite EN 301 893 mask (also the IEEE 802.11ac mask) level at the punctured area is -25 dBr level. For the case of two punctured channels, the composite EN 301 893 mask (also the IEEE 802.11ac mask) level in the punctured area is -28dbr

Figure 3. Wide-band Spectral power mask with multiple punctured channels at the side of a 80MHz channel (RBW=1MHz) [1].

Figure 3 represents an 80 MHz bandwidth mask with 1, 2 or 3 contiguous punctured channels at the side. For this case, the composite EN 301 893 mask (also the IEEE 802.11ac mask) level at the punctured area is -28 dbr.

Proposal 3: We propose the level at the punctured area to be -28 dbr for the case of 1, 2 or 3 punctured channels at the side of a 80 MHz composite EN 301 893 mask (also the IEEE 802.11ac mask). This is alternative 5 of WF [1].

Conclusions

- In this paper, we have made three proposals to mitigate the impact of NR-U wider bandwidth operations enabling fair coexistence between already deployed Wi-Fi generations and NR-U, between NR-U and LTE-LAA, and between different NR-U systems in the 5 GHz band. These proposals addresses the general spectrum emission mask and the cases where single, two contiguous and three contiguous punctured channels occurred

Proposal 1: Consider the emission mask proposal in Figure 1 and the corresponding values for the interception points as the NR-U general out of carrier spectral emission mask.

Proposal 2: We propose for the case of one punctured channel, the composite EN 301 893 mask (also the IEEE 802.11ac mask) level at the punctured area is -25 dBr level. For the case of two punctured channels, the composite EN 301 893 mask (also the IEEE 802.11ac mask) level in the punctured area is -28dbr

Proposal 3: We propose the level at the punctured area to be -28 dbr for the case of 1, 2 or 3 punctured channels at the side of a 80 MHz composite EN 301 893 mask (also the IEEE 802.11ac mask). This is alternative 5 of WF [1].

# References

[1] R4-1910535 WF on NR-U emission requirement Source: Nokia, Nokia Shanghai Bell