3GPP TSG-RAN4 Meeting #92bis	R4-1911876

# Chong Qing, China, 14 - 18 oct. 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the TCI state switching (section 8.10) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-09-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - It is agreed in RAN4#91 (R4-1907305 ad hoc minutes); Agreement: - The known TCI state is defined by the time from the latest measurement resource for the TCI to when TCI activation command is received, - If the period is less than X, the TCI state is considered as known. Otherwise the TCI state is considered as unknown. - Before when TCI activation command is received - L1 RSRP measurement for the RS to which the target TCI is QCLed with should be reported The QCLed condition shall be specified in the specification. - Normalize the unit |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The reported RS for the L1-RSRP is the reported RS for the L1-RSRP measurement is the RS in the target TCI state or to which the RS in target TCI state is QCLed. - Correct the unit |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification is not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

8.10	Active TCI state switching delay

8.10.1	Introduction

The requirements in this section apply for a UE configured with one or more TCI state configurations on serving cell in MR-DC or standalone NR. UE shall complete the switch of active TCI state within the delay defined in this section.

8.10.2	Known conditions for TCI state

The TCI state is known if it has been meeting the following conditions:

-	TCI state switch command is received within [1280] ms of the last transmission of the RS resource for beam reporting or measurement for the target TCI state

-	The UE has sent at least 1 L1-RSRP measurement report for the target TCI state, where the reported RS for the L1-RSRP measurement is the RS in the target TCI state or to which the RS in target TCI state is QCLed.

-	The TCI state remain detectable during the TCI state switching period

-	The SSB associated with the TCI state remain detectable during the TCI switching period

-	SNR of the TCI state is ≥ -3dB

Otherwise, the TCI state is unknown.

8.10.3	MAC-CE based TCI state switch delay

If the target TCI state is known, upon receiving PDSCH carrying MAC-CE activation command at slot n, UE shall be able to receive PDCCH with target TCI state of the serving cell on which TCI state switch occurs no later than at slot n+ (THARQ +3 ms +TOk*(Tfirst-SSB + TSSB-proc)) / NR slot length. The UE shall be able to receive on the old TCI state until slot n+ (THARQ +3 ms +TOk*(Tfirst-SSB)) / NR slot length.

Where THARQ is the timing between DL data transmission and acknowledgement as specified in TS 38.321 [7];

Tfirst-SSB is time to first SSB transmission after MAC CE command is decoded by the UE;

TSSB-proc = 2 ms;

TOk = 1 if target TCI state is not in the active TCI state list for PDSCH, 0 otherwise.

If the target TCI state is unknown, upon receiving PDSCH carrying MAC-CE activation command at slot n, UE shall be able to receive PDCCH with target TCI state of the serving cell on which TCI state switch occurs no later than at slot n+ (THARQ +3 ms + TL1-RSRP +TOuk*(Tfirst-SSB+ TSSB-proc)) / NR slot length. The UE shall be able to receive on the old TCI state until slot n+ (THARQ +3 ms+ TL1-RSRP +TOuk*(Tfirst-SSB)) / NR slot length.

Where T L1-RSRP is the time for L1-RSRP measurement for Rx beam refinement, defined as

-	TL1-RSPR_Measurement_Period_SSB for SSB as specified in clause 9.5.4.1,

-	with the assumption of M=1

-	with TReport = 0

-	TL1-RSRP_Measurement_Period_CSI-RS for CSI-RS as specified in clause 9.5.4.2

-	with the assumption of M=1 for periodic CSI-RS

-	for aperiodic CSI-RS if number of resources in resource set at least equal to MaxNumberRxBeam

-	with TReport = 0

TOuk = 1 for CSI-RS based L1-RSRP measurement, and 0 for SSB based L1-RSRP measurement

TL1-RSRP_Measurement_Period_SSB = 0 for SSB in FR2 and TL1-RSRP_Measurement_Period_CSI-RS = 0 for CSI-RS in FR2, provided that the TCI state switching involves QCL-TypeA, QCL-TypeB or CQL-TypeC only.

During MAC CE based TCI state switch the UE is allowed an interruption due to one shot timing adjustment on the serving or any activated serving cells as defined in clause 8.2.

8.10.4	DCI based TCI switch delay

If the target TCI state is known, when a UE is configured with the higher layer parameter tci-PresentInDCI which is set as 'enabled' for the CORESET scheduling the PDSCH at slot n, UE shall be able to receive PDSCH or transmit PUSCH with target TCI state of the serving cell on which TCI state switch occurs no later than at slot n+timeDurationForQCL, where, timeDurationForQCL is the time required by the UE to perform PDCCH reception and applying spatial QCL information received in DCI for PDSCH processing as described in TS 38.214 [26], the value of timeDurationForQCL is defined in TS 38.306 [14].

The known condition for TCI state defined in clause 8.10.2 is applied.

During DCI based TCI state switch the UE is allowed an interruption due to one shot timing adjustment on the serving or any activated serving cells as defined in clause 8.2.

8.10.5	RRC based TCI state delay

If the target TCI state is known, upon receiving PDSCH carrying RRC activation command at slot n, UE shall be able to receive PDCCH with target TCI state of the serving cell on which TCI state switch occurs no later than at slot n+ (TRRC_processing +TOk*(Tfirst-SSB + TSSB-proc)) / NR slot length. Where TRRC_processing is the RRC processing delay, Tfirst-SSB, TSSB-proc and TOk are as defined in clause 8.10.3. The UE is not required to receive PDCCH/PDSCH or transmit PUCCH/PUSCH until the end of switching period.

If the target TCI state is unknown, upon receiving PDSCH carrying RRC activation command at slot n, UE shall be able to receive PDCCH with target TCI state of the serving cell on which TCI state switch occurs no later than at slot n+ (TRRC_processing +TL1-RSRP +TOuk*(Tfirst-SSB + TSSB-proc)) / NR slot length . Where TRRC_processing is the RRC processing delay, Tfirst-SSB and TOuk are as defined in clause 8.10.3. The UE is not required to receive PDCCH/PDSCH or transmit PUCCH/PUSCH until the end of switching period.

During RRC based TCI state switch the UE is allowed an interruption due to one shot timing adjustment on the serving or any activated serving cells as defined in clause 8.2.

8.10.6	Active TCI state list update delay

If the target TCI state is known, upon receiving PDSCH carrying MAC-CE active TCI state list update at slot n, UE shall be able to receive PDCCH to schedule PDSCH with the new target TCI state no later than slot n+ THARQ +3ms +TOk*(Tfirst-SSB + TSSB-proc). Where THARQ, Tfirst-SSB, TSSB-proc and TOk are as defined in clause 8.10.3.

<End of Change 1>