3GPP TSG-RAN4 Meeting #92bis	R4-1911922

# Chongqing, China, 14 - 18 October 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 6658 | rev | - | Current version: | 15.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on inter-frequency RSTD measurement requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-09-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed e?planations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | RAN4 have agreed that Inter-frequency E-UTRA RSTD measurement shall be taken into account during determing CSSFwithin_gap,i in previous discussion. However, requirements for inter-frequency RSTD measurementis under EN-DC mode is still missing in 36.133. So technically speaking UE doesn’t know how to use CSSF when determing the measurement and report delay for inter-frequency RSTD measurements. To fix this problem, we purpose to reuse requirements for inter-frequency RSTD measurements under LTE SA (section 8.1.2.6) as the requirement for inter-frequency RSTD measurements under EN-DC except taking CSSF into consideration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new subsection is added in section 8.17 (measurements for E-UTRA-NR dual connectivity) to contain requirements for inter-frequency RSTD measurements under EN-DC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE doesn’t know how to use CSSF when determing the measurement and report delay for inter-frequency RSTD measurements under EN-DC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.17.15 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of first modified section>

8.17.15	E-UTRAN Inter-Frequency RSTD Measurements when Configured with E-UTRA-NR Dual Connectivity

All inter-frequency RSTD measurement requirements specified in Sections 8.17.15.1-8.17.15.4 shall apply, provided that

-	the UE is capable of inter-frequency RSTD measurements for OTDOA [24], and

-	the measurement gap pattern ID # 0 specified in TS 38.133 [50] clause 9.1.2

All inter-frequency RSTD measurement requirements specified in Sections 8.17.15.1-8.17.15.4 shall apply without DRX as well as for any DRX or eDRX_CONN cycles specified in TS 36.331 [2].

The UE capable of SRS carrier based switching when configured to perform SRS transmission and/or RACH transmission over one or more SCells without PUSCH shall meet the requirements defined in Section 8.17.15 provided the following condition is met:

-	all positioning subframes indicated in the OTDOA assistance data and specified in Section 9.1.10 are available for RSTD measurements at the UE in the measured and reference cells.

The requirements in this section shall also appy, when the UE is configured to perform SRS carrier based switching and using measurement gaps.

When using CRS, in addition to PRS, is enabled in the OTDOA assistance data, it is up to UE implementation whether to use or not the CRS for RSTD measurements, but in either case the RSTD measurements reported by the UE shall meet the requirements specified in this section.

If network-based CRS interference mitigation is enabled in a cell for which the UE performs RSTD measurements, the UE capable of supporting network-based CRS interference mitigation shall perform RSTD measurements and meet all the requirements in this section, provided the CRS are available within at least the PRS bandwidth in the subframes with PRS during all positioning occasions within the RSTD measurement period.

8.17.15.1	E-UTRAN FDD-FDD Inter-Frequency RSTD Measurements when Configured with E-UTRA-NR Dual Connectivity

When the physical layer cell identities of neighbour cells together with the OTDOA assistance data are provided, the UE shall be able to detect and measure inter-frequency RSTD, specified in TS 36.214 [4], for at least n=16 cells, including the reference cell, within TRSTD InterFreqFDD, EN-DC ms as given below:

TRSTD InterFreqFDD, EN-DC=TPRS∙M-1+Δ  ms,

Where

TRSTD InterFreqFDD, EN-DC is the total time for detecting and measuring at least n cells,

TPRS is the the largest value of the cell-specific positioning subframe configuration period, defined in TS 36.211 [16], among the measured n cells including the reference cell,

M is the number of PRS positioning occasions as defined in Table 8.17.15.1-1, where a PRS positioning occasion is as defined in clause 8.1.2.5.1,

CSSFinterFreq is the carrier-specific scaling factor for the inter-frequency RSTD measurements and is determined according to CSSFwithin_gap,i defined in TS 38.133 [50] clause 9.1.5.2,

Δ=160∙nM ms is the measurement time for a single PRS positioning occasion which includes the sampling time and the processing time, and

The n cells are distributed on up to two carrier frequencies including a serving carrier frequency and one inter-frequency carrier.

Table 8.17.15.1-1: Number of PRS positioning occasions within TRSTD InterFreqFDD, EN-DC

| Positioning subframe configuration period TPRS | Number of PRS positioning occasions M |  |
| --- | --- | --- |
|  | f2 Note1 | f1 and f2 Note2 |
| 160 ms | 16 × CSSFinterFreq | 32 ×CSSFinterFreq |
| >160 ms | 8 ×CSSFinterFreq | 16 ×CSSFinterFreq |
| Note 1: When inter-frequency RSTD measurements are performed over the reference cell and neighbour cells, which belong to the FDD inter-frequency carrier frequency f2. Note 2: When inter-frequency RSTD measurements are performed over the reference cell and the neighbour cells, which belong to the serving FDD carrier frequency f1 and the FDD inter-frequency carrier frequency f2 respectively. |  |  |

The UE physical layer shall be capable of reporting RSTD for the reference cell and all the neighbor cells i out of at least (n-1) neighbor cells within TRSTD InterFreqFDD, E-UTRAN provided:

PRS EsIotref≥-6  dB for all Frequency Bands for the reference cell,

PRS EsIoti≥-13 dB for all Frequency Bands for neighbour cell i,

PRS EsIotref and PRS EsIoti conditions apply for all subframes of at least L=M2 PRS positioning occasions,

PRP 1,2|dBm according to Annex B.2.6 for a corresponding Band

PRS EsIot is as defined in Clause 8.1.2.5.1.

The time TRSTD InterFreqFDD, EN-DC starts from the first subframe of the PRS positioning occasion closest in time after both the OTDOA-RequestLocationInformation message and the OTDOA assistance data in the OTDOA-ProvideAssistanceData message as specified in TS 36.355 [24], are delivered to the physical layer of the UE.

If the inter-frequency handover occurs while inter-frequency RSTD measurements are being performed, and the inter-frequency carrier on which RSTD is measured becomes the new serving carrier frequency after the inter-frequency handover, the UE shall complete the ongoing OTDOA measurement session. The UE shall also meet the inter-frequency OTDOA measurement and accuracy requirements. However in this case the RSTD measurement period (TRSTD InterFreqFDD, EN-DC,HO) shall be according to the following expression:

TRSTD InterFreqFDD, EN-DC,HO=TRSTD InterFreqFDD, EN-DC+K×TPRS+THO  ms,

where:

K is the number of times the inter-frequency handover occurs during TRSTD InterFreqFDD, E-UTRAN,HO,

THO is the time during which the inter-frequency RSTD measurement may not be possible due to inter-frequency handover; it can be up to 45 ms.

The RSTD measurement accuracy for all measured neighbor cells i shall be fulfilled according to the accuracy as specified in the sub-clause 9.1.10.2.

Furthermore, due to the inter-frequency handover the UE shall meet the RSTD measurement accuracy for a PRS bandwidth which is not larger than the minimum channel bandwidth of those PCells on whose carriers RSTD measurement is performed during the RSTD measurement period.

8.17.15.1.1	RSTD Measurement Reporting Delay

This requirement assumes that the measurement report is not delayed by other LPP signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PUSCH with subframe or slot or subslot duration. This measurement reporting delay excludes any delay caused by no UL resources for UE to send the measurement report. When the UE is configured to perform SRS carrier based switching, an additional delay can be expected.

8.17.15.2	E-UTRAN TDD-FDD Inter-Frequency RSTD Measurements when Configured with E-UTRA-NR Dual Connectivity

When the physical layer cell identities of neighbour cells together with the OTDOA assistance data are provided, the UE shall be able to detect and measure inter-frequency RSTD, specified in TS 36.214 [4], for at least n=16 cells, including the reference cell, within TRSTD InterFreqTDDFDD, EN-DC ms as given below:

TRSTD InterFreqTDDFDD, EN-DC=TPRS∙M-1+Δ  ms,

where

TRSTD InterFreqTDDFDD, EN-DC is the total time for detecting and measuring at least n cells,

TPRS is the largest value of the cell-specific positioning subframe configuration period, defined in TS 36.211 [16], among the measured n cells including the reference cell,

M is the number of PRS positioning occasions as defined in Table 8.17.15.2-1, where a PRS positioning occasion is as defined in clause 8.1.2.5.1,

CSSFinterFreq is the carrier-specific scaling factor for the inter-frequency RSTD measurements and is determined according to CSSFwithin_gap,i defined in TS 38.133 [50] clause 9.1.5.2,

Δ=160∙nM ms is the measurement time for a single PRS positioning occasion which includes the sampling time and the processing time, and the n cells are distributed on up to two carrier frequencies including a serving carrier frequency and one inter-frequency carrier.

Table 8.17.15.2-1: Number of PRS positioning occasions within TRSTD InterFreqTDDFDD, EN-DC

| Positioning subframe configuration period TPRS | Number of PRS positioning occasions M |  |
| --- | --- | --- |
|  | f2 Note1 | f1 and f2 Note2 |
| 160 ms | 16 × CSSFinterFreq | 32 × CSSFinterFreq |
| >160 ms | 8 × CSSFinterFreq | 16 × CSSFinterFreq |
| NOTE 1: When inter-frequency RSTD measurements are performed over the reference cell and neighbour cells, which belong to the FDD inter-frequency carrier frequency f2. NOTE 2: When inter-frequency RSTD measurements are performed over the reference cell and the neighbour cells, which belong to the serving TDD carrier frequency f1 and the FDD inter-frequency carrier frequency f2 respectively. |  |  |

The UE physical layer shall be capable of reporting RSTD for the reference cell and all the neighbor cells i out of at least (n-1) neighbor cells within TRSTD InterFreqTDDFDD, EN-DC, provided:

PRS EsIotref≥-6 dB for all Frequency Bands for the reference cell,

PRS EsIoti≥-13 dB for all Frequency Bands for neighbour cell i,

PRS EsIotref and PRS EsIoti conditions apply for all subframes of at least L=M2 PRS positioning occasions,

PRP 1,2|dBm according to Annex B.2.6 for a corresponding Band,

PRS EsIot is as defined in Clause 8.1.2.5.1.

The time TRSTD InterFreqTDDFDD, EN-DC starts from the first subframe of the PRS positioning occasion closest in time after both the OTDOA-RequestLocationInformation message and the OTDOA assistance data in the OTDOA-ProvideAssistanceData message as specified in TS 36.355 [24], are delivered to the physical layer of the UE.

If the inter-frequency handover occurs while inter-frequency RSTD measurements are being performed, and the inter-frequency carrier on which RSTD is measured becomes the new serving carrier frequency after the inter-frequency handover, the UE shall complete the ongoing OTDOA measurement session. The UE shall also meet the inter-frequency OTDOA measurement and accuracy requirements. However in this case the RSTD measurement period (TRSTD InterFreqTDDFDD, EN-DC, HO) shall be according to the following expression:

TRSTD InterFreqTDDFDD, EN-DC,HO=TRSTD InterFreqTDDFDD, EN-DC+K×TPRS+THO  ms,

Where:

K is the number of times the inter-frequency handover occurs duringTRSTD InterFreqTDDFDD, EN-DC,HO,

THO is the time during which the inter-frequency RSTD measurement may not be possible due to inter-frequency handover; it can be up to 45 ms.

The RSTD measurement accuracy for all measured neighbor cells i shall be fulfilled according to the accuracy as specified in the sub-clause 9.1.10.2.

Furthermore, due to the inter-frequency handover the UE shall meet the RSTD measurement accuracy for a PRS bandwidth which is not larger than the minimum channel bandwidth of those PCells on whose carriers RSTD measurement is performed during the RSTD measurement period.

The inter-frequency requirements in this clause (8.17.15.2) shall apply for all TDD special subframe configurations specified in TS 36.211 [16] and for the TDD uplink-downlink configurations as specified in Table 8.17.15.2-2.

Table 8.17.15.2-2: TDD uplink-downlink subframe configurations applicable for TDD-FDD inter-frequency requirements

| PRS Transmission Bandwidth [RB] | Applicable TDD uplink-downlink configurations |
| --- | --- |
| 6, 15 | 1, 2, 3, 4 and 5 |
| 25, 50, 75, 100 | 0, 1, 2, 3, 4, 5 and 6 |
| NOTE: Uplink-downlink configurations are specified in Table 4.2-2 in TS 36.211 [16]. |  |

8.17.15.2.1	RSTD Measurement Reporting Delay

This requirement assumes that that the measurement report is not delayed by other LPP signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PUSCH with subframe or slot or subslot duration. This measurement reporting delay excludes any delay caused by no UL resources for UE to send the measurement report. When the UE is configured to perform SRS carrier based switching, an additional delay can be expected.

8.17.15.3	E-UTRAN TDD-TDD Inter-Frequency RSTD Measurements when Configured with E-UTRA-NR Dual Connectivity

When the physical layer cell identities of neighbour cells together with the OTDOA assistance data are provided, the UE shall be able to detect and measure inter-frequency RSTD, specified in TS 36.214 [4], for at least n=16 cells, including the reference cell, within TRSTD InterFreqTDD, EN-DC ms as given below:

TRSTD InterFreqTDD, EN-DC=TPRS∙M-1+Δ  ms,

Where

TRSTD InterFreqTDD, EN-DC is the total time for detecting and measuring at least n cells,

TPRS is the largest value of the cell-specific positioning subframe configuration period, defined in TS 36.211 [16], among the measured n cells including the reference cell,

M is the number of PRS positioning occasions as defined in Table 8.17.15.1-1, where a PRS positioning occasion is as defined in clause 8.1.2.5.1,

CSSFinterFreq is the carrier-specific scaling factor for the inter-frequency RSTD measurements and is determined according to CSSFwithin_gap,i defined in TS 38.133 [50] clause 9.1.5.2,

Δ=160∙nM ms is the measurement time for a single PRS positioning occasion which includes the sampling time and the processing time, and

The n cells are distributed on up to two carrier frequencies including a serving carrier frequency and one inter-frequency carrier.

Table 8.17.15.3-1: Number of PRS positioning occasions within TRSTD InterFreqTDD, EN-DC

| Positioning subframe configuration period TPRS | Number of PRS positioning occasions M |  |
| --- | --- | --- |
|  | f2 Note1 | f1 and f2 Note2 |
| 160 ms | 16 × CSSFinterFreq | 32 × CSSFinterFreq |
| >160 ms | 8 × CSSFinterFreq | 16 × CSSFinterFreq |
| Note 1: When inter-frequency RSTD measurements are performed over the reference cell and neighbour cells, which belong to the TDD inter-frequency carrier frequency f2. Note 2: When inter-frequency RSTD measurements are performed over the reference cell and the neighbour cells, which belong to the serving TDD carrier frequency f1 and the TDD inter-frequency carrier frequency f2 respectively. |  |  |

The inter-frequency requirements in this clause (8.17.15.3) shall apply for all TDD special subframe configurations specified in TS 36.211 [16] and for the TDD uplink-downlink configurations as specified in Table 8.17.15.3-2.

Table 8.17.15.3-2: TDD uplink-downlink subframe configurations applicable for inter-frequency requirements

| PRS Transmission Bandwidth [RB] | Applicable TDD uplink-downlink configurations |
| --- | --- |
| 6, 15 | 3, 4 and 5 |
| 25 | 1, 2, 3, 4, 5 and 6 |
| 50, 75, 100 | 0, 1, 2, 3, 4, 5 and 6 |
| Note 1: Uplink-downlink configurations are specified in Table 4.2-2 in TS 36.211 [16]. Note2: For UEs capable of performing inter-frequency measurements without measurement gaps, TDD uplink-downlink subframe configurations as specified in Table 8.1.2.5.2-2 shall apply. |  |

The UE physical layer shall be capable of reporting RSTD for the reference cell and all the neighbor cells i out of at least (n-1) neighbor cells within TRSTD InterFreqTDD, EN-DC provided:

PRS EsIotref≥-6 dB for all Frequency Bands for the reference cell,

PRS EsIoti≥-13 dB for all Frequency Bands for neighbour cell i,

PRS EsIotref and PRS EsIoti conditions apply for all subframes of at least L=M2 PRS positioning occasions,

PRP 1,2|dBm according to Annex B.2.6 for a corresponding Band

PRS EsIot is as defined in Clause 8.1.2.5.1.

The time TRSTD InterFreqTDD, EN-DC starts from the first subframe of the PRS positioning occasion closest in time after both the OTDOA-RequestLocationInformation message and the OTDOA assistance data in the OTDOA-ProvideAssistanceData message as specified in TS 36.355 [24], are delivered to the physical layer of the UE.

If the inter-frequency handover occurs while inter-frequency RSTD measurements are being performed, and the inter-frequency carrier on which RSTD is measured becomes the new serving carrier frequency after the inter-frequency handover, the UE shall complete the ongoing OTDOA measurement session. The UE shall also meet the inter-frequency OTDOA measurement and accuracy requirements. However in this case the RSTD measurement period (TRSTD InterFreqTDD, EN-DC, HO) shall be according to the following expression:

TRSTD InterFreqTDD, EN-DC,HO=TRSTD InterFreqTDD, EN-DC+K×TPRS+THO  ms.

Where:

K is the number of times the inter-frequency handover occurs during TRSTD InterFreqTDD, EN-DC, HO,

THO is the time during which the inter-frequency RSTD measurement may not be possible due to inter-frequency handover; it can be up to 45 ms.

The RSTD measurement accuracy for all measured neighbor cells i shall be fulfilled according to the accuracy as specified in the sub-clause 9.1.10.2.

Furthermore, due to the inter-frequency handover the UE shall meet the RSTD measurement accuracy for a PRS bandwidth which is not larger than the minimum channel bandwidth of those PCells on whose carriers RSTD measurement is performed during the RSTD measurement period.

8.17.15.3.1	RSTD Measurement Reporting Delay

This requirement assumes that that the measurement report is not delayed by other LPP signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PUSCH with subframe or slot or subslot duration. This measurement reporting delay excludes any delay caused by no UL resources for UE to send the measurement report. When the UE is configured to perform SRS carrier based switching, an additional delay can be expected.

8.17.15.4	E-UTRAN FDD-TDD Inter-Frequency RSTD Measurements when Configured with E-UTRA-NR Dual Connectivity

When the physical layer cell identities of neighbour cells together with the OTDOA assistance data are provided, the UE shall be able to detect and measure inter-frequency RSTD, specified in TS 36.214 [4], for at least n=16 cells, including the reference cell, within TRSTD InterFreqFDDTDD, EN-DC ms as given below:

TRSTD InterFreqFDDTDD, EN-DC=TPRS∙M-1+Δ  ms,

Where

TRSTD InterFreqFDDTDD, EN-DC is the total time for detecting and measuring at least n cells,

TPRS is the largest value of the cell-specific positioning subframe configuration period, defined in TS 36.211 [16], among the measured n cells including the reference cell,

M is the number of PRS positioning occasions as defined in Table 8.17.15.4-1, where  a PRS positioning occasion is as defined in clause 8.1.2.5.1,

CSSFinterFreq is the carrier-specific scaling factor for the inter-frequency RSTD measurements and is determined according to CSSFwithin_gap,i defined in TS 38.133 [50] clause 9.1.5.2,

Δ=160∙nM ms is the measurement time for a single PRS positioning occasion which includes the sampling time and the processing time, and

The n cells are distributed on up to two carrier frequencies including a serving carrier frequency and one inter-frequency carrier.

Table 8.17.15.4-1: Number of PRS positioning occasions within TRSTD InterFreqFDDTDD, EN-DC

| Positioning subframe configuration period TPRS | Number of PRS positioning occasions M |  |
| --- | --- | --- |
|  | f2 Note1 | f1 and f2 Note2 |
| 160 ms | 16 × CSSFinterFreq | 32 × CSSFinterFreq |
| >160 ms | 8 × CSSFinterFreq | 16 × CSSFinterFreq |
| Note 1: When inter-frequency RSTD measurements are performed over the reference cell and neighbour cells, which belong to the TDD inter-frequency carrier frequency f2. Note 2: When inter-frequency RSTD measurements are performed over the reference cell and the neighbour cells, which belong to the serving FDD carrier frequency f1 and the TDD inter-frequency carrier frequency f2 respectively. |  |  |

The UE physical layer shall be capable of reporting RSTD for the reference cell and all the neighbor cells i out of at least (n-1) neighbor cells within TRSTD InterFreqFDDTDD, EN-DC, provided:

PRS EsIotref≥-6 dB for all Frequency Bands for the reference cell,

PRS EsIoti≥-13 dB for all Frequency Bands for neighbour cell i,

PRS EsIotref and PRS EsIoti conditions apply for all subframes of at least L=M2 PRS positioning occasions,

PRP 1,2|dBm according to Annex B.2.6 for a corresponding Band

PRS EsIot is as defined in Clause 8.1.2.5.1.

The time TRSTD InterFreqFDDTDD, EN-DC starts from the first subframe of the PRS positioning occasion closest in time after both the OTDOA-RequestLocationInformation message and the OTDOA assistance data in the OTDOA-ProvideAssistanceData message as specified in TS 36.355 [24], are delivered to the physical layer of the UE.

If the inter-frequency handover occurs while inter-frequency RSTD measurements are being performed, and the inter-frequency carrier on which RSTD is measured becomes the new serving carrier frequency after the inter-frequency handover, the UE shall complete the ongoing OTDOA measurement session. The UE shall also meet the inter-frequency OTDOA measurement and accuracy requirements. However in this case the RSTD measurement period (TRSTD InterFreqFDDTDD, EN-DC, HO) shall be according to the following expression:

TRSTD InterFreqFDDTDD, EN-DC,HO=TRSTD InterFreqFDDTDD, EN-DC+K×TPRS+THO  ms,

Where:

K is the number of times the inter-frequency handover occurs during TRSTD InterFreqFDDTDD, EN-DC, HO,

THO is the time during which the inter-frequency RSTD measurement may not be possible due to inter-frequency handover; it can be up to 45 ms.

The RSTD measurement accuracy for all measured neighbor cells i shall be fulfilled according to the accuracy as specified in the sub-clause 9.1.10.2.

Furthermore, due to the inter-frequency handover the UE shall meet the RSTD measurement accuracy for a PRS bandwidth which is not larger than the minimum channel bandwidth of those PCells on whose carriers RSTD measurement is performed during the RSTD measurement period.

The inter-frequency requirements in this clause (8.17.15.4) shall apply for all TDD special subframe configurations specified in TS 36.211 [16] and for the TDD uplink-downlink configurations as specified in Table 8.17.15.4-2.

Table 8.17.15.4-2: TDD uplink-downlink subframe configurations applicable for inter-frequency requirements

| PRS Transmission Bandwidth [RB] | Applicable TDD uplink-downlink configurations |
| --- | --- |
| 6, 15 | 3, 4 and 5 |
| 25 | 1, 2, 3, 4, 5 and 6 |
| 50, 75, 100 | 0, 1, 2, 3, 4, 5 and 6 |
| Note 1: Uplink-downlink configurations are specified in Table 4.2-2 in TS 36.211 [16]. Note2: For UEs capable of performing inter-frequency measurements without measurement gaps, TDD uplink-downlink subframe configurations as specified in Table 8.1.2.5.2-2 shall apply. |  |

8.17.15.4.1	RSTD Measurement Reporting Delay

This requirement assumes that that the measurement report is not delayed by other LPP signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PUSCH with subframe or slot or subslot duration. This measurement reporting delay excludes any delay caused by no UL resources for UE to send the measurement report. When the UE is configured to perform SRS carrier based switching, an additional delay can be expected.

<End of second modified section>