3GPP TSG-RAN4 Meeting #92	R4-1912000

# Chongqing, China, 14th – 18th October 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR |  | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on quality reporting in Msg3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eMTC5-Core |  |  |  |  |  | Date: |  |  | 2019-09-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Core requirements for DL quality reporting should be defined for Rel-16 eMTC, including the hypothetical MPDCCH parameters and the measurement period. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define core requirements for DL quality reporting for Rel-16 eMTC for idle mode. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | DL quality reporting for Rel-16 eMTC for idle mode is not supported from RRM perspective. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New section 6.2.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

6.2.3.1	MSG3-based channel quality report for UE Category M1

The requirements in this clause shall apply for UE supporting DL channel quality reporting for UE Category M1 as defined in TS TBD section TBD.

The DL channel quality provides the serving eNB with information about the minimum MPDCCH repetition level or aggregation level to satisfy the hypothetical MPDCCH block error rate of 1% with the parameters specified in Table 6.2.3.1-1.

Table 6.2.3.1-1: MPDCCH transmission parameters for downlink quality reporting

| Attribute | Normal coverage/CEModeA | Enhanced coverage/CEModeB |
| --- | --- | --- |
| DCI format | 6-1A | 6-1B |
| Starting OFDM symbols | 2; Bandwidth >= 10MHz 3; 3MHz <= Bandwidth < 10MHz 4; Bandwidth = 1.4MHz |  |
| MPDCCH repetition level Note1 | 1 | n.a. |
| MPDCCH Aggregation level (ECCE) Note2 | 24 |  |
| M-PDCCH Transmission type | Distributed |  |
| NOTE 1: Not applicable if repetition number in DL quality information is larger than 1. NOTE 2: Not applicable if repetition number in DL quality information equals to 1. |  |  |

When UE is in idle mode, the reported MPDCCH repetition level or aggregation level shall be derived from the channel quality measured in the period T2 in the narrowband where the random access response is transmitted, where

-	T2 is the period from the beginning of the random access response to the beginning of PUSCH for DL channel quality reporting.

The MPDCCH repetition level or aggregation level is chosen from the supported MPDCCH repetition levels and aggregation levels [3]. When UE is in idle mode, the report mapping is defined in section TBD.

The UE shall satisfy the downlink channel quality measurement accuracy requirements as specified in section TBD.

<End of Change 1>