3GPP TSG RAN WG4 Meeting #92Bis	R4-1912032

Chongqing, China, October 14-18, 2019

Agenda Item:	10.4.4

Source:	Futurewei

Title:	TP to TR 38.820: system parameters for 7-24 GHz study

Document for:	Approval

# 1	Introduction

After RAN4#92, the updated version of the TR [1] captured the following note

Editor’s note: Study applicable maximum bandwidths and SCS for SSB/PBCH block and control/data

This TP is provided for approval.

# 2	Conclusion

Based on the discussion in [1], a proposed TP to TR 38.820 on system parameters is provided.

# References

- R4-1912031, “Discussion of system parameters for 7-24 GHz study”, Futurewei, RAN4#92Bis, Oct. 14-18, 2019

## 5.7	NR system parameters analysis for 7-24 GHz range

### 5.7.a	SSB

The design of SSB for the 7-24 GHz range should consider the minimum channel bandwidth requirements, which is a combination of the occupied bandwidth of the SSB (a function of the number of RBs and SCS) and the smallest channel bandwidth within the frequency band. The resources used for CORESET0 (time location, frequency location, frequency span, SCS) may be determined once the minimum bandwidth is established. In addition, requirements for beam management, beam recovery, measurements, and beam switching should be considered to determine the number of SSBs per half frame.

### 5.7.b	Maximum bandwidth

The maximum bandwidth for the 7-24 GHz range for a UE is determined by the SCS, maximum FFT size, sampling rate, and spectrum efficiency. In addition, regulatory requirements may limit the allowed channel bandwidth supported. Another factor for the maximum bandwidth is the number of carriers used for carrier aggregation.

### 5.7.c	Shared / Control channels

### In general, the shared and control channels are independent of frequency and are scalable in SCS. However, for the 7-24 GHz range, factors affecting demodulation performance should be examined: phase noise, reference signal density and carrier tracking. This will be used to specify minimum reference signal requirements.