3GPP TSG-RAN WG4 Meeting #92-Bis 	R4-1912096

# Chongqing, P. R. of China, 14th – 18th October, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 6662 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Interruptions requirements with SRS carrier based switching in 36.133 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_Enh-Core |  |  |  |  |  | Date: |  |  | 2019-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | No interruptions requirements are defined for SRS carrier based switching |  |  |  |  |  |  |  |  |
| Summary of change: |  | Interruptions requirements for SRS carrier based switching are introduced |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No interruptions requirements are defined for SRS carrier based switching |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.32, 7.36 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

--- Start of change 1 ---

## 7.32	Interruptions with EN-DC

### 7.32.1	Introduction

This section contains the requirements related to the interruptions on PCell, and MCG SCell when

NR PSCell is added or released, or

transitions between active and non-active during NR PSCell DRX, or

transitions from NR PSCell non-DRX to DRX, or

SCell in either E-UTRA MCG or NR SCG is added or released, or

SCell in either E-UTRA MCG or NR SCG is activated or deactivated, or

measurements on SCC with deactivated SCell in either E-UTRA MCG or NR SCG, or

a downlink bandwidth part (BWP) and/or an uplink BWP is switched in NR PSCell or in any NR SCell, or

SRS carrier based switching is performed between E-UTRAN carriers, or

SRS carrier based switching is performed between NR carriers, or

SRS carrier based switching is performed between E-UTRAN carriers and between NR carriers.

The requirements shall apply for EN-DC.

This section contains interruption requirements when the victim cell is PCell or SCell belonging to MCG. Requirements for interruptions where victim cell is the NR PSCell or an NR SCell belonging to SCG are specified in [50].

For a UE which does not support per-FR measurement gaps, interruptions to the PCell or active MCG SCells may be caused by NR PSCell or NR SCells on any frequency range. For UE which support per-FR gaps, interruptions to the PCell or active MCG SCells may be caused by NR PSCell or NR SCells on FR1 only.

### 7.32.2	Requirements

--- unchaned sections ---

##### 7.32.2.8	Interruptions at SRS carrier based switching between E-UTRAN carriers

A PUSCH-less SCell is a TDD SCell without PUCCH/PUSCH configured. When a UE needs to transmit periodic or aperiodic SRS [16] and/or non-contention based PRACH on a PUSCH-less SCell, the UE can perform SRS carrier based switching to one or more CCs with PUSCH-less SCells from a CC with PUSCH or from another CC with PUSCH-less SCell prior to transmitting SRS and/or PRACH, provided that:

-	switching is between E-UTRAN carriers: from a configured CC to another CC with activated TDD SCell, and

-	the CCs with PUSCH-less SCells to which SRS carrier based switching is performed is indicated by DCI SRS request field for aperiodic SRS transmission or configured via RRC [2] for periodic SRS transmission or indicated by PDCCH for PRACH, and

-	the serving cell, from which SRS carrier based switching is performed and whose UL transmission may therefore be interrupted, is indicated by srs-SwitchFromServCellIndex [2], and

-	the SRS switching is not colliding with any other transmission with higher priority defined in [3], and

-	the SRS switching is not colliding with PDCCH in subframe 0 and 5 as specified in [3], and

-	for UE, which does not support simultaneous reception and transmission for inter-band TDD CA specified in TS 36.331 [2], and is compliant to the requirements for inter-band CA with uplink in one E-UTRA band and without simultaneous Rx/Tx specified in TS 36.101 [5], the SRS or RACH transmission are not simultaneously scheduled with DL subframe #0 or DL subframe #5 on other CCs.

The UE shall not perform SRS carrier based switching if all the above conditions cannot be met.

When performing SRS carrier based switching the UE shall meet the following interruptions requirements:

The interruption on PCell and each of the activated SCells during the switching to the PUSCH-less SCell shall not exceed 2 subframes including the first subframe where SRS transmission is configured on the PUSCH-less SCell.

The interruption on PCell and each of the activated SCells during the switching from the PUSCH-less SCell shall not exceed 2 subframes including the last subframe where SRS transmission is configured on the PUSCH-less SCell.

NOTE: interruptions requirements on NR PSCell and NR SCells in EN-DC due to the SRS switching between the E-UTRAN carriers are defined in TS 38.133 [50].

##### 7.32.2.9	Interruptions at SRS carrier based switching between NR carriers

A PUSCH-less SCell is a TDD SCell without PUCCH/PUSCH configured. When a UE needs to transmit periodic or aperiodic NR SRS [42] and/or non-contention based PRACH on a PUSCH-less SCell, the UE can perform SRS carrier based switching to one or more CCs with NR PUSCH-less SCells from an NR CC with PUSCH or from another CC with NR PUSCH-less SCell prior to transmitting SRS and/or PRACH, provided that:

-	switching is between NR carriers: from a configured CC to another activated TDD SCell,

-	other conditions are TBD.

The UE shall not perform SRS carrier based switching if all the above conditions cannot be met.

When performing SRS carrier based switching the UE shall meet the following interruptions requirements:

Editor’s note: requirements are TBD.

NOTE: interruptions requirements on NR PCell and NR SCells in EN-DC due to the SRS switching between the NR carriers are defined in TS 38.133 [50].

##### 7.32.2.10	Interruptions at SRS carrier based switching between E-UTRAN carriers and between NR carriers

Editor’s note: requirements are TBD.

NOTE: interruptions requirements on NR PSCell and NR SCells in EN-DC due to the SRS switching between E-UTRAN carriers and between the NR carriers are defined in TS 38.133 [50].

--- End of change 1 ---

--- Start of change 2 ---

## 7.36	Interruptions with NE-DC

### 7.36.1	Introduction

This clause contains the requirements related to the interruptions on PSCell and SCG SCells when

transitions between active and non-active during NR PCell DRX, or

transitions from NR PCell non-DRX to DRX, or

SCell in either NR MCG or E-UTRA SCG is added or released, or

SCell in either NR MCG or E-UTRA SCG is activated or deactivated, or

measurements on SCC with deactivated SCell in either NR MCG or E-UTRA SCG, or

a downlink bandwidth part (BWP) and/or an uplink BWP is switched in NR PCell or in any NR SCell, or

SRS carrier based switching is performed between E-UTRAN carriers, or

SRS carrier based switching is performed between NR carriers, or

SRS carrier based switching is performed between E-UTRAN carriers and between NR carriers.

The requirements shall apply for NE-DC.

This clause contains interruption requirements when the victim cell is PSCell or SCell belonging to SCG. Requirements for interruptions where victim cell is the NR PCell or an NR SCell belonging to MCG are specified in TS 38.133 [50].

For a UE which does not support per-FR measurement gaps, interruptions to the PSCell or active SCG SCells may be caused by NR PCell or NR SCells on any frequency range. For UE which support per-FR gaps, interruptions to the PSCell or active SCG SCells may be caused by NR PCell or NR SCells on FR1 only.

### 7.36.2	Requirements

--- unchaned sections ---

##### 7.36.2.7	Interruptions at SRS carrier based switching between E-UTRAN carriers

A PUSCH-less SCell is a TDD SCC without PUCCH/PUSCH configured. When a UE needs to transmit periodic or aperiodic SRS [16] and/or non-contention based PRACH on a PUSCH-less SCell, the UE can perform carrier based switching to one or more CCs with PUSCH-less SCells from a CC with PUSCH or from another PUSCH-less SCell prior to transmitting SRS and/or PRACH, provided that:

-	switching is between E-UTRAN carriers: from a configured CC to another CC with activated TDD SCell, and

-	the CC with PUSCH-less SCells to which SRS carrier based switching is performed is indicated by DCI SRS request field for aperiodic SRS transmission or configured via RRC [2] for periodic SRS transmission or indicated by PDCCH for PRACH, and

-	the serving cell, from which SRS carrier based switching is performed and whose UL transmission may therefore be interrupted, is indicated by srs-SwitchFromServCellIndex [2], and

-	the SRS switching is not colliding with any other transmission with higher priority defined in [3], and

-	the SRS switching is not colliding with PDCCH in subframe 0 and 5 as specified in [3], and

-	for UE, which does not support simultaneous reception and transmission for inter-band TDD CA specified in TS 36.331 [2], and is compliant to the requirements for inter-band CA with uplink in one E-UTRA band and without simultaneous Rx/Tx specified in TS 36.101 [5], the SRS or RACH transmission are not simultaneously scheduled with DL subframe #0 or DL subframe #5 on other CCs.

The UE shall not perform SRS carrier based switching if all the above conditions cannot be met.

When performing SRS carrier based switching the UE shall meet the following interruptions requirements:

The interruption on PSCell and each of the activated SCells during the switching to the PUSCH-less SCell shall not exceed 2 subframes including the first subframe where SRS transmission is configured on the PUSCH-less SCell.

The interruption on PSCell and each of the activated SCells during the switching from the PUSCH-less SCell shall not exceed 2 subframes including the last subframe where SRS transmission is configured on the PUSCH-less SCell.

NOTE: interruptions requirements on NR PCell and NR SCells in NE-DC due to the SRS switching between the E-UTRAN carriers are defined in TS 38.133 [50].

##### 7.36.2.8	Interruptions at SRS carrier based switching between NR carriers

A PUSCH-less SCell is a TDD SCC without PUCCH/PUSCH configured. When a UE needs to transmit periodic or aperiodic NR SRS [42] and/or non-contention based PRACH on a PUSCH-less SCell, the UE can perform carrier based switching to one or more NR CCs with PUSCH-less SCells from an NR CC with PUSCH or from another NR CC with PUSCH-less SCell prior to transmitting SRS and/or PRACH, provided that:

-	switching is between NR carriers: from a configured CC to another CC with activated TDD CC,

-	other conditions are TBD.

The UE shall not perform SRS carrier based switching if all the above conditions cannot be met.

When performing SRS carrier based switching the UE shall meet the following interruptions requirements:

Editor’s note: requirements are TBD.

NOTE: interruptions requirements on NR PCell and NR SCells in NE-DC due to the SRS switching between the NR carriers are defined in TS 38.133 [50].

##### 7.36.2.9	Interruptions at SRS carrier based switching between E-UTRAN carriers and between NR carriers

Editor’s note: requirements are TBD.

NOTE: interruptions requirements on NR PCell and NR SCellss in NE-DC due to the SRS switching between E-UTRAN carriers and between the NR carriers are defined in TS 38.133 [50].

--- End of change 2 ---