3GPP TSG-RAN4 Meeting #92bis	R4-1912161

# Chongqing, China, 14th - 18th October 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-1 | CR |  | rev |  | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.141-1: Introduction of PUSCH requirements at 30% throughput point |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_perf_enh-Perf |  |  |  |  |  | Date: |  |  | 2019-10-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Conformance tests with 30% throughput should be added to the specifications |  |  |  |  |  |  |  |  |
| Summary of change: |  | 30% throughput test points added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirement coverage incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.2.1, 8.2.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 38.104 CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.141-2 CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 8.1.2.1	Applicability of PUSCH performance requirements

##### 8.1.2.1.1	Applicability of requirements for different subcarrier spacings

Unless otherwise stated, PUSCH requirement tests shall apply only for each subcarrier spacing declared to be supported (see D.14 in table 4.6-1).

For tests at 30% throughput, tests shall only be applied with the lowest supported subcarrier spacing.

#### 8.2.1.5	Test Requirement

The throughput measured according to subclause 8.2.1.4.2 shall not be below the limits for the SNR levels specified in table 8.2.1.5-1 to 8.2.1.5-14.

Table 8.2.1.5-1: Test requirements for PUSCH, Type A, 5 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-8 | pos1 | [-1.6] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-8 | pos1 | [10.8] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-8 | pos1 | [13.2] |
|  |  | Normal | TDLA30-10 Low | 30 % | G-FR1-A5-8 | pos1 | TBD |
|  | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-8 | pos1 | [-5.2] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-8 | pos1 | [7.0] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-8 | pos1 | [9.6] |
|  |  | Normal | TDLA30-10 Low | 30 % | G-FR1-A5-8 | pos1 | TBD |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-8 | pos1 | [-8.0] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-8 | pos1 | [3.7] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-8 | pos1 | [6.5] |
|  |  | Normal | TDLA30-10 Low | 30 % | G-FR1-A5-8 | pos1 | TBD |
| 2 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-22 | pos1 | [1.4] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-22 | pos1 | [19.0] |
|  | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-22 | pos1 | [-1.9] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-22 | pos1 | [11.4] |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-22 | pos1 | [TBD] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-22 | pos1 | [7.2] |

Table 8.2.1.5-2: Test requirements for PUSCH, Type A, 10 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-9 | pos1 | [-1.8] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-9 | pos1 | [11.0] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-9 | pos1 | [13.2] |
|  | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-9 | pos1 | [-5.3] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-9 | pos1 | [7.1] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-9 | pos1 | [9.4] |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-9 | pos1 | [-8.1] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-9 | pos1 | [3.8] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-9 | pos1 | [6.3] |
| 2 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-23 | pos1 | [2.1] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-23 | pos1 | [19.3] |
|  | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-23 | pos1 | [-1.0] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-23 | pos1 | [11.6] |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-23 | pos1 | [-5.0] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-23 | pos1 | [7.3] |

Table 8.2.1.5-3: Test requirements for PUSCH, Type A, 20 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-10 | pos1 | [-1.4] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-10 | pos1 | [10.9] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-10 | pos1 | [13.3] |
|  | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-10 | pos1 | [-4.9] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-10 | pos1 | [7.1] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-10 | pos1 | [9.5] |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-10 | pos1 | [-7.9] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-10 | pos1 | [3.7] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-10 | pos1 | [6.3] |
| 2 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-24 | pos1 | [2.5] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-24 | pos1 | [19.2] |
|  | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-24 | pos1 | [-1.4] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-24 | pos1 | [11.5] |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-24 | pos1 | [-4.9] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-24 | pos1 | [7.3] |

Table 8.2.1.5-4: Test requirements for PUSCH, Type A, 10 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-11 | pos1 | [-2.1] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-11 | pos1 | [10.8] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-11 | pos1 | [12.7] |
|  |  | Normal | TDLA30-10 Low | 30 % | G-FR1-A5-11 | pos1 | TBD |
|  | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-11 | pos1 | [-5.4] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-11 | pos1 | [6.7] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-11 | pos1 | [8.9] |
|  |  | Normal | TDLA30-10 Low | 30 % | G-FR1-A5-11 | pos1 | TBD |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-11 | pos1 | [-8.3] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-11 | pos1 | [3.6] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-11 | pos1 | [5.8] |
|  |  | Normal | TDLA30-10 Low | 30 % | G-FR1-A5-11 | pos1 | TBD |
| 2 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-25 | pos1 | [1.7] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-25 | pos1 | [18.8] |
|  | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-25 | pos1 | [-1.8] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-25 | pos1 | [11.6] |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-25 | pos1 | [-4.8] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-25 | pos1 | [7.4] |

Table 8.2.1.5-5: Test requirements for PUSCH, Type A, 20 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-12 | pos1 | [-2.4] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-12 | pos1 | [10.7] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-12 | pos1 | [12.8] |
|  | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-12 | pos1 | [-5.7] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-12 | pos1 | [6.6] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-12 | pos1 | [8.8] |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-12 | pos1 | [-8.5] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-12 | pos1 | [3.5] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-12 | pos1 | [5.8] |
| 2 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-26 | pos1 | [1.7] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-26 | pos1 | [18.9] |
|  | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-26 | pos1 | [-1.8] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-26 | pos1 | [11.7] |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-26 | pos1 | [-4.9] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-26 | pos1 | [7.3] |

Table 8.2.1.5-6: Test requirements for PUSCH, Type A, 40 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-13 | pos1 | [-2.2] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-13 | pos1 | [TBD] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-13 | pos1 | [12.3] |
|  | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-13 | pos1 | [-5.5] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-13 | pos1 | [6.6] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-13 | pos1 | [8.9] |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-13 | pos1 | [-8.4] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-13 | pos1 | [3.4] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-13 | pos1 | [5.7] |
| 2 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-27 | pos1 | [1.4] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-27 | pos1 | [18.9] |
|  | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-27 | pos1 | [-1.8] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-27 | pos1 | [11.7] |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-27 | pos1 | [-4.8] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-27 | pos1 | [7.4] |

Table 8.2.1.5-7: Test requirements for PUSCH, Type A, 100 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-14 | pos1 | [-2.3] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-14 | pos1 | [10.8] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-14 | pos1 | [13.3] |
|  | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-14 | pos1 | [-5.4] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-14 | pos1 | [6.8] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-14 | pos1 | [9.3] |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-14 | pos1 | [-8.4] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-14 | pos1 | [3.5] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-14 | pos1 | [6.1] |
| 2 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-28 | pos1 | [1.9] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-28 | pos1 | [19.4] |
|  | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-28 | pos1 | [-1.8] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-28 | pos1 | [12.1] |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-28 | pos1 | [-4.8] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-28 | pos1 | [7.5] |

Table 8.2.1.5-8: Test requirements for PUSCH, Type B, 5 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-8 | pos1 | [-2.0] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-8 | pos1 | [10.5] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-8 | pos1 | [12.7] |
|  |  | Normal | TDLA30-10 Low | 30 % | G-FR1-A5-8 | pos1 | TBD |
|  | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-8 | pos1 | [-5.4] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-8 | pos1 | [6.6] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-8 | pos1 | [9.1] |
|  |  | Normal | TDLA30-10 Low | 30 % | G-FR1-A5-8 | pos1 | TBD |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-8 | pos1 | [-8.4] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-8 | pos1 | [3.2] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-8 | pos1 | [6.0] |
|  |  | Normal | TDLA30-10 Low | 30 % | G-FR1-A5-8 | pos1 | TBD |
| 2 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-22 | pos1 | [1.9] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-22 | pos1 | [18.7] |
|  | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-22 | pos1 | [-1.9] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-22 | pos1 | [11.5] |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-22 | pos1 | [-5.0] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-22 | pos1 | [7.2] |

Table 8.2.1.5-9: Test requirements for PUSCH, Type B, 10 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-9 | pos1 | [-2.0] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-9 | pos1 | [10.7] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-9 | pos1 | [13.1] |
|  | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-9 | pos1 | [-5.5] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-9 | pos1 | [6.7] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-9 | pos1 | [9.1] |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-9 | pos1 | [-8.8] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-9 | pos1 | [3.5] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-9 | pos1 | [6.1] |
| 2 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-23 | pos1 | [TBD] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-23 | pos1 | [19.5] |
|  | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-23 | pos1 | [-1.9] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-23 | pos1 | [11.7] |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-23 | pos1 | [TBD] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-23 | pos1 | [7.4] |

Table 8.2.1.5-10: Test requirements for PUSCH, Type B, 20 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-10 | pos1 | [-1.8] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-10 | pos1 | [10.7] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-10 | pos1 | [12.8] |
|  | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-10 | pos1 | [-5.5] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-10 | pos1 | [6.6] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-10 | pos1 | [9.1] |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-10 | pos1 | [-8.2] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-10 | pos1 | [3.3] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-10 | pos1 | [5.9] |
| 2 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-24 | pos1 | [1.9] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-24 | pos1 | [19.0] |
|  | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-24 | pos1 | [-1.6] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-24 | pos1 | [11.6] |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-24 | pos1 | [-4.9] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-24 | pos1 | [7.3] |

Table 8.2.1.5-11: Test requirements for PUSCH, Type B, 10 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-11 | pos1 | [-2.2] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-11 | pos1 | [10.7] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-11 | pos1 | [12.7] |
|  |  | Normal | TDLA30-10 Low | 30 % | G-FR1-A5-11 | pos1 | TBD |
|  | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-11 | pos1 | [-5.5] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-11 | pos1 | [6.6] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-11 | pos1 | [8.9] |
|  |  | Normal | TDLA30-10 Low | 30 % | G-FR1-A5-11 | pos1 | TBD |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-11 | pos1 | [-8.5] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-11 | pos1 | [3.5] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-11 | pos1 | [5.9] |
|  |  | Normal | TDLA30-10 Low | 30 % | G-FR1-A5-11 | pos1 | TBD |
| 2 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-25 | pos1 | [1.5] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-25 | pos1 | [18.9] |
|  | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-25 | pos1 | [-2.1] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-25 | pos1 | [11.7] |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-25 | pos1 | [-5.2] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-25 | pos1 | [7.4] |

Table 8.2.1.5-12: Test requirements for PUSCH, Type B, 20 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-12 | pos1 | [-2.4] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-12 | pos1 | [10.7] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-12 | pos1 | [12.7] |
|  | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-12 | pos1 | [-5.7] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-12 | pos1 | [6.6] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-12 | pos1 | [8.9] |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-12 | pos1 | [-8.7] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-12 | pos1 | [3.4] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-12 | pos1 | [5.9] |
| 2 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-26 | pos1 | [1.7] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-26 | pos1 | [19.0] |
|  | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-26 | pos1 | [-1.9] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-26 | pos1 | [11.6] |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-26 | pos1 | [-5.0] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-26 | pos1 | [7.4] |

Table 8.2.1.5-13: Test requirements for PUSCH, Type B, 40 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-13 | pos1 | [-2.2] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-13 | pos1 | [10.6] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-13 | pos1 | [12.8] |
|  | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-13 | pos1 | [-5.5] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-13 | pos1 | [6.5] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-13 | pos1 | [8.9] |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-13 | pos1 | [-8.5] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-13 | pos1 | [3.3] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-13 | pos1 | [5.8] |
| 2 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-27 | pos1 | [2.1] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-27 | pos1 | [19.1] |
|  | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-27 | pos1 | [-1.7] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-27 | pos1 | [11.6] |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-27 | pos1 | [-4.8] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-27 | pos1 | [7.3] |

Table 8.2.1.5-14: Test requirements for PUSCH, Type B, 100 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-14 | pos1 | [-2.2] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-14 | pos1 | [10.7] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-14 | pos1 | [13.4] |
|  | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-14 | pos1 | [-5.5] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-14 | pos1 | [6.6] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-14 | pos1 | [9.4] |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-14 | pos1 | [-8.5] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-14 | pos1 | [3.4] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-14 | pos1 | [6.1] |
| 2 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-28 | pos1 | [2.0] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-28 | pos1 | [19.6] |
|  | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-28 | pos1 | [-1.8] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-28 | pos1 | [12.0] |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-28 | pos1 | [-4.9] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-28 | pos1 | [7.5] |