TSG-RAN Working Group 4 (Radio) meeting #92bis	R4-1912165

Chongqing, China, 14th  October – 18th October 2019

Source:	Ericsson

Title:	Discussion on BS type 2-O PUSCH demodulation requirements with TBD and near 20dB SNR

Agenda item:	8.17.2.3

Document for:	Discussion

# Introduction

The BS type 2-O PUSCH requirements with transform precoding disabled was discussed in the last meeting, and new requirement values are also agreed [1]. But there are still some test cases (2 transmission layers and SCS=60kHz) have TBDs and around 20dB SNR values. In this paper, we will discuss the requirements for those test cases based on simulation result with MCS12.

# Discussion

## 2.1 Background

From the previous discussion, most of requirements have been agreed for BS type 2-O PUSCH with transform precoding disabled cases [1]. But there are still some TBD values and around 20dB values which means untestable for now. All these cases are summarized as following table.

Table 2.1 Summary of problematic test cases

| BW (MHz) | SCS (kHz) | Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | PT-RS | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50 | 60 | 2 | 2 | Normal | TDLA30-300 Low | 70 % | G-FR2-A4-6 | pos0 | Yes | [TBD] |
|  |  |  |  |  |  |  |  |  | No | [TBD] |
|  |  |  |  |  |  |  | G-FR2-A4-16 | pos1 | Yes | [19.6] |
|  |  |  |  |  |  |  |  |  | No | [18.14] |
| 100 |  | 2 | 2 | Normal | TDLA30-300 Low | 70% | G-FR2-A4-7 | pos0 | Yes | [TBD] |
|  |  |  |  |  |  |  |  |  | No | [TBD] |
|  |  |  |  |  |  |  | G-FR2-A4-17 | pos1 | Yes | [18.83] |
|  |  |  |  |  |  |  |  |  | No | [18.5] |
| 50 | 120 | 2 | 2 | Normal | TDLA30-300 Low | 70 % | G-FR2-A4-8 | pos0 | Yes | [21.1] |
|  |  |  |  |  |  |  |  |  | No | [18.86] |
|  |  |  |  |  |  |  | G-FR2-A4-18 | pos1 | Yes | [19.6] |
|  |  |  |  |  |  |  |  |  | No | [17.86] |
| 100 |  | 2 | 2 | Normal | TDLA30-300 Low | 70 % | G-FR2-A4-9 | pos0 | Yes | [20.8] |
|  |  |  |  |  |  |  |  |  | No | [19.34] |
|  |  |  |  |  |  |  | G-FR2-A4-19 | pos1 | Yes | [18.58] |
|  |  |  |  |  |  |  |  |  | No | [18.10] |
| 200 |  | 2 | 2 | Normal | TDLA30-300 Low | 70 % | G-FR2-A4-10 | pos0 | Yes | [21.5] |
|  |  |  |  |  |  |  |  |  | No | [20.72] |
|  |  |  |  |  |  |  | G-FR2-A4-20 | pos1 | Yes | [19.0] |
|  |  |  |  |  |  |  |  |  | No | [18.42] |

For those test cases with TBD values, the 70% of maximum throughput can’t be reached based on the simulation. For other cases, the requirements are more than or very close to 20dB which is agreed as the SNR upper limit for uplink conformance test. Since we agreed to keep these test cases in specification, lower MCS should be used and then new FRC should be defined.

## 2.2 New FRC definition and simulation result

We propose using MCS12 instead of MCS16 for these problematic cases and the corresponding FRC definition as following tables.

| Table 2.2-1: FRC parameters for FR2 PUSCH performance requirements, transform precoding disabled, Additional DM-RS position = pos0 and 2 transmission layers (16QAM, R=434/1024) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference channel | G-FR2-A4-6 | G-FR2-A4-7 | G-FR2-A4-8 | G-FR2-A4-9 | G-FR2-A4-10 |
| Subcarrier spacing [kHz] | 60 | 60 | 120 | 120 | 120 |
| Allocated resource blocks | 66 | 132 | 32 | 66 | 132 |
| CP-OFDM Symbols per slot (Note 1) | 9 | 9 | 9 | 9 | 9 |
| Modulation | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM |
| Code rate (Note 2) | 434/1024 | 434/1024 | 434/1024 | 434/1024 | 434/1024 |
| Payload size (bits) | 24072 | 48168 | 11784 | 24072 | 48168 |
| Transport block CRC (bits) | 24 | 24 | 24 | 24 | 24 |
| Code block CRC size (bits) | 24 | 24 | 24 | 24 | 24 |
| Number of code blocks - C | 3 | 6 | 2 | 3 | 6 |
| Code block size including CRC (bits) (Note 2) | 8056 | 8056 | 5928 | 8056 | 8056 |
| Total number of bits per slot | 57024 | 114048 | 27648 | 57024 | 114048 |
| Total symbols per slot | 14256 | 28512 | 6912 | 14256 | 28512 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, Additional DM-RS position = pos1 with l0= 0 and l =8 as per Table 6.4.1.1.3-3 of TS 38.211 [5]. NOTE 2: Code block size including CRC (bits) equals to K' in sub-clause 5.2.2 of TS 38.212 [15]. |  |  |  |  |  |

| Table 2.2-2: FRC parameters for FR2 PUSCH performance requirements, transform precoding disabled, Additional DM-RS position = pos1 and 2 transmission layers (16QAM, R=434/1024) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference channel | G-FR2-A4-16 | G-FR2-A4-17 | G-FR2-A4-18 | G-FR2-A4-19 | G-FR2-A4-20 |
| Subcarrier spacing [kHz] | 60 | 60 | 120 | 120 | 120 |
| Allocated resource blocks | 66 | 132 | 32 | 66 | 132 |
| CP-OFDM Symbols per slot (Note 1) | 8 | 8 | 8 | 8 | 8 |
| Modulation | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM |
| Code rate (Note 2) | 434/1024 | 434/1024 | 434/1024 | 434/1024 | 434/1024 |
| Payload size (bits) | 21504 | 43032 | 10504 | 21504 | 43032 |
| Transport block CRC (bits) | 24 | 24 | 24 | 24 | 24 |
| Code block CRC size (bits) | 24 | 24 | 24 | 24 | 24 |
| Number of code blocks - C | 3 | 6 | 2 | 3 | 6 |
| Code block size including CRC (bits) (Note 2) | 7200 | 7200 | 5288 | 7200 | 7200 |
| Total number of bits per slot | 50688 | 101376 | 24576 | 50688 | 101376 |
| Total symbols per slot | 12672 | 25344 | 6144 | 12672 | 25344 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, Additional DM-RS position = pos1 with l0= 0 and l =8 as per Table 6.4.1.1.3-3 of TS 38.211 [5]. |  |  |  |  |  |
| NOTE 2: Code block size including CRC (bits) equals to K' in sub-clause 5.2.2 of TS 38.212 [15]. |  |  |  |  |  |

Figure 2.1  FR2 PUSCH MCS12 2Tx2Rx demodulation with transform precoding disabled

Table 2.2-3 Summary of ideal simulation result

| BW (MHz) | SCS (kHz) | Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | PT-RS | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  | ideal | implementation |
| 50 | 60 | 2 | 2 | Normal | TDLA30-300 Low | 70 % | G-FR2-A4-6 | pos0 | Yes | [11.9] | [14.9] |
|  |  |  |  |  |  |  |  |  | No | [11.3] | [14.3] |
|  |  |  |  |  |  |  | G-FR2-A4-16 | pos1 | Yes | [10.4] | [13.4] |
|  |  |  |  |  |  |  |  |  | No | [10.0] | [13.0] |
| 100 |  | 2 | 2 | Normal | TDLA30-300 Low | 70% | G-FR2-A4-7 | pos0 | Yes | [12.2] | [15.2] |
|  |  |  |  |  |  |  |  |  | No | [11.6] | [14.6] |
|  |  |  |  |  |  |  | G-FR2-A4-17 | pos1 | Yes | [10.4] | [13.4] |
|  |  |  |  |  |  |  |  |  | No | [10.0] | [13.0] |
| 50 | 120 | 2 | 2 | Normal | TDLA30-300 Low | 70 % | G-FR2-A4-8 | pos0 | Yes | [10.9] | [13.9] |
|  |  |  |  |  |  |  |  |  | No | [10.5] | [13.5] |
|  |  |  |  |  |  |  | G-FR2-A4-18 | pos1 | Yes | [10.4] | [13.4] |
|  |  |  |  |  |  |  |  |  | No | [10.0] | [13.0] |
| 100 |  | 2 | 2 | Normal | TDLA30-300 Low | 70 % | G-FR2-A4-9 | pos0 | Yes | [10.4] | [13.4] |
|  |  |  |  |  |  |  |  |  | No | [10.0] | [13.0] |
|  |  |  |  |  |  |  | G-FR2-A4-19 | pos1 | Yes | [9.7] | [12.7] |
|  |  |  |  |  |  |  |  |  | No | [10.1] | [13.1] |
| 200 |  | 2 | 2 | Normal | TDLA30-300 Low | 70 % | G-FR2-A4-10 | pos0 | Yes | [10.6] | [13.6] |
|  |  |  |  |  |  |  |  |  | No | [10.2] | [13.2] |
|  |  |  |  |  |  |  | G-FR2-A4-20 | pos1 | Yes | [10.2] | [13.2] |
|  |  |  |  |  |  |  |  |  | No | [9.8] | [12.8] |

- Observation

- From Figure 2.1, only 60kHz SCS with DM-RS additional position 0 cases can’t reach 100% of maximum throughput, but they keep stable performance after reaching its highest point like other cases. There are a lot of margin between highest performance point and 70% of maximum performance point, so we can say the requirements based on these results are reliable.

- In Table 2.2-3, we deliver all ideal results and implementation results (adding 3dB to ideal results). From implementation results, all SNR at 70% of maximum throughput are around 13~15dB which is still quite below 20dB. Then all these cases become testable and reproducible if we apply FRC definition in Table 2.2-1 and Table 2.2-2.

It can be confirmed that MCS12 is more suitable for those TBD and around 20dB cases than MCS16. We can use FRC definition like Table 2.2-1 and Table 2.2-2.

# Conclusion

The FR2 PUSCH 2Tx2Rx demodulation with transform precoding disabled test cases have some TBD values and also have risk of not been tested due to around 20dB SNR.

If we replace MCS16 by MCS12 for these cases, we can get more reliable and practicable SNR at 70% of maximum throughput according to the simulation results.

# References

[1]	R4-1912166 “Draft CR to TS 38.104: BS 2-O PUSCH demodulation with transform precoding disabled”