3GPP TSG-RAN WG4 Meeting #92-Bis	R4-1912168

Chongqing, CN, 14 – 18 October 2019

Source:	Ericsson

Title:	Initial simulation results for Rel-16 NR HST PUSCH demodulation performance at UE speed of 350 km/h

Agenda item:	8.17.2.3

Document for:	Discussion

# Introduction

In this contribution, we presented our initial simulation results for NR HST PUSCH demodulation performance.

# Initial simulation assumptions and simulation results

In this section, our presented our simulation assumptions based on [1] in Table 1 and the corresponding ideal simulation results and results with impairments are given in Table 2 and Table 3.

| Parameters | Values |
| --- | --- |
|  | FR1 |
| Waveform | CP-OFDM |
| Number of Tx and Rx | 1Tx 2Rx |
| DMRS type | type 1 |
| Number of DMRS symbols | 1+1+1 DM-RS symbols with l0 = 2 |
| Number of symbols | 14 |
| Time domain resource allocation type | type A |
| Frequency domain resource | full RB allocation of the applicable BW |
| MCS index | 16 |
| Carrier frequency (GHz) | fc = 2.1 GHz for SCS = 15 kHz fc = 3.6 GHz for SCS = 30 kHz |
| Propagation channel | HST tunnel channel model: Ds = 300 m and Dmin = 2 m HST open space channel model option 1: Ds = 1000 m and Dmin = 50 m HST open space channel model option 2: Ds = 700 m and Dmin = 150 m |
| SCS and BW | 15kHz: 5 MHz & 20 MHz 30kHz: 10 MHz & 100 MHz |
| Speed (v) | 350 km / h |
| Maximum Doppler shift (fd) | 15 kHz: 1340 Hz 30 kHz: 2334 Hz |
| Testing metric | SNR @70% of maximum throughput |

Table 1. Simulation assumptions for HST scenarios at 350 km/h.

Table 2. Ideal simulation results for NR HST PUSCH demodulation performance.

| Scenario | SCS | BW | SNR [dB] @ 70% of maximum throughput |
| --- | --- | --- | --- |
| Tunnel | 15 kHz | 5 MHz | 6.8 |
|  |  | 20 MHz | 6.7 |
|  | 30 kHz | 10 MHz | 6.2 |
|  |  | 100 MHz | 6.2 |
| Open space 1 | 15 kHz | 5 MHz | 6.6 |
|  |  | 20 MHz | 6.6 |
|  | 30 kHz | 10 MHz | 6.2 |
|  |  | 100 MHz | 6.2 |
| Open space 2 | 15 kHz | 5 MHz | 6 |
|  |  | 20 MHz | 6.7 |
|  | 30 kHz | 10 MHz | 6 |
|  |  | 100 MHz | 6 |

Table 3. Results with impairments for NR HST PUSCH demodulation performance.

| Scenario | SCS | BW | SNR [dB] @ 70% of maximum throughput |
| --- | --- | --- | --- |
| Tunnel | 15 kHz | 5 MHz | 9.3 |
|  |  | 20 MHz | 9 |
|  | 30 kHz | 10 MHz | 8.3 |
|  |  | 100 MHz | 8.3 |
| Open space 1 | 15 kHz | 5 MHz | 8.7 |
|  |  | 20 MHz | 9 |
|  | 30 kHz | 10 MHz | 8 |
|  |  | 100 MHz | 8.2 |
| Open space 2 | 15 kHz | 5 MHz | 8.1 |
|  |  | 20 MHz | 9 |
|  | 30 kHz | 10 MHz | 8.4 |
|  |  | 100 MHz | 8.3 |

# Conclusion

In this contribution, we summarized our initial simulation results for NR HST PUSCH demodulation performance at UE speed of 350 km/h.

# References

R4-1910126, Way forward on NR HST BS demodulation requirements, Huawei, HiSilicon.