3GPP TSG-RAN WG4 Meeting #92-Bis	R4-1912170

Chongqing, CN, 14 – 18 October 2019

Source:	Ericsson

Title:	Discussion on maximum Doppler shifts required for NR HST PUSCH demodulation requirements for 500 km/h

Agenda item:	8.17.2.3

Document for:	Discussion

# Introduction

During RAN4#92 meeting, consensus on maximum Doppler shifts at 500 km/h could not be reached [1] and several options remain open:

- Single tap HST 500km/h

  - 15kHz SCS:

    - Option 1: 1944Hz

    - Option 2: 1500Hz

    - Option 3: 1400Hz

    - Option 4: 1250Hz

  - 30kHz SCS:

    - Option 1: 3334Hz

    - Option 2: 2600Hz

    - Option 3: 3000Hz

    - Option 4: 2300Hz

In this contribution, we present out view on the requirement for maximum Doppler shifts at 500 km/h.

# Discussion

Table 1. Single tap channel models [1].

| Parameter | Value |  |  |
| --- | --- | --- | --- |
|  | Tunnel channel model | Open space option 1 | Open space option 2 |
| Ds | 300 m | 1000 m | 700 m |
| Dmin | 2 m | 50 m | 150 m |
| v | 350 km/h, and 500 km/h FFS |  |  |
| fc | 15 kHz SCS: 2.1 GHz 30 kHz SCS: 3.6 GHz |  |  |
| fd | 15 kHz SCS: max expected ideal Doppler shift (at v = 500 km/h)? 30 kHz SCS: max expected ideal Doppler shift (at v = 500 km/h)? |  |  |

Currently, three channel models shown in Table 1 are still under discussion. However, there are no difference in how BS should start frequency offset estimations and performing compensation between different channel models defined in Table 1. But, the choice of channel models would set the lower bound on the maximum Doppler shifts required to be measured at particular HST speed.

Among the channel models given in Table 1, tunnel scenario has the largest Dmin / Ds ratio and BS is expected to experience largest Doppler shift with this deployment.

Ds/2

Ds/2

RRU0

RRU1

RRU2

θ

v

Figure 1. HST deployment.

Assuming, idealistically, BS would only start frequency offset estimation when the signal is strongest at respective RRU when UE enters the midpoint between 2 RRUs as shown in Figure 1. Then, from signal tap channel model defined in 36.104 [2], this would set the lower bound on the maximum frequency offset estimation to:

fs,lower bound=fdcosθmin,

With tunnel channel model, θmin≅0.760 for the idealistic condition stated above. Then the lower bound for the maximum Doppler shift would become:

fs,lower bound=fdcosθmin≈3334 Hz for ν = 500 km/h and fc = 3.6 GHz, and

fs,lower bound=fdcosθmin≈1944 Hz for ν = 500 km/h and fc = 2.1 GHz.

However, in reality, there is an impairment contributed by oscillator residual error which is about 0.1 ppm. This would lead to the requirement for the maximum Doppler shift of:

fs,lower bound=fdcosθmin+fosc_error =3334+360=3694Hz for ν = 500 km/h and fc = 3.6 GHz, and

fs,lower bound=fdcosθmin+fosc_error =1944+210=2154Hz for ν = 500 km/h and fc = 2.1 GHz

Proposal 1: As tunnel deployment is a realistic scenario, in our view, the maximum Doppler shifts to be defined for 500 km/h should be set to 3334 Hz for fc = 3.6 GHz and 1944 Hz for fc = 2.1 GHz when oscillator residual error is not considered.

Proposal 2: Consider oscillator residual error (i.e. 0.1 ppm of the carrier frequency) margin in the maximum frequency offsets.

In Figure 2, we illustrate the feasibility of the demodulation performance with the proposed maximum frequency offsets (including oscillator residual error).

Observation 1: With MCS = 16, the performance is stable, and the requirement is feasible with single-tap channel model at maximum frequency offsets = 3694 Hz for SCS = 30 kHz and at maximum frequency offsets = 2154 Hz for SCS = 15 kHz.

Figure 2. PUSCH demodulation performance at fc = 2.1 GHz with fd = 1944 + fosc_error and fc = 3.6 GHz with fd = 3334 + fosc_error in the signal-tap HST tunnerl scenario.

# Conclusion

In this contribution, we presented our view on the maximum Doppler shifts necessary for HST scenario for 500 km/h and our proposal is provided:

Proposal 1: As tunnel deployment is a realistic scenario, in our view, the maximum Doppler shifts to be defined for 500 km/h should be set to 3334 Hz for fc = 3.6 GHz and 1944 Hz for fc = 2.1 GHz when oscillator residual error is not considered.

Proposal 2: Consider oscillator residual error (i.e. 0.1 ppm of the carrier frequency) margin in the maximum frequency offsets.

In Figure 2, we illustrate the feasibility of the demodulation performance with the proposed maximum frequency offsets (including oscillator residual error).

Observation 1: With MCS = 16, the performance is stable, and the requirement is feasible with single-tap channel model at maximum frequency offsets = 3694 Hz for SCS = 30 kHz and at maximum frequency offsets = 2154 Hz for SCS = 15 kHz.

# References

R4-1910126, Way forward on NR HST BS demodulation requirements, Huawei, HiSilicon.

TS 36.104, V15.2.0.