3GPP TSG RAN-WG4 Meeting #92bis	R4-1912199

Chongqing, China, October 14-18, 2019

Agenda Item:	8.5.4.1

Source: 	Ericsson

Title:	Further analysis of OTA timing alignment for IAB

Document for:	Discussion

- Introduction

In the last RAN4 meeting RAN4 provided an initial response in [1] to the RAN1 LS related to the Over-The-Air (OTA) timing alignment for IAB [2]. The RAN4 initial response LS provided responses to all RAN1 questions except one question related to the minimum and maximum values of the T_delta range.

In this paper we suggest a wayforward to resolve the remaining issue related to minimum and maximum values of the T_delta range and provide the final response to RAN1.

# Analysis of T_delta

RAN4 has been discussing the T_delta range including the minimum and maximum values for the last several meetings. The minimum and maximum T_delta values have been the most controversial issue since there is large variation in the proposed values from different companies [3-9]. Our proposed minimum and maximum T_delta values are extensively anayzed in [3] is reproduced in table 1. The analysis is not repeated here for simplicity.

Table 1: Minimum and maximum values of T_delta proposed in [3]

| SCS (kHz) | T_delta min (Tc) | T_delta max (Tc) |
| --- | --- | --- |
| 15 | - NTA offset /2 -35328 | - NTA offset /2 +256 |
| 30 | - NTA offset /2 -17664 | - NTA offset /2 +128 |
| 60 | - NTA offset /2 - 8832 | - NTA offset /2 + 64 |
| 120 | - NTA offset /2 - 4416 | - NTA offset /2 + 32 |

The formula used for deriving the min/max values differ among the companies since companies have different views as to which parameters (e.g. symbol length, TAres_error, CP length, cell phase sync etc) should be included for deriving the T_delta range. Eventually the numerical values of the range as function of NTA offset will be defined. Furthermore, it is up to the IAB parent to use particular T_delta value for its IAB child in given scenario. The standard should therefore provide sufficient implementation flexibility in choosing appropriate value of T_delta. In order to ensure sufficient IAB implementation flexibility we suggest that the minimum T_delta is based on the minimum of all the lower bound values proposed by different companies and similarly the maximum minimum T_delta is based on the maximum of all the upper bound values proposed by different companies. The minimum of all the minimum T_delta values was proposed in [3], which is also expressed in table 1. The maximum of all the maxmum Tel_delta values was proposed in [4], which is also expressed in table 2.

Table 2: Minimum and maximum values of T_delta proposed in [4]

| SCS (kHz) | T_delta min (Tc) | T_delta max (Tc) |
| --- | --- | --- |
| 15 | - NTA offset /2 - 6256 | - NTA offset /2 + 6256 |
| 30 | - NTA offset /2 - 6128 | - NTA offset /2 + 6128 |
| 60 | - NTA offset /2 - 6032 | - NTA offset /2 + 6032 |
| 120 | - NTA offset /2 – 6032 | - NTA offset /2 + 6032 |

Following the compromise approach the proposed minimum and maximum ranges of T_delta are shown in table 3.

Table 3: Compromised proposal on minimum and maximum values of T_delta

| SCS (kHz) | T_delta min (Tc) | T_delta max (Tc) |
| --- | --- | --- |
| 15 | - NTA offset /2 -35328 | - NTA offset /2 + 6256 |
| 30 | - NTA offset /2 -17664 | - NTA offset /2 + 6128 |
| 60 | - NTA offset /2 - 8832 | - NTA offset /2 + 6032 |
| 120 | - NTA offset /2 - 4416 | - NTA offset /2 + 6032 |

- Summary and Wayforward

In this paper we have discussed a wayfoward to derive the minimum and the maximum values of T_delta range which is the only remaining issue related to the RAN1 LS on OTA time alignment [2]. Following is the main observation and compromised proposal:

- Observation # 1: There is large variation in the minimum and the maximum T_delta values proposed by different companies [3-9].

- Observation # 2: The minimum and the maximum T_delta values should be defined to ensure sufficient implementation flexibilility in configuring the T_delta value in differen scenarios.

- Proposal # 1: Derive minimum and maximum T_delta values as follows and the corresponding values in table 3:

  - minimum T_delta is the minimum of all the minimum T_delta values proposed by different companies.

  - maximum T_delta is the maximum of all the maximum T_delta values proposed by different companies.

Table 3: Minimum and maximum T_delta values

| SCS (kHz) | T_delta min (Tc) | T_delta max (Tc) |
| --- | --- | --- |
| 15 | - NTA offset /2 -35328 | - NTA offset /2 + 6256 |
| 30 | - NTA offset /2 -17664 | - NTA offset /2 + 6128 |
| 60 | - NTA offset /2 - 8832 | - NTA offset /2 + 6032 |
| 120 | - NTA offset /2 - 4416 | - NTA offset /2 + 6032 |

A LS response to RAN1 based on the above proposal is provided in [10].

# References

- R4-1909999, Reply LS on OTA timing alignment for IAB, Samsung, RAN4 LS to RAN1/RAN2

- R4-1905306/R1-1905842, LS on clarification of OTA timing alignment for IAB, RAN1 LS to RAN4

- R4-1909509, Futher analysis of OTA timing alignment for IAB, Ericsson

- R4-1909924, OTA timing alignment for IAB Networks, Qualcomm

- R4-1908091, Further Discussion on OTA timing alignment for IAB, Samsung

- R4-1908920, Range and graularity of T_delta for IAB, Nokia, Nokia Shanghai Bell

- R4-1908921, T_delta definition for IAB	Nokia, Nokia Shanghai Bell

- R4-1909026, Discussion on the OTA alignment for IAB, Huawei, HiSilicon

- R4-1909541, draft Reply LS on OTA timing alignment for IAB, ZTE

- R4-1912200, LS response on Max/Min T_delta values for OTA timing alignment for IAB, Ericsson