3GPP TSG-RAN WG4 Meeting #92bis	R4-1912253

Chongqing, China, 14 October – 18 October 2019

Source:	Ericsson, Swisscom

Title:	TP for TR 38.716-02-00: CA_n1-n3

Agenda item:			9.2.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.716-02-00 to include CA_n1-n3 as defined in WID [1].

# 2. Text Proposal

# ---Start of changes---

## 6.x	n1-n3

### 6.x.1	Common for 1 band UL and 2 bands UL CA

#### 6.x.1.1		Operating bands for CA

Table 6.x.1.1-1: CA band combination of band n1 and n3

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | TDD |
| n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz |  |

#### 6.x.1.2		Channel bandwidths per operating band for CA

Table 6.x.1.2-1: Supported bandwidths per CA band combination of band n1 and n3

| NR CA configuration | Uplink CA configuration | NR Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n3A | CA_n1A-n3A | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |

#### 6.x.1.3		UE co-existence studies

Table 6.x.1.3-1 lists up to 7th harmonics for n1A-n3A.

Table 6.x.1.3-1: Impact of UL/DL Harmonic

|  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  | 6th Harmonic |  | 7th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n1 | 1920 | 1980 | 3840 | 3960 | 5760 | 5940 | 7680 | 7920 | 9600 | 9900 | 11520 | 11880 | 13440 | 13860 |
| n3 | 1710 | 1785 | 3420 | 3570 | 5130 | 5355 | 6840 | 7140 | 8550 | 8925 | 10260 | 10710 | 11970 | 12495 |

#### 6.x.1.4		∆TIB and ∆RIB values

For CA_n1-n3, the TIB,c and RIB,c values are derived from LTE combination CA_1-3 and are given in the tables below.

# Table 6.x.1.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| n1-n3 | n1 | 0.3 |
|  | n3 | 0.3 |

# Table 6.x.1.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| n1-n3 | n1 | 0 |
|  | n3 | 0 |

#### 6.x.1.5 		REFSENS requirements

As can be seen in the co-existence studies in 6.x.1.3 there are no harmonics issues.

### 6.x.2	Specific for 2 bands UL CA

#### 6.x.2.1		UE co-existence studies

Table 6.x.2.1-1 lists Band n1 + Band n3 2UL bands CA  2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

# Table 6.x.2.1-1: Band n1 and Band n3 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high | 2*fx_low | 2*fx_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high | 3*fx_low | 3*fx_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 5760 | 5940 |
| 4th harmonics frequency limits | 4* fy_low | 4* fy_high | 4*fx_low | 4*fx_high |
| 4th harmonics frequency limits (MHz) | 6840 | 7140 | 7680 | 7920 |
| 5th harmonics frequency limits | 5* fy_low | 5* fy_high | 5*fx_low | 5*fx_high |
| 5th harmonics frequency limits (MHz) | 8550 | 8925 | 9600 | 9900 |
| 6th harmonics frequency limits | 6* fy_low | 6* fy_high | 6*fx_low | 6*fx_high |
| 6th harmonics frequency limits (MHz) | 10260 | 10710 | 11520 | 11880 |
| 7th harmonics frequency limits | 7* fy_low | 7* fy_high | 7*fx_low | 7*fx_high |
| 7th harmonics frequency limits (MHz) | 11970 | 12495 | 13440 | 13860 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 135 | 270 | 3630 | 3765 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2055 | 2250 | 1440 | 1650 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5550 | 5745 | 5340 | 5550 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 270 | 540 | 7260 | 7530 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3975 | 4230 | 3150 | 3435 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7470 | 7725 | 7050 | 7335 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5220 | 4860 | 6210 | 5895 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8760 | 9120 | 9390 | 9705 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 1515 | 1170 | 2520 | 2190 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8970 | 9315 | 9180 | 9510 |

Based on Table 6.x.2.1-1 there are IMD3 issues affecting own Rx frequencies of band n1.

Table 6.x.2.1-2 lists the protected bands required for the 2UL bands CA configuration as to be used in Table 6.5A.3.2.3-1 of TS 38.101-1

# Table 6.x.2.1-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n1-n3 | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 38, 40, 41, 43, 44, 50, 51, 65, 67, 72, 73, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA band 22, 42, 52 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 6 |
|  | Frequency range | 1880 |  | 1895 | -40 | 1 | 4, 14 |
|  | Frequency range | 1895 |  | 1915 | -15.5 | 5 | 4, 7, 14 |
|  | Frequency range | 1915 |  | 1920 | +1.6 | 5 | 4, 7, 14 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. NOTE 6: This requirement is applicable for any channel bandwidths within the range 1920 – 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 – 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 14: This requirement is applicable in the case of a 10 MHz NR carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. |  |  |  |  |  |  |  |

#### 6.x.2.2		REFSENS requirements

Table 6.x.2.2-1 lists the MSD required due to IMD3 for the dual uplink configuration as to be used in Table 7.3A.5-1 of TS 38.101-1.

# Table 6.x.2.2-1: MSD due to IMD issue

| Operating band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | Operating band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1A-n3A | n1 | 1950 | 5 | 25 | 2140 | [23] | FDD | IMD3 |
|  | n3 | 1760 | 5 | 25 | 1855 | N/A | FDD | N/A |

---End of changes---

# Reference

[1] 		RP-190494, “Revised WID on Rel-16 NR Inter-band Carrier Aggregation/Dual Connectivity for 2 bands DL with x bands UL (x=1,2)”, ZTE Corporation