3GPP TSG-RAN WG4 Meeting #92bis	R4-1912258

Chongqing, China, 14 October – 18 October 2019

Source:	Ericsson, Swisscom

Title:	TP for TR 38.716-03-01: CA_n1-n28-n78

Agenda item:			9.10.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.716-03-01 v2 to include CA_n1-n28-n78 as defined in WID [1].

# 2. Text Proposal

# ---Start of changes---

## 6.x	CA_n1A-n28A-n78A

### 6.x.1	Operating bands for CA

Table 6.x.1-1: 3DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n1-n28-n78 | n1 | 1920MHz | – | 1980MHz | 2110MHz | – | 2170MHz | FDD |
|  | n28 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n78 | 3300MHz | – | 3800MHz | 3300MHz | – | 3800MHz | TDD |

### 6.x.2	Channel bandwidths per operating band for CA

Table 6.x.2-1: Supported channel bandwidths per CA configuration for 3DL inter-band CA

| NR CA Configuration | UL Config | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n28A-n78A | - | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes |  | Yes |  | Yes2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes |  | Yes |  | Yes2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes1 | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes1 | Yes |  |
| NOTE 1: This UE channel bandwidth is optional in this release of the specification. NOTE 2: For the 20 MHz bandwidth, the minimum requirements are specified for NR UL carrier frequencies confined to either 713-723 MHz or 728-738 MHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.x.3	Co-existence studies

Table 6.x.3-1 summarizes frequency ranges where harmonics occur due to 3DL bands CA with 1 UL. No issues are found.

Table 6.x.3-1: Harmonic Interference for 3DLs/1UL

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n1 | 1920 | 1980 | 2110 | 2170 | 3840 | 3960 | 5760 | 5940 | 7680 | 7920 |
| n28 | 703 | 748 | 758 | 803 | 1406 | 1496 | 2109 | 2244 | 2812 | 2992 |
| n78 | 3300 | 3800 | 3300 | 3800 | 6600 | 7600 | 9900 | 11400 | 13200 | 15200 |

Table 6.x.3-2 gives harmonic mixing issue for the 3DL bands CA with 1 UL. No issues are found.

Table 6.x.3-2 Harmonic mixing for 3DLs/1UL

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n1 | 1920 | 1980 | 2110 | 2170 | 4220 | 4340 | 6330 | 6510 | 8440 | 8680 |
| n28 | 703 | 748 | 758 | 803 | 1516 | 1606 | 2274 | 2409 | 3032 | 3212 |
| n78 | 3300 | 3800 | 3300 | 3800 | 6600 | 7600 | 9900 | 11400 | 13200 | 15200 |

For single uplink, the UE coexistence is already considered for these bands in TS 38.101-1 [3].

### 6.x.4	∆TIB,c and ∆RIB,c values

For three simultaneous DLs and one UL of Band n1, n28 and n78, the TIB,c and RIB,c  values are shown in table 6.x.4-1 and  table 6.x.4-2, respectively. Values are derived from DC_1-28_n78.

Table 6.x.4-1: ΔTIB,c for 3DL aggregation

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n1-n28-n78 | n1 | 0.3 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |

Table 6.x.4-2: ΔRIB,c for 3DL aggregation

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n1-n28-n78 | n1 | 0 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |

### 6.x.5	REFSENS requirements

Additional REFSENS requirements are not needed to be defined.

---End of changes---

# Reference

[1] 		RP-192245, “Revised WID: Rel-16 NR inter-band CA for 3 bands DL with 1 band UL”, CATT