3GPP TSG-RAN WG4 Meeting #92bis	R4-1912292

# Chongqing, China, 14 – 18 October 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.104 | CR | 0047 | rev |  | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.104 on Corrections from endorsed draft CRs (Rel-16) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The “Big CR” (R4-1908441) agreed after RAN4#92 in Ljubljana has some errors, where in some parts the endorsed draft CRs were not correcly implemented: - From R4-1907940, Table A.2-1: Word “size” missing - From R4-1910084, 11.2.2.1.2: Duplicate values in tables - From R4-1910094, Table 8.3.6.1-1: Some changes not implemented Table 8.3.6.2-2: One value incorrect [-6.4] |  |  |  |  |  |  |  |  |
| Summary of change: |  | The correct text from the endorsed Draft CRs R4-1907940, R4-1910084 and R4-1910094 is implemented. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The incorrect implementation of the endorsed text would remain. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.6.1, 8.3.6.2, 11.2.2.1.2, A.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 8.3.6	Performance requirements for PUCCH format 4

#### 8.3.6.1	General

The performance is measured by the required SNR at UCI block error probability not exceeding 1%.

The UCI block error probability is defined as the conditional probability of incorrectly decoding the UCI information when the UCI information is sent. The UCI information does not contain CSI part 2.

The transient period as specified in TS 38.101-1 [17] subclause 6.3.3.1 is not taken into account for performance requirement testing, where the RB hopping is symmetric to the CC center, i.e. intra-slot frequency hopping is enabled.

Table 8.3.6.1-1: Test parameters

| Parameter | Value |
| --- | --- |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of symbols | 14 |
| The number of UCI information bits | 22 |
| First symbol | 0 |
| Length of the orthogonal cover code | n2 |
| Index of the orthogonal cover code | n0 |

#### 8.3.6.2	Minimum requirement

The UCI block error probability shall not exceed 1% at the SNR given in Table 8.3.6.2-1 and Table 8.3.6.2-2.

Table 8.3.6.2-1: Required SNR for PUCCH format 4 with 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic Prefix | Propagation conditions and correlation matrix (Annex G) | Additional DM-RS configuration | Channel bandwidth / SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 5 MHz | 10 MHz | 20 MHz |
| 1 | 2 | Normal | TDLC300-100 Low | No additional DM-RS | [1.8] | [2.6] | [2.2] |
|  |  |  |  | Additional DM-RS | [1.6] | [2.4] | [1.8] |
|  | 4 | Normal | TDLC300-100 Low | No additional DM-RS | [-2.3] | [-1.9] | [-2.2] |
|  |  |  |  | Additional DM-RS | [-2.9] | [-2.6] | [-2.7] |
|  | 8 | Normal | TDLC300-100 Low | No additional DM-RS | [-5.9] | [-5.7] | [-5.8] |
|  |  |  |  | Additional DM-RS | [-6.6] | [-6.4] | [-6.3] |

Table 8.3.6.2-2: Required SNR for PUCCH format 4 with 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic Prefix | Propagation conditions and correlation matrix (Annex G) | Additional DM-RS configuration | Channel bandwidth / SNR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 10 MHz | 20 MHz | 40 MHz | 100 MHz |
| 1 | 2 | Normal | TDLC300-100 Low | No additional DM-RS | [3.1] | [3.0] | [3.1] | [2.7] |
|  |  |  |  | Additional DM-RS | [2.7] | [2.3] | [3.1] | [2.2] |
|  | 4 | Normal | TDLC300-100 Low | No additional DM-RS | [-1.7] | [-1.9] | [-1.7] | [-2.1] |
|  |  |  |  | Additional DM-RS | [-2.0] | [-2.5] | [-2.5] | [-2.4] |
|  | 8 | Normal | TDLC300-100 Low | No additional DM-RS | [-5.6] | [-5.4] | [-5.5] | [-5.5] |
|  |  |  |  | Additional DM-RS | [-6.2] | [-6.1] | [-6.1] | [-6.2] |

*********************End of change*****************

*********************Next changed section*****************

##### 11.2.2.1.2	Minimum requirements

The throughput shall be equal to or larger than the fraction of maximum throughput stated in the tables 11.2.2.1.2-1 to 11.2.2.1.2-5 at the given SNR for 1Tx and for 2Tx two-layer spatial multiplexing transmission.

Table 11.2.2.1.2-1: Minimum requirements for PUSCH, 50 MHz channel bandwidth, 60 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | PT-RS | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-300 Low | 70 % | G-FR2-A3-1 | pos0 | No | [-2.0] |
|  |  |  |  |  | G-FR2-A3-13 | pos1 | No | [-2.2] |
|  |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A4-1 | pos0 | Yes | [12.0] |
|  |  |  |  |  |  |  | No | [11.5] |
|  |  |  |  |  | G-FR2-A4-11 | pos1 | Yes | [10.7] |
|  |  |  |  |  |  |  | No | [10.7] |
|  |  | Normal | TDLA30-75 Low | 70 % | G-FR2-A5-1 | pos0 | Yes | [13.7] |
|  |  |  |  |  |  |  | No | [13.1] |
|  |  |  |  |  | G-FR2-A5-6 | pos1 | Yes | [13.4] |
|  |  |  |  |  |  |  | No | [13.1] |
| 2 |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A3-6 | pos0 | No | [1.5] |
|  |  |  |  |  | G-FR2-A3-18 | pos1 | No | [1.2] |
|  |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A4-6 | pos0 | Yes | [TBD] |
|  |  |  |  |  |  |  | No | [TBD] |
|  |  |  |  |  | G-FR2-A4-16 | pos1 | Yes | [19.6] |
|  |  |  |  |  |  |  | No | [18.1] |

Table 11.2.2.1.2-2: Minimum requirements for PUSCH, 100 MHz channel bandwidth, 60 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | PT-RS | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-300 Low | 70 % | G-FR2-A3-2 | pos0 | No | [-2.1] |
|  |  |  |  |  | G-FR2-A3-14 | pos1 | No | [-2.4] |
|  |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A4-2 | pos0 | Yes | [12.2] |
|  |  |  |  |  |  |  | No | [11,2] |
|  |  |  |  |  | G-FR2-A4-12 | pos1 | Yes | [11.2] |
|  |  |  |  |  |  |  | No | [10.6] |
|  |  | Normal | TDLA30-75 Low | 70 % | G-FR2-A5-2 | pos0 | Yes | [14.2] |
|  |  |  |  |  |  |  | No | [13.3] |
|  |  |  |  |  | G-FR2-A5-7 | pos1 | Yes | [13.7] |
|  |  |  |  |  |  |  | No | [13.1] |
| 2 |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A3-7 | pos0 | No | [1.5] |
|  |  |  |  |  | G-FR2-A3-19 | pos1 | No | [1.2] |
|  |  | Normal | TDLA30-300 Low | 70% | G-FR2-A4-7 | pos0 | Yes | [TBD] |
|  |  |  |  |  |  |  | No | [TBD] |
|  |  |  |  |  | G-FR2-A4-17 | pos1 | Yes | [18.3] |
|  |  |  |  |  |  |  | No | [18.5] |

Table 11.2.2.1.2-3: Minimum requirements for PUSCH, 50 MHz channel bandwidth, 120 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | PT-RS | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-300 Low | 70 % | G-FR2-A3-3 | pos0 | No | [-2.1] |
|  |  |  |  |  | G-FR2-A3-15 | pos1 | No | [-2.4] |
|  |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A4-3 | pos0 | Yes | [11.3] |
|  |  |  |  |  |  |  | No | [10.9] |
|  |  |  |  |  | G-FR2-A4-13 | pos1 | Yes | [11.2] |
|  |  |  |  |  |  |  | No | [10.7] |
|  |  | Normal | TDLA30-75 Low | 70 % | G-FR2-A5-3 | pos0 | Yes | [14.1] |
|  |  |  |  |  |  |  | No | [13.4] |
|  |  |  |  |  | G-FR2-A5-8 | pos1 | Yes | [13.7] |
|  |  |  |  |  |  |  | No | [13.3] |
| 2 |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A3-8 | pos0 | No | [1.4] |
|  |  |  |  |  | G-FR2-A3-20 | pos1 | No | [1.3] |
|  |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A4-8 | pos0 | Yes | [21.1] |
|  |  |  |  |  |  |  | No | [18.6] |
|  |  |  |  |  | G-FR2-A4-18 | pos1 | Yes | [19.6] |
|  |  |  |  |  |  |  | No | [17.6] |

Table 11.2.2.1.2-4: Minimum requirements for PUSCH, 100 MHz channel bandwidth, 120 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | PT-RS | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-300 Low | 70 % | G-FR2-A3-4 | pos0 | No | [-2.4] |
|  |  |  |  |  | G-FR2-A3-16 | pos1 | No | [-2.5] |
|  |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A4-4 | pos0 | Yes | [11.9] |
|  |  |  |  |  |  |  | No | [10.5] |
|  |  |  |  |  | G-FR2-A4-14 | pos1 | Yes | [11.1] |
|  |  |  |  |  |  |  | No | [10.5] |
|  |  | Normal | TDLA30-75 Low | 70 % | G-FR2-A5-4 | pos0 | Yes | [13.5] |
|  |  |  |  |  |  |  | No | [12.9] |
|  |  |  |  |  | G-FR2-A5-9 | pos1 | Yes | [13.4] |
|  |  |  |  |  |  |  | No | [12.8] |
| 2 |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A3-9 | pos0 | No | [1.4] |
|  |  |  |  |  | G-FR21-A3-21 | pos1 | No | [1.2] |
|  |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A4-9 | pos0 | Yes | [20.8] |
|  |  |  |  |  |  |  | No | [19.4] |
|  |  |  |  |  | G-FR2-A4-19 | pos1 | Yes | [18.5] |
|  |  |  |  |  |  |  | No | [18.0] |

Table 11.2.2.1.2-5: Minimum requirements for PUSCH, 200 MHz channel bandwidth, 120 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | PT-RS | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-300 Low | 70 % | G-FR2-A3-5 | pos0 | No | [-2.1] |
|  |  |  |  |  | G-FR2-A3-17 | pos1 | No | [-2.4] |
|  |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A4-5 | pos0 | Yes | [11.3] |
|  |  |  |  |  |  |  | No | [10.9] |
|  |  |  |  |  | G-FR2-A4-15 | pos1 | Yes | [11.2] |
|  |  |  |  |  |  |  | No | [10.7] |
|  |  | Normal | TDLA30-75 Low | 70 % | G-FR2-A5-5 | pos0 | Yes | [14.1] |
|  |  |  |  |  |  |  | No | [13.4] |
|  |  |  |  |  | G-FR2-A5-10 | pos1 | Yes | [13.7] |
|  |  |  |  |  |  |  | No | [13.3] |
| 2 |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A3-10 | pos0 | No | [1.4] |
|  |  |  |  |  | G-FR2-A3-22 | pos1 | No | [1.1] |
|  |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A4-10 | pos0 | Yes | [21.5] |
|  |  |  |  |  |  |  | No | [20.2] |
|  |  |  |  |  | G-FR2-A4-20 | pos1 | Yes | [19.0] |
|  |  |  |  |  |  |  | No | [18.2] |

*********************End of change*****************

*********************Next changed section*****************

Annex A (normative):
Reference measurement channels

# A.1	Fixed Reference Channels for reference sensitivity level, ACS, in-band blocking, out-of-band blocking, receiver intermodulation and in-channel selectivity (QPSK, R=1/3)

The parameters for the reference measurement channels are specified in table A.1-1 for FR1 reference sensitivity level, ACS, in-band blocking, out-of-band blocking, receiver intermodulation, in-channel selectivity, OTA sensitivity, OTA reference sensitivity level, OTA ACS, OTA in-band blocking, OTA out-of-band blocking, OTA receiver intermodulation and OTA in-channel selectivity.

The parameters for the reference measurement channels are specified in table A.1-2 for FR2 OTA reference sensitivity level, OTA ACS, OTA in-band blocking, OTA out-of-band blocking, OTA receiver intermodulation and OTA in-channel selectivity.

Table A.1-1: FRC parameters for FR1 reference sensitivity level, ACS, in-band blocking, out-of-band blocking, receiver intermodulation, in-channel selectivity, OTA sensitivity, OTA reference sensitivity level, OTA ACS, OTA in-band blocking, OTA out-of-band blocking, OTA receiver intermodulation and OTA in-channel selectivity

| Reference channel | G-FR1-A1-1 | G-FR1-A1-2 | G-FR1-A1-3 | G-FR1-A1-4 | G-FR1-A1-5 | G-FR1-A1-6 | G-FR1-A1-7 | G-FR1-A1-8 | G-FR1-A1-9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Subcarrier spacing (kHz) | 15 | 30 | 60 | 15 | 30 | 60 | 15 | 30 | 60 |
| Allocated resource blocks | 25 | 11 | 11 | 106 | 51 | 24 | 15 | 6 | 6 |
| CP-OFDM Symbols per slot (Note 1) | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Modulation | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Code rate (Note 2) | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| Payload size (bits) | 2152 | 984 | 984 | 9224 | 4352 | 2088 | 1320 | 528 | 528 |
| Transport block CRC (bits) | 16 | 16 | 16 | 24 | 24 | 16 | 16 | 16 | 16 |
| Code block CRC size (bits) | - | - | - | 24 | - | - | - | - | - |
| Number of code blocks - C | 1 | 1 | 1 | 2 | 1 | 1 | 1 | 1 | 1 |
| Code block size including CRC (bits) (Note 3) | 2168 | 1000 | 1000 | 4648 | 4376 | 2104 | 1336 | 544 | 544 |
| Total number of bits per slot | 7200 | 3168 | 3168 | 30528 | 14688 | 6912 | 4320 | 1728 | 1728 |
| Total symbols per slot | 3600 | 1584 | 1584 | 15264 | 7344 | 3456 | 2160 | 864 | 864 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS, additional DM-RS position = pos1 with l0= 2, l = 11 as per table 6.4.1.1.3-3 of TS 38.211 [5]. NOTE 2: MCS index 4 and target coding rate = 308/1024 are adopted to calculate payload size. NOTE 3: Code block size including CRC (bits) equals to K' in sub-clause 5.2.2 of TS 38.212 [15]. |  |  |  |  |  |  |  |  |  |

Table A.1-2: FRC parameters for FR2 OTA reference sensitivity level, OTA ACS, OTA in-band blocking, OTA out-of-band blocking, OTA receiver intermodulation and OTA in-channel selectivity

| Reference channel | G-FR2-A1-1 | G-FR2-A1-2 | G-FR2-A1-3 | G-FR2-A1-4 | G-FR2-A1-5 |
| --- | --- | --- | --- | --- | --- |
| Subcarrier spacing (kHz) | 60 | 120 | 120 | 60 | 120 |
| Allocated resource blocks | 66 | 32 | 66 | 33 | 16 |
| CP-OFDM Symbols per slot (Note 1) | 12 | 12 | 12 | 12 | 12 |
| Modulation | QPSK | QPSK | QPSK | QPSK | QPSK |
| Code rate (Note 2) | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| Payload size (bits) | 5632 | 2792 | 5632 | 2856 | 1416 |
| Transport block CRC (bits) | 24 | 16 | 24 | 16 | 16 |
| Code block CRC size (bits) | - | - | - | - | - |
| Number of code blocks - C | 1 | 1 | 1 | 1 | 1 |
| Code block size including CRC (bits) (Note 3) | 5656 | 2808 | 5656 | 2872 | 1432 |
| Total number of bits per slot | 19008 | 9216 | 19008 | 9504 | 4608 |
| Total symbols per slot | 9504 | 4608 | 9504 | 4752 | 2304 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS, additional DM-RS position = pos1 with l0 = 2, l = 11 as per table 6.4.1.1.3-3 of TS 38.211 [5]. NOTE 2: MCS index 4 and target coding rate = 308/1024 are adopted to calculate payload size. NOTE 3: Code block size including CRC (bits) equals to K' in sub-clause 5.2.2 of TS 38.212 [15]. |  |  |  |  |  |