3GPP TR 37.824 V0.2.0 (2019-10)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

NR and E-UTRA;

Coexistence between NB-IoT and NR

(Release 16)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

<keyword[, keyword]>

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2017, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	5

2	References	5

3	Definitions, symbols and abbreviations	5

3.1	Definitions	5

3.2	Symbols	5

3.3	Abbreviations	5

4	Background	7

4.1	Task description	7

5	NR and NB-IoT coexistence	8

5.1	General	8

5.2	NB-IoT operating in NR in-band	8

5.2.1.1	Convention	9

5.2.1.2	NB-IoT channel raster	9

5.2.1.3	NR channel raster	10

5.2.1.4	NR SS Blocks	11

5.2.1.5	NB-IoT and NR coexistence	12

5.2.1.6	NR 100 kHz channel raster	13

5.2.1.6.1	DL considerations	13

5.2.1.6.1.1	Carrier frequency	13

5.2.1.6.1.2	RB alignment	13

5.2.1.6.1.3	Channel bandwidth	14

5.2.1.6.1.4	Conclusion	15

5.2.1.6.1.5	Example	15

5.2.1.6.2	NR FDD bands– UL considerations	15

5.2.1.6.3	NR TDD bands – UL considerations	16

5.2.1.6.4	Example	16

5.2.1.7	NR SCS based channel raster	17

5.2.1.7.1	DL considerations	17

5.2.1.7.1.1	Carrier frequency	17

5.2.1.7.1.2	RB alignment	17

5.2.1.7.1.3	Channel bandwidth	18

5.2.1.7.1.4	Conclusion	18

5.2.1.7.2	UL considerations	19

5.2.1.7.3	Example	20

5.3	NB-IoT operating in NR guard-band	20

5.3.1	Spectrum utilization consideration	20

5.3.2	Refarming consideration	20

5.3.3 	Conclusion	21

5.4	NB-IoT standalone operating	21

5.5	Synchronization between NR and NB-IoT	21

5.6	Specification and conformance impact	21

6	Summary	23

Annex	A:	24

Annex B (informative):	46

Change history	46

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document is a technical report for the work item on additional enhancements for NB-IoT, covering the study on the coexistence of NB-IoT with NR, which is one of the objectives of the WI [2].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	RP-1907573: “Additional enhancements for NB-IoT”

[3]	3GPP TS 36.104: "Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception".

[4]	3GPP TS 36.101: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception".

[5]	3GPP TS 38.213: "NR; Physical layer procedures for control".

[6]	3GPP TS 38.104: "NR: Base Station (BS) radio transmission and reception".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

# 4	Background

## 4.1	Task description

The objective on NR and NB-IoT coexistence is,

- Study aspects of FDD/TDD NB-IoT coexistence with NR [RAN4, RAN1, RAN2]

- For NB-IoT in-band, guard band and standalone operation co-existence with NR, including the case of NR configured in 15kHz SS Block SCS and the case of 30kHz SS Block SCS as specified in 38.101-1, investigate the following: [RAN4]

    - 15KHz, 30KHz, and 60KHz numerologies for NR FR1 concerned bands, with higher priority given first to 15kHz and then to 30kHz

    - Channel raster, PRB and subcarrier grid alignment between NB-IoT and NR

    - Study feasible NB-IoT carrier(s) placement allocation without RF backward compatibility impact and compatible with Rel’13 NB-IoT and Rel’15 NR, to operate simultaneously within various NR channel bandwidths

    - Study if the +6dB downlink RE power boosting can still be allowed for both in-band and guard band operation modes when co-existing with NR

    - Synchronization issue between NR and NB-IoT, including timing advance

    - Frequency band support in NB-IoT and NR

    - Testability applicable to RF

    - Coexistence between R15 NR and R13/R14/R15 NB-IoT.

Note: After RAN1 concludes the objective on R16 NB-IoT coexistence aspects, evaluate coexistence between R15 NR and R16 NB-IoT.

# 5	NR and NB-IoT coexistence

## 5.1	General

### 5.1.1	System scenarios

For NB-IoT operating in LTE, system scenarios have been divided into 3 following operations,

- Category NB1/NB2 stand-alone operation: category NB1/NB2 is operating standalone when it utilizes its own spectrum, for example the spectrum used by GERAN systems as a replacement of one or more GSM carriers, as well as scattered spectrum for potential IoT deployment.

- Category NB1/NB2 guard band operation: category NB1/NB2 is operating in guard band when it utilizes the unused resource block(s) within an E-UTRA carrier’s guard-band.

- Category NB1/NB2 in-band operation: category NB1/NB2 is operating in-band when it utilizes the resource block(s) within a normal E-UTRA carrier.

When it comes to NB-IoT co-existence with NR, depend on the utilized spectrum for NB-IoT operations, 3 system scenarios should be considered,

- NB-IoT operation in NR in-band: NB-IoT is operating in-band when it is located within a NR transmission bandwidth configuration plus 15 kHz at each edge but not within the NR minimum guard band GBChannel.

- NB-IoT operation in NR guard band: NB-IoT is operating in guard band when it is located within a NR BS channel bandwidth but is not NB-IoT In-band operation.

- NB-IoT standalone operation: NB-IoT is operating standalone when it utilizes its own spectrum, for example the spectrum currently being used by GERAN systems as a replacement of one or more GSM carriers, as well as scattered spectrum for potential IoT deployment.

.

Figure 5.1.1-1: Definition of channel bandwidth and transmission bandwidth configuration for one NR channel

## 5.2	NB-IoT operating in NR in-band

Text will be added

### 5.2.1	Channel raster, PRB and subcarrier grid alignment

#### 5.2.1.1	Convention

In the following sub-sections, focus is on NR 15 kHz SCS. Coexistence with NR 30kHz and 60kHz SCS is nevertheless addressed, mentioning additional guard band would be needed in between NR and NB-IoT. Also, PRBs alignment won’t really make sense anymore as orthogonality could not be preserved when considering 30kHz and 60 kHz SCS.

In the following sub-sections, here after tone and RB index rules are assumed:

- Tone (or subcarrier) #0 of one RB is the first tone (or subcarrier) of that RB, as shown on Figure 5.2.1.2-1 with NB-IoT RB.

- If NRB is the number of RBs for a certain NR channel bandwidth:

  - RB #0 is the RB hosting the NR RF reference frequency.

    - Subcarrier #0 is the first subcarrier of RB #0.

  - As shown on Figure 5.2.1.3-1, if NRB is even:

    - RB #(-NRB/2) is the RB on the lower edge of NR RF channel BW

      - Subcarrier #(-NRB/2*12) is the first subcarrier at lower edge of NR RF channel BW.

    - RB #(NRB/2-1) is the RB on the upper edge of NR RF channel BW.

      - Subcarrier #((NRB/2)*12-1) is the last subcarrier at upper edge of NR RF channel BW.

  - As shown on Figure 5.2.1.3-2, if NRB is odd:

    - RB #(-(NRB-1)/2) is the RB on the lower edge of NR RF channel BW

      - Subcarrier #(-(NRB-1)/2*12) is the first subcarrier at lower edge of NR RF channel BW.

    - RB #((NRB-1)/2) is the RB on the upper edge of NR RF channel BW.

      - Subcarrier #((NRB+1)/2*12-1) is the last subcarrier at upper edge of NR RF channel BW.

For example, for a NR 10MHz channel bandwidth signal:

  - RB#0 is the RB in the middle hosting the RF reference frequency (channel raster).

  - RB#(-26) is the RB on the lower edge of NR RF channel BW.

  - RB#25 is the RB on the upper edge of NR RF channel BW.

#### 5.2.1.2	NB-IoT channel raster

The NB-IoT carrier frequency position is defined on a 100 kHz channel raster based as follow:

FDL = FDL_low + 0.1(NDL – NOffs-DL) + 0.0025*(2MDL+1)

FUL = FUL_low + 0.1(NUL – NOffs-UL) + 0.0025*(2MUL)

The specified carrier frequency is located in the middle of NB-IoT RB, i.e. the frequency in between tones 5 and 6 of the RB (for 15 kHz SCS), as shown on Figure 5.2.1.2-1.

Figure 5.2.1.2-1: NB-IoT carrier location

MDL and MUL values ranges are specified in TS 36.104 [3] or TS 36.101 [4].

The anchor NB-IoT RB (broadcasting NPSS/NSSS) position is defined with a limited MDL range to {-2,-1,0,1}.

#### 5.2.1.3	NR channel raster

NR has considered 2 types of channel raster:

- 100 kHz based channel raster, mainly used for NR bands re-farmed from LTE ones.

- SCS based channel raster, 15 or 30 kHz for FR1.

Note that all FDD bands in FR1 have a channel raster based on 100 kHz.

The RF reference frequency defined by this channel raster identifies the RF channel position depending on the number of specified RBs (NRB) by the spectrum utilization for a given SCS and channel bandwidth:

- If this number is even, then it points to the subcarrier #0 (first one) of RB index #0, as shown in Figure 2.

- If this number is odd, the it points to the subcarrier #6 of RB index #0, as shown in Figure 5.2.1.3-1.

Figure 5.2.1.3-1: NR RF reference frequency position

#### 5.2.1.4	NR SS Blocks

The presence of the NR Synchronization Signal (SS) blocks would restrict possible options when placing a NB . Those blocks contain synchronization signals (PSS and SSS) which are used for UE to synchronize and connect to the BS. It’s so key that NB-IoT is not overlapping those blocks to not impact NR UE synchronization performance.

The SS block pattern for 15 kHz SCS is according to Case A (specified in TS 38.213 [5]) and its structure is given in Figure 5.2.1.4-1. SSB occupies 20 RBs in frequency domain and 4 OFDM symbols in time domain.

Figure 5.2.1.4-1: Structure of SS Block

Considering NB-IoT is currently specified only for frequency bands below 3 GHz, following equation gives the frequency position of the synchronization raster:

SSREF=1200N+50M,  kHz					(4.1)

where N∈{1,2,..2499} is an integer, and M∈{1,3,5}.

SSREF refers to the first subcarrier of the RB #10 of the SSB block (which corresponds to 11th RB of the SSB block).

Therefore, the minimum and maximum frequencies of the SSB (i.e., SSB edges), based on Figure 5.2.1.4-2, are:

FSSB,min=SSREF-10×180-7.5,  kHz		(4.2)

FSSB,max=FSSB,min+20×180,        kHz			(4.3)

Figure 5.2.1.4-2: Position of SSREF in SSB.

Which also means NB-IoT RB, shall also not be overlapping the following frequency range, as illustrated on Figure 5.2.1.4-3:

[ 1200N+50M-10×180-7.5 , 1200N+50M+10×180-7. 5] kHz

Figure 5.2.1.4-3: Feasible locations of an NB-IoT RB to avoid overlap with NR SSB in coexistence scenario.

#### 5.2.1.5	NB-IoT and NR coexistence

As mentioned before, NB-IoT carrier frequency and NR RF reference frequency refers to different locations in the frequency domain:

- NB-IoT carrier frequency is referring to a frequency located in between 2 NB-IoT tones.

- NR RF reference frequency is referring to a frequency located in the middle of a tone.

To align sub-carriers, considering NR 15 kHz SCS, a 7.5 kHz offset shall therefore be considered in between reference frequencies.

#### 5.2.1.6	NR 100 kHz channel raster

##### 5.2.1.6.1	DL considerations

##### 5.2.1.6.1.1	Carrier frequency

Based on previous considerations (NB-IoT carrier frequency, preserving perfect orthogonality in between NB-IoT and NR tones), the following equation should be resolved:

100n+2.5*2MDL+1=100m+15k-7.5  	(6.1)

With:

- MDL being the Offset of NB-IoT Channel Number to EARFCN.

- n being an integer identifying NB-IoT carrier on the 100kHz grid

- m being an integer identifying NR RF reference frequency on the 100 kHz grid

- k being an integer representing a subcarrier index from NR RF reference frequency.

This would then result in following potential subcarrier index:

k= 20* n-m + MDL+23		(6.2)k= 20* n-m + MDL+23k= 20* n-m + MDL+23 k= 20* n-m + MDL-13

All possible positions are determined by finding (n-m) values so that k is an integer that would solve this equation (6.2).

##### 5.2.1.6.1.2	RB alignment

NB-IoT RB and NR RB should be aligned to optimize resource utilization, avoiding that a NB-IoT RB overlaps 2 NR RBs.

As mentioned in 5.2.1.5, NR reference frequency refers to different subcarriers of the middle RB depending on spectrum utilization, and more specifically on the parity of the number of RBs allocated for a considered channel bandwidth.

Based on this, to ensure RBs alignment, considering p being an integer representing the RB index from NR reference frequency (p = 0 for the NR RB hosting the NR Reference frequency), if:

- The maximum number of RBs for a given channel bandwidth is even: NB-IoT carrier frequency should be further offset with 90 kHz, and equation (6.1) would become:

100n+2.5*2MDL+1=100m+180p-7.5 +90100n+2.5*2MDL+1=100m+15k-7.5 +90		(6.3)

Figure 5.2.1.6.1.2-1: Possible “RB aligned” frequency positions for NB-IoT (number of NR RBs is even).

Equation (6.2) would become:

p= 20* n-m + MDL-1636				(6.4)

- The maximum number of RBs for a given channel bandwidth is odd: no further offset shall be considered, equation (6.1) would become:

100n+2.5*2MDL+1=100m+180p-7.5		(6.5)

Figure 5.2.1.6.1.2-2: Possible “RB aligned” frequency positions for NB-IoT (number of NR RBs is odd).

Equation (6.2) would become:

p= 20* n-m + MDL+236					(6.6)

##### 5.2.1.6.1.3	Channel bandwidth

In previous equations (6.4) and (6.6), (n-m) represents the frequency separation in between NB-IoT carrier and NR RF reference frequency on a 100 kHz grid.

So, for a given NR RF reference frequency, (n-m) would indicate the relative frequency position of NB-IoT carrier from that NR reference frequency. And vice-versa, for a given NB-IoT carrier frequency, (n-m) would indicate the relative frequency position of NR reference frequency from that NB-IoT carrier.

Also, p being the RB index from NR reference frequency, p has limited value determined from NR Channel bandwidth and from the corresponding spectrum utilization.

So, if NRB is the maximum number of RBs for a certain channel bandwidth (as specified by the transmission bandwidth configuration in TS 38.104 [6], considering p = 0 for the centre RB in that channel bandwidth, p will be bounded. Equations (6.4) and (6.6) would have following additional conditions:

- If NRB is even:

p= 20* n-m + MDL-1636 	with -NRB/2 ≤p ≤(NRB/2)-1-NRB/2≥p≥(NRB/2)-1		(6.7)

- If NRB is odd:

p= 20* n-m + MDL+236		with -(NRB-1)/2≤p≤(NRB-1)/2-(NRB-1)/2≥p≥(NRB-1)/2	(6.8)

Those equations could be used to determine the position of NB-IoT anchor RB (MDL as then limited set of values {-2,-1,0,1}), or of any non-anchor RB (MDL possible values would be then in the range of {-10,-9,-8,-7,-6,-5,-4,-3,-2,-1,0,1,2,3,4,5,6,7,8,9}).

##### 5.2.1.6.1.4	Conclusion

NB-IoT and NR operating with 15 kHz SCS could coexist in NR bands supporting 100 kHz channel raster while optimizing resource utilization by aligning respective RBs. The RB index to deploy NB-IoT, for a given channel bandwidth with a transmission bandwidth configuration of NRB, is given by:

- If NRB is even:

p= 20* n-m + MDL-1636 	with  -NRB/2 ≤p ≤(NRB/2)-1-NRB/2≥p≥(NRB/2)-1    (Eq6.1)

- If NRB is odd:

p= 20* n-m + MDL+236		with -(NRB-1)/2≤p≤(NRB-1)/2-(NRB-1)/2≥p≥(NRB-1)/2    (Eq6.2)

With the additional condition those candidates RBs shall not overlap in the following range:

[ 1200N+50M-10×180-7.5 , 1200N+50M+10×180-7. 5] kHz	    (Cond6.1)

where N∈{1,2,..2499} is an integer, and M∈{1,3,5}.

From those equations, it’s then possible to determine where to deploy NB-IoT in-band of a given NR frequency deployment. Or vice-versa, it’s possible to determine how to deploy NR for a given NB-IoT frequency deployment, with NB-IoT operating in-band NR.

Those equations could be used to determine the position of NB-IoT anchor RB (MDL as then limited set of values {-2,-1,0,1}), or of any non-anchor RB (MDL possible values would be then in the range of {-10,-9,-8,-7,-6,-5,-4,-3,-2,-1,0,1,2,3,4,5,6,7,8,9}).

##### 5.2.1.6.1.5	Example

Based on previous equations, the maximum channel bandwidth for 15 kHz SCS being 50 MHz (NRB=270), p (anchor PRB position) could take one of the following values: [-131, -126, -121, -116, -111, …, -11, -6, -1, 4, 9, 14, 19, …124, 129, 134]

##### 5.2.1.6.2	NR FDD bands– UL considerations

Due the DC component in DL but not in UL, there is a 7.5 kHz shift in between DL and UL for E-UTRA. As NB-IoT might be operated in E-UTRA in-band, this shift exists also in between NB-IoT DL and UL.

NR has no DC component in its design, so such 7.5 kHz shift doesn’t exist in between DL and UL. Nevertheless, to facilitate NR and E-UTRA coexistence and SUL feature, the possibility to have this 7.5 kHz offset has also been introduced for all FDD bands (see TS 38.104 [6] section 5.2.4.1):

For SUL bands and for the uplink of all FDD bands defined in table 5.2-1,

FREF,shift = FREF + Δshift,  Δshift = 0 kHz or 7.5 kHz

where Δshift is signalled by the network in higher layer parameter frequencyShift7p5khz [11].

Using this 7.5 kHz shift it is possible to align NB-IoT RB when operating in NR in-band with NR RBs and so maintain orthogonality.

##### 5.2.1.6.3	NR TDD bands – UL considerations

Similar to FDD, for E-UTRA TDD, there is a 7.5 kHz offset in between DL and UL.

Unfortunately, the optional 7.5 kHz shift in between UL and DL was not agreed for NR TDD bands. This can’t be used then to compensate the offset in between UL and DL for NB-IoT.

However, in the scope of Rel-15 and the introduction of TDD support for NB-IoT, anticipating NR, the range of possible position frequency positions of NB-IoT TDD UL carrier has been extended in TS 36.104 [3].

The NB-IoT UL frequency position was initially specified with:

FUL = FUL_low + 0.1(NUL – NOffs-UL) + 0.0025*(2MUL)

with MUL in the range of {-10,-9,-8,-7,-6,-5,-4,-3,-2,-1,0,1,2,3,4,5,6,7,8,9}.

While from Rel-15, and the introduction of NB-IoT TDD support, the specification is:

FUL = FUL_low + 0.1(NUL – NOffs-UL) + 0.0025*(2MUL)

with MUL in the range of {-11,-10,-9.5,-9,-8.5,-8,-7.5,-7,-6.5,-6,-5.5,-5,  -4.5,-4,-3.5,-3,-2.5,-2,-1.5,-1,-0.5,0,0.5,1,1.5,2,2.5,3,3.5,4,4.5,5,5.5,6,6.5,7,7.5,8,8.5,9,9.5,10, 11} for TDD.

It’s then possible for NB-IoT TDD to align UL with DL carrier.

NR UL and DL carriers being aligned, with a 0 kHz offset, this means NB-IoT TDD DL and UL could coexist operating in-band with NR DL and UL, keeping orthogonality in between tones.

The only drawback is that NB-IoT UL carrier can’t remain at exact same frequency position when migrating from E-UTRA TDD to NR TDD but shall be shifted with an offset of 7.5 kHz.

##### 5.2.1.6.4	Example

Let’s consider a NR 10MHz channel bandwidth (NRB=52) deployed in band n1. This band uses 100 kHz channel raster.

Let’s assume the RF reference frequency for this deployment is NREF=427000, so FREF = 2,135 MHz.

NRB being even, such deployment would be in between 2,135,000-7.5-4,680=2,130,312.5 kHz (which corresponds to lower edge of RB #(-26)) and 2,135,000-7.5+4,680=2,139,672.5 kHz (which corresponds to upper edge of RB #25)..

As seen in 5.2.1.4, the SS block would occupy a frequency block from 1200*N+50*M-10*180-7.5 kHz to 1200*N+50*M+10*180-7.5 kHz.

Assuming N=1778 and M=5 (GSCN=5333), being subcarrier aligned, the SS block would occupy frequency range [2,132,042.5 ; 2,135,642.5] kHz. In other words, SS block would overlap NR RBs #(-17) to #3.

Considering NB-IoT and NR RB alignment, NB-IoT anchor RB could then be deployed in one of the following NR DL RBs: [-26, -21, 4, 9, 14, 19, 24].

Considering sub-carrier alignment only (with overlapping of NB-IoT RB over 2 NR RBs then) would offer more options for possible NB-IoT RB location, with subcarriers [-306, -300, -299, … -13, -6, 0, 1, 7, …294, 300, 301].

#### 5.2.1.7	NR SCS based channel raster

##### 5.2.1.7.1	DL considerations

##### 5.2.1.7.1.1	Carrier frequency

Based on previous considerations (NB-IoT carrier frequency, preserving perfect orthogonality in between NB-IoT and NR tones, which also means NR SCS=15 kHz), the following equation should be resolved:

100n+2.5*2MDL+1=k* Scs+5*I-7.5100n+2.5*2MDL+1=k* Scs+5*Step-7.5			(7.1)

With:

- MDL: Offset of NB-IoT Channel Number to EARFCN.

- n: integer identifying NB-IoT carrier on the 100kHz grid

- k: subcarrier index.

- m: integer identifying NR RF reference frequency

- I : step size as specified in Table 5.4.2.3-1 of TS 38.104, Step = {1, 2, 3, 6} depending on carrier frequency and sub-carrier spacing.

This would then result in following potential subcarrier index:

k =  20* n + MDL-I +2 Scs/5k =  20* n + MDL-I +2 Scs/5	(7.2)

All possible positions are determined by finding (n,m) values so that k is an integer that would solve this equation (7.2).

##### 5.2.1.7.1.2	RB alignment

When considering 15 kHz SCS, NB-IoT RB and NR RB should be aligned to optimize resource utilization, avoiding that a NB-IoT RB overlaps 2 NR RBs.

When considering other SCS, RBs alignment is not obvious anymore. Orthogonality will be lost, and some guard would be needed in between NR RBs and NB-IoT RB. Depending on the required guard, it might be more optimal to center NB-IoT RB with one or 2 punctured NR RBs.

In the following, considering no requirement has to be specified for mixed numerology, we will only investigate NR 15 kHz SCS.

As mentioned in 5.2.1.5, NR reference frequency refers to different subcarriers of the middle RB depending on spectrum utilization, and more specifically on the parity of the number of RBs allocated for a considered channel bandwidth.

Based on this, to ensure RBs alignment, considering p being an integer representing the RB index from NR reference frequency (p = 0 for the NR RB hosting the NR Reference frequency), if:

- The maximum number of RBs for a given channel bandwidth is even: NB-IoT carrier frequency should be further offset with 90 kHz, and equation (1) would become:

100n+2.5*2MDL+1=p*12* Scs+5*I-7.5+12*Scs2100n+2.5*2MDL+1=100m+180p-7.5 +90100n+2.5*2MDL+1=100m+15k-7.5 +90(7.3)

With I15  = 3 for channel raster below 3 GHz and I15  = 1 for channel raster above 3 GHz

Equation (7.2) would become:

(7.4)

Or  for channel raster below 3 GHz.

- The maximum number of RBs for a given channel bandwidth is odd: no further offset shall be considered, equation (7.1) would become:

(7.5)

With I15  = 3 for channel raster below 3 GHz and I15  = 1 for channel raster above 3 GHz

Equation (7.2) would become:

(7.6)

Or	 for channel raster below 3 GHz.

##### 5.2.1.7.1.3	Channel bandwidth

Also, p being the RB index from NR reference frequency, p has limited value determined from NR Channel bandwidth and from the corresponding spectrum utilization.

So, if NRB is the maximum number of RBs for a certain channel bandwidth (as specified by the transmission bandwidth configuration in TS 38.104 ([6]), considering p = 0 for the centre RB in that channel bandwidth, p will be bounded. Equations (7.4) and (7.6) would have following additional conditions:

- If NRB is even:

with -NRB/2 ≤p ≤(NRB/2)-1-NRB/2≥p≥(NRB/2)-1	(7.7)

- If NRB is odd:

with -(NRB-1)/2≤p≤(NRB-1)/2-(NRB-1)/2≥p≥(NRB-1)/2	(7.8)

Comparing to 100 kHz channel raster for both NR and NB-IoT, with NR SCS based channel raster, the possible positions of NB-IoT RB depend here of NR carrier frequency position. For a given NR channel bandwidth signal, it’s then not possible to give the candidate list of NB-IoT RB as it also depends on NR-ARFCN value.

##### 5.2.1.7.1.4	Conclusion

NB-IoT and NR operating could coexist in NR bands supporting SCS based channel raster, but also optimizing resource utilization by aligning respective RBs for 15 kHz SCS. The RB index to deploy NB-IoT, for a given channel bandwidth with a transmission bandwidth configuration of NRB, is given by:

- If NRB is even:

with -NRB/2 ≤p ≤(NRB/2)-1-NRB/2≥p≥(NRB/2)-1	       (Eq7.1)

- If NRB is odd:

with -(NRB-1)/2≤p≤(NRB-1)/2-(NRB-1)/2≥p≥(NRB-1)/2       (Eq7.2)

With the additional condition those candidates RBs shall not overlap in the following range:

[ 1200N+50M-10×180-7.5 , 1200N+50M+10×180-7. 5] kHz	      (Cond7.1)

where N∈{1,2,..2499} is an integer, and M∈{1,3,5}.

From those equations, it’s then possible to determine where to deploy NB-IoT in-band of a given NR frequency deployment. Or vice-versa, it’s possible to determine how to deploy NR for a given NB-IoT frequency deployment, with NB-IoT operating in-band NR.

Those equations could be used to determine the position of NB-IoT anchor RB (MDL as then limited set of values {-2,-1,0,1}), or of any non-anchor RB (MDL possible values would be then in the range of {-10,-9,-8,-7,-6,-5,-4,-3,-2,-1,0,1,2,3,4,5,6,7,8,9}).

##### 5.2.1.7.2	UL considerations

All SCS based channel raster are TDD bands, so the following UL consideration is focusing on TDD bands only.

With E-UTRA, there is a 7.5 kHz offset in between DL and UL.

In the scope of Rel-15 and the introduction of TDD support for NB-IoT, anticipating NR, the range of possible position frequency positions of NB-IoT TDD UL carrier has been extended in TS 36.104 [3].

The NB-IoT UL frequency position was initially specified with:

FUL = FUL_low + 0.1(NUL – NOffs-UL) + 0.0025*(2MUL)

with MUL in the range of {-10,-9,-8,-7,-6,-5,-4,-3,-2,-1,0,1,2,3,4,5,6,7,8,9}.

While from Rel-15, and the introduction of NB-IoT TDD support, the specification is:

FUL = FUL_low + 0.1(NUL – NOffs-UL) + 0.0025*(2MUL)

with MUL in the range of {-11,-10,-9.5,-9,-8.5,-8,-7.5,-7,-6.5,-6,-5.5,-5,  -4.5,-4,-3.5,-3,-2.5,-2,-1.5,-1,-0.5,0,0.5,1,1.5,2,2.5,3,3.5,4,4.5,5,5.5,6,6.5,7,7.5,8,8.5,9,9.5,10, 11} for TDD.

It’s then possible for NB-IoT TDD to align UL with DL carrier.

NR UL and DL carriers being aligned, with a 0 kHz offset, this means NB-IoT TDD DL and UL could coexist operating in-band with NR DL and UL, keeping orthogonality in between tones.

The only drawback is that NB-IoT UL carrier can’t remain at exact same frequency position when migrating from E-UTRA TDD to NR TDD but shall be shifted with an offset of 7.5 kHz.

##### 5.2.1.7.3	Example

Let’s consider a NR 10MHz channel bandwidth (NRB=52) deployed in band n41. This band uses 15 kHz or 30 kHz SCS based channel raster, we’ll consider 15 kHz SCS in the following.

Let’s assume the RF reference frequency for this deployment is NREF=512400, so FREF = 2,562 MHz.

NRB being even, such deployment would be in between 2,562,000-7.5-4,680=2,557,312.5 kHz (which corresponds to lower edge of RB #(-26)) and 2,520,000-7.5+4,680=2,556,672.5 kHz (which corresponds to upper edge of RB #25).

As seen in 2.5.4, the SS block would occupy a frequency block from 1200*N+50*M-10*180-7.5 kHz to 1200*N+50*M+10*180-7.5 kHz.

Assuming N=2134 and M=3 (GSCN=6402), being subcarrier aligned, the SS block would occupy frequency range [2,559,142.5 ; 2,562,742.5] kHz. In other words, SS block would overlap NR RBs #(-17) to #4.

Considering NB-IoT and NR RB alignment, NB-IoT RB could then be deployed in one of the following NR DL RBs: [-26, -21, 9, 14, 19, 24].

### 5.2.2	Power boosting

Text will be added

### 5.2.3	Numerologies

As NR could support several numerology, e.g. 15KHz, 30KHz and 60KHz in FR1 which might be different from NB-IoT numerologies (15KHz SCS in downlink and 15KHz/3.75KHz SCS in the uplink), there might exist mixed numerology configuration which would result in interference between NR subcarrier and NB-IoT subcarrier when NB-IoT carrier is deployed within NR carrier. In general, guard band between different numerology should be reserved for performance protection, however this should be up to the BS implementation. Similar as the mixed numerology in NR scenarios, no requirement is defined for mixed numerology between NR and NB-IoT.

## 5.3	NB-IoT operating in NR guard-band

### 5.3.1	Spectrum utilization consideration

Compared to LTE which has fixed spectrum utilization for channel bandwidth larger than and equal to 3 MHz, the spectrum utilization has been improved for NR especially for 15 kHz SCS. Spectrum utilization values in Rel-15 are the results of companies’ compromise. NB-IoT operating in NR guard band need further increase the spectrum utilization hence NB-IoT should not be considered for operating in NR guard band for 15 kHz SCS. For SCS other than 15 kHz, in some scenarios, the spectrum utilization may be less than that in LTE. For those scenarios some extra guard need to be considered for the co-existence of mixed numerology. As agreed in Clause 5.2.3, no requirement would be specified for mixed numerology. Again the spectrum utilization was optimized for 30 kHz and 60 kHz SCS, the operation in guard band will depend on the implementation margin and need to be investigated case by case.

### 5.3.2	Refarming consideration

When refarming the LTE carrier to NR carrier with the existence of downlink anchor guard-band NB-IoT operations, keeping same NB-IoT carrier frequency, NB-IoT might partially operate in NR in-band and partially in NR guard band as shown in figure 5.3.1.

Figure 5.3.1: Downlink anchor guard-band NB-IoT operation for 10MHz LTE channel bandwidth refarming to 15kHz SCS with 100kHz channel raster NR

As shown in figure 5.3.1, due to the carrier shift by 15kHz, the guard-band NB-IoT PRB operating in the upper half of the LTE 10MHz channel bandwidth will not be aligned with NR PRB 51, which results in one NB-IoT subcarrier falling into NR guard band. However, the frequency distance from NB-IoT PRB edge to NR channel bandwidth edge is not smaller than the minimum NR guard band. Thus there will be no issue for the radio to support this case.

So it is concluded that considering legacy deployments, even if one NB-IoT 15 kHz sub-carrier would operate in NR guard band, still consider this case as NB-IoT operation in NR in-band.

### 5.3.3 	Conclusion

NB-IoT operating in NR guard band would be handled as implementation issue and RF requirements will not be specified in Rel-15 (nor in Rel-16 unless new Rel-16 features make this essential).

## 5.4	NB-IoT standalone operating

In Rel-13 when introduction of NB-IoT in the specifications, for stand-alone operation mode, the coexistence between NB-IoT and legacy LTE/UMTS/GSM systems was verified both for the downlink and uplink in TR 36.802. The summary of system level simulation results for coexistence study between Rel-15 NR and Rel-13/14/15 NB-IoT standalone operation is provided in annex A. Based on coexistence study, the RF requirements such as ACLR and ACS for NB-IoT was defined to guarantee coexistence with the legacy systems. And for FR1 the LTE ACLR and ACS requirements are reused for NR. Hence there should be no issue for NB-IoT standalone coexistence with NR.

In addition, in Rel-15 MSR specification, standalone NB-IoT+LTE+NR Multi-Standard operation is supported with capability set (CS17). The corresponding core requirements and conformance testing are already handled in MSR specifications.

## 5.5	Synchronization between NR and NB-IoT

Text will be added

## 5.6	Specification and conformance impact

MSR BS supporting NR and LTE+NB-IoT (LTE in-band/guard band) could also meet current RF requirements when operating NR+(LTE)+NB-IoT (NR in-band) with following limitations:

- Same NR channel bandwidth as LTE

- NR uses 15 kHz SCS

- NB-IoT carrier frequency kept when operating with LTE and NR, or shifted closer to NR carrier.

Therefore, MSR BS could operate NB-IoT in NR in-band with above limitations and no need to redo conformance testing in this case.

To remove any limitation, specification update is needed to add support for NB-IoT operating in NR in-band in TS 38.104, TS 38.141-1, TS 37.104 and TS 37.141 in Rel-16 the latest.

# 6	Summary

Text will be added

# Annex	A:

Coexistence simulation

A.1	Downlink coexistence simulation results using legacy LTE/UMTS/GSM channel model

The system level simulation methodology and assumptions in TR 36.802 for coexistence study between NB-IoT standalone operation and legacy LTE/UMTS/GSM systems are used here. 10MHz channel bandwidth are used for both LTE and NR here, and 15kHz SCS is used for NR as it has a higher spectrum utilization than 30kHz SCS for 10MHz channel bandwidth. Note that the NB-IoT BS ACLR2 of 50dBc is used here because the NB-IoT BS ACLR1 will fall into the 500kHz internal guard-band of the 10MHz LTE channel, as well as the 312.5kHz minimum guard-band of the 10MHz 15kHz SCS NR channel. Consequently, the overall ACIR is dominate by the LTE or NR UE ACS1 of 33dBc.

The CDFs of the coupling loss between victim LTE UE and BS and the DL SINR of victim LTE UE for 900MHz and 2GHz carrier frequency are shown in Figure A.1-1 below.

(a) With 900MHz carrier frequency

(b) With 2GHz carrier frequency

Figure A.1-1: For LTE victim UE

The CDFs of the coupling loss between victim NR UE and BS and the DL SINR of victim NR UE for 900MHz and 2GHz carrier frequency are shown in Figure A.1-2 below.

(a) With 900MHz carrier frequency

(b) With 2GHz carrier frequency

Figure A.1-2: For NR victim UE

The average and 5%-tile DL throughput loss of the victim UE for the simulated cases are summarized in Table A.1-1 below. It can be seen that the average and 5%-tile DL throughput loss of the victim LTE and NR UE are equal (with four decimal point precision) in all simulated cases. This can be explained by the fact that the only difference between the victim LTE and NR UE is the DL bandwidth configuration which accounts to only [10log10(9.36e6/9e6)=]0.17dB difference in the UE receiver noise floor, which is minimal compared to the cumulative interference caused by the interfering NB-IoT BS to the DL SINR of victim LTE and NR UE. Therefore, the average and 5%-tile DL throughput loss of the victim NR UE are equal to those of the victim LTE UE (with four decimal point precision).

Table A.1-1: Average and 5%-tile DL throughput loss of the victim UE

| Victim UE | LTE | NR |
| --- | --- | --- |
| Average throughput loss (900MHz carrier frequency) | 0.30% | 0.30% |
| 5%-tile throughput loss (900MHz carrier frequency) | 1.03% | 1.03% |
| Average throughput loss (2GHz carrier frequency) | 0.31% | 0.31% |
| 5%-tile throughput loss (2GHz carrier frequency) | 1.00% | 1.00% |

A.2	Downlink coexistence simulation results using NR channel model

Further simulation results are obtained using the system level simulation methodology and assumptions in TR 36.802 but with the NR channel model in TR 38.901 instead of the legacy LTE/UMTS/GSM channel model. Only 2GHz carrier frequency is simulated here as the use of BS antenna array in 900MHz has limitation considering the required antenna array size with the longer wavelength. A (4x8) antenna array is assumed here for the NR BS, while legacy antenna is assumed for the UE as well as the LTE and NB-IoT BS.

The CDFs of the coupling loss between victim LTE UE and BS and the DL SINR of victim LTE UE are shown in Figure A.2-1 below.

Figure A.2-1: For LTE victim UE

The CDFs of the coupling loss between victim NR UE and BS and the DL SINR of victim NR UE are shown in Figure A.2-2 below.

Figure A.2-2: For NR victim UE

The average and 5%-tile DL throughput loss of the victim UE for the simulated cases are summarized in Table A.2-1 below. It can be seen that the average and 5%-tile DL throughput loss of the victim LTE and NR UE are higher than those using the legacy channel mode in Table A.1-1. This can be explained by the fact that no MCL is applied for the NR channel model, and thus the coupling loss between the victim UE and the interfering NB-IoT BS can be much smaller using the NR channel model (below 50dB as shown in Figures A.2-1 and A.2-2) compared to those using the legacy channel model in Figures A.1-1 and A.1-2 (where MCL is 70dB). This means that the impact from the interfering NB-IoT BS to the DL SINR of victim LTE and NR UE can be much larger, especially for the 5%-tile DL throughput where the near-far effect is more severe. The 5%-tile DL throughput loss of the victim UE will be greatly reduced if 70dB MCL is also applied to the NR channel model, as shown in Table A.2-1 below. On the other hand, it can be seen that the average DL throughput loss of the victim NR UE is similar to those of the victim LTE UE, while the 5%-tile DL throughput loss of the victim NR UE is lower than those of the victim LTE UE. This shows that the UE specific beamforming used in the NR BS can effectively reduce the impact from the interfering NB-IoT BS to the DL SINR of victim NR UE at the cell edge.

Table A.2-1: Average and 5%-tile DL throughput loss of the victim UE

| Victim UE | LTE | NR |
| --- | --- | --- |
| Average throughput loss (without MCL) | 2.21% | 2.13% |
| 5%-tile throughput loss (without MCL) | 10.50% | 7.80% |
| Average throughput loss (with 70dB MCL) | 1.35% | 1.48% |
| 5%-tile throughput loss (with 70dB MCL) | 4.51% | 1.90% |

A.3	Uplink coexistence simulation results using legacy LTE/UMTS/GSM channel model

The system level simulation methodology and assumptions in TR 36.802 for coexistence study between NB-IoT standalone operation and legacy LTE/UMTS/GSM systems are used here. 10MHz channel bandwidth are used for both LTE and NR here, and 15kHz SCS is used for NR as it has a higher spectrum utilization than 30kHz SCS for 10MHz channel bandwidth. The number of active users in both victim and interfering systems is 3. Note that the NB-IoT UE UL power control parameter PLx-ile is bandwidth scaled targeting 15 dB uplink SNR at the serving BS, and the NB-IoT UE ACLR of 37dBc is bandwidth scaled by the factor 10*log10(9e6 or 9.36e6/3/180e3) to account for the asymmetrical transmission bandwidth. The LTE or NR BS ACS is 45dBc. Consequently, the overall ACIR is dominate by the NB-IoT UE ACLR.

The CDFs of the LTE and NB-IoT UE transmit power and the UL SINR of victim LTE UE for 900MHz and 2GHz carrier frequency are shown in Figure A.3-1 below.

(a) With 900MHz carrier frequency

(b) With 2GHz carrier frequency

Figure A.3-1: For LTE victim UE

The CDFs of the NR and NB-IoT UE transmit power and the UL SINR of victim NR UE for 900MHz and 2GHz carrier frequency are shown in Figure A.3-2 below.

(a) With 900MHz carrier frequency

(b) With 2GHz carrier frequency

Figure A.3-2: For NR victim UE

The average and 5%-tile UL throughput loss of the victim UE for the simulated cases are summarized in Table A.3-1 below. It can be seen that the average and 5%-tile UL throughput loss of the victim LTE and NR UE are similar in all simulated cases. This can be explained by the fact that the only difference between the victim LTE and NR UE is the UL bandwidth configuration which accounts to only [10log10(9.36e6/9e6)=]0.17dB difference in the BS receiver noise floor, which is minimal compared to the cumulative interference caused by the interfering NB-IoT UE to the UL SINR of victim LTE and NR UE. Therefore, the average and 5%-tile DL throughput loss of the victim NR UE are similar to those of the victim LTE UE.

Table A.3-1: Average and 5%-tile UL throughput loss of the victim UE

| Victim UE | LTE | NR |
| --- | --- | --- |
| Average throughput loss (900MHz carrier frequency) | 0.73% | 0.76% |
| 5%-tile throughput loss (900MHz carrier frequency) | 0.30% | 0.31% |
| Average throughput loss (2GHz carrier frequency) | 0.72% | 0.75% |
| 5%-tile throughput loss (2GHz carrier frequency) | 0.34% | 0.36% |

A.4	Uplink coexistence simulation results using NR channel model

Further simulation results are obtained using the system level simulation methodology and assumptions in TR 36.802 but with the NR channel model in TR 38.901 instead of the legacy LTE/UMTS/GSM channel model. Note that 70dB MCL is also applied to the NR channel model as explained in annex A.2. Only 2GHz carrier frequency is simulated here as the use of BS antenna array in 900MHz has limitation considering the required antenna array size with the longer wavelength. A (4x8) antenna array is assumed here for the NR BS, while legacy antenna is assumed for the UE as well as the LTE and NB-IoT BS.

The CDFs of the LTE and NB-IoT UE transmit power and the UL SINR of victim LTE UE are shown in Figure A.4-1 below.

Figure A.4-1: For LTE victim UE

The CDFs of the NR and NB-IoT UE transmit power and the UL SINR of victim NR UE are shown in Figure A.4-2 below.

Figure A.4-2: For NR victim UE

The average and 5%-tile UL throughput loss of the victim UE for the simulated cases are summarized in Table A.4-1 below. It can be seen that the average and 5%-tile UL throughput loss of the victim NR UE are higher than those of the victim LTE UE. This can be explained by the fact that the NB-IoT UE ACLR of 37dBc is bandwidth scaled by a larger factor the factor 10*log10 (9.36e6/3/180e3) instead of 10*log10(9e6/3/180e3) to account for the higher spectrum utilization of the NR BS compared to that of the LTE BS. However, the average and 5%-tile UL throughput loss of the victim NR UE are still lower than the 5% threshold for acceptable throughput degradation.

Table A.4-1: Average and 5%-tile DL throughput loss of the victim UE

| Victim UE | LTE | NR |
| --- | --- | --- |
| Average throughput loss (with 70dB MCL) | 4.59% | 4.68% |
| 5%-tile throughput loss (with 70dB MCL) | 2.58% | 2.69% |

# Annex B (informative):

# Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2019-04 | RAN4#90BIS | R4-1905109 | - | - | - | First version of the draft TR | 0.0.1 |
| 2019-08 | RAN4#92 | R4-1908370 |  |  |  | Second version of the draft TR Implemented following TPs endorsed during RAN4#91 meeting: R4-1906926, R4-1907810, R4-1907811, R4-1907812 | 0.1.0 |
| 2019-10 | RAN4#92-BIS | R4-191 |  |  |  | Third version of the draft TR Implemented following TPs endorsed during RAN4#91 meeting: R4-1908314, R4-1908315, R4-1910486, R4-1910487, R4-1910488, R4-1910489 | 0.2.0 |