3GPP TSG RAN WG4 Meeting #92-Bis	R4-1912307

Chongqing, China 	14th – 18th October, 2019

Agenda Item:	8.4.2.2

Source:	Futurewei

Title:	TP on Indevice Coexistence

Document for:	Approval

# Introduction

This contribution provides text proposal for the section 6 Evaluation of Indevice Coexistence in the TR 38.886 [1]. The scenario description for evaluation are described.

************* Start of the TP in sub clause 6 of TR 38.886 ***************

# 6	Evaluation of In-device Coexistence

## 6.1 In-device coexistence scenarios

From the UE perspective, coexistence between LTE V2X and NR V2X SLs is studied when there are coordinated procedures between LTE and NR, and half-duplex constraints are assumed.

Solutions based on TDM or FDM between LTE and NR SLs are the focus of the study.  TDM solutions are those that involve overlapping or simultaneous NR and LTE V2X SL transmissions.  FDM solutions are those that involve simultaneous NR and LTE V2X SL transmissions, and define solutions for sharing the total power between the two.

RAN4 conducted indevice coexistence for two scenarios:

- when LTE SL and NR SL in the ITS band, and without LTE or NR Uu in another band on the same device

- when LTE SL and NR SL in the ITS band, and with LTE or NR Uu in another band on the same device

For the scenario 1: no self-interference issues are identified for TDM operation in 5.9 GHz ITS spectrum.  The long-term coexistence is found feasible without any issues.  For short-term switching times for TDM operation needs to be studied.

<End of Changes>

# References

TR 38.886. Title: V2X Services based on NR; User Equipment (UE) radio transmission and reception