# 3GPP TSG-RAN WG4 Meeting #92bis															R4-1912341

# Chongqing , China, 14th – 18th Oct 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-1 | CR | - | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to 38.141-1 with editorial corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This CR provide purely editorial changes in 38.141-1 specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Following editorial corrections are made: - 6.3.3.4.1 - Test models acronim corrected - 6.3.3.5 – reference to table corrected - 6.5.3.4.2 - Test models acronim corrected - 6.5.4.4.2 - Test models acronim corrected - 6.6.5.4.2 - Test models acronim corrected - 6.7.4.2 - Test models acronim corrected - 7.6.4.2 - Test models acronim corrected - 8.1.2.3.3 – typo corrected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification may be misleading. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.3.4.1, 6.3.3.5, 6.5.3.4.2, 6.5.4.4.2, 6.6.3.4.2, 6.6.4.4.2, 6.6.5.4.2, 6.7.4.2, 7.6.4.2, 8.1.2.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of TP >>

##### 6.3.3.4.1	Initial conditions

Test environment: Normal, see annex B.2.

RF channels to be tested:	M; see subclause 4.9.1.

Set the channel set-up of the connector under test transmitted signal according to NR-FR1-TM3.1.

<< next modified section >>

##### 6.3.3.4.2	Procedure

For BS type 1-H where there may be multiple TAB connectors, they may be tested one at a time or multiple TAB connectors may be tested in parallel as shown in annex D.3.1. Whichever method is used the procedure is repeated until all TAB connectors necessary to demonstrate conformance have been tested.

1)	Connect the single-band connector(s) under test as shown in annex D.1.1 for BS type 1-C and in annex D.3.1 for BS type 1-H. All connectors not under test shall be terminated.

2)	Set each connector under test to transmit according to the applicable test configuration in subclause 4.8 using the corresponding test models in subclause 4.9.2 at rated carrier output power Prated,c,AC for BS type 1-C and Prated,c,TABC for BS type 1-H (D.21).

3)	For BS type 1-C and BS type 1-H, set the BS to transmit a signal according to:

-	NR-FR1-TM3.1a if 256QAM is supported by BS without power back off, or

-	NR-FR1-TM3.1 if 256QAM is supported by BS with power back off, or

-	NR-FR1-TM3.1 if 256QAM is not supported by BS.

4)	Measure the average OFDM symbol power as defined in the annex H.

5)	For BS type 1-C and BS type 1-H, set the BS to transmit a signal according to:

NR-FR1-TM2a if 256QAM is supported by BS, or

NR-FR1-TM2 if 256QAM is not supported by BS;

6)	Measure the average OFDM symbol power as defined in the annex H. The measured OFDM symbols shall not contain RS or SSB.

In addition, for multi-band connectors, the following steps shall apply:

7)	For a multi-band connectors and single band tests, repeat the steps above per involved operating band where single band test configurations and test models shall apply with no carrier activated in the other operating band.

#### 6.3.3.5	Test requirements

The downlink (DL) total power dynamic range for each NR carrier shall be larger than or equal to the level in table 6.3.3.5-1.

Table 6.3.3.5-1: BS total power dynamic range

| NR channel bandwidth (MHz) | Total power dynamic range (dB) |  |  |
| --- | --- | --- | --- |
|  | 15 kHz SCS | 30 kHz SCS | 60 kHz SCS |
| 5 | 13.5 | 10 | N/A |
| 10 | 16.7 | 13.4 | 10 |
| 15 | 18.5 | 15.3 | 12.1 |
| 20 | 19.8 | 16.6 | 13.4 |
| 25 | 20.8 | 17.7 | 14.5 |
| 30 | 21.6 | 18.5 | 15.3 |
| 40 | 22.9 | 19.8 | 16.6 |
| 50 | 23.9 | 20.8 | 17.7 |
| 60 | N/A | 21.6 | 18.5 |
| 70 | N/A | 22.3 | 19.2 |
| 80 | N/A | 22.9 | 19.8 |
| 90 | N/A | 23.4 | 20.4 |
| 100 | N/A | 23.9 | 20.9 |

NOTE:	Additional test requirements for the EVM at the lower limit of the dynamic range are defined in subclause 6.5.4.

<< next modified section >>

##### 6.5.3.4.2	Procedure

The minimum requirement is applied to all antenna connectors or TAB connectors, they may be tested one at a time or multiple antenna connectors or TAB connectors may be tested in parallel as shown in annex D.1.1 for BS type 1-C or in annex D.3.1 for BS type 1-H. Whichever method is used the procedure is repeated until all antenna connectors or TAB connectors necessary to demonstrate conformance have been tested.

1)	For an antenna connector or TAB connector declared to be capable of single carrier operation only (D.16), set the antenna connector or the TAB connector under test to transmit a signal according to the applicable test configuration in subclause 4.8 using the corresponding test models:

-	NR-FR1-TM3.1a if 256QAM is supported by BS without power back off, or

-	NR-FR1-TM3.1a at manufacturer's declared rated output power if 256QAM is supported by BS with power back off, and NR-FR1-TM3.1 at maximum power, or

-	NR-FR1-TM3.1 if highest modulation order supported by BS is 64QAM, or

-	NR-FR1-TM3.2 if highest modulation order supported by BS is 16QAM, or

-	NR-FR1-TM3.3 if highest modulation order supported by BS is QPSK.

For an antenna connector or TAB connector declared to be capable of multi-carrier and/or CA operation (D.15-D.16), set the antenna connector or the TAB connector under test to transmit according to the applicable test configuration and corresponding power setting specified in subclauses 4.7 and 4.8 using the corresponding test models on all carriers configured:

-	NR-FR1-TM3.1a if 256QAM is supported by BS without power back off, or

-	NR-FR1-TM3.1a at manufacturer's declared rated output power if 256QAM is supported by BS with power back off, and NR-FR1-TM3.1 at maximum power, or

-	NR-FR1-TM3.1 if highest modulation order supported by BS is 64QAM, or

-	NR-FR1-TM3.2 if highest modulation order supported by BS is 16QAM, or

-	NR-FR1-TM3.3 if highest modulation order supported by BS is QPSK.

For NR-FR1-TM3.1a, power back-off shall be applied if it is declared.

2)	Measure the EVM and frequency error as defined in annex H.

3)	Repeat steps 1 and 2 for NR-FR1-TM2 if 256QAM is not supported by BS or for NR-FR1-TM2a if 256QAM is supported by BS. For NR-FR1-TM2 and NR-FR1-TM2a the OFDM symbol power shall be at the lower limit of the dynamic range according to the test procedure in subclause 6.3.3.4 and test requirements in subclause 6.3.3.5.

In addition, for multi-band connector(s), the following steps shall apply:

4)	For multi-band connectors and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

<< next modified section >>

##### 6.5.4.4.2	Procedure

For BS type 1-C antenna connectors to be tested are for a specific set of signals/transmitter configuration/transmission mode.

For BS type 1-H TAB connectors to be tested are identified from the declared sets of TAB connector beam forming groups in the TAE groups declaration (D.31).

Compliance is to be demonstrated between all pairs of single-band connectors and/or multi-band connectors, however it is not required to exhaustively measure TAE between every combination of pairs of representative connectors. Compliance can be demonstrated by comparison of a reduced set of representative measurement results.

1)	Conducted measurement setup:

- For BS type 1-C: Connect two antenna connectors to the measurement equipment according to annex D.1.3. Terminate any unused antenna connector(s).

- For BS type 1-H: Connect two representative TAB connectors one from separate TAE group (D.31) to the measurement equipment according to annex D.3.4. Terminate any unused TAB connector(s).

2)	Set the connectors under test to transmit NR-FR1-TM1.1 or any DL signal using MIMO transmission or carrier aggregation.

NOTE:	For MIMO transmission, different ports may be configured in NR-FR1-TM1.1 (using PDSCH DMRS ports 1000 and 1001).

3)	For a connectors declared to be capable of single carrier operation only (D.16), set the representative connectors under test to transmit according to the applicable test configuration in subclause 4.8 using the corresponding test models in subclause 4.9.2 at rated carrier output power (Prated,c,AC, or Prated,c,TABC, D.21).

If the connector under test supports intra band contiguous or non-contiguous CA, set the representative connectors to transmit using the applicable test configuration and corresponding power setting specified in subclauses 4.7 and 4.8.

If the BS supports inter band CA, set the representative connectors to transmit, for each band, a single carrier or all carriers, using the applicable test configuration and corresponding power setting specified in subclauses 4.7 and 4.8.

For a connector declared to be capable of multi-carrier operation (D.15), set the BS to transmit according to the applicable test signal configuration and corresponding power setting specified in subclauses 4.7 and 4.8 using the corresponding test model in subclause 4.9.2 on all carriers configured.

4)	Measure the time alignment error between the different PDSCH demodulation reference signals on different antenna ports belonging to different connectorson the carrier(s) from the representative connectors under test.

5)	Repeat step 1 - 4 for any other configuration of connectors, which could be required to demonstrate compliance.

In addition, for multi-band connectors, the following steps shall apply:

6)	For a multi-band connectors and single band tests, repeat the steps above per involved operating band where single band test configurations and test models shall apply with no carrier activated in the other operating band.

<< next modified section >>

##### 6.6.3.4.2	Procedure

For BS type 1-H where there may be multiple TAB connectors, they may be tested one at a time or multiple TAB connectors may be tested in parallel as shown in annex D.1.1 for BS type 1-C or in annex D.3.1 for BS type 1-H. Whichever method is used the procedure is repeated until all TAB connectors necessary to demonstrate conformance have been tested.

1)	Connect the single-band connector or multi-band connector under test to measurement equipment as shown in annex D.1.1 for BS type 1-C and in annex D.3.1 for BS type 1-H. All connectors not under test shall be terminated.

The measurement device characteristics shall be:

-	Measurement filter bandwidth: defined in subclause 6.6.3.5.

-	Detection mode: true RMS voltage or true average power.

2)	For a connectors declared to be capable of single carrier operation only (D.16), set the representative connectors under test to transmit according to the applicable test configuration in subclause 4.8 using the corresponding test models NR-FR1-TM1.1 in subclause 4.9.2 at rated carrier output power Prated,c,AC for BS type 1-C and Prated,c,TABC for BS type 1-H (D.21).

For a connector under test declared to be capable of multi-carrier and/or CA operation (D.15-D.16) set the connector under test to transmit on all carriers configured using the applicable test configuration and corresponding power setting specified in subclauses 4.7 and 4.8 using the corresponding test models or set of physical channels in subclause 4.9.2.

3)	Measure ACLR for the frequency offsets both side of channel frequency as specified in table 6.6.3.5.2-1. In multiple carrier case only offset frequencies below the lowest and above the highest carrier frequency used shall be measured.

4)	For the ACLR requirement applied inside sub-block gap for non-contiguous spectrum operation, or inside Inter RF Bandwidth gap for multi-band operation:

a)	Measure ACLR inside sub-block gap or Inter RF Bandwidth gap as specified in subclause 6.6.3.5.2, if applicable.

b)	Measure CACLR inside sub-block gap or Inter RF Bandwidth gap as specified in subclause 6.6.3.5.2, if applicable.

5)	Repeat the test with the channel set-up according to NR-FR1-TM1.2 in subclause 4.9.2.

In addition, for multi-band connectors, the following steps shall apply:

6)	For a multi-band connectors and single band tests, repeat the steps above per involved operating band where single band test configurations and test models shall apply with no carrier activated in the other operating band.

<< next modified section >>

##### 6.6.4.4.2	Procedure

For BS type 1-H where there may be multiple TAB connectors, they may be tested one at a time or multiple TAB connectors may be tested in parallel as shown in annex D.3.1. Whichever method is used the procedure is repeated until all TAB connectors necessary to demonstrate conformance have been tested.

1)	Connect the single-band connector or multi-band connector under test to measurement equipment as shown in annex D.1.1 for BS type 1-C or in annex D.3.1 for BS type 1-H. All connectors not under test shall be terminated.

As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity, efficiency and avoiding e.g. carrier leakage, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

The measurement device characteristics shall be:

-	Detection mode: True RMS.

2)	For a connectors declared to be capable of single carrier operation only, set the representative connectors under test to transmit according to the applicable test configuration in subclause 4.8 at rated carrier output power Prated,c,AC for BS type 1-C and Prated,c,TABC for BS type 1-H (D.21). Channel set-up shall be according to NR-FR1-TM1.1.

For a connector under test declared to be capable of multi-carrier and/or CA operation set the connector under test to transmit on all carriers configured using the applicable test configuration and corresponding power setting specified in subclauses 4.7 and 4.8 using the corresponding test models or set of physical channels in subclause 4.9.2.

3)	Step the centre frequency of the measurement filter in contiguous steps and measure the emission within the specified frequency ranges with the specified measurement bandwidth. For connector under test declared to operate in multiple bands or non-contiguous spectrum, the emission within the Inter RF Bandwidth or sub-block gap shall be measured using the specified measurement bandwidth from the closest RF Bandwidth or sub block edge.

4)	Repeat the test for the remaining test cases, with the channel set-up according to NR-FR1-TM1.2.

In addition, for multi-band connectors, the following steps shall apply:

5)	For a multi-band connectors and single band tests, repeat the steps above per involved operating band where single band test configurations and test models shall apply with no carrier activated in the other operating band.

<< next modified section >>

##### 6.6.5.4.2	Procedure

For BS type 1-H where there may be multiple TAB connectors, they may be tested one at a time or multiple TAB connectors may be tested in parallel as shown in annex D.3.1. Whichever method is used the procedure is repeated until all TAB connectors necessary to demonstrate conformance have been tested.

1)	Connect the single-band connector or multi-band connector under test to measurement equipment as shown in annex D.1.1 for BS type 1-C and in annex D.3.1 for BS type 1-H. All connectors not under test shall be terminated.

2)	Measurements shall use a measurement bandwidth in accordance to the conditions in subclause 6.6.5.5.

The measurement device characteristics shall be:

-	Detection mode: True RMS.

3)	For a connectors declared to be capable of single carrier operation only (D.16), set the representative connectors under test to transmit according to the applicable test configuration in subclause 4.8 at rated carrier output power (Prated,c,AC, or Prated,c,TABC, D.21). Channel set-up shall be according to NR-FR1-TM1.1.

For a connector under test declared to be capable of multi-carrier and/or CA operation (D.15-D.16) set the connector under test to transmit on all carriers configured using the applicable test configuration and corresponding power setting specified in subclauses 4.7 and 4.8 using the corresponding test models or set of physical channels in subclause 4.9.2.

4)	Measure the emission at the specified frequencies with specified measurement bandwidth.

In addition, for multi-band connectors, the following steps shall apply:

5)	For a multi-band connectors and single band tests, repeat the steps above per involved operating band where single band test configurations and test models shall apply with no carrier activated in the other operating band.

#### 6.7.4.2	Procedure

For BS type 1-H where there may be multiple TAB connectors, they may be tested one at a time or multiple TAB connectors may be tested in parallel as shown in annex D.3.1. Whichever method is used the procedure is repeated until all TAB connectors necessary to demonstrate conformance have been tested.

1)	Connect the single-band connector or multi-band connector under test to measurement equipment as shown in annex D.1.2 for BS type 1-C and in annex D.3.2 for BS type 1-H. All connectors not under test shall be terminated.

2)	The measurement device characteristics shall be:

-	Detection mode: True RMS.

3)	For a connectors declared to be capable of single carrier operation only (D.16), set the representative connectors under test to transmit according to the applicable test configuration in subclause 4.8 at rated carrier output power Prated,c,AC for BS type 1-C and Prated,c,TABC for BS type 1-H (D.21). Channel set-up shall be according to NR-FR1-TM1.1.

For a connector under test declared to be capable of multi-carrier and/or CA operation (D.15-D.16) set the connector under test to transmit on all carriers configured using the applicable test configuration and corresponding power setting specified in subclauses 4.7 and 4.8 using the corresponding test models or set of physical channels in subclause 4.9.2.

4)	Generate the interfering signal according to NR-FR1-TM1.1, as defined in subclause 4.9.2, with the minimum channel bandwidth (BWChannel) with 15 kHz SCS of the band defined in subclause 5.3.5 of TS 38.104 [2] and a centre frequency offset from the lower/upper edge of the wanted signal or edge of sub-block inside a sub-block gap , for n = 1, 2 and 3, but exclude interfering frequencies that are outside of the allocated downlink operating band or interfering frequencies that are not completely within the sub-block gap or within the Inter RF Bandwidth gap.

5)	Adjust ATT attenuator (as in the test setup in annex D.1.2 for BS type 1-C and in annex D.3.2 for BS type 1-H) so that level of the interfering signal is as defined in subclause 6.7.5.

6)	Perform the unwanted emission tests specified in subclauses 6.6.3 and 6.6.4 for all third and fifth order intermodulation products which appear in the frequency ranges defined in subclauses 6.6.3 and 6.6.4. The width of the intermodulation products shall be taken into account.

7)	Perform the transmitter spurious emissions test as specified in subclause 6.6.5, for all third and fifth order intermodulation products which appear in the frequency ranges defined in subclause 6.6.5. The width of the intermodulation products shall be taken into account.

8)	Verify that the emission level does not exceed the required level in subclause 6.7.5 with the exception of interfering signal frequencies.

9)	Repeat the test for the remaining interfering signal centre frequency offsets according to step 4.

10)	Repeat the test for the remaining test signals defined in subclause 6.7.5 for additional requirements and for BS type 1-H intra-system requirements.

In addition, for multi-band connectors, the following steps shall apply:

11)	For a multi-band connectors and single band tests, repeat the steps above per involved operating band where single band test configurations and test models shall apply with no carrier activated in the other operating band.

NOTE:	The third order intermodulation products are centred at 2F1F2 and 2F2F1. The fifth order intermodulation products are centred at 3F12F2, 3F22F1, 4F1F2, and 4F2F1 where F1 represents the test signal centre frequency or centre frequency of each sub-block and F2 represents the interfering signal centre frequency. The widths of intermodulation products are:

-	(n*BWF1 + m* BWF2) for the nF1mF2 products;

-	(n* BWF2 + m* BWF1) for the nF2mF1 products;

where BWF1 represents the test wanted signal RF bandwidth or channel bandwidth in case of single carrier, or sub-block bandwidth and BWF2 represents the interfering signal channel bandwidth.

<< next modified section >>

#### 7.6.4.2	Procedure

The minimum requirement is applied to all connectors under test,

For BS type 1-H where there may be multiple TAB connectors they may be tested one at a time or multiple TAB connectors may be tested in parallel as shown in annex D.4.4. Whichever method is used the procedure is repeated until all TAB connectors necessary to demonstrate conformance have been tested.

1)	Connect the connector under test to measurement equipment as shown in annex D.2.6 for BS type 1-C and in annex D.4.4 for BS type 1-H.

2)	For separate RX only connectors with single carrier operation set the connector under test to transmit at manufacturers declared rated carrier output power (PRated,c,AC or PRated,c,TABC, D.21). Channel set-up shall be according to NR-FR1-TM1.1.

For separate RX only connectors declared to be capable of multi-carrier and/or CA operation (D.15-D.16) set the connector under test to transmit on all carriers configured using the applicable test configuration and corresponding power setting specified in subclauses 4.7 and 4.8 using the corresponding test models or set of physical channels in subclause 4.9.

For TDD connectors capable of transmit and receive ensure the transmitter is OFF.

3)	Set the measurement equipment parameters as specified in table 7.6.5.1-1.

4)	Measure the spurious emissions over each frequency range described in table 7.6.5.1-1.

In addition, for a multi-band connector, the following steps shall apply:

5)	For multi-band connector and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

<< next modified section >>

##### 8.1.2.3.3	Applicability of requirements for different channel bandwidths

Unless otherwise stated, for the subcarrier spacing to be tested, the tests shall apply only for anyone channel bandwidth declared to be supported (see D.14 in table 4.6-1).

<< next modified section >>