3GPP TSG-RAN4  #92bis																     	R4-1912370

Chongqing, China, 14th - 18th Oct 2019

Source: 	Qualcomm Incorporated

Title: 	PI/2 BPSK DMRS

Agenda item:	8.11.1

Document for:	Discussion

# Introduction

The introduction of Pi/2 BPSK modulation for PUSCH in Rel. 15 enabled the use of lower PAPR UL waveforms relative to QPSK and QAM. However, the full benefit of BPSK could not be realized due to the DMRS of these waveforms which used a higher PAPR Zadoff-Chu sequence. In [1] a WI was initiated to investigate the use of different modulation schemes for DMRS to lower its PAPR to be closer to that of the PUSCH Pi/2 BPSK modulation.

# Discussion

In [2,3,4,5] several agreements were made pertaining to DMRS using pi/2 BPSK modulation. Some of the major agreements are captured below:

- For PUSCH/PUCCH DMRS for pi/2 modulation, new DMRS sequences are specified in Rel.16 to reduce the PAPR to the same level as for data symbols

- For sequences with length 30 or larger, DMRS for ππ/2 BPSK modulation for PUSCH is generated based on Gold-sequence followed by π/2 BPSK modulation followed by transform precoding resulting in a DMRS Type 1 comb structure.

- For sequences with length 30 or larger, DMRS for π/2 BPSK modulation for PUCCH is generated based on Gold-sequence followed by π/2 BPSK modulation followed by transform precoding.

- For sequences with allocation length 12,18 and 24 CGS is used for DMRS for π/2 BPSK modulation in case of PUSCH and PUCCH

- For sequence with allocation length 6 CGS 8PSK modulation is used in case of PUSCH and PUCCH

- For the case of CGS sequence in case of two adjacent symbol DMRS, the same CGS sets are used for both single and two symbol DMRS for pi/2 BPSK modulation.

Similar to Rel. 15 the Gold code or CGS sequences after modulation are passed through a filter having an impulse response given in section 6.4.2.4.1 of TS38.101-1. A Slot can have up to 4 DMRS symbols.

The performance of two SCS=30kHz, BW=20MHz and RB allocations =[0 50] waveforms having all symbols of either Zadoff-Chu or Pi/2 BPSK DMRS are compared when passed through a typical sub6 PA.

The PAPRs of the 2 input waveforms are given below:

| Waveform | DMRS | Signal Bandwidth (MHz) | PAPR (dB) |
| --- | --- | --- | --- |
| 1 | Zadoff-Chu | 20 | 4.0 |
| 2 | Pi/2 BPSK | 20 | 2.5 |

Each of these waveforms were created by filtering the modulated DMRS/PUSCH data with a 3-tap filter having coefficients [-0.26 0.93 -0.26] prior to sub-carrier mapping. The Pi/2 BPSK DMRS waveform was created using a Gold code sequence of length 31.

The PA ACLR, EVM, SEM and spur performance at the UE antenna is given below:

|  | Zadoff-Chu |  |  |  |  | Pi/2 BPSK |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pwr Antenna (dBm) | DMRS EVM (dBc) | ACLR (dB) | SEM margin (dB) | Spur margin (dB) | Pwr Antenna (dBm) | DMRS EVM (dBc) | ACLR (dB) | SEM margin (dB) | Spur margin (dB) |
| 16.1 | -29.9 | 43.3 | 23.4 | 31.0 | 16.1 | -45.3 | 51.2 | 31.8 | 47.2 |
| 18.0 | -30.2 | 43.3 | 21.9 | 29.6 | 18 | -46.4 | 52.3 | 31.0 | 44.4 |
| 19.8 | -30.2 | 43.0 | 19.9 | 24.4 | 19.9 | -48.3 | 54.1 | 30.2 | 46.0 |
| 21.6 | -26.5 | 39.0 | 13.8 | 18.1 | 21.8 | -45.7 | 51.4 | 26.6 | 39.9 |
| 23.1 | -20.3 | 32.7 | 5.6 | 16.1 | 23.5 | -37.5 | 43.3 | 17.0 | 34.8 |
| 24.1 | -14.2 | 26.8 | -1.6 | 9.0 | 24.8 | -30.7 | 36.7 | 8.9 | 27.9 |
| 24.6 | -10.9 | 23.6 | -5.0 | 4.6 | 25.3 | -24.3 | 30.8 | 2.1 | 15.9 |
| 24.7 | -9.8 | 22.4 | -6.2 | 0.7 | 25.4 | -24 | 30.4 | 1.6 | 15.8 |

The ACLR was compared to a specification of 30dBc, the EVM to a specification of 30% while the SEM and spur levels were compared against the values in TS38.101-1 tables 6.5.2.2.-1 and 6.5.3.1-2 respectively. Figure 1 illustrates the EVM for the DMRS symbols for increasing PA power while figure 2 illustrates the DMRS ACLR performance. As can be seen for a given PA power the DMRS EVM is ~10dB lower with Pi/2 BPSK modulation compared to Zadoff-Chu modulation while the ACLR is also ~5-7dB larger for Pi/2 BPSK. For this PA the Zadoff-Chu waveform fails the ACLR spec for power levels greater than 23.5dBm.

Figure 1 – EVM for Pi/2 BPSK and Zadoff-Chu DMRS

Figure 2 – ACLR for Pi/2 BPSK and Zadoff-Chu DMRS

Figure 3 and 4 shows the impact of DMRS EVM and ACLR as a function of the PA power for a 3-tap filter, [-c 1 -c] where the coefficient, c, is varied from -0.05 to -0.3.  For all coefficients the EVM pass the spec while the ACLR fails for the lower filter tap coefficients at power levels exceeding ~24.5dBm.

Figure 3 – EVM for Pi/2 BPSK as a function of 3 tap filter coefficients [-c 1 -c]

Figure 4 – ACLR for Pi/2 BPSK as a function of 3 tap filter coefficients [-c 1 -c]

The EVM and ACLR performance of a SCS=120kHz waveform having a BW=100MHz and RB allocation of [0 64] shaped using a 3 tap filter=[-0.26 0.93 -0.26] having all DMRS symbols in a slot was compared for both Pi/2 BPSK and Zadoff-Chu modulations as a function of the PA output power when passed through a typical mmWave PA. The results given in figure 5 indicate that the waveform having a Pi/2 BPSK modulated DMRS gives approximately a 15dB improvement in EVM compared with the Zadoff-Chu modulated signal. It is seen that the Zadoff-Chu waveform fails the EVM spec at power levels exceeding 23dBm. Also, figure 6 shows that the ACLR of the Pi/2 BPSK signal is ~8-10dB better than that of the Zadoff-Chu signal.

Figure 5 – EVM for Pi/2 BPSK and Zadoff-Chu DMRS

Figure 6 – ACLR for Pi/2 BPSK and Zadoff-Chu DMRS

Conclusion: The waveform having Pi/2 BPSK modulated DMRS has better EVM and ACLR performance compared with the Zadoff-Chu modulated DMRS waveform.

In RAN1 there will be UE signaling capability bit that indicates the support of Pi/2 BPSK DMRS. When this bit is set to 1 the eNB will use RRC signaling to indicate to the UE whether to use Rel-15 Zadoff-Chu DMRS or Pi/2 BPSK. When set to 0 only Zadoff-Chu DMRS will be signaled. Benefit of this new DMRS should result in  better UE EVM and ACLR capability and hence lower interference and better transmit signal quality.

Conclusion: The waveform having Pi/2 BPSK modulated DMRS has better EVM and ACLR performance compared with the Zadoff-Chu modulated DMRS waveform.

# Conclusion

The newly defined DMRS created using Pi/2 BPSK modulation was simulated with Pi/2 BPSK PUSCH data and was seen to have better EVM and ACLR compared to a similar waveform having a Zadoff-Chu modulated DMRS both for  sub-6 and mmWave PAs.

Conclusion: The waveform having Pi/2 BPSK modulated DMRS has better EVM and ACLR performance compared with the Zadoff-Chu modulated DMRS waveform.

# Reference

- RP-182863, “Revised WID: Enhancements on MIMO for NR”, RAN1#82, Sorrento, Italy, December 2018

- RAN1 95 Chairman’s notes, Spokane, USA, November 2018

- RAN1 1901 Ah meeting Chairman’s notes, Taipei, Taiwan, January 2019

- RAN1 96 meeting Chairman’s notes, Athens, Greece, March 2019

- RAN1 96b meeting Chairman’s notes, Xi’an, China, April 2019