3GPP TSG-RAN Working Group 4 (Radio) meeting #92bis	R4-1912381

Chongqing, China, 14th– 18th Oct, 2019

Title:	Draft reply LS on AGC settling time and RB size of PSFCH

Response to:	R1-1909922

Release:	Release 16

Work Item:	5G_V2X_NRSL-Core

Source:	3GPP RAN WG4

To:	3GPP RAN WG1

Cc:

Contact Person:

#### Name:	Markus Pettersson

E-mail Address:	markus.pettersson@lge.com

Attachments:	None.

1. Overall Description:

RAN WG4 already sent the reply LS on the NR V2X RF parameters to RAN1 in [2], [3] and [4]. This LS is answering the questions regarding PSFCH waveforms in R1-1909922 [5], where

RAN1 respectfully asks RAN4 to provide feedback on the AGC settling time for the reception of PSFCH and the minimum RB size required for it.

The related RAN1 agreement is as follows:

Agreements: - A sequence-based PSFCH format with one symbol (not including AGC training period) is supported. - This is applicable for unicast and groupcast including options 1/2. - Sequence of PUCCH format 0 is the starting point. - FFS: 1 PRB or multiple PRBs is/are used for this PSFCH format - FFS: feasible number of HARQ-ACK bits, mapping of HARQ-ACK bit

RAN4 has studied the AGC settling time for PSFCH signals generated from low PAPR sequences with length of 12 and 24, which are transmitted in 1 or 2 RBs. The results indicate that AGC settling time similar to 10RB wide CP-OFDMA signal can be achieved with PSFCH signals that are 2 RBs wide.

- AGC settling time for PSFCH can be:

  - For single CC using PSFCH sequences transmitted in 2 RBs

    - 35 usec for 15 kHz SCS

    - 35 usec for 30 kHz SCS

    - 18 usec for 60 kHz SCS

  - PSFCH12 transmitted in interleaved manner in 2 RBs provides AGC vice better performance than PSFC24 transmitted in 2 RBs.

  - RAN4 has not studied the multicarrier cases.

2. Actions:

To RAN WG1:

RAN WG4 respectfully asks RAN WG1 to evaluate the NR V2X sidelink operation taking into account the above information.

3. Date of Next TSG-RAN Meetings:

3GPP RAN WG4 #93	    18th – 22th November 2019				Reno, Nevada, US

Reference

[1] R1-1810006 “LS on IBE model for V2X” RAN WG1

[2] R4-1816666, “Reply LS on NR V2X RF parameters for RAN1 physical layer design aspects” RAN WG4

[3] R4-1905241, “reply LS on NR V2X UE RF parameters for NR V2X service” RAN WG4

[4] R4-1906062, “reply LS on NR V2X UE RF parameters for NR V2X service” RAN WG4

[5] R1-1909922, “LS on AGC settling time and RB size of PSFCH”, LG Electronics, RAN1 #98, August 2019