3GPP TSG-RAN4  #92bis															     	R4-1912392

Chongqing, China, 14th - 18th Oct 2019

Source: 	Qualcomm Incorporated

Title: 	Variable duplexer spacing

Agenda item:		9.20.1

Document for:	Approval

# Background

In RAN #92 4 new FDD bands were created using existing SUL and SDL bands n75, n75, n81 and n82. These bands denoted as n91, n92, n93, and n94 can have variable duplexer spacing and support asymmetric channel BW operation on the UL and DL.

# Discussion

In [1] the minimum TX-RX separations and the asymmetric BWs that should be supported by these bands were calculated. As the UL and DL frequencies can be set independently the TX-RX separation has a frequency range as calculated below:

| NR Operating Band | UL Frequency Range (MHz) | DL Frequency Range (MHz) | Minimum supported channel BW (MHz) | Minimum TX-RX separation (MHz) | Maximum TX-RX separation (MHz) |
| --- | --- | --- | --- | --- | --- |
| n91 | 832 – 862 | 1427 - 1432 | 5 | 570 | 595 |
| n92 | 832 - 862 | 1432 - 1517 | 5 | 575 | 680 |
| n93 | 880 - 915 | 1427 - 1432 | 5 | 517 | 547 |
| n94 | 880 - 915 | 1432 - 1517 | 5 | 522 | 632 |

In the above table the minimum and maximum TX-RX separations are calculated using a minimum BW of 5MHz.

These new bands should be included in TS38.101-1 section 5.4.4 in table 5.4.4-1 as follows

Table 5.4.4-1: Default UE TX-RX frequency separation

| NR Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| n1 | 190 MHz |
| n2 | 80 MHz |
| n3 | 95 MHz |
| n5 | 45 MHz |
| n7 | 120 MHz |
| n8 | 45 MHz |
| n12 | 30 MHz |
| n20 | -41 MHz |
| n25 | 80 MHz |
| n28 | 55 MHz |
| n66 | 400 MHz |
| n70 | 295,3001 MHz |
| n71 | -46 MHz |
| n74 | 48 MHz |
| n91 | 582.5MHz2 |
| n92 | 627.5MHz3 |
| n93 | 532MHz4 |
| n94 | 577MHz5 |
| NOTE 1: Default TX-RX carrier centre frequency separation. NOTE 2: Range of TX-RX frequency separation 570-595MHz NOTE 3: Range of TX-RX frequency separation 575-680MHz NOTE 4: Range of TX-RX frequency separation 517-547MHz NOTE 5: Range of TX-RX frequency separation 522-632MHz |  |

Observation: Bands n91, n92, n93 and n94 can have a range of TX-RX frequency separations

Proposal 1: Bands n91, n92, n93 and n94 should be included in table 5.4.4-1 with default TX-RX frequency separations

If asymmetric BWs are used, then section 5.3.6 of 38.101-1 states:

In asymmetric channel bandwidth operation, the narrower carrier shall be confined within the frequency range of the wider channel bandwidth. In FDD, the confinement is defined as a deviation to the default Tx-Rx carrier center frequency separation (defined in table 5.4.4-1) as following:

ΔFTX-RX = | (BWDL – BWUL)/2 |

Also, in TS38.101-1 section 5.3.6 the BW combinations for these new bands should be added to table 5.3.6-1 as follows:

Table 5.3.6-1: FDD asymmetric UL and DL channel bandwidth combinations

| NR Band | Channel bandwidths for UL (MHz) | Channel bandwidths for DL (MHz) |
| --- | --- | --- |
| n66 | 5, 10 | 20, 40 |
|  | 20 | 40 |
| n70 | 5 | 10, 15 |
|  | 5, 10, 15 | 20, 25 |
| n71 | 5 | 10 |
|  | 10 | 15 |
|  | 15 | 20 |
| n91 | 10 | 5 |
| n92 | 5 | 10,15,20 |
|  | 10 | 15,20 |
| n93 | 10 | 5 |
| n94 | 5 | 10,15,20 |
|  | 10 | 15,20 |

Proposal 2: For bands n91, n92, n93 and n94 the TX-RX separation should be modified for asymmetric UL/DL BWs using the above formula for ΔFTX-RX

For bands with variable duplex capability the reference sensitivity for the default TX-RX spacings should be defined in the table below

Table 7.3.2-1: Two antenna case reference sensitivity QPSK PREFSENS for variable duplex bands (default TX-RX spacing)

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) | Duplex Mode |
| n91 | 15 | [-100.0] |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n92 | 15 | [-100.0] | [-96.8] | [-95.0] | [-93.8] |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | [-97.1] | [-95.1] | [-94.0] |  |  |  |  |  |  |  |  |  |
|  | 60 |  | [-97.5] | [-95.4] | [-94.2] |  |  |  |  |  |  |  |  |  |
| n93 | 15 | [-100.0] |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n94 | 15 | [-100.0] | [-96.8] | [-95.0] | [-93.8] |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | [-97.1] | [-95.1] | [-94.0] |  |  |  |  |  |  |  |  |  |
|  | 60 |  | [-97.5] | [-95.4] | [-94.2] |  |  |  |  |  |  |  |  |  |

Proposal 3: For bands with variable duplex the reference sensitivities with the default TX-RX spacing should be defined for each SCS and BW

Also, for bands with variable duplex capability the worst-case reference sensitivity should be defined in the table below and the corresponding TX-RX spacing should be noted.

Table 7.3.2-2: Two antenna port worst case reference sensitivity QPSK PREFSENS for variable duplex bands

|  | Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) | TX-RX spacing (MHz) | Duplex Mode |
| n91 | 15 | [-100.0] |  |  |  |  |  |  |  |  |  |  |  | 570 | FDD |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n92 | 15 | [-100.0] | [-96.8] | [-95.0] | [-93.8] |  |  |  |  |  |  |  |  | 575 | FDD |
|  | 30 |  | [-97.1] | [-95.1] | [-94.0] |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  | [-97.5] | [-95.4] | [-94.2] |  |  |  |  |  |  |  |  |  |  |
| n93 | 15 | [-100.0] |  |  |  |  |  |  |  |  |  |  |  | 517 | FDD |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n94 | 15 | [-100.0] | [-96.8] | [-95.0] | [-93.8] |  |  |  |  |  |  |  |  | 522 | FDD |
|  | 30 |  | [-97.1] | [-95.1] | [-94.0] |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  | [-97.5] | [-95.4] | [-94.2] |  |  |  |  |  |  |  |  |  |  |

Proposal 4: For bands with variable duplex the worst-case reference sensitivities should be defined for each SCS and BW

# Conclusion

This contribution outlines how to introduce new variable duplex bands into TS38.101-1 and makes the following observations and proposals.

Observation: Bands n91, n92, n93 and n94 have a range of TX-RX frequency separations

Proposal 1: Bands n91, n92, n93 and n94 should be included in table 5.4.4-1 with default TX-RX frequency separations

Proposal 2: For bands n91, n92, n93 and n94 the TX-RX separation should be modified for asymmetric UL/DL BWs using the above formula for ΔFTX-RX

Proposal 3: For bands with variable duplex the reference sensitivities with the default TX-RX spacing should be defined for each SCS and BW

Proposal 4: For bands with variable duplex the worst-case reference sensitivities should be defined for each SCS and BW

# Reference

[1] R4-1908736, “UE RF requirements analysis for variable duplex”, Huawei, HiSilicon, Vodafone, Orange, 3GPP TSG RAN WG4 Meeting #92, Ljubljana, Slovenia, 26 - 30 October, 2019