3GPP TSG-RAN WG4 Meeting #92-Bis               	R4-1912416

Chongqing, China, October 14 – 18, 2019

Source:	Nokia, Nokia Shanghai Bell

Title:	System-level simulation results for RSTD in NR

Agenda item:	8.8.2.1.1

Document for:	Discussion

Introduction

At RAN4 #92, work for the NR positioning Rel-16 work item [1] was started. Agreement was found in [2] to specify RRM requirements for RSTD in RRC_CONNECTED based on DL PRS. Simulation assumptions and performance metrics for system level simulations, in order to derive RSRP and SINR distributions for the best N cells (N ≥ 5) for subsequent link level simulations, were agreed in [3].

This contribution lists our preliminary simulation results according to the agreed simulation setup in [3].

Simulation assumptions

Simulation assumptions and performance metrics agreed in [3] are replicated in this section. The simulation setup aligns to the NR positioning study item in RAN1 [4]. The simulation results in section 3 refer to a subset of these simulation assumptions, as depicted at the end of this section.

Table 1: Common scenario parameters applicable for all scenarios

|  | FR1 Specific Values | FR2 Specific Values |
| --- | --- | --- |
| Carrier frequency, GHz | 2 GHz, 4 GHz – Note 1 | 30 GHz – Note 1 |
| Bandwidth, MHz | 5 MHz, 50 MHz for 2 GHz 100 MHz for 4 GHz | 100 MHz, 400 MHz |
| Subcarrier spacing, kHz | 15 kHz for 5 MHz and 50 MHz 30 kHz for 100 MHz | 120 kHz |
| gNB model parameters |  |  |
| gNB noise figure, dB | 5 dB | 7 dB |
| UE max. TX power, dBm | 23 dBm – Note 1 | 23 dBm – Note 1 EIRP should not exceed 43 dBm. |
| UE model parameters |  |  |
| UE noise figure, dB | 9 dB – Note 1 | 13 dB – Note 1 |
| UE antenna configuration | Panel model 1 – Note 1 Mg = 1, Ng = 1, P = 2, dH = 0.5λ, (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) | Multi-panel Configuration 1 and Panel Configuration a – Note 1 - Multi-panel Configuration 1: (Mg, Ng) = (1, 2); Θmg,ng=90°; Ω0,1=Ω0,0+180°; (dg,H, dg,V)=(0,0) - Panel Configuration a: - Each antenna array has shape dH=dV=0.5λ - Config a: (M, N, P) = (2, 4, 2), - the polarization angles are 0° and 90° - The antenna elements of the same polarization of the same panel is virtualized into one TXRU |
| UE antenna radiation pattern | Omni, 0 dBi | Antenna model according to Table 2 |
| PHY/link level abstraction | Explicit simulation of all links, individual parameters estimation is applied. Companies to provide description of applied algorithms for estimation of signal location parameters. |  |
| Note 1: According to 3GPP TR 38.802 Note 2: According to 3GPP TR 38.901 |  |  |

Table 2: UE antenna radiation pattern model 1 (FR2)

| Parameter | Values |
| --- | --- |
| Antenna element radiation pattern in θ'' dim (dB) |  |
| Antenna element radiation pattern in φ'' dim (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 5 dBi |
|  | Note: are in local coordinate system. |

Table 3: Scenario 1. Indoor office scenario parameters

|  | FR1 Specific Values | FR2 Specific Values |
| --- | --- | --- |
| Layout | Indoor floor: (12 BSs per 120 m x 50 m), TRP number per floor:12, Inter-gNB distance = 20 m - Note 1 |  |
| Total gNB TX power, dBm | 24 dBm | 24 dBm EIRP should not exceed 58 dBm |
| gNB antenna configuration | (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), dH=dV=0.5λ – Note 1 | (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), dH=dV=0.5λ – Note 1 One TXRU per polarization per panel is assumed |
| gNB antenna radiation pattern | Single sector – Note 1 | 3-sector antenna configuration – Note 1 |
| Channel model | Indoor open office – Note 2 |  |
| Peneteration loss | 0 dB |  |
| Number of floors | 1 |  |
| UE drop procedure | 100% indoor, uniformly distributed over the horizontal area |  |
| UE mobility | 3 km/h |  |
| UE antenna height | 1.5 m |  |
| Min gNB-UE distance (2D), m | 0 m |  |
| gNB antenna height | 3 m |  |
| Note 1: According to 3GPP TR 38.802 Note 2: According to 3GPP TR 38.901 |  |  |

Table 4: Scenario 2: Urban micro (UMi) scenario parameters

|  | FR1 Specific Values | FR2 Specific Values |
| --- | --- | --- |
| Layout | Hexagonal grid, 19 or 7 macro sites, 3 sectors per site, ISD = 200 m, Note 1 Wrap-around is applied, Note 2 |  |
| Total gNB TX power, dBm | 44dBm | 37dBm per panel EIRP should not exceed 73 dBm |
| gNB antenna configuration | (M, N, P, Mg, Ng) = (8, 8, 2, 1, 1), (dH, dV) = (0.5, 0.8)λ, - Note 1 | (M, N, P, Mg, Ng) = (4, 8, 2, 2, 2), (dH, dV) = (0.5, 0.5)λ, (dg,H,dg,V) = (4.0, 2.0)λ, - Note 1 |
| gNB antenna radiation pattern | Directional, 8dBi – Note 1 Table 6.1.1-5 | Directional, 8dBi – Note 1 Table 6.1.1-5 |
| Channel model | UMi Street Canyon – Note 3 |  |
| Penetration loss | For outdoor UEs: 0dB |  |
| Number of floors | All UEs are on the ground. |  |
| UE drop procedure | 100% outdoor uniformly distributed over the horizontal area |  |
| UE mobility (for modeling Doppler effects) | Outdoor: 3km/h |  |
| UE height, m | 1.5m |  |
| Min. gNB-UE distance (2D), m | 10m |  |
| gNB antenna height | 10 m by default. Companies can bring results with uniform distribution [5-20]m​ |  |
| Note 1: According to 3GPP TR 38.802 Note 2: In case if interference considerations are not properly taken into account for 7 sites companies are encouraged to provide results for 19 sites. Note 3: According to 3GPP TR 38.901 |  |  |

Table 5: gNB antenna patterns for UMi/UMa

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) |  |
| Antenna element horizontal radiation pattern (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 8dBi |

Table 6: Scenario 3: Outdoor macro (UMa) scenario parameters

|  | FR1 Specific Values |
| --- | --- |
| Layout | Hexagonal grid, 3 sectors per site, 7 or 19 macro sites, ISD = 500m – Note 1 Wrap-around is applied. Note 2 |
| Total gNB TX power, dBm | 49dBm |
| gNB antenna configuration | (M, N, P, Mg, Ng) = (8, 8, 2, 1, 1), (dH, dV) = (0.5, 0.8)λ – Note 1 Applicable for 2GHz and 4 GHz carrier frequency. |
| gNB antenna radiation pattern | Directional, 8dBi – Note 1, Table 6.1.1-5 |
| Channel model | UMa scenario – Note 3 |
| Penetration loss | For outdoor UEs: 0dB For indoor UEs: 20dB+0.5d2D-in – Note 3 |
| Number of floors, (floor height) | 8, (3m) |
| Antenna Height: | Uniformly distributed [20-50] m – Note 4 25m + α, where α~uniform[-5, 25] |
| UE Height | 3(nfl – 1) + 1.5 m – Note 4 where, nfl ~ uniform(1,Nfl) and Nfl = 8 |
| UE dropping procedure | 50% indoor and 50% outdoor uniformly distributed over the horizontal area (separate statistic) |
| Min. gNB-UE distance (2D), m | 35m |
| UE mobility (for modeling Doppler effects) | For indoor UEs: 3km/h For outdoor UEs: 60km/h |
| Note 1: According to 3GPP TR 38.802 Note 2: In case if interference considerations are not properly taken into account for 7 sites companies are encouraged to provide results for 19 sites. Note 3: According to 3GPP TR 38.901 Note 4: According to 3GPP TR 37.857 |  |

Existing channel models as provided in the TR 38.901 are used for NR positioning evaluation as a baseline.

For performance metrics, at least RSRP and SINR distributions for each of the first, second, …, Nth best cells (e.g., N>=5) should be presented.

In this contribution, RSRP distributions for the best 5 cells for the above three scenarios are evaluated for the following setting:

- FR1 (4 GHz), Bandwidth: 100 MHz, SCS: 30 kHz

- Network layout for Urban Micro / Outdoor Macro: 7 sites

- Simulation length: 2500 subframes, RSRP is averaged over all subframes

- simulated UE’s: ~ 300

Simulation results

Simulation results for the above described three scenarios and above listed setting are depicted in this section.

## 3.1 	Scenario 1: Indoor Office

The CDF curves for RSRP of the 5 strongest cells for Scenario 1 are depicted in Figure 1.

Figure 1: RSRP CDF curves for 5 strongest cells (Scenario 1).

The calculated median levels are the following ones: -70 dBm, -74 dBm, -76 dBm, -78 dBm and -80 dBm.

## 3.2 Scenario 2: Urban Micro

The CDF curves for RSRP of the 5 strongest cells for Scenario 2 are depicted in Figure 2.

Figure 2: RSRP CDF curves for 5 strongest cells (Scenario 2).

The calculated median levels are the following ones: -79 dBm, -87 dBm, -93 dBm, -97 dBm and -100 dBm.

## 3.3 	Scenario 3: Outdoor Macro

The CDF curves for RSRP of the 5 strongest cells for Scenario 3 are depicted in Figure 3.

Figure 3: RSRP CDF curves for 5 strongest cells (Scenario 3).

The calculated median levels are the following ones: -94 dBm, -101 dBm, -107 dBm, -110 dBm and -114 dBm.

Discussion

The above depicted results provide the RSRP distributions for the strongest 5 cells for indoor office, urban micro and outdoor macro scenarios in view of using appropriate power levels for link-level simulations for RSTD in NR. The following proposal is made:

- Proposal: 	Take into account the system level simulation results reported in section 3 in the RAN4 discussion.

Further system-level simulation results with increased number of simulated UE’s and including SINR distributions for the above three scenarios are planned to be contributed to the next meeting.

Conclusion

This contribution lists our preliminary system-level simulation results for RSRP distributions for the strongest 5 cells for indoor office, urban micro and outdoor macro scenarios in view of using appropriate power levels for link-level simulations for RSTD in NR. The following proposal is made:

- Proposal: 	Take into account the system level simulation results reported in section 3 in the RAN4 discussion.

References

- RP-191156, "Revised WID: NR Positioning Support", source: Intel, Ericsson

- R4-1910561, "On RAN4 work scope for NR positioning", source: Intel

- R4-1910002, "Time plan and system-level assumptions for RSTD simulations in NR", source: Ericsson

- 3GPP TR 38.855, "Study on NR positioning support", Rel-16