3GPP TSG-RAN WG4 Meeting #92bis	R4-1912431

# Chongqing, China, 14-18 October 2019

Source:	Sprint, SouthernLINC, Dish Network

Title:	n26 Emissions requirements

Agenda item:	9.23.1

Document for:	Approval

# 1	Introduction

At RAN#85 a new WID for the creation of NR band n26 was approved [1]. This document discusses the band specific emissions requirements for n26.

# 2	Discussion

The channel bandwidths and SCS for n26 are as follows as per the WID:

Table 1 SCS for n26

| NR Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| --- | --- | --- | --- | --- | --- |
| n26 | 15 | Yes | Yes | Yes | Yes |
|  | 30 |  | Yes | Yes | Yes |
|  | 60 |  |  |  |  |

Any emission requirements and A-MPR simulations should use these channel bandwidths.

Observation 1: The spurious emissions for UE coexistence requirements for n26 are as follows in 36.101, Table 6.6.3.2-1:

| 26 | E-UTRA Band 1, 2, 3, 4, 5, 10, 11, 12, 13, 14, 17, 18,19, 21, 24, 25, 26, 29, 30, 31, 34, 39, 40, 42, 43, 48, 50, 51, 53, 65, 66, 70, 71, 73,74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | E-UTRA Band 41 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 703 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 15 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |

Proposal 1: Re-use the LTE Band 26 spurious emissions for UE coexistence requirements for n26.

For LTE band 26 there are 4 NS values for additional spectrum emissions requirements. NS_12 and NS_13 cover the same protection level, but different channel bandwidths. For NR they can be combined into one NS value.

Observation 2: The additional emissions requirements LTE band 26 are as follows with the addition of a range indicator in the first column:

| Range 1 | 806 ≤ f ≤ 813.5 | -42 | 6.25 kHz |
| --- | --- | --- | --- |
| Range 2 | 806 ≤ f ≤ 816 | -42 | 6.25 kHz |
| Range 3 | 851 ≤ f ≤ 859 | -53 | 6.25 kHz |

Range 1 is for the protection of the narrowband public safety uplink in the southeastern United States. Range 2 is for the protection of the narrow band public safety uplink in the US other than the southeast. Range 3 is for4 protection of the public safety downlink in 851-859 MHz.

Proposal 2: Re-use the LTE Band 26 additional emissions requirements for n26.

For A-MPR simulations it would be best to capture at a minimum, the primary operator frequency allocation scenarios based on the studies in 37.806 [2].

Primary scenarios of interest:

Range 1 (806 ≤ f ≤ 813.5) additional emissions requirements:

- 5 MHz carrier in 814-819 MHz (SouthernLINC)

- 10 MHz carrier in 814-824 MHz (SouthernLINC)

Range 2 (806 ≤ f ≤ 816) additional emissions requirements:

- 5 MHz carrier in 817-822 MHz (Sprint)

- 5 MHz carrier in 819-824 MHz (Sprint)

- 5 MHz carrier between 824 and 849 MHz

- 10 MHz carrier between 824 and 849 MHz

- 15 MHz carrier between 824 and 849 MHz

- 20 MHz cattier between 824 and 849 MHz

Range 3 (851 ≤ f ≤ 859) additional emissions requirements:

- 5 MHz carrier between 814 and 824 MHz

- 5 MHz carrier between 824 and 849 MHz

- 10 MHz carrier in 814-824 MHz

- 10 MHz carrier in 825-835 MHz (Region 2 A block)

- 10 MHz carrier in 835-845 MHz (Region 2 B block)

- 10 MHz carrier in 839-849 MHz

- 15 MHz carrier in 824-839 MHz

- 15 MHz carrier in 834-849 MHz

- 20 MHz carrier in 824-844 MHz

- 20 MHz carrier in 829-849 MHz

Proposal 3: A-MPR simulations and tables for protected Range 1, Range 2 and Range 3 should at a minimum cover and be optimized for scenarios 1-18 listed above.

# 3	Conclusions

Observation 1: The spurious emissions for UE coexistence requirements for n26 are as follows in 36.101, Table 6.6.3.2-1:

| 26 | E-UTRA Band 1, 2, 3, 4, 5, 10, 11, 12, 13, 14, 17, 18,19, 21, 24, 25, 26, 29, 30, 31, 34, 39, 40, 42, 43, 48, 50, 51, 53, 65, 66, 70, 71, 73,74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | E-UTRA Band 41 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 703 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 15 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |

Observation 2: The additional emissions requirements LTE band 26 are as follows with the addition of a range indicator in the first column:

| Range 1 | 806 ≤ f ≤ 813.5 | -42 | 6.25 kHz |
| --- | --- | --- | --- |
| Range 2 | 806 ≤ f ≤ 816 | -42 | 6.25 kHz |
| Range 3 | 851 ≤ f ≤ 859 | -53 | 6.25 kHz |

Proposal 1: Re-use the LTE Band 26 spurious emissions for UE coexistence requirements for n26.

Proposal 2: Re-use the LTE Band 26 additional emissions requirements for n26.

Primary scenarios of interest:

Range 1 (806 ≤ f ≤ 813.5) additional emissions requirements:

- 5 MHz carrier in 814-819 MHz (SouthernLINC)

- 10 MHz carrier in 814-824 MHz (SouthernLINC)

Range 2 (806 ≤ f ≤ 816) additional emissions requirements:

- 5 MHz carrier in 817-822 MHz (Sprint)

- 5 MHz carrier in 819-824 MHz (Sprint)

- 5 MHz carrier between 824 and 849 MHz

- 10 MHz carrier between 824 and 849 MHz

- 15 MHz carrier between 824 and 849 MHz

- 20 MHz cattier between 824 and 849 MHz

Range 3 (851 ≤ f ≤ 859) additional emissions requirements:

- 5 MHz carrier between 814 and 824 MHz

- 5 MHz carrier between 824 and 849 MHz

- 10 MHz carrier in 814-824 MHz

- 10 MHz carrier in 825-835 MHz (Region 2 A block)

- 10 MHz carrier in 835-845 MHz (Region 2 B block)

- 10 MHz carrier in 839-849 MHz

- 15 MHz carrier in 824-839 MHz

- 15 MHz carrier in 834-849 MHz

- 20 MHz carrier in 824-844 MHz

- 20 MHz carrier in 829-849 MHz

Proposal 3: A-MPR simulations and tables for protected Range 1, Range 2 and Range 3 should at a minimum cover and be optimized for scenarios 1-18 listed above.

# 4	References

- RP-192273, “New WID on Introduction of NR Band n26,” Sprint, AT&T, Dish Network

- TR 37.306, “Extending 850 MHz study Technical Report,” 3GPP