3GPP TSG-RAN WG4 Meeting #92bis	R4-1912448

Chongqing, China, 14th– 18th October 2019

Agenda item:	9.16.2	BS RF (38.104)	[NR_n259-Core]

Source: 		NTT DOCOMO, INC.

Title: 	BS RF requirement and test tolerance for n259

Document for:	Approval

- Introduction

In RAN4#92, the impact to BS RF requirements due to the introduction of n259 were discussed. However, some aspects related to BS RF were missing. In this contribution, we further investigate on the expected impact to the BS RF specifications for the introduction of n259.

2.	Discussion

According to revised WID [1], BS core and conformance requirements are included in the objective of this work item as below.

Core part

The purpose of this work item is to specify a new NR TDD operating band, i.e. Band n259, covering at least the range 39.5-43.5 GHz taking into account the potential benefit of an overlap with Band n260. The lower limit of the band will be further analysed and decided by considering spectrum allocations in different regions, operator demands and RF implementation but should not exceed 39.5 GHz.

The objectives are to define:

- Determine the lower limit of Band n259 if lower than 39.5 GHz.

- Transmitter and receiver characteristics requirements for the UE (38.101-2)

- Transmitter and receiver characteristics requirements for the BS (38.104)

Performance part

The objectives are to define:

- The new band to band group for RRM measurement accuracy requirements

- Conformance requirements for BS (38.141)

- Changes to be added to release independence TS 38.307

Observation 1: WI objectives include defining BS transmitter and receiver core and conformance requirements for the introduction of n259.

In RAN4 #91, it was agreed to define the frequency range of n259 from 39.5-43.5GHz (FDL,high – FDL,low = FUL_high – FUL_low = 4000 MHz) [2]. And required changes for general sections were clarified and agreed as below [3]. However, the necessity of changes for BS transmitter and receiver core and conformance requirements has not been discussed so far.

Table 1: Required changes in TS 38.104 for n259

| Section | Requirement | Required changes in TS 38.104 |
| --- | --- | --- |
| 5.2 | Operating bands | Operating band of n259 shall be added in to Table Table 5.2-2. |
| 5.3.5 | BS channel bandwidth per operating band | Channel bandwidths and corresponding SCS for n259 shall be added in Table 5.3.5-2. |
| 5.4.2.3 | Channel raster entries for each operating band | Applicable NR-ARFCN for n259 shall be added into Table 5.4.2.3-2 once the lower frequency limit for n259 is determined. |
| 5.4.3.3 | Synchronization raster entries for each operating band | Applicable SS raster entries for n259 shall be added into Table 5.4.3.3-2 once the lower frequency limit for n259 is determined. |

Observation 2: The impact to BS transmitter and receiver core and conformance requirements has not been discussed.

In the following sections, we discuss the necessity of changes for core and conformance specifications based on the objectives agreed in WID.

2.1	Changes for core specification

Table 1 summarizes the frequency ranges that apply to the requirements of each transmitter and receiver.

Table 1: Summary of the necessity of changes for BS RF conformance specification

| Section (TS 38.104) | Requirement | Frequency range of operating band applicable to core requirement (NOTE 1) | Necessity to be updated for introduction of n259 |
| --- | --- | --- | --- |
| Radiated transmitter characteristics |  |  |  |
| 9.2 | Radiated transmit power | Not mentioned | No |
| 9.3 | OTA base station output power | Not mentioned | No |
| 9.4.3 | OTA total power dynamic range | Not mentioned | No |
| 9.5.2 | OTA transmitter OFF power | Refer to OTA transient period | No |
| 9.5.3 | OTA transient period | Not mentioned | No |
| 9.6.1 | OTA frequency error | Not mentioned | No |
| 9.6.2 | OTA modulation quality | Not mentioned | No |
| 9.6.3 | OTA time alignment error | Not mentioned | No |
| 9.7.1 | General for OTA unwanted emissions | Not mentioned (However the definition of ΔfOBUE is not enough) | Yes (Need to update the definition of ΔfOBUE) |
| 9.7.2 | OTA occupied bandwidth | Not mentioned | No |
| 9.7.3 | OTA Adjacent Channel Leakage Power Ratio (ACLR) | 24.25 – 33.4 GHz 37 – 52.6 GHz | No |
| 9.7.4 | OTA operating band unwanted emissions | 24.25 – 33.4 GHz 37 – 52.6 GHz | No |
| 9.7.5 | OTA transmitter spurious emissions | Not mentioned (Step frequency for Cat.B does not cover n259) | Need to check (Need to add n259 to table 9.7.5.3.2.3-2 if necessary) |
| Radiated receiver characteristics |  |  |  |
| 10.2 | OTA sensitivity | Not mentioned | No |
| 10.3 | OTA reference sensitivity level | Not mentioned | No |
| 10.5.1 | OTA adjacent channel selectivity | 24.25 – 33.4 GHz 37 – 52.6 GHz | No |
| 10.5.2 | OTA in-band blocking | Not mentioned (However the definition of ΔfOOB is not enough) | Yes (Need to update the definition of ΔfOOB) |
| 10.6 | OTA out-of-band blocking | Not mentioned | No |
| 10.7 | OTA receiver spurious emissions | Not mentioned | No |
| 10.8 | OTA receiver intermodulation | Not mentioned | No |
| 10.9 | OTA in-channel selectivity | Not mentioned | No |
| NOTE 1: “Not mentioned” imply applicable to 24.25 – 52.6GHz. |  |  |  |

In our analysis, existing core requirements cover frequency range of n259. However, the definition of ΔfOBUE and ΔfOOB need to be updated for the following reason.

- ΔfOBUE and ΔfOOB

In TS 38.104, ΔfOBUE and ΔfOOB are defined as below.

Table 9.7.1-1: Maximum offset ΔfOBUE outside the downlink operating band

| BS type | Operating band characteristics | ΔfOBUE (MHz) |
| --- | --- | --- |
| BS type 1-O | FDL,high – FDL,low < 100 MHz | 10 |
|  | 100 MHz ≤ FDL,high – FDL,low ≤ 900 MHz | 40 |
| BS type 2-O | FDL,high – FDL,low ≤ 3250 MHz | 1500 |

Table 10.5.2.3-0: ΔfOOB offset for NR operating bands in FR2

| BS type | Operating band characteristics | ΔfOOB (MHz) |
| --- | --- | --- |
| BS type 2-O | FUL_high – FUL_low ≤ 3250 MHz | 1500 |

As agreed in [2], the frequency range definition of n259 is from 39.5-43.5GHz and FDL,high – FDL,low = FUL_high – FUL_low = 4000 MHz. On the other hand, the current definition covers up to 3250 MHz operating bandwidth and it is not enough for n259. Therefore, in TS 38.104, the definition of ΔfOBUE and ΔfOOB shall be updated. In addition, corresponding changes shall be done for other BS RF specification (i.e., TS38.141-2).

Proposal 1: In TS 38.104, RAN4 updates the definition of ΔfOBUE and ΔfOOB for the introduction of band n259.

Furthermore, RAN4 need to further check whether to add n259 to the table for step frequency for category B spurious emissions.

- Step frequency for category B spurious emissions requirement

In TS 38.104, Step frequency for category B spurious emissions requirement are defined as below.

Table 9.7.5.3.2.3-2: Step frequencies for defining the BS radiated Tx spurious emission limits in FR2 (Category B)

| Operating band | Fstep,1 [GHz] | Fstep,2 [GHz] | Fstep,3 [GHz] (Note 2) | Fstep,4 [GHz] (Note 2) | Fstep,5 [GHz] | Fstep,6 [GHz] |
| --- | --- | --- | --- | --- | --- | --- |
| n258 | 18 | 21 | 22.75 | 29 | 30.75 | 40.5 |
| NOTE 1: Fstep,X are based on ERC Recommendation 74-01 [19], Annex 2. NOTE 2: Fstep,3 and Fstep,4 are aligned with the values for ΔfOBUE in Table 9.7.1-1. |  |  |  |  |  |  |

If band n259 is necessary in category B region, the definition of step frequency for n259 shall be added.

Proposal 2: In TS 38.104, RAN4 adds n259 to table 9.7.5.3.2.3-2 to define step frequency corresponding to the operating band if necessary.

2.2	Changes for conformance specification

Table 1 summarizes the frequency ranges that apply to the test requirements, test system uncertainty and test tolerance of each transmitter and receiver requirements.

Table 2: Summary of the necessity of changes for BS RF conformance specification

| Section (TS 38.141-2) | Requirement | Frequency range of operating band applicable to test requirement (NOTE 1) | Frequency range applicable to test system uncertainty (MU) for test requirements (Sec. 4.1.2) (NOTE 1) | Frequency range applicable to test tolerance (TT) for test requirements (Annex C) (NOTE 1) | Necessity to update for introduction of n259 |
| --- | --- | --- | --- | --- | --- |
| Radiated transmitter characteristics |  |  |  |  |  |
| 6.2 | Radiated transmit power | 24.15 GHz < f ≤ 29.5 GHz 37 GHz < f ≤ 40 GHz | 24.25 – 29.5 GHz 37 – 40 GHz | 24.15 GHz < f ≤ 29.5 GHz 37 GHz < f ≤ 40 GHz | Yes |
| 6.3 | OTA base station output power | 24.15 GHz < f ≤ 29.5 GHz 37 GHz < f ≤ 40 GHz | 24.25 – 29.5 GHz 37 – 40 GHz | 24.15 GHz < f ≤ 29.5 GHz 37 GHz < f ≤ 40 GHz | Yes |
| 9.4.3 | OTA total power dynamic range | Not mentioned | Not mentioned | Not mentioned | No |
| 6.5.1 | OTA transmitter OFF power | Refer to OTA transient period | 24.25 – 29.5 GHz 37 – 40 GHz | 24.15 GHz < f ≤ 29.5 GHz 37 GHz < f ≤ 40 GHz | Yes |
| 6.5.2 | OTA transient period | 24.15 GHz < f ≤ 29.5 GHz 37 GHz < f ≤ 40 GHz | - | - | Yes |
| 6.6.2 | OTA frequency error | Not mentioned | Not mentioned | Not mentioned | No |
| 6.6.3 | OTA modulation quality | Not mentioned | Not mentioned | Not mentioned | No |
| 6.6.4 | OTA time alignment error | Not mentioned | Not mentioned | Not mentioned | No |
| 6.7.1 | General for OTA unwanted emissions | Not mentioned (However the definition of ΔfOBUE is not enough) | - | - | Yes (Need to update the definition of ΔfOBUE) |
| 6.7.2 | OTA occupied bandwidth | Not mentioned | Not mentioned | Not mentioned | No |
| 6.7.3 | OTA Adjacent Channel Leakage Power Ratio (ACLR) | 24.25 – 33.4 GHz 37 – 52.6 GHz | 24.25 – 29.5 GHz 37 – 40 GHz | 24.15 GHz < f ≤ 29.5 GHz 37 GHz < f ≤ 40 GHz | Yes |
| 6.7.4 | OTA operating band unwanted emissions | 24.25 – 33.4 GHz 37 – 52.6 GHz | 24.25 – 29.5 GHz 37 – 40 GHz | 24.15 GHz < f ≤ 29.5 GHz 37 GHz < f ≤ 40 GHz | Yes |
| 6.7.5 | OTA transmitter spurious emissions | Not mentioned (Step frequency for Cat.B does not cover n259) | Not mentioned | Not mentioned | Need to check (Need to add n259 to Table 6.7.5.2.5.2.3-2 if necessary) |
| Radiated receiver characteristics |  |  |  |  |  |
| 7.2 | OTA sensitivity | Not mentioned |  | - | No |
| 7.3 | OTA reference sensitivity level | Not mentioned | 24.15 GHz < f ≤ 29.5 GHz 37 GHz < f ≤ 40 GHz | 24.25 GHz < f ≦ 33.4 GHz 37 GHz < f ≦ 52.6 GHz | Yes |
| 7.5.1 | OTA adjacent channel selectivity | 24.25 – 33.4 GHz 37 – 52.6 GHz | 24.15 GHz < f ≤ 29.5 GHz 37 GHz < f ≤ 40 GHz | Not mentioned | Yes |
| 7.5.2 | OTA in-band blocking | Not mentioned (However the definition of ΔfOOB is not enough) | 24.15 GHz < f ≤ 29.5 GHz 37 GHz < f ≤ 40 GHz | Not mentioned | Yes (Need to update the definition of ΔfOOB) |
| 7.6 | OTA out-of-band blocking | Not mentioned | Not mentioned | Not mentioned | No |
| 7.7 | OTA receiver spurious emissions | Not mentioned | Not mentioned | Not mentioned | No |
| 7.8 | OTA receiver intermodulation | Not mentioned | 24.15 GHz < f ≤ 29.5 GHz 37 GHz < f ≤ 40 GHz | Not mentioned | Yes |
| 7.9 | OTA in-channel selectivity | Not mentioned | 24.15 GHz < f ≤ 29.5 GHz 37 GHz < f ≤ 40 GHz | 24.25 GHz < f ≦ 33.4 GHz 37 GHz < f ≦ 52.6 GHz | Yes |
| NOTE 1: “Not mentioned” imply applicable to 24.25 – 52.6GHz. |  |  |  |  |  |

There are four cases where there are problems when introducing n259 as the follows:

Case 1: 	Test requirement and corresponding MU and TT do not support the frequency range of n259 (highlighted in blue).

- Radiated transmit power

- OTA base station output power

- OTA transmitter OFF power/OTA transient period

Case 2:	Test requirement supports the frequency range of n259 but corresponding MU and TT do not support the range (highlighted in yellow).

- OTA ACLR

- OTA OBUE

- OTA ACS

Case 3:	MU supports the frequency range of n259 but TT does not support the range(highlighted in green).

- OTA reference sensitivity level

- OTA in-channel selectivity

Case 4:	MU does not support the frequency range of n259 (highlighted in gray).

- OTA in-band blocking

- OTA receiver intermodulation

For case 1(highlighted in blue), the test requirements and corresponding MU and TT are defined in the frequency ranges 24.25 – 29.5 GHz and 37 – 40 GHz. To support n259, upper frequency limit shall be expanded to at least 43.5GHz. In general, the MU may increase as the frequency increases, and the MU affected by the change in frequency is mainly the MU of the test equipment (signal generator, spectrum analyzer, network analyzer). However, if we take different MU value for 40-43.5GHz, the different test requirements will be applied to n259 corresponding to the frequency allocation regardless of the same operating band. In order to avoid confusion, the same MU of the test equipment might be used for the frequency range of n259. Therefore, the existing MU and TT shall be used for at least up to 43.5GHz. Whether to be able to expand up to 52.6GHz or not might be discussed in TEI15 if necessary.

For case 2 (highlighted in yellow), the requirements have been already defined up to 52.6GHz and covered n259. For alignment with the test requirements, the upper frequency limit for MU and TT shall be revisited to 52.6GHz.

For case 3 (highlighted in green), there is no alignment between MU and TT. Basically, TT is defined based on MU and TT=MU can be applied for these requirements. Therefore, the upper frequency limit for TT shall be revisited to 52.6GHz.

For case 4(highlighted in gray), a similar approach can be used since these requirements are TT = 0 and only MU value should be discussed. From the same reason of case 1, the existing MU shall be used for at least up to 43.5GHz. Whether to be able to expand up to 52.6GHz or not might be discussed in TEI15 if necessary.

Proposal 3: The existing test requirements, test system uncertainty (MU) and test tolerance (TT) shall be applied to n259 (39.5 GHz – 43.5 GHz).

- ΔfOBUE and ΔfOOB

Same issue as TS 38.104 can be seen in TS 38.141-2.

Proposal 4: In TS 38.141-2, RAN4 updates the definition of ΔfOBUE and ΔfOOB for the introduction of band n259.

- Step frequency for category B spurious emissions requirement

Same issue as TS 38.104 can be seen in TS 38.141-2.

Proposal 5: In TS 38.141-2, RAN4 adds n259 to Table 6.7.5.2.5.2.3-2 to define step frequency corresponding to the operating band if necessary.

3.	Conclusion

In this contribution, we further analyse the necessity to update TS 38.104 and TS 38.141-2 for the introduction of n259.

Observation 1: Objectives include defining BS transmitter and receiver core and conformance requirements for the introduction of n259.

Observation 2: The impact to BS transmitter and receiver core and conformance requirements has not been discussed.

Proposals for TS 38.104:

Proposal 1: In TS 38.104, RAN4 updates the definition of ΔfOBUE and ΔfOOB for the introduction of band n259.

Proposal 2: In TS 38.104, RAN4 adds n259 to table 9.7.5.3.2.3-2 to define step frequency corresponding to the operating band if necessary.

Proposals for TS 38.141-2:

Proposal 3: The existing test requirements, test system uncertainty (MU) and test tolerance (TT) shall be applied to n259 (39.5 GHz – 43.5 GHz).

Proposal 4: In TS 38.141-2, RAN4 updates the definition of ΔfOBUE and ΔfOOB for the introduction of band n259.

Proposal 5: In TS 38.141-2, RAN4 adds n259 to Table 6.7.5.2.5.2.3-2 to define step frequency corresponding to the operating band if necessary.

References

- RP-191880, “Revised WID:  Introduction of NR band n259”, Ericsson, RAN #85, September 2019

- R4-1906861,	“On frequency range for NR band n259”, Huawei, HiSilicon, RAN4 #91, May 2019

- R4-1906862,	“BS RF requirements for n259”, Huawei, HiSilicon, RAN4 #91, May 2019