3GPP TSG-RAN4 Meeting #92-bis	R4-1912458

Chongqing , CN

Source: 	Qualcomm Incorporated, Nokia, Ericsson, Nokia Shanghai Bell

Title: 	Location of transient period when a UE declares it’s capability

Agenda item:	6.5.4.4

Document for:	Approval

# Introduction

In [1], the WF lists the concerns from different companies related to transient period capability. One of the concerns were to show the location of the transient period when a UE declares a capability shorter than the allowed transient period.

In this paper we address the same.

# Discussion

In the current standard [2], there are three types of transient periods:

- Transient period is symmetrically shared across the slot boundary between two symbols:

2. Transient period is located on one side of a slot boundary:

3. Blanking is allowed for 60 kHz SCS when there is a transient period on both sides of a symbol

When a UE declares a capability of a shorter transient period than allowed then:

Proposal 1: Currently if a transient period is symmetrically shared, it will still be symmetrically shared

Proposal 2: Currently if a transient period is on one side of a slot boundary, it will still on that side of the slot boundary

Proposal 3: Currently if blanking is allowed then transient period will be symmetrically shared, and blanking will be removed.

Proposal 4: The value of the transient period in the above Proposals 1-3 will now be the transient period value that the UE supports according to its UE capability or minimum UE requirements

# Conclusion

This paper addresses the concern of the location of the transient period when a UE indicates its capability for transient period in Proposals 1-4.

Proposal 1: Currently if a transient period is symmetrically shared, it will still be symmetrically shared

Proposal 2: Currently if a transient period is on one side of a slot boundary, it will still on that side of the slot boundary

Proposal 3: Currently if blanking is allowed then transient period will be symmetrically shared, and blanking will be removed.

Proposal 4: The value of the transient period in the above Proposals 1-3 will now be the transient period value that the UE supports according to its UE capability or minimum UE requirements

# References

[1] R4-1910312, WF on Transient period capability, 3GPP TSG-RAN WG4 Meeting #92, Ljubljana, Slovenia, 26-30 Aug 2019

[2] 3GPP TS 38.101-1