3GPP TSG-RAN4 Meeting #92-bis	R4-1912462

Chongqing , CN

Source: 	Qualcomm Incorporated, Ericsson, Nokia, Nokia Shanghai Bell

Title: 	Values for Transient period capability

Agenda item:	6.5.4.4

Document for:	Approval

# Introduction

Transient period capability has been discussed widely in RAN4 for the past year. In this paper, we discuss the values a UE could choose to report for its capability. These were also discussed in [1].

# Discussion

In the past few meetings, there were different proposals for the values of the capability. In our opinion, a few discrete values need to be selected and it might be reasonable to choose values relative to CP duration. Alternatively, to save the number of bits for signaling UE could signal its improved capability of 2 usec or less only.

For FR1, 60 kHz SCS has the smallest CP duration of approximately 1 usec. SCS of 30 kHz and 15 kHz have a CP duration of approximately 2.5 usec and 5 usec respectively.

Proposal 1: The options for improved Transient period capability to choose from are: 1 usec, 2 usec or 4 usec.

Alternative Proposal 1: 2 usec (to reduce the number of bits needed for signaling)

Proposal 2: This capability should be defined per band.

# Conclusion

In this paper, we propose the values for the transient period capability that a UE could use to report in Proposals 1. We believe that this value should be band dependent to allow the UE vendors to pick and choose the PAs for each band based on their requirements.

# References

[1] R4-1909740, Transient period capability, 3GPP TSG-RAN4 Meeting #92, Ljubljana , SI