3GPP TSG-RAN WG4 Meeting #92bis	R4-1912464

Chongqing, P.R. China 14th – 18th October 2019

Source: 	Skyworks Solutions, Inc.

Title: 	Input to WF on Region 1 Extension for Power Class 3 Operation in FR2 and Output Power Boosting

Agenda Item: 			8.14.9 Improvement of UE MPR  [NR_RF_FR2_req_enh]

Document for:	Approval

# 1	Introduction

In previous RAN4 meeting a WF [1] was approved aiming to study possible 0 dB MPR region extension from current Region 1 to an inner 1/3 FR1 like MPR concept [2]. Companies were also invited to evaluate the concept of power boosting by removing In-Band Emissions (IBE) as gating factor [3]. This paper presents and update of our measurements results only for power boosting concept since this experimental data set was that which was used to make original proposal of 0dB MPR Region 1 extension for PC3 operation in FR2 [2,4].

# 2	Discussion

## 2.1	Region 1 Extension for Power Class 3 Operation

For 0dB MPR Region 1 extension proposal, no further measurements are needed as we believe C-IM3 terms are not likely to impact the observations and proposals made in [2,4]. As pointed in [1], these measurements results seem to be in good agreement with simulation data from [5,6,7].

Proposal 1: The Region 1 0dB MPR for PC3 operation in FR2 at channel bandwidth less or equal to 200MHz is extended as described in [1].

## 2.2	Output Power Boosting

Output power boosting concept relies on the fact that if IBE gating factor is omitted, it should be possible to boost UE transmit power for RB allocations in the actual 0 dB MPR Region 1. This is justified by the fact that for these RB allocations, once IBE is ignored, the next gating factor should be EVM for Pi/2 BPSK and for QPSK modulation schemes, for which there usually remains a margin with regards to the requirement specifications. In this section, we present the measurement results for possible output power boosting in case IBE is omitted on the restricted set of data presented in [4].

Test measurement conditions are [4]:

Reference 0 dB MPR PA calibration point is that agreed in [1], ie BW = 100 MHz, SCS = 120 KHz, DFT-S-OFDM QPSK, 20RB23 for which all gating factors are considered, including IBE. We use the following assumptions:

- Power class 3 UE

- Carrier frequency: 24 300 MHz

- Channel BW = 100 MHz, SCS = 120 kHz

- Modulation: QPSK

- Waveforms: DFT-S-OFDM

- Carrier suppression 25 dB, calibrated for each waveform,

- Image suppression 25 dB, calibrated for each waveform,

- RB allocation test points (TP)

  - DFT-S-OFDM test points:1RB0, 1RB23, 5RB5, 10RB10, 20RB20, 20RB23, 32RB16, 50RB8, 64RB0. RB allocations corresponding to new 0dB MPR inner 1/3 Region 1 proposal are 1RB23, 5RB5, 10RB10, 20RB20, 20RB23.

For each TP, the NR transmit power level, denoted PNR is increased and measurement is stopped as soon as one of the following gating factor fails:

- ACLR according to 38.101-2

- EVM according to 38.101-2,

- In-Band Emissions (IBE): accounted for in 1st measurement results, then IBE is ignored in second set of data according to [1],

- Occupied bandwidth limit according to 38.101-2,

Power levels of the second harmonic are also systematically captured for each power sweep.

Measurement test points are summarised in Figure 1 below (orange RB allocations with black dots).

Figure 1: DFT-S-OFDM test points (orange with black dot) vs actual Edge allocation (yellow ‘E’) vs actual MPRnarrow (blue ‘N’) vs actual 0dB MPR inner Region 1 allocation (‘R1’ within red contour) vs new inner 1/3 0dB MPR Region 1 extension proposal [1] (green with black contour) vs LCrb vs RBstart for the example of PC3 operation at 100MHz channel bandwidth and SCS 120 kHz. 0 dB MPR reference waveform is 20RB23.

We reproduce in Table 1 below measurements from [4] which confirm that for the measurement points corresponding to the new proposed 0dB MPR Region 1 contour, i.e. for 1RB23, 5RB5, 10RB10, 20RB20:

- IBE is indeed the limiting factor,

- A small margin is available compared to the 0dB MPR reference test point.

It is worth noting that this contour seems to provide sufficient margin for 32RB16 and 50RB8 since they are also IBE limited despite being located within the actual “Edge” region.

Table 1: Gating Factor and Measured MPR Taking All Gating Factors into Account

|  | QPSK DFT-S-OFDM |  | Measurement results |  |
| --- | --- | --- | --- | --- |
|  | LCRB | RBstart | Back-off vs 0dB ref. | Gator |
| new inner 1/3 Region 1 | 1 | 23 | -0.6 | IBE |
|  | 5 | 5 | -0.5 | IBE |
|  | 10 | 10 | -0.2 | IBE |
|  | 20 | 20 | -0.3 | IBE |
|  | 20 | 23 | 0dB ref. | IBE |
| Other RB allocations | 1 | 0 | -1.9 | EVM |
|  | 32 | 16 | -0.3 | IBE |
|  | 50 | 8 | 0.7 | IBE |
|  | 64 | 0 | 0.7 | OBW |

Observation 1: IBE is the limiting gating factor for selected TP fulfilling the new 0dB MPR inner 1/3 Region 1 extension proposal [1]. Other points in the “Edge” allocation regions are also IBE limited, indicating that new Region 1 proposal provides sufficient margin.

In Table 2, we present the measured power boosting that can be achieved if IBE is removed for the TP fulfilling new 0 dB MPR Region1 proposal. In these measurements, the 20RB23 0 dB MPR reference waveform calibration is maintained as currently defined, in this case it is IBE limited. The measured power boosting ranges from 1.6 to 2.5 dB for QSPK DFT-S-OFDM waveforms. Further studies are needed to confirm if boosting is also applicable to Pi/2 BPSK waveforms. Considering the data set is very small compared the large number of RB allocation combinations, these results should be considered only as a way to verify that the 1 dB power boosting proposed in [3] seems to be feasible. In any case, if boosting was to be adopted, boosting level could only be based on adopting the minimum boosting level across all RB allocations eligible to the new Region 1 extension proposal.

Table 2: Gating Factor and Measured MPR Ignoring IBE Gating Factor for Inner 1/3 TP

|  | QPSK DFT-S-OFDM |  | Measurement results |  |
| --- | --- | --- | --- | --- |
|  | LCRB | RBstart | Back-off vs 0dB ref. | Gator |
| new inner 1/3 Region 1 | 1 | 23 | -1.8 | EVM |
|  | 5 | 5 | -2.5 | EVM |
|  | 10 | 10 | -2.4 | EVM |
|  | 20 | 20 | -1.6 | EVM |
|  | 20 | 23 | 0dB ref. | IBE |

It should be noted that the 0 dB MPR waveform is also eligible to power boosting since it is IBE limited. The measurements indicate a 1.5 dB power boosting for 20RB23 QPSK DFT-S-OFDM.

Observation 2: For QPSK DFT-S-OFDM RB allocations which are eligible to the new 0 dB MPR Region 1 contour [1], the 1dB power boosting proposed in [3] seems to be feasible.

Proposal 2: Power Boosting Concept is applicable for QPSK DFT-S-OFDM RB allocations eligible to the new 0 dB MPR Region 1 extension proposal. Applicability for Pi/2 BPSK is FFS.

# 3	Conclusion

In this contribution we have presented measurements results for which we believe Region 1 extension proposal is still valid and based on these we propose:

Proposal 1: The Region 1, 0 dB MPR for PC3 operation in FR2 at channel bandwidth less or equal to 200MHz is extended as described in [1].

As for the power boosting concept, based on our results we observe that:

Observation 1: IBE is the limiting gating factor for the selected TP fulfilling the new 0 dB MPR inner 1/3 Region 1 extension proposal [1]. Other points in the “Edge” allocation regions are also IBE limited, indicating that new Region 1 proposal provides sufficient margin.

Observation 2: For QPSK DFT-S-OFDM RB allocations which are eligible to the new 0 dB MPR Region 1 contour [1], the 1 dB power boosting proposed in [3] seems to be feasible.

And therefore propose:

Proposal 2: Power Boosting Concept is applicable for QPSK DFT-S-OFDM RB allocations which are eligible to the new 0 dB MPR Region 1 extension proposal. Applicability for Pi/2 BPSK is FFS.

# 4	References

[1] R4-1910280, WF on Region 1 Extension for Power Class 3 Operation in FR2, Skyworks Solutions Inc., Nokia, Qualcomm, RAN4#92

[2] R4-1909945	Inner RB Allocations Region 1 Extension for Power Class 3 Operation in FR2, R4 #92, Skyworks Solutions Inc.

[3] R4-1909927	Power Boost and IBE in FR2, R4 #92, Qualcomm.

[4]	R4-1907163	FR2 PC3 MPR PA Measurements for DFT-S-OFDM and CP-OFDM for Rel-16, Skyworks Solutions Inc.

[5] R4-1903524 FR2 MPR due to OBW, R4 #90 bis Xi’an China, Nokia, Nokia Shanghai Bell .

[6]	R4-1902848 On FR2 PC1 MPR and OBW, R4 #90 bis Xi’an China, Qualcomm.

[7]	R4-1902930 On PC1 MPR and CA MPR, R4 #90 bis Xi’an China, Intel.