3GPP TSG-RAN WG4 Meeting #92bis	R4-1912479

Chongqing, China, 14 – 18 October 2019

Source:	Nokia, Nokia Shanghai Bell

Title:	BS demodulation simulation results of PUSCH for Rel-16 LTE HST

Agenda item:	7.13.4.1

Document for:	Approval

# 1	Introduction

During the RAN4#92 meeting [1], the way forward on BS demodulation requirement for HST enhancement [1] was approved, which captured some agreements and open issues. The open issues are simulation evaluation of PUSCH and PRACH for HST speed of 500 km/h.

This document addresses the PUSCH simulation evaluation issue. At the last RAN#4 meeting, no agreement was reached despite several contributions on simulation results for the PUSCH performance. A companion document [2] provides a detailed analysis as to what causes the difference in the contributed PUSCH simulation results. In addition, the companion document proposes to consider the DM-RS of PUCCH in the simulation as a means to boost BS demodulation performance for HST instead of an implementation-dependent Doppler compensation algorithm. In this document, simulation results based the proposal are presented.

# 2	Discussion

From [1], the agreed initial simulation assumptions and HST deployment scenarios for evaluation of the PUSCH performance for HST are provided in Tables 1 and 2, respectively.

Table 1: Simulation assumptions [1]

| Parameter | Unit | Value |
| --- | --- | --- |
| Bandwidth | MHz | 10 |
| MCS for PUSCH |  | QPSK 1/3 |
| Propagation condition and correlation matrix |  | HST scenarios, see Table 2 |
| Antenna configuration |  | 1x2 |
| Reference receiver |  | MRC |
| Noise estimation |  | Practical |
| Time and frequency track |  | Practical |

Table 2: HST deployment scenarios [1]

| Parameter | Value |  |
| --- | --- | --- |
|  | Open space | Tunnel |
| Ds | 1000 m | 300 m |
| Dmin | 50 m | 2 m |
| v | 500 km/h | 500 km/h |
| fd | 1944 Hz, 1894 Hz, 1844 Hz, 1794 Hz, 1744 Hz, 1694 Hz |  |

According to [2], the DM-RS structure of PUCCH Format 2/2a/2b/3 is assumed in the PUSCH simulation. As such, the theoretical limit for the Doppler shift is 1750 Hz instead of 1 kHz as in the case of PUSCH.

Figure 1 shows the simulation results for PUSCH. For the open space scenario, Doppler shifts fd = 1894 Hz and 1944 Hz fail to achieve 70% of maximum throughput while Doppler shift = 1844 Hz achieves 70% of maximum throughput at SNR = 0 dB. For the tunnel scenario, Doppler shift fd = 1844 Hz fails to achieve 70% throughput while Doppler shift fd = 1794 Hz achieves 70% of maximum throughput at SNR = 2 dB. Note, both Doppler shifts are above the theoretical limit.

From TS 36.101 in subsection 6.5.1, the UE frequency error is ±0.1 PPM, which is not taken into account in the simulation. Hence, the maximum Doppler shift for specifying PUSCH performance requirements should account for the UE frequency error and other implementation margins.

Observation 1: The maximum Doppler shift should take into account the UE frequency error = ±0.1 PPM and other implementation margins.

Proposal 1: For the HST open space scenario, it is proposed to adopt the maximum Doppler shift fd=1744 Hz for QPSK 1/3 and 500 km/h.

Proposal 2: For the HST tunnel scenario, it is proposed to adopt maximum Doppler shift fd=1694 Hz for QPSK 1/3 and 500 km/h.

Figure 1: Simulation results for HST open space and tunnel scenarios for bi-directional channel model, QPSK 1/3 at 500 km/h

# 3	Conclusions

Simulation results for BS demodulation for HST open space and tunnel scenarios under bi-directional model have been presented. As a result, the following observation and proposal can be made:

Observation 1: The maximum Doppler shift should take into account the UE frequency error = ±0.1 PPM and other implementation margins.

Proposal 1: For the HST open space scenario, it is proposed to adopt the maximum Doppler shift fd=1744 Hz for QPSK 1/3 and 500 km/h.

Proposal 2: For the HST tunnel scenario, it is proposed to consider Doppler shift fd=1694 Hz for QPSK 1/3 and 500 km/h.

# References

- R4-1910102, Way forward on LTE BS demodulation performance requirements, NTT DoCoMo

- R4-1912478, On PUSCH simulation results for Rel-16 LTE HST, Nokia, Nokia Shanghai Bell

- R4-1909887, BS demodulation simulation results of PUSCH for Rel-16 LTE HST, Nokia, Nokia Shanghai Bell