3GPP TSG-RAN WG4 Meeting #92bis	R4-1912502

# Chongqing, China, 14- 18 October, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | --- | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on demodulation performance requirements for EN-DC including FR1 and FR2 CCs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-10-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There is no demodulation performance requirements specified for EN-DC including both FR1 and FR2 NR CCs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify FR1-FR2 EN-DC demodulation performance requirements following the existing principles to define EN-DC (only verifying the NR CC performance) and FR1-FR2 band combination (only verifying FR2 NR CC performance) performance requirements. In that way the UE supporting EN-DC including FR1 NR and FR2 NR CCs shall be verified under the EN-DC fallback mode including FR1 NR CC only or FR2 NR CC only against the corresponding EN-DC performance requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The demodulation performance requirements for EN-DC including both FR1 and FR2 CCs are missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes>>

9.2	PDSCH Demodulation

9.2A	PDSCH demodulation for CA

9.2A.1	NR CA between FR1 and FR2

(Void)

9.2B	PDSCH demodulation for DC

9.2B.1	EN-DC

9.2B.1.1	EN-DC within FR1

9.2B.1.1.1	PDSCH

The test setup for E-UTRA PCell is specified in Section 9.1.2. The NR PDSCH demodulation performance requirements for NR are specified in Section 5.2. During the test, only the PDSCH performance on the NR cell(s) shall be verified.

9.2B.1.2	EN-DC including FR2 NR carrier only

9.2B.1.2.1	PDSCH

The test setup for E-UTRA PCell is specified in Section 9.1.2. The NR PDSCH demodulation performance requirements for NR are specified in Section 7.2. During the test, only the PDSCH performance on the NR cell(s) on FR2 carriers shall be verified.

9.2B.1.3	EN-DC including FR1 and FR2 NR carriers

The test setup for E-UTRA PCell is specified in Section 9.1.2. During the test, only the PDSCH performance on the NR cell(s) shall be verified. The performance on FR1 carriers and FR2 carriers shall be verified separately. The test coverage is considered fulfilled, if the demodulation performance requirements specified in Section 9.2B.1.1 for EN-DC within FR1 and the demodulation performance requirements specified in Section 9.2B.1.2 for EN-DC including FR2 NR carrier only are executed separately for UE under test in the fallback mode of supported EN-DC configurations including FR1 and FR2 NR carriers.

9.2B.2	NR DC between FR1 and FR2

9.3	PDCCH demodulation

9.3A	PDCCH demodulation for CA

9.3A.1	NR CA between FR1 and FR2

(Void)

9.3B	PDCCH demodulation for DC

9.3B.1	EN-DC

9.3B.1.1	EN-DC within FR1

9.3B.1.1.1	PDCCH

The test setup for E-UTRA PCell is specified in Section 9.1.2. The NR PDCCH demodulation performance requirements for NR are specified in Section 5.3. During the test, only the PDCCH performance on the single NR cell shall be verified.

9.3B.1.2	EN-DC including FR2 NR carrier only

9.3B.1.2.1	PDCCH

The test setup for E-UTRA PCell is specified in Section 9.1.2. The NR PDCCH demodulation performance requirements are specified in Section 7.3. During the test, only the PDCCH performance on the single NR cell shall be verified.

9.3B.1.3	EN-DC including FR1 and FR2 NR carriers

The test setup for E-UTRA PCell is specified in Section 9.1.2. During the test, only the PDCCH performance on the NR cell(s) shall be verified. The performance on FR1 carriers and FR2 carriers shall be verified separately. The test coverage is considered fulfilled, if the demodulation performance requirements specified in Section 9.3B.1.1 for EN-DC within FR1 and the demodulation performance requirements specified in Section 9.3B.1.2 for EN-DC including FR2 NR carrier only are executed separately for UE under test in the fallback mode of supported EN-DC configurations including FR1 and FR2 NR carriers.

9.3B.2	NR DC between FR1 and FR2

<< End of changes >>