3GPP TSG-RAN4 Meeting #92-Bis	R4-1912546

# Chongqing, China, 10th – 14th October, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | 5550 | rev | - | Current version: | 15.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 36.101 rel. 15 to fix Out-of-band Blocking issue for bands 38, 41 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm, Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15 |  |  |  |  |  | Date: |  |  | 2019-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Bands 38 and 41 have been designed to be used as a single band using the same RF hardware. However, OOBB has been defined in such a way that an extra filter would be required for band 38 fulfil OOBB. The CR corrects this issue for UEs supporting both bands 38 and 41 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding note specifying that for the FDL_high and FDL_low of band 41 applies to band 38 for UEs supporting both bands 38 and 41. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Supporting band 38 requires additional hardware effort and might not be supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.6.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.521 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <Start of Changes>

#### 7.6.2.1	Minimum requirements

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Tables 7.6.2.1-1 and 7.6.2.1-2. For operating bands with an unpaired DL part (as noted in Table 5.5-1), the requirements only apply for carriers assigned in the paired part.

For Table 7.6.2.1-2 in frequency range 1, 2 and 3, up to exceptions are allowed for spurious response frequencies in each assigned frequency channel when measured using a 1MHz step size, where  is the number of resource blocks in the downlink transmission bandwidth configuration (see Figure 5.6-1). For these exceptions the requirements of subclause 7.7 Spurious response are applicable.

For Table 7.6.2.1-2 in frequency range 4, up to exceptions are allowed for spurious response frequencies in each assigned frequency channel when measured using a 1MHz step size, where  is the number of resource blocks in the downlink transmission bandwidth configurations (see Figure 5.6-1) and  is the number of resource blocks allocated in the uplink. For these exceptions the requirements of clause 7.7 spurious response are applicable.

Table 7.6.2.1-1: Out-of-band blocking parameters

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |  |
|  |  | 6 | 6 | 6 | 6 | 7 | 9 |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L at the minimum uplink configuration specified in Table 7.3.1-2 with PCMAX_L as defined in subclause 6.2.5. NOTE 2: Reference measurement channel is specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2. NOTE 3: The REFSENS power level is specified in Table 7.3.1-1 and Table 7.3.1-1a for two and four antenna ports, respectively. NOTE 4: For DL category M1 and M2 UE, the reference sensitivity for category M1 in table 7.3.1E-3 and category M2 in Table 7.3.1E-8 should be used as REFSENS for the power in Transmission Bandwidth Configuration. NOTE5: For DL category M1 and M2 UE, the parameters for the applicable channel bandwidth apply. |  |  |  |  |  |  |  |

Table 7.6.2.1-2: Out of band blocking

| E-UTRA band | Parameter | Units | Frequency |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Range 1 | Range 2 | Range 3 | Range 4 |
|  | PInterferer | dBm | -44 | -30 | -15 | -15 |
| 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 30, 31, 33, 34, 35, 36, 37, 38, 39, 40, 41, 42 (NOTE 2), 43 (NOTE 2), 44, 45, 48 (NOTE 2), 50, 51, 52 (NOTE 6), 65, 66, 68, 70, 71, 72, 73, 74, 85 | FInterferer (CW) | MHz | FDL_low -15 to FDL_low -60 | FDL_low -60 to FDL_low -85 | FDL_low -85 to 1 MHz | - |
|  |  |  | FDL_high +15 to FDL_high + 60 | FDL_high +60 to FDL_high +85 | FDL_high +85 to +12750 MHz | - |
| 2, 5, 12, 17, 85 | FInterferer | MHz | - | - | - | FUL_low - FUL_high (NOTE 5) |
| NOTE 1: For the UE which supports both Band 11 and Band 21 the out of blocking is FFS. NOTE 2: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 2800 MHz and FInterferer < 4400 MHz. The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 2800 MHz and FInterferer < 4800 MHz when UE supports both E-UTRA band B42 and NR bands n77, n78. NOTE 3: For the UE that supports both Band 4 and Band 66, the out-of-blocking frequency range for Band 4 is defined relative to FDL_low and FDL_high of Band 66. NOTE 4: For a UE supporting CA_20A-28A, CA_1A-3A-7A-20A-28A, CA_1A-3A-20A-28A, CA_1A-3A-3A-20A-28A, CA_1A-7A-20A-28A, CA_1A-20A-28A, CA_3A-7A-20A-28A, CA_3A-20A-28A or CA_7A-20A-28A the requirements for Band 20 and Band 28 apply with FDL_low given by the lower limit of the restricted operating frequency range in Band 28 and FDL_high by Band 20 (Table 5.5A-2). NOTE 5: Range 4 requirement does not apply to category M1 and M2. NOTE 6: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 2700 MHz and FInterferer < 4000 MHz. NOTE 7: For band 51 the FDL_high of band 50 is applied as FDL_high for band 51. NOTE 8: For UEs supporting both bands 38 and 41, the FDL_high and FDL_low of band 41 is applied as FDL_high and FDL_low for band 38. |  |  |  |  |  |  |

## <End of Changes>