3GPP TSG-RAN WG4 RAN4#92bis 	R4-1912549

Chongqing, China October 26th – 30th, 2019

Agenda item:	8.5.4.2

Source: 	Qualcomm, AT&T, Docomo

Title: 	Defining RRM requirements for IAB nodes

Document for:	Discussion

# Introduction

RAN4 made the following agreement during a previous meeting [1].

Agreement: - Access Link (AL): - A UE served by an IAB node on the access link (Uu) shall meet existing RRM requirements for the access link (Uu) defined in 38.133 and 36.133 (related to EN-DC or NE-DC). - No additional requirements related to UE operation on the access link are needed. - Backhaul link (BL): - It is FFS whether any RRM requirement need to be specified for IAB node and identify type of RRM requirement(s) (if need to be specified).

During the last meeting, a WF [6] was submitted to resolve this issue but there was no conclusion. This contribution focuses on the need and the type of RRM requirements that need to be specified for IAB nodes.

# Necessity of Defining IAB RRM Requirements

## Guidance from RAN Plenary

RAN plenary agreed to the following in the work item document of IAB for NR [5]

- Define RRM core requirements for both backhaul and access links of IAB node. - Define latency requirements for switching between communication over parent backhaul link (i.e. MT) and child backhaul/access links (i.e. DU). - Define timing requirements for IAB specific network synchronization. This may include (a) requirement for “case 1” timing (e.g. accuracy of DL transmission timing alignment between an IAB-node and its parent node), and (b) cell phase synchronization accuracy for multi-hop IAB network.

Observation 1: RAN plenary has already agreed to define RRM core requirements for both backhaul and access links of IAB nodes.

## Interoperability in IAB Networks

An IAB network can consist of many IAB nodes. Each IAB node will have a mobile-termination (MT) and gNB-DU functionality [1]. The MT part of an IAB node acts as the child of a parent node. The DU part provides gNB functionality to children nodes. Figure 1 shows an example of IAB network. Both IAB node 2 and IAB node 3 are children of IAB node 1. The MTs of these two IAB nodes receive data from the DU of IAB node 1. UE node 1 and UE node 2 are children of IAB node 2 and IAB node 3 respectively. The dotted line between IAB node 2 and IAB node 3 means that the link between IAB node 2 and 3 is good; and IAB node 2 might be able to act as a parent of IAB node 3.

Figure 1: An IAB network

Different IAB nodes of an IAB network can be designed by different infra vendors. Inter-operability among different IAB nodes is significant to guarantee the performance of an IAB network. This inter-operability can be achieved by defining RRM requirements for IAB nodes.

Observation 2: Ensuring inter-operability among different IAB nodes is very important to guarantee the performance of an IAB network. Inter-operability can be achieved by defining RRM requirements for IAB nodes.

## Deployment of IAB in FR2

FR2 is the main use case for Rel-16 IAB networks. Defining requirements for beam failure detection, beam failure recovery, radio link monitoring, etc. are essential to guarantee performance in FR2 IAB network.

Observation 3: FR2 is the main use case for Rel-16 IAB networks. Defining beam management related requirements is essential to guarantee performance in FR2 IAB network.

## IAB RRM Discussion in RAN1

RAN1 has made several RRM related agreements for IAB MTs that are different from that of UEs.

In Rel-15, a UE can be configured with only up to two SMTC windows in intra-frequency carriers and one SMTC window in inter-frequency. The maximum periodicity of an SMTC window can be 160 ms.

Different parts of 38.133 [2] have been written based on above assumptions. Some texts from 38.133 are given below:

On the other hand, for IAB node discovery and measurement, an IAB MT can be configured with up to four SMTC windows. The periodicity of SMTC can be up to 1280 ms. RAN1 also agreed that each SMTC window is provided its own independent configuration (e.g. periodicity, offset, duration). RAN1 made the following agreements regarding this [3]:

Agreements: The configurable values of the parameters in the SSB reception configurations including SMTC for IAB node discovery and measurement are provided in the following - SMTC window periodicity: - 5ms, 10ms, 20ms, 40ms, 80ms, 160ms, 320ms, 640ms, 1280ms - SMTC window timing offset: - [0, ..., (number of subframes within SMTC window periodicity) – 1] - SMTC window duration: - [1, …, 5] (subframes) - FFS larger than 5 - List of physical cell IDs to be measured - [cell ID 0, …, cell ID M-1 ] - FFS value of M - SSB-ToMeasure - Same as Rel-15 Agreements: Each SMTC window is provided its own independent configuration (e.g. periodicity, offset, duration)

The updated number of SMTC configurations and SMTC periodicity needs to be captured in IAB RRM specs.

Observation 4: RAN1 has agreed to allow a different number of SMTC configurations and SMTC periodicities for IAB MT nodes compared to that of UE nodes.

- These updated numbers need to be captured in RAN4 RRM specs.

# List of Individual IAB RRM requirements

Proposal 1: RAN4 supports table 1 to define the RRM requirements in IAB networks.

Table 1: RRM requirements in IAB networks

| Requirements | Features | Requirement needed? | Reason |
| --- | --- | --- | --- |
| Cell reselection | - Intra-freq - Inter-freq | No | Rel-16 IAB nodes are not expected to operate at idle mode |
| Handover |  | Yes | - Interoperability between parent and child |
| RRC Connection Mobility Control | - RRC re-establishment | Yes | - Same as above |
|  | - Random access | Yes | - Same as above |
|  | - RRC release with redirection | Yes | - Same as above |
| SCell activation/deactivation delays |  | Yes | - Same as above |
| Interruptions related to SCells addition, release, activation, deactivation |  | Yes | - Same as above |
| PSCell addition/release delays |  | Yes | - Same as above |
| Active BWP switching | - Interruptions at active BWP switching - Active BWP switching delay | Yes | - Same as above |
| RLM | Requirements for RLM based on SSB, CSI-RS or both, and for PCell/PSCell | Yes | - Interoperability between parent and child - ‘No DRX’ mode prioritized |
| TCI switching | TCI switching delay | Yes | - Interoperability between parent and child - Beam management in FR2 |
| Link recovery procedures | Requirements for BFD and CBD for PCell/PSCell | Yes | - Same as above - ‘No DRX’ mode prioritized |
| Measurements requirements | Measurement evaluation periods | Yes | - Interoperability between parent and child - ‘No DRX’ mode prioritized |
| Measurement accuracy requirements |  |  | - Interoperability between parent and child |
| MT timing related requirements | - MT maximum receive timing difference | Yes | - Same as above |
|  | - MT transmit timing | Yes | - Same as above |
|  | - MT maximum transmit timing difference | Yes | - Same as above |
|  | - Timing Advance | Yes | - Same as above |
| Switching time between MT and DU | --- | Yes | - Helps scheduling of MT and semi-static resource configuration between upstream and downstream links - Already agreed at RAN plenary |
| DU timing related requirements | - Sync accuracy | - Yes | - RAN4 has already agreed to support -+ 3 us synchronization accuracy among different IAB nodes that have overlapping coverage |

# Conclusion

Observation 1: RAN plenary has already agreed to define RRM core requirements for both backhaul and access links of IAB nodes.

Observation 2: Ensuring inter-operability among different IAB nodes is very important to guarantee the performance of an IAB network. Inter-operability can be achieved by defining RRM requirements for IAB nodes.

Observation 3: FR2 is the main use case for Rel-16 IAB networks. Defining beam management related requirements is essential to guarantee performance in FR2 IAB network.

Observation 4: RAN1 has agreed to allow a different number of SMTC configurations and SMTC periodicities for IAB MT nodes compared to that of UE nodes.

- These updated numbers need to be captured in RAN4 RRM specs.

Proposal 1: RAN4 supports table 1 to define the RRM requirements in IAB networks.

# References

- Chairman’s notes, 3GPP TSG RAN WG4 Meeting #91.

- 3GPP TS 38.133, V 15.5.0, Requirements for support of radio resource management (Release 15)

- 3GPP TSG RAN WG1 Meeting #97, Chairman’s Notes.

- 3GPP TSG RAN WG1 Meeting #96bis, Chairman’s Notes.

- RP-190712, Revised WID: Integrated Access and Backhaul for NR.

- R4-1909978, “Placeholder for WF on IAB RRM requirements”.