3GPP TSG-RAN WG4 Meeting #92bis	R4-1912607

Chongqing, China, 14 October – 18 October 2019

Source:	Ericsson, Swisscom

Title:	TP for TR 37.716-21-11: DC_3-20_n38

Agenda item:			9.4.2

Document for:	Approval

# 1. Introduction

Rel-16 LTE-NR DC combinations LTE 2 band + NR 1 band combination are defined in the WID [1]. This contribution is a text proposal for TR 37.716-21-11 to add DC_3A-20A_n38A.

# 2. Text Proposal

# ---Start of changes---

## 5.1.x	3A-20A_n38A

### 5.1.x.1	Operating bands for EN-DC

Table 5.1B.5.3-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 3-20_n38 | CA_3-20 | n38 | No |

### 5.1.x.2	Configuration for DC

Table 5.5B.5.3-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-20A_n38A | DC_3A_n38A DC_20A_n38A | CA_3A-20A | n38 |

### 5.1.x.3		∆TIB and ∆RIB values

For DC_3-20_n38, the TIB,c and RIB,c values are reused from the LTE combination CA_3-7-20, and are given in the tables below.

Table 5.1.x.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-20_n38 | 3 | 0.5 |
|  | 20 | 0.3 |
|  | n38 | 0.5 |

Table 5.1.x.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-20_n38 | 3 | 0 |
|  | 20 | 0 |
|  | n38 | 0 |

### 5.1.x.4	REFSENS requirements

Co-existence analysis from DC_3_n38 shows that there are IMD2 and IMD3 impact from DC_3_n38 UL to Band 20 DL. The MSD values for the impact on band 20 DL are reused from DC_3A-28A_n41A, which also have IMD2 and IMD3 to the DL of the low band (but only have IMD2 specified).

Co-existence analysis from DC_20_n38 shows that there no impact from DC_20_n38 UL to Band 3 DL.

Table 7.3B.2.3.5.2-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A-20A_n38A | 3 | 1779 | 5 | 25 | 1874 | N/A | FDD | N/A |
|  | 20 | 852 | 10 | 20 | 811 | 26.0 | FDD | IMD21 |
|  | n38 | 2590 | 10 | 50 | 2590 | N/A | FDD | N/A |
| NOTE 1: This band is subject to IMD3 also which MSD is not specified. |  |  |  |  |  |  |  |  |

---End of changes---

# Reference

[1]	RP-192087, “Revised WID on Dual Connectivity (EN-DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL)”, Huawei