3GPP TSG-RAN WG4 Meeting #92Bis                                                    R4-1912669

Chongqing, China, 14 – 18 August 2019

Source:	Huawei, HiSilicon

Title:	Ad Hoc minutes for NR Rel-15 and Rel-16 BS demodulation

Agenda Item:	6.12.2, 8.18.2

Document for:	Discussion

General Issues for Rel-15

Contributions list and summary of proposals

| Tdoc | Title | Source | Proposal |
| --- | --- | --- | --- |
| R4-1910800 | Remaining general issues for BS demodulation requirements | China Telecom | Proposal 2: Align BS FR1 OTA demodulation test direction in AAS specification with TS 38.141-2. Proposal 3: Align the test setup and procedure steps split in 38.141-2 and AAS specification with 38.141-1. Observation 2: In this meeting, decide the tentative requirements with [] for all the cases agreed before the RAN4#90 meeting. Proposal 4: Discuss the remaining TBDs case by case, by slightly adapting the selection criteria. |
| R4-1911108 | Discussion on NR Rel-15 BS demodulation requirements | Huawei, HiSilicon | Observation 1: The PUSCH performances with the same test configurations but only different TDD UL-DL configurations are very similar. Proposal 1: Remove the statement about the test facility calibration and measurement uncertainty in the test procedures in section 8 Radiated performance requirements. Proposal 2: Do not move the original test steps 1 to 4 in the procedure section to the initial conditions section and keep aligned for all tests in the specification TS 38.141-2. |
| R4-1911173 | Handling remaining TBDs in BS performance requirements | ZTE Wistron Telecom AB | Proposal 1: Remove configurations with error code 100 from the performance requirements list. Proposal 2: Apply a similar outlier removal process on the configurations with error code 102. Proposal 3: Remove configurations with error code 103 from the performance requirements list. |
| R4-1911188 | On NR Rel-15 BS demodulation requirements | Nokia, Nokia Shanghai Bell | Proposal 2: The UL baseband CA manufacturer declaration are to contain all carrier combinations that support UL baseband CA for each SCS. Proposal 4: In the BS demodulation performance test, method of test chapters of TS 38.141-2, move the procedure steps up to and including connecting the BS tester generating the wanted signal, multipath fading simulators and AWGN generators, to the initial conditions section. Proposal 5: List the direction to be tested for both BS Type 1-O and Type 2-O in TS 38.141-2 minimum performance requirements, even if the directions are the same. Observation 2: Demodulation branches to polarization mapping in BS demod OTA test procedure currently not aligned with the method/text in the AAS test specification. Proposal 6: RAN4 to change OTA test procedure text from “ach of the demodulation branch signals should be transmitted on each polarization” to “each of the demodulation branch signals should be transmitted on one polarization”. |
| R4-1911815 | BS demodulation - remaining open issues | Ericsson | Proposal 1: Adopt values proposed in Table 1 for the remaining TBDs. Proposal 2: Add following UL CA declaration for BS demodulation: D.108; UL CA combination for performance testing; For the highest supported SCS, the carrier combination with the largest aggregated bandwidth. If there is more than one combination, the carrier combination with the largest number of carriers shall be declared.; c; x; x. Proposal 3: Update TS 38.141-1 and TS 38.141-2 BS demodulation “Method of test” sub-sections according to above proposal. |

Discussions

Issue 1: Declarations for PUSCH UL baseband CA support

Agreements in previous meetings:

- RAN4#91 (R4-1907239):

  - Test applicability for CA

    - Define CA demodulation requirements for CP-OFDM PUSCH only

      - For a BS supporting UL carrier aggregation, for the highest SCS in each FR, the CC combination with the largest aggregated bandwidth is used for the test.

      - If there is more than one combination, the CC combination with the largest number of CCs is used for the test.

      - Mixed SCS CA combination is not to be tested for within a frequency range, considering the test complexity

      - Verify the performance per CC basis

      - Note: when capture the agreement in the draft CR, it should be considered that there is no requirement defined for FR1 60kHz.

  - Declaration of the support of UL baseband CA

    - FFS: the existing RF declarations (D.38/39/40 in 38.141-1 and D.60/1/2 in 38.141-2) can be reused

- RAN4#92 (R4-1910063):Declaration of support UL CA

  - Add new declarations of the support of UL CA for PUSCH demodulation performance tests

Open issues:

1: How to define the declaration of support UL CA for demodulation performance requirements:

- Option 1: Declare all supported UL CA configurations that baseband would support and specify the corresponding applicability rule (Nokia)

- Option 2: a single yes/no declaration: support UL CA or not

- Option 3: Only declare the carrier combination that should be used for BS demodulation testing, the agreed test applicability rule should be part of this new declaration to clearly identify what should be tested (Ericsson)

Discussion:

TS 38.141-1/2:

Table 4.6-1 Manufacturer declarations for BS type 1-C and BS type 1-H conducted test requirements [3] Declaration identifier; Declaration; Description; Applicability. BS type 1-C; BS type 1-H. D.14; NR supported channel bandwidths and SCS; NR supported SCS and channel bandwidths per supported SCS. Declared per supported operating band, per antenna connector for BS type 1-C, or TAB connector for BS type 1-H.; x; x. D.15; CA only operation; Declaration of CA-only operation (with equal power spectral density among carriers) but not multiple carriers, declared per operating band per antenna connector for BS type 1-C, or TAB connector for BS type 1-H.; x; x. <Other Items Skipped>. D.38; Inter-band CA; Band combinations declared to support inter-band CA (per CA capable multi-band connector(s), as in D.15). Declared for every multi-band connector which support CA.; x; x. D.39; Intra-band contiguous CA; Bands declared to support intra-band contiguous CA (per CA capable single band connector(s) or multi-band connector(s), as in D.15). Declared per antenna connector for BS type 1-C, or TAB connector for BS type 1-H.; x; x. D.40; Intra-band non-contiguous CA; Bands declared to support intra-band non-contiguous CA (per CA capable single band connector(s) or multi-band connector(s), as in D.15). Declared per antenna connector for BS type 1-C, or TAB connector for BS type 1-H.; x; x.

Table 4.6-1 Manufacturers declarations for BS type 1-H, BS type 1-O and BS type 2-O radiated test requirements [4] Declaration identifier; Declaration; Description; Applicability (Note 1). BS type 1-H (Note 2); BS type 1-O; BS type 2-O. D.7; BS channel band width and SCS support; BS supported SCS and channel bandwidth per supported SCS. Declared for each beam (D.3) and each operating band (D.4).; c; x; x. D.52; Operating band combination support; List of operating bands combinations supported by single-band RIB(s) and/or multi-band RIB(s) of the BS.; c; x; n/a. <Other Items Skipped>. D.60; Inter-band CA; Declaration of operating band(s) combinations supporting inter-band CA. Declared per operating band combination (D.52).; c; x; x. D.61; Intra-band contiguous CA; Declaration of operating band(s) supporting intra-band contiguous CA. Declared per operating band with CA support.; c; x; x. D.62; Intra-band non-contiguous CA; Declaration of operating band(s) supporting intra-band non-contiguous CA. Declared per operating band with CA support.; c; x; x.

Nokia [R4-1911188]:

In RAN4#92, the question was raised, if the declaration of support for UL CA needs to contain the bands that are supported for CA, or if a single yes/no declaration is necessary.

Proposal 2: The UL baseband CA manufacturer declaration are to contain all carrier combinations that support UL baseband CA for each SCS.

| Declaration identifier | Declaration | Description | Applicability |  |
| --- | --- | --- | --- | --- |
|  |  |  | BS type 1-C | BS type 1-H |
| <<Unchanged Items Skipped>> |  |  |  |  |
| D.1XX | UL baseband CA | Declaration of support of uplink baseband carrier aggregation for the supported SCS(s), i.e., the carrier combination(s) supported for CA per SCS. | x | x |

Ericsson [R4-1911815]:

This new declaration is supposed to determine which UL CA configuration should be used when testing BS demodulation requirements. To do so, there are basically 2 approaches:

- The first approach would be to declare all supported CA configurations that baseband would support and specify the new applicability rule agreed in [6].

- The second approach would be to only declare the UL CA configuration that should be used for BS demodulation testing, the agreed rule in XX should then be part of this new declaration to clearly identify what should be tested.

There are pros and cons with both approaches:

- The first one is more future proof and could give the opportunity to re-use this declaration for a future usage.

- The second one is more straight forward and would avoid declaring a large list of CA configurations that a baseband could support. Moreover, it would reduce the risk of confusion in between this new declaration and the existing ones (D.38, D.39 and D40 for FR1 – D.60, D.61 and D.62 for FR2).

Considering this comparison, we would propose to consider the 2nd approach, only declaring the UL CA configuration to be tested and make following proposal:

Proposal 2: Add following UL CA declaration for BS demodulation:

| D.108 | UL CA combination for performance testing | For the highest supported SCS, the carrier combination with the largest aggregated bandwidth. If there is more than one combination, the carrier combination with the largest number of carriers shall be declared. | c | x | x |
| --- | --- | --- | --- | --- | --- |

Declare a band combination with the largest aggregated BW and the largest number of CC for test?

Nokia: continue the email discussion. If not list all separate carrier combinations, no test applicability rule is needed. You can declare what you want to test. Constraint the test applicability rule.

Ericsson: to avoid the complexity, based on the test applicability, only list the carrier combination with the largest number of CC.

DCM: different carrier aggregation from RF and baseband, separate RF and baseband supplier.

Nokia: go with Ericsson’s proposal, but need concrete text proposal.

DCM: If you have radio A and B equipment, but baseband support band combinations of A+B+C.

2:

I’d like to add one additional open issue, related with CA and total BW of aggregated carrier, especially for FR2.

Current link budget analysis is done with 200MHz Carrier as assumption and margin from system noise floor is basically comes from PA assumption (usable PA on those freq).

And this analysis is setting limit of SNR <20dB requirement due to margin from system noise floor.

And also, requirement is applied for each individual CC.

So, when total aggregated BW is larger than 200MHz, for example, say 400MHz as example, and total signal power is limited by PA, then can’t increase total Power.

This causes each CC, or for example, 200MHz has power level decreases, in this example 3dB, makes level of signal and noise comes closer to system noise floor 3dB (in this example).

When aggregated BW increases, test system can’t correctly keep assumed margin from system noise floor, means individual CC requirement becomes tighter then total BW larger.

This needs more consideration for solution but like to raise this as open issue for today’s discussion.

Takao Miyake

keysight

Keysight: CA test with per compoment carrier at one compoment carrier time. No need other compoment carrier,

DCM: CA with multiple carrier.

Nokia：The SNR should include the interference from all CC. whether to clarify the receiver implementation is active for complete aggregated bandwidth from all CC in the specification.

Keysight: receiver from baseband from all CC,

Samsung/Huawei：this is the common understanding.

Common understanding: the receiver implementation is active for complete aggregated bandwidth from all CC.

Agreements:

Agree the following declaration text proposal：

| D.108 | UL CA combination for performance testing | For the highest supported SCS, the carrier combination with the largest aggregated bandwidth. If there is more than one combination, the carrier combination with the largest number of carriers shall be declared. | c | x | x |
| --- | --- | --- | --- | --- | --- |

Issue 2: Direction for BS type 1-O OTA test

Agreements in last meeting RAN4#92 (R4-1910063):

- Direction for BS type 1-O OTA test

    - OTA REFSENS receiver target reference direction (D.54)

Open issues:

1:  Align BS FR1 OTA demodulation test direction in AAS specification TS 37.145-2 with TS 38.141-2 (CTC)

2: OTA REFSENS receiver target reference direction (D.54) to be captured separately or combined for BS Type1-O and Type2-O in TS 38.141-2.

- Option 1: Combined format that are captured in the current specification TS 38.141-2

  - Direction to be tested: OTA REFSENS receiver target reference direction (see D.54 in table 4.6-1). (Samsung)

- Option 2: Separate format for BS type 1-O and type 2-O (Nokia)

  - Direction to be tested:

    - For BS type 1-O, OTA REFSENS receiver target reference direction (see D.54 in table 4.6-1).

    - For BS type 2-O, OTA REFSENS receiver target reference direction (see D.54 in table 4.6-1).

Both format exist in the specification TS 38.141-2

Discussion:

1: To be handled in RF session for alignment of AAS specification TS 37.145-2?

2: Example of section 7.5.2.4.1 given in R4-1911188 (Nokia)

##### 7.5.2.4.1 Initial conditions Test environment: Normal, see annex B.2. RF channels to be tested for single carrier: M; see subclause 4.9.1. Base Station RF Bandwidth edge position to be tested for multi-carrier and/or CA: - MRFBW in single-band operation, see subclause 4.9.1; - BRFBW_T'RFBW and B'RFBW_TRFBW in multi-band operation, see subclause 4.9.1. Directions to be tested: For BS type 1-O: - receiver target reference direction for the minSENS OSDD (D.31), - OTA REFSENS conformance test directions (D.55), For BS type 2-O: - OTA REFSENS receiver target reference direction (D.54), - OTA REFSENS conformance test directions (D.55).

Nokia：which one to align?

Ericsson：no strong view on which format to use. Check if PUSCH FR2 still used the seprate format for the direction declaration.

Agreements:

One declaration for the direction to test for both BS type 1-O and 2-O:

Direction to be tested:

OTA REFSENS receiver target reference direction (see D.54 in table 4.6-1).

Issue 4: Test setup and procedure alignment between TS 38.141-1 and TS 38.141-2

Agreements in last meeting RAN4#92 (R4-1910063):

Background information: R4-1908132 proposed the following updates:

Open issues:

1: Whether move test steps 1~4 to initial conditions section in TS 38.141-2:

- Option 1: Yes (CTC, Nokia)

- Option 2: No (Huawei, Ericsson)

If yes, additionally the same updates to AAS specification (CTC)

If no,

2: Whether need to add in TS 38.141-1 or move in test procedure of TS 38.141-2 about the statement of the test facility calibration and measurement uncertainty to the section of initial condition

- Option 1: Remove the statement about the test facility calibration and measurement uncertainty in the test procedure of section 8 in TS 38.141-2 (Huawei)

#### 8.3.5.4 Method of test ##### 8.3.5.4.1 Initial conditions Test environment: Normal; see annex B.2. RF channels to be tested for single carrier (SC): M; see subclause 4.9.1 Direction to be tested: - For BS type 1-O, OTA REFSENS receiver target reference direction (see D.54 in table 4.6-1). - For BS type 2-O, OTA REFSENS receiver target reference direction (see D.54 in table 4.6-1). ##### 8.3.5.4.2 Procedure 1) Place the BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex E.3.

- Option 2: Keep/Move the statement about the test facility calibration and measurement uncertainty in the initial condition part (Ericsson)

#### 8.3.5.4 Method of test ##### 8.3.5.4.1 Initial conditions Test environment: Normal; see annex B.2. RF channels to be tested for single carrier (SC): M; see subclause 4.9.1 Direction to be tested: - For BS type 1-O, OTA REFSENS receiver target reference direction (see D.54 in table 4.6-1). - For BS type 2-O, OTA REFSENS receiver target reference direction (see D.54 in table 4.6-1). OTA test requires correct use of an appropriate test facility which has been calibrated and is capable of performing measurements within the measurement uncertainties in subclause 4.1.2.4. ##### 8.3.5.4.2 Procedure 1) Place the BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex E.3.

3: TS 38.141-1, move “connect the BS tester generating the wanted signal…” in the initial conditions section to the test procedure? (Ericsson R4-1911815)

##### 8.3.3.1.4 Method of test ###### 8.3.3.1.4.1 Initial Condition Test environment: Normal, see annex B.2. RF channels to be tested: M; see subclause 4.9.1 ###### 8.3.3.1.4.2 Procedure 1) Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to all BS antenna connectors for diversity reception via a combining network as shown in annex D.5 and D.6 for BS type 1-C and type 1-H respectively 2) Adjust the AWGN generator, according to the channel bandwidth defined in table 8.3.3.1.4.2-1.

Discussion:

Section 4.13 of TS 38.141-2:

## 4.13 Format and interpretation of tests Each test has a standard format: … X.4 Method of test X.4.1 General In some cases there are alternative test procedures or initial conditions. In such cases, guidance for which initial conditions and test procedures can be applied are stated here. In the case only one test procedure is applicable, that is stated here. X.4.2y First test method X.4.2y.1 Initial conditions This subclause defines the initial conditions for each test, including the test environment, the RF channels to be tested and the basic measurement set-up. The OTA Test System is assumed to be correctly calibrated as part of the initial conditions. Calibration is not explicitly mentioned. X.4.2y.2 Procedure This subclause describes the steps necessary to perform the test and provides further details of the test definition like domain (e.g. frequency-span), range, weighting (e.g. bandwidth), and algorithms (e.g. averaging). The procedure may comprise data processing of the measurement result before comparison with the test requirement (e.g. average result from several measurement positions) #### 4.1.2.1 General The maximum acceptable uncertainty of the OTA Test System is specified below for each radiated test defined explicitly in the present specification, where appropriate. The OTA Test System shall enable the stimulus signals in the test case to be adjusted to within the specified tolerance and the DUT to be measured with an uncertainty not exceeding the specified values. All tolerances and uncertainties are absolute values, and are valid for a confidence level of 95 %, unless otherwise stated.

Nokia: TS 38.141-1 needs to be changed. Agree with Ericsson’s proposal. No strong view about the 38.141-2 part.

CTC: 38.141-1 is following the format used by LTE. Ok to change part 1 spec, just remind the NR spec is not aligned with LTE spec.

Ericsson: Ok to remove the TE calibration and measurement uncertainty part from the procedure part proposed by Huawei.

Agreements:

TS 38.141-1 updates, agreed to use the following format:

##### 8.3.3.1.4 Method of test ###### 8.3.3.1.4.1 Initial Condition Test environment: Normal, see annex B.2. RF channels to be tested: M; see subclause 4.9.1 ###### 8.3.3.1.4.2 Procedure 1) Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to all BS antenna connectors for diversity reception via a combining network as shown in annex D.5 and D.6 for BS type 1-C and type 1-H respectively 2) Adjust the AWGN generator, according to the channel bandwidth defined in table 8.3.3.1.4.2-1.

TS 38.141-2 updates: remove the statements about the TE calibration and measurement uncertainty part from the procedure as below:

#### 8.3.5.4 Method of test ##### 8.3.5.4.1 Initial conditions Test environment: Normal; see annex B.2. RF channels to be tested for single carrier (SC): M; see subclause 4.9.1 Direction to be tested: - For BS type 1-O, OTA REFSENS receiver target reference direction (see D.54 in table 4.6-1). - For BS type 2-O, OTA REFSENS receiver target reference direction (see D.54 in table 4.6-1). ##### 8.3.5.4.2 Procedure 1) Place the BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex E.3.

Issue 5: Mapping of demodulation branches to polarizations

Agreements in last meeting RAN4#92:

Open issues:

1: Misalignment of the statement about the mapping of demodulation branches to polarization between NR specification TS 38.141-2 and AAS specification TS 37.105

- Each of the demodulation branch signals should be transmitted on each polarization of the test antenna(s).(TS 38.141-2)

- When tested for two demodulation branches, each demodulation branch maps to one polarization (TS 37.105)

Option 1: Change to each demodulation branch signals should be transmitted on one polarization (Nokia)

Option 2:

Discussion:

Ericsson/Samsung/ZTE: agree with Nokia.

Agreements:

Change the test step of below for the demodulation branches to polarization as following：

4)	Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex E.3. Each of the demodulation branch signals should be transmitted on one polarization of the test antenna(s).

Issue 6: How to handle the remaining requirements with TBD

Agreements in last meeting:

RAN4#90Bis：R4-1904713 WF on requirement SNR derivation procedure, Nokia, Nokia Shanghai Bell, ZTE

- Procedure to derive the performance requirements:

– Only inputs that consist of a pair of ideal and impaired results can be taken into account.

– If the ideal span <= [2]dB:

• The AVERAGE impairment results can be used for the performance requirement with [] in the draftCRs/CRs;

– Else if the ideal span is larger than [2]dB:

• The results farthest from the AVERAGE value is taken out for the AVERAGE and SPAN re-calculation until the ideal span is <=2dB but still with at least 3 companies’ results available:

– The ultimate AVERAGE impairment results with corresponding ideal span <=2dB can be used for performance requirement with [] in the draftCRs/CRs.

• Otherwise put TBD for the related performance requirements.

– If the span of the impairment results after removal the outliers (if any) are larger than 4dB, then the procedure cannot be applied, related performance requirement remain TBD.

RAN4#91: R4-1907239, Way forward on General part of NR BS demodulation performance, Huawei

- For test cases agreed before the RAN4#90 meeting:

    - Set deadline for simulation results submission for Rel-15 minimum requirements to October meeting this year (RAN4#92bis).
No more results will be accepted unless technical issues with the requirements setup are identified.

    - Remove the square bracket for performance requirements at November meeting (RAN4#93)

- For the remaining test cases agreed after the RAN4#90 meeting, i.e.,

    - Multi-slot PUCCH

    - HST PUSCH, HST PRACH

    - UCI multiplexing on PUSCH

Aim to stabilise and align the simulation results until RAN4#92 (August). Set deadline for simulation result submission for RAN4#93 (November) and aim to also remove square brackets at RAN4#93 (November).

RAN4#92 (R4-1910063): Discuss how to handle the possible requirements still with TBD case by case in the next RAN4#92Bis meeting.

Open issues:

Handle the requirements with TBD case by case

Discussion:

R4-1911173 (ZTE):

| Configurations with TBDs | Not enough inputs (Error Code: 100) | Too large impairment span (Error Code: 102) | Too few effective inputs after outlier removal (Error Code: 103) |
| --- | --- | --- | --- |
| PUSCH – 1T2R |  | 1 |  |
| PUSCH – 2T2R | 4 | 1 |  |
| PUSCH – 2T8R |  | 2 |  |
| PUCCH – 1T2R |  | 3 |  |
| PUCCH – FR2 |  |  | 5 |
| Suggested Action | Remove the related performance requirements | Apply the similar outlier removal process by removing the impairment results with span>4dB | Remove the related performance requirements |

R4-1911815 (Ericsson):

| Type | Ant. | FR or Format | CBW - SCS | Misc | MCS | DM-RS | Error code | Analysis | Proposed values |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Ideal | Imp. |
| PUSCH | 1T2R | FR1 | 40 MHz – 30 kHz | Type A | MCS16 | 1+1 | -102 | One company has impaired results < idea results. Don’t consider those results | 10.0 | 10.6 |
| PUSCH | 2T2R | FR1 | 10MHz – 15 kHz | Type B | MCS2 | 1+1 | -102 | One company is having -1,05 as ideal, and -0,73 as impairment. Tbc but might be a typo and the value to be considered would be 0.73 instead? | 1.9 | 2.7 |
| PUSCH | 2T2R | FR2 | 100MHz 60kHz | With PTRS | MCS16 | 1 | -100 | No reliable result (SNR too high) Remove those requirements | N/A | N/A |
| PUSCH | 2T2R | FR2 | 100MHz 60kHz | Without PTRS | MCS16 | 1 | -100 | No reliable result (SNR too high) Remove those requirements | N/A | N/A |
| PUSCH | 2T2R | FR2 | 50MHz 60kHz | Wth PTRS | MCS16 | 1 | -100 | No reliable result (SNR too high) Remove those requirements | N/A | N/A |
| PUSCH | 2T2R | FR2 | 50MHz 60kHz | Without PTRS | MCS16 | 1 | -100 | No reliable result (SNR too high) Remove those requirements | N/A | N/A |
| PUSCH | 2T8R | FR1 | 10MHz 15 kHz |  | MCS2 | 1+1 | -102 | Current spam is 4.41, should be acceptable for this case. | -7.76 | -5.55 |
| PUSCH | 2T8R | FR1 | 5MHz 15kHz |  | MCS2 | 1+1 | -102 | Current spam is 4.12, should be acceptable for this case | -7.98 | -5.72 |
| PUCCH | 1T2R | F1 | 20MHz 30kHz | NACK to ACK | - |  | -102 | One company has impairments margin from 8 to 11 dB! Don’t consider corresponding values | -3.3 | -2.7 |
| PUCCH | 1T2R | F1 | 100MHz 30kHz | ACK miss | - |  | -102 | One company has impairments margin from 8 to 11 dB! Don’t consider corresponding values | -4.2 | -3.6 |
| PUCCH | 1T2R | F1 | 100MHz 30kHz | NACK to ACK | - |  | -102 | One company has impairments margin from 8 to 11 dB! Don’t consider corresponding values | -3.5 | -2.9 |
| PUCCH | FR2 | F1 | 50MHz 60kHz | Nack to Ack | - |  | -103 | Don’t consider only the value with highest spread. Then impaired values spam is less than 4dB. | -4.14 | -1.24 |
| PUCCH | FR2 | F2 | 50MHz 60kHz | ACK miss | - |  | -103 | Don’t consider only the value with highest spread. Then impaired values spam is less than 4dB. | 3.83 | 6.70 |
| PUCCH | FR2 | F4 | 50MHz 120kHz |  | - | 1+1 | -103 | Don’t consider only the value with highest spread. Then impaired values spam is less than 4 dB | 0.81 | 3.58 |
| PUCCH | FR2 | F4 | 200MHz 120kHz |  | - | 1 | -103 | Don’t consider only the value with highest spread. Then impaired values spam is less than 4 dB | 0.70 | 3.45 |
| PUCCH | FR2 | F4 | 200MHz 120kHz |  | - | 1+1 | -103 | Don’t consider only the value with highest spread. Then impaired values spam is less than 4 dB | 0.50 | 3.20 |

Summary of the proposal:

| Configurations with TBDs | Not enough inputs (Error Code: 100) | Too large impairment span (Error Code: 102) | Too few effective inputs after outlier removal (Error Code: 103) |
| --- | --- | --- | --- |
| Suggested Action | Remove the related performance requirements | Apply the similar outlier removal process by removing the impairment results with span >4 dB The span of 4.12 dB and 4.41 dB for some cases are acceptable? Typo for case PUSCH 2T2R FR1 10MHz/15kHz typeB with MCS2, DMRS1+1? | Option 1: Remove the related performance requirements Option 2: Don’t consider only the value with highest spread |

Samsung: Updated their results and submitted before the meeting.

CTC: submitted the summary spreadsheet, still 60 cases with TBD

ZTE: case by case but with the same principle for the case with the same error code. How to handle the cases without enough inputs, leave it TBD or other way? The door is still open for the principle to improve.

Ericsson: generic principle but not change what we agreed before. Case by case to handle.

CTC: Strong view on case by case to remove the performance requirements for error code 100/103. Not sure about the principle for error code 102 that was discussed before.

Ericsson: similar view with CTC.

Samsung: ok with ZTE. Change the span. For cases with error code 100, just use the submitted results for the requirements definition but with notes that no enough inputs for this.

CTC: First take a look at the results, 4 cases for 100 code, some companies did not find the SNR point for 70% TP in their simulation. E.g. ok to remove some cases without enough inputs but against to make RAN4 agreements to remove all cases without enough inputs.

Nokia: Email discussion about the cases with TBD by listing all the cases with TBD.

Agreements:

CTC initiates the discussion by listing all the cases with TBD and decide it during this meeting.(R4-1910801)

Issue 7: AWGN power level setting in TS 38.141-2

Agreements in last meeting RAN4#92:

Open issues:

Different formats still exist in the specification.

PUSCH and PUCCH format 3&4:

Table 8.2.1.4.2-2: AWGN power level at the BS input

| BS type | Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| BS type 1-O | 15 | 5 | -86.5 - ΔOTAREFSENS dBm / 4.5 MHz |
|  |  | 10 | -83.3 - ΔOTAREFSENS dBm / 9.36 MHz |
|  |  | 20 | -80.2 - ΔOTAREFSENS dBm / 19.08 MHz |
|  | 30 | 10 | -83.6 - ΔOTAREFSENS dBm / 8.64 MHz |
|  |  | 20 | -80.4 - ΔOTAREFSENS dBm / 18.36 MHz |
|  |  | 40 | -77.2 - ΔOTAREFSENS dBm / 38.16 MHz |
|  |  | 100 | -73.1 - ΔOTAREFSENS dBm / 98.28 MHz |
| BS type 2-O | 60 | 50 | EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 47.52MHz |
|  |  | 100 | EISREFSENS_50M + ΔFR2_REFSENS + 18 dBm / 95.04 MHz |
|  | 120 | 50 | EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 46.08 MHz |
|  |  | 100 | EISREFSENS_50M + ΔFR2_REFSENS + 18 dBm / 95.04 MHz |
|  |  | 200 | EISREFSENS_50M + ΔFR2_REFSENS + 21 dBm / 190.08 MHz |
| NOTE 1: ΔOTAREFSENS as declared in D.53 in table 4.6-1 and subclause 7.1. NOTE 2: ΔFR2_REFSENS = -3 dB as described in subclause 7.1, since the OTA REFSENS reference direction (as declared in D.54 in table 4.6-1) is used for testing. NOTE 3: EISREFSENS_50M as declared in D.28 in table 4.6-1. |  |  |  |

Table 8.3.5.4.2-2: AWGN power level at the BS input

| BS type | Subcarrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| BS type 1-O | 15 | 5 | -83.5 dBm - ΔOTAREFSENS/ 4.5 MHz |
|  |  | 10 | -80.3 dBm - ΔOTAREFSENS/ 9.36 MHz |
|  |  | 20 | -77.2 dBm - ΔOTAREFSENS/ 19.08 MHz |
|  | 30 | 10 | -80.6 dBm - ΔOTAREFSENS/ 8.64 MHz |
|  |  | 20 | -77.4 dBm - ΔOTAREFSENS/ 18.36 MHz |
|  |  | 40 | -74.2 dBm - ΔOTAREFSENS/ 38.16 MHz |
|  |  | 100 | -70.1 dBm - ΔOTAREFSENS/ 98.28 MHz |
| BS type 2-O | 60 | 50 | EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm/ 47.52 MHz |
|  |  | 100 | EISREFSENS_50M + ΔFR2_REFSENS + 18 dBm/ 95.04 MHz |
|  | 120 | 50 | EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm/ 46.08 MHz |
|  |  | 100 | EISREFSENS_50M + ΔFR2_REFSENS + 18 dBm/ 95.04 MHz |
|  |  | 200 | EISREFSENS_50M + ΔFR2_REFSENS + 21 dBm/ 190.08 MHz |
| NOTE 1: ΔOTAREFSENS as declared in D.53 in table 4.6-1 and subclause 7.1. NOTE 2: ΔFR2_REFSENS = -3 dB as described in subclause 7.1, since the OTA REFSENS receiver target reference direction (as declared in D.54 in table 4.6-1) is used for testing. NOTE 3: EISREFSENS_50M as declared in D.28 in table 4.6-1. |  |  |  |

UCI multiplexing on PUSCH, square brackets still exist:

Table 8.2.3.4.2-2: AWGN power level at the BS input

| BS type | Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| BS type 1-O | 30 | 10 | [-83.6] - ΔOTAREFSENS dBm / 8.64MHz |
| BS type 2-O | 120 | 50 | [EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 46.08 MHz] |
| NOTE 1: ΔOTAREFSENS as declared in D.53 in table 4.6-1 and subclause 7.1. NOTE 2: ΔFR2_REFSENS = -3 dB as declared in subclause 7.1. NOTE 3: EISREFSENS_50M as declared in D.28 in table 4.6-1. |  |  |  |

PUCCH format 0:

Table 8.3.1.4.2-2: AWGN power level at the BS input

| BS type | Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| BS type 1-O | 15 | 5 | -83.5 dBm - ΔOTAREFSENS / 4.5 MHz |
|  |  | 10 | -80.3 dBm - ΔOTAREFSENS / 9.36 MHz |
|  |  | 20 | -77.2 dBm - ΔOTAREFSENS / 19.08 MHz |
|  | 30 | 10 | -80.6 dBm - ΔOTAREFSENS / 8.64 MHz |
|  |  | 20 | -77.4 dBm - ΔOTAREFSENS / 18.36 MHz |
|  |  | 40 | -74.2 dBm - ΔOTAREFSENS / 38.16 MHz |
|  |  | 100 | -70.1 dBm - ΔOTAREFSENS / 98.28 MHz |
| BS type 2-O | 60 | 50 | EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 47.52 MHz |
|  |  | 100 | EISREFSENS_50M + ΔFR2_REFSENS + 18 dBm / 95.04 MHz |
|  | 120 | 50 | EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 46.08 MHz |
|  |  | 100 | EISREFSENS_50M + ΔFR2_REFSENS + 18 dBm / 95.04 MHz |
|  |  | 200 | EISREFSENS_50M + ΔFR2_REFSENS + 21 dBm / 190.08 MHz |
| NOTE 1: ΔOTAREFSENS as declared in D.53 in table 4.6-1 and subclause 7.1 NOTE 2: ΔFR2_REFSENS = -3 dB as described in subclause 7.1 since the OTA REFSENS reference direction (as declared in D.54 in table 4.6-1) is used for testing. NOTE 3: EISREFSENS_50M as declared in D.28 in table 4.6-1. |  |  |  |

PUCCH format 1 (correct subscript format):

Table 8.3.2.1.4.2-2: AWGN power level at the BS input

| BS type | Subcarrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| BS type 1-O | 15 kHz | 5 | -83.5 dBm - ΔOTAREFSENS/ 4.5 MHz |
|  |  | 10 | -80.3 dBm - ΔOTAREFSENS/ 9.36 MHz |
|  |  | 20 | -77.2 dBm - ΔOTAREFSENS/ 19.08 MHz |
|  | 30 kHz | 10 | -80.6 dBm - ΔOTAREFSENS/ 8.64 MHz |
|  |  | 20 | -77.4 dBm - ΔOTAREFSENS/ 18.36 MHz |
|  |  | 40 | -74.2 dBm - ΔOTAREFSENS/ 38.16 MHz |
|  |  | 100 | -70.1 dBm - ΔOTAREFSENS/ 98.28 MHz |
| BS type 2-O | 60 kHz | 50 | EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 47.52 MHz |
|  |  | 100 | EISREFSENS_50M + ΔFR2_REFSENS + 18 dBm / 95.04 MHz |
|  |  | 50 | EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 46.08 MHz |
|  |  | 100 | EISREFSENS_50M + ΔFR2_REFSENS + 18 dBm / 95.04 MHz |
|  |  | 200 | EISREFSENS_50M + ΔFR2_REFSENS + 21 dBm / 190.08 MHz |
| NOTE 1: ΔOTAREFSENS as declared in D.53 in table 4.6-1 and subclause 7.1. NOTE 2: ΔFR2_REFSENS = -3 dB as described in subclause 7.1, since the OTA REFSENS reference direction (as declared in D.54 in table 4.6-1) is used for testing. NOTE 3: EISREFSENS_50M as declared in D.28 in table 4.6-1. |  |  |  |

PUCCH format 2 (square brackets):

Table 8.3.3.1.4.2-2: AWGN power level at the BS input

| BS type | Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| BS type 1-O | 15 kHz | 5 | [-83.5] - ΔOTAREFSENS dBm / 4.5 MHz |
|  |  | 10 | [-80.3] - ΔOTAREFSENS dBm / 9.36 MHz |
|  |  | 20 | [-77.2] -ΔOTAREFSENS dBm / 19.08 MHz |
|  | 30 kHz | 10 | [-80.6] - ΔOTAREFSENS dBm / 8.64 MHz |
|  |  | 20 | [-77.4] - ΔOTAREFSENS dBm / 18.36 MHz |
|  |  | 40 | [-74.2] - ΔOTAREFSENS dBm / 38.16 MHz |
|  |  | 100 | [-70.1] - ΔOTAREFSENS dBm / 98.28 MHz |
| BS type 2-O | 60 kHz | 50 | [EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 47.52MHz] |
|  |  | 100 | [EISREFSENS_50M + ΔFR2_REFSENS + 18 dBm / 95.04 MHz] |
|  | 120 kHz | 50 | [EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 46.08 MHz] |
|  |  | 100 | [EISREFSENS_50M + ΔFR2_REFSENS + 18 dBm / 95.04 MHz] |
|  |  | 200 | [EISREFSENS_50M + ΔFR2_REFSENS + 21 dBm / 190.08 MHz] |
| NOTE 1: ΔOTAREFSENS as declared in D.53 in table 4.6-1 and subclause 7.1. NOTE 2: ΔFR2_REFSENS = -3 dB as declared in subclause 7.1. NOTE 3: EISREFSENS_50M as declared in D.28 in table 4.6-1. |  |  |  |

Discussion:

The unified formula format for the AWGN power level setting:

Table 8.2.1.4.2-2: AWGN power level at the BS input

| BS type | Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| BS type 1-O | 15 | 5 | -86.5 - ΔOTAREFSENS dBm / 4.5 MHz |
|  |  | 10 | -83.3 - ΔOTAREFSENS dBm / 9.36 MHz |
|  |  | 20 | -80.2 - ΔOTAREFSENS dBm / 19.08 MHz |
|  | 30 | 10 | -83.6 - ΔOTAREFSENS dBm / 8.64 MHz |
|  |  | 20 | -80.4 - ΔOTAREFSENS dBm / 18.36 MHz |
|  |  | 40 | -77.2 - ΔOTAREFSENS dBm / 38.16 MHz |
|  |  | 100 | -73.1 - ΔOTAREFSENS dBm / 98.28 MHz |
| BS type 2-O | 60 | 50 | EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 47.52MHz |
|  |  | 100 | EISREFSENS_50M + ΔFR2_REFSENS + 18 dBm / 95.04 MHz |
|  | 120 | 50 | EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 46.08 MHz |
|  |  | 100 | EISREFSENS_50M + ΔFR2_REFSENS + 18 dBm / 95.04 MHz |
|  |  | 200 | EISREFSENS_50M + ΔFR2_REFSENS + 21 dBm / 190.08 MHz |
| NOTE 1: ΔOTAREFSENS as declared in D.53 in table 4.6-1 and subclause 7.1. NOTE 2: ΔFR2_REFSENS = -3 dB as described in subclause 7.1, since the OTA REFSENS reference direction (as declared in D.54 in table 4.6-1) is used for testing. NOTE 3: EISREFSENS_50M as declared in D.28 in table 4.6-1. |  |  |  |

Keysight: for 50MHz/120kHz SCS, the 46.08MHz BW is very small high density for CA, is it ok to company?

Nokia: we cannot change the PRB allocation. Smaller noise power.

Agreements:

1: Use the following format for all cases:

Table 8.2.1.4.2-2: AWGN power level at the BS input

| BS type | Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| BS type 1-O | 15 | 5 | -86.5 - ΔOTAREFSENS dBm / 4.5 MHz |
|  |  | 10 | -83.3 - ΔOTAREFSENS dBm / 9.36 MHz |
|  |  | 20 | -80.2 - ΔOTAREFSENS dBm / 19.08 MHz |
|  | 30 | 10 | -83.6 - ΔOTAREFSENS dBm / 8.64 MHz |
|  |  | 20 | -80.4 - ΔOTAREFSENS dBm / 18.36 MHz |
|  |  | 40 | -77.2 - ΔOTAREFSENS dBm / 38.16 MHz |
|  |  | 100 | -73.1 - ΔOTAREFSENS dBm / 98.28 MHz |
| BS type 2-O | 60 | 50 | EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 47.52MHz |
|  |  | 100 | EISREFSENS_50M + ΔFR2_REFSENS + 18 dBm / 95.04 MHz |
|  | 120 | 50 | EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 46.08 MHz |
|  |  | 100 | EISREFSENS_50M + ΔFR2_REFSENS + 18 dBm / 95.04 MHz |
|  |  | 200 | EISREFSENS_50M + ΔFR2_REFSENS + 21 dBm / 190.08 MHz |
| NOTE 1: ΔOTAREFSENS as declared in D.53 in table 4.6-1 and subclause 7.1. NOTE 2: ΔFR2_REFSENS = -3 dB as described in subclause 7.1, since the OTA REFSENS reference direction (as declared in D.54 in table 4.6-1) is used for testing. NOTE 3: EISREFSENS_50M as declared in D.28 in table 4.6-1. |  |  |  |

PUSCH for Rel-15

Contributions list and summary of proposals

| Tdoc | Title | Source | Proposal |
| --- | --- | --- | --- |
| R4-1910800 | Remaining general issues for BS demodulation requirements | China Telecom | Observation 1: In theory, with shortest slot offset between two HARQ (re)transmissions in FDD, PUSCH performance with FDD is poorer than PUSCH with any TDD pattern. Observation 2: The link-level performance difference at 70% PUSCH throughput is negligible for FDD, TDD 7:1:2 and TDD 3:1:1. Proposal 1: The PUSCH performance requirements can also be applied to other TDD patterns, and update the PUSCH test parameters as follows. |
| R4-1911188 | On NR Rel-15 BS demodulation requirements | Nokia, Nokia Shanghai Bell | - Relative TPUT performance differences for different slot patterns are neglectable (below 0.2 dB). - NR Rel-15 BS demodulation PUSCH minimum performance requirements are applicable to all TDD UL-DL patterns and TDD. The pattern definition can be removed from the specifications or replaced by “N/A”. |
| R4-1911815 | BS demodulation - remaining open issues | Ericsson | Proposal 4: For BS demodulation requirements, keep current specified TDD configuration for each SCS. BS TDD demodulation should be tested using those TDD configurations. |
| R4-1911187 | NR Rel-15 BS demodulation simulations on TDD patterns | Nokia, Nokia Shanghai Bell |  |
| R4-1912441 | Discussion on applicable TDD configuration for NR BS performance | NTT DOCOMO, INC. | Proposal 1: If the performance difference is negligible among different TDD configurations, apply the same performance requirements to all TDD configuration patterns. Proposal 2: If the performance difference is not negligible among different TDD configurations, study the way on how to define the performance requirements for such TDD configurations. |

Discussions

Issue 1: PUSCH performance requirements for different TDD UL-DL patterns

Agreements in the previous meeting:

RAN4#92 (R4-1910063):

- Performance for different duplex mode and TDD UL-DL patterns

  - Apply the current PUSCH performance requirements to FDD mode

  - Further discuss whether the requirements can be applied to other TDD configurations in the next meeting.

Open issues:

1: Whether the PUSCH performance requirements can be applied to other agreed TDD configurations:

- Option 1: Yes (CTC, Nokia)

- Option 2: No

2: How to draft the specification:

- Option 1: No specific TDD configuration is specified

  - Option 1-1: Add one Note: The same set of requirements is applicable to FDD and TDD with different uplink-downlink allocations (CTC)

  - Option 1-2: Set “N/A” for the UL-DL allocation for TDD configuration (Nokia)

- Option 2: Specify one default TDD configuration (the one captured in the specification) to easy the test vendor implementation and testing with one note specifying the requirements also applicable to other agreed TDD configurations (Huawei)

- Option 3: Keep current specified TDD configuration for each SCS. BS TDD demodulation should be tested using those TDD configurations. (Ericsson)

3: Applicability rule to choose which TDD configuration for gNB test if no specific TDD configurations are specified, BS should not be mandated to support all TDD configurations, the one in the specification is supported by all BS vendors.

Discussion:

CTC: Some typical TDD configurations, CTC/Nokia/Huawei provided results with small margin. FDD with smallest retransmission interval, TDD should has larger retransmission interval, so it should be applicable all TDD configuration.

Nokia: Requirements with different TDD configuration has small marge, but the original agreement to use the pattern in the specs as baseline. No need to change the spec.

Ericsson: Requirements are applicable for other TDD configurations. But one configuration is used for test setup.

Keysight: which TDD configuration is mandate to test is not clear, one of those should be tested. FDD: same CL, short test time; prefer to mandate one TDD configuration to be specified in the spec.

Samsung: similar with Ericsson and Huawei, one TDD configuration is specified for LTE. We did evaluations for those agreed TDD configurations and cannot conclude to applicable for all TDD configuration.

CTC: NR is different from LTE, operators may use different TDD patterns, test complexity, different operators have the same TDD patterns in the same band, and our interested TDD pattern is not included. The PUSCH performance difference between different TDD patterns is negligible, how to capture it in the spec can further discuss.

DCM: It is not necessary to mandate TDD configuration for test. We change the TDD configuration later, it is different from the original proposal for the TDD patterns,

Ericsson: Requirements are applicable for all TDD patterns, but only one TDD pattern is used for test.

CTC: test applicability for all TDD patterns, applicable to the TDD patterns declared to support.

CMCC: Need TDD patterns declared to support clarification, because one BS vendors may support serval TDD patterns.

CTC: From our point of view, fine to test with one TDD pattern per SCS.

Requirements are applicable for all TDD patterns, test is conducted only for one TDD pattern per SCS, further discussion on the TDD patterns per SCS supported by BS vendors.

Ericsson: complex for the declaration and prefer to keep the agreed TDD patterns.no need to declare the supported TDD patterns.

Keysight: suggest to change “test is conducted only for one TDD pattern per SCS declared to supported by BS”

CTC/DCM: Ok to keysight’s proposal.

DCM: Is it common understanding that requirements are applicable for all TDD patterns?

ZTE: much time will be spent on the discussion on the declaration item definition.

CTC: No strong view on the declaration for the supported TDD patterns definition

Samsung: Same way as LTE.

CTC: No need to align with LTE for all aspects.

Nokia: Keep the test parameters in the spec and specified it is used for the requirements derivation.

Agreements:

Requirements are applicable for all TDD patterns, test is conducted only for one TDD pattern per SCS, FFS how to capture it in the specification.

PUCCH for Rel-15

Contributions list and summary of proposals

| Tdoc | Title | Source | Proposal |
| --- | --- | --- | --- |
| R4-1911188 | On NR Rel-15 BS demodulation requirements | Nokia, Nokia Shanghai Bell | Proposal 3: RAN4 to either remove all “for single carrier (SC)” statements for all PUCCH formats, or to re-introduce this statement for PUCCH F0. |

Discussions

Issue 1: Initial conditions for PUCCH

Agreements in last meeting:

In the endorsed draftCR R4-1910070:

###### 8.3.3.1.4.1 Initial Condition Test environment: Normal, see annex B.2. RF channels to be tested:): M; see subclause 4.9.1 1) Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to all BS antenna connectors for diversity reception via a combining network as shown in annex D.5 and D.6 for BS type 1-C and type 1-H respectively.

Open issues:

Whether to remove “for single carrier (SCS)” statement for all PUCCH formats:

- Option 1: Remove “for single carrier (SCS)” statement for all PUCCH formats

- Option 2: Re-introduce the statement for PUCCH format 2

Discussion:

Check subclause 4.9.1 of TS 38.141-1/2:

- For single carrier testing with appropriate frequencies: RF channels B (bottom), M (middle) and T (top)

- Tests with the maximum Base Station RF Bandwidth: BRFBW (bottom), MRFBW (middle) and TRFBW (top)

- Test for multi-band and dual-band operation: BRFBW_T'RFBW and B'RFBW_TRFBW

- Tests with the aggregated BS channel bandwidth and sub-block bandwidths: BBW Channel CA(bottom), MBW Channel CA (middle) and TBW Channel CA (top)

###### 8.3.3.1.4.1 Initial Condition Test environment: Normal, see annex B.2. RF channels to be tested: see subclause 4.9.1 1) Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to all BS antenna connectors for diversity reception via a combining network as shown in annex D.5 and D.6 for BS type 1-C and type 1-H respectively.

Samsung: CR to remove the single carrier in the initial condition

Nokia: keep it.

Ericsson: keep it for single carrier.

Samsung: fine to keep it, but better to align with LTE only with M.

DCM: per CC test for CA. it means that 3 tests for CA with 3 carriers?

Agreements:

Keep it “for single carrier: M” in the initial condition part

Further discuss how to specify it for CA cases

PRACH for Rel-15

Contributions list and summary of proposals

| Tdoc | Title | Source | Proposal |
| --- | --- | --- | --- |

Discussions

Issue 1: Alignment with other parts

Agreements in the last meeting:

Open issues:

The following parts need to align with other sections for demodulation performance in the specification:

- The initial condition

- The test procedure

- The AWGN power level setting

Discussion:

During the contributions review, we prepared late draftCR for these, is it necessary?

Agreements:

Huawei will prepare draftCR for PRACH part to align with other sections during this meeting.

Draft CRs and TPs for Rel-15

Contributions list and summary of proposals

1) Draft CRs for applicability rules and manufacture declarations

| Tdoc | Title | Source | Notes |
| --- | --- | --- | --- |
| R4-1910802 | Draft CR to TS 38.141-1: Further update of applicability rule for BS conducted demodulation test | China Telecom | revised |
| R4-1910803 | Draft CR to TS 38.141-2: Further update of applicability rule for BS radiated demodulation test | China Telecom | revised |
| R4-1911109 | draftCR: Updates to manufacture's declarations for demodulation requirements in TS 38.141-1 | Huawei, HiSilicon | revised |
| R4-1911110 | draftCR: Updates to manufacture's declarations for demodulation requirements in TS 38.141-2 | Huawei, HiSilicon | revised |
| R4-1911115 | Remove the square brackets for uncertainty and TT for OTA tests in 38.141-2 | Huawei, HiSilicon |  |
| R4-1911371 | Draft CR on correction on FRC table for FR1 PUSCH performance requirements for TS 38.104 | Samsung, China Telecom | Agreeable for company |
| R4-1911372 | Draft CR on correction on FRC table for FR1 PUSCH conducted performance requirements for TS 38.141-1 | Samsung, China Telecom | Agreeable for company |
| R4-1911373 | Draft CR on correction on FRC table for FR1 PUSCH radicated performance requirements for TS 38.141-2 | Samsung, China Telecom | Agreeable for company |

Company needs to double check the FRC table defined for MCS2, if the BG1 or BG2 is assumed before the number of CB is calculated.

2) Draft CRs to TS 38.104, 38.141-1, 38.141-2

| Contents | Tdoc | Title | Source | Notes |
| --- | --- | --- | --- | --- |
| DFT-s-OFDM based PUSCH | R4-1910804 | Draft CR to TS 38.104: Update of performance requirements for DFT-s-OFDM based PUSCH | China Telecom |  |
|  | R4-1910805 | Draft CR to TS 38.141-1: Update of conducted test requirements for DFT-s-OFDM based PUSCH | China Telecom |  |
|  | R4-1910806 | Draft CR to TS 38.141-2: Update of radiated test requirements for DFT-s-OFDM based PUSCH | China Telecom |  |
| CP-OFDM based PUSCH in FR1 | R4-1911189 | draftCR for 38.104 on PUSCH requirements with CP-OFDM and FR1 | Nokia, Nokia Shanghai Bell |  |
|  | R4-1911190 | draftCR for 38.141-1: Conducted test requirements for CP-OFDM based PUSCH in FR1 | Nokia, Nokia Shanghai Bell |  |
|  | R4-1911191 | draftCR for TS 38.141-2: Radiated test requirements for CP-OFDM based PUSCH in FR1 | Nokia, Nokia Shanghai Bell |  |
| CP-OFDM based PUSCH in FR2 | R4-1911813 | Draft CR to TS 38.104 BS demodulation CP-OFDM PUSCH FR2 requirements | Ericsson |  |
|  | R4-1911814 | Draft CR to TS 38.141-2 BS demodulation CP-OFDM PUSCH FR2 requirements | Ericsson |  |
| PUCCH format 0 | R4-1911810 | Draft CR to TS 38.104 BS demodulation PUCCH format 0 requirements | Ericsson |  |
|  | R4-1911811 | Draft CR to TS 38.141-1 BS demodulation PUCCH format 0 requirements | Ericsson |  |
|  | R4-1911812 | Draft CR to TS 38.141-2 BS demodulation PUCCH format 0 requirements | Ericsson |  |
| PUCCH format 1 | R4-1911174 | Draft CR for 38.141-1 Conducted test requirements for NR PUCCH format 1 | ZTE Wistron Telecom AB |  |
|  | R4-1911175 | Draft CR for TS 38.141-2 Radiated test requirements for NR PUCCH format 1 | ZTE Wistron Telecom AB |  |
|  | R4-1911176 | Draft CR for 38.104: Performance requirements for NR PUCCH format 1 | ZTE Wistron Telecom AB |  |
| PUCCH format 2 | R4-1911365 | Draft CR on NR PUCCH format2 performance requirements for TS 38.104 | Samsung |  |
|  | R4-1911366 | Draft CR on NR PUCCH format2 conducted performance requirements for TS 38.141-1 | Samsung |  |
|  | R4-1911367 | Draft CR on NR PUCCH format2 radiated performance requirements for TS 38.141-2 | Samsung |  |
| PUCCH format 3 and 4 | R4-1911112 | draftCR: Updates for PUCCH formats 3 and 4 performance requirements in TS 38.104 | Huawei, HiSilicon |  |
|  | R4-1911113 | draftCR: Updates for PUCCH formats 3 and 4 conducted conformance testing in TS 38.141-1 | Huawei, HiSilicon |  |
|  | R4-1911114 | draftCR: Updates for PUCCH format 3 and 4 radiated conformance testing in TS 38.141-2 | Huawei, HiSilicon |  |
| PRACH | R4-191xxxx | draftCR: Updates to PRACH conducted conformance testing in TS 38.141-1 | Huawei, HiSilicon | Late contribution to keep alignment with other parts during this meeting |
|  | R4-191xxxx | draftCR: Updates to PRACH radiated conformance testing in TS 38.141-2 | Huawei, HiSilicon | Late contribution |
| UCI on PUSCH | R4-1911368 | Draft CR on NR UCI on PUSCH performance requirements for TS 38.104 | Samsung |  |
|  | R4-1911369 | Draft CR on NR UCI on PUSCH conducted performance requirements for TS 38.141-1 | Samsung |  |
|  | R4-1911370 | Draft CR on NR UCI on PUSCH radiated performance requirements for TS 38.141-2 | Samsung |  |
|  | R4-1912496 | CR on performance requirements for UCI multiplexed on PUSCH | Huawei, HiSilicon | Add DTX to UCI requirement (false alarm) when nothing is sent noted |
| Multi-slot PUCCH | R4-1911177 | Draft CR for 38.104: Performance requirements for NR multi-slot PUCCH | ZTE Wistron Telecom AB |  |
|  | R4-1911178 | Draft CR for 38.141-1 Conducted test requirements for NR multi-slot PUCCH format 1 | ZTE Wistron Telecom AB |  |
|  | R4-1911179 | Draft CR for TS 38.141-2 Radiated test requirements for NR multi-slot PUCCH | ZTE Wistron Telecom AB |  |

UCI:

Samsung: the specific bits are specified, no false alarm is needed and also LTE did not define false alarm for UCI. We only counter the UCI to be sent and assume the UCI is transmitted contiguously.

Ericsson: this create new requirements, it is late to consider it in Rel-15.

Nokia: PF2, ACK missed detection, no false alarm,

Company can check if the false alarm detection is considered in the simulation.

Note:

|  |  | 38.104 |  | 38.141-1 | 38.141-2 |
| --- | --- | --- | --- | --- | --- |
|  |  | Conducted and radiated, FR1 | Radiated, FR2 | Conducted, FR1 | Radiated, FR1 and FR2 |
| Test applicability |  | NA | NA | China Telecomm | China Telecomm |
| Manufacture declarations |  | NA | NA | Ericsson | Ericsson |
| PUSCH | CP-OFDM | Nokia | Ericsson | Nokia | Nokia, Ericsson |
|  | DFT-S-OFDM | China Telecom | China Telecom | China Telecom | China Telecom |
|  | UCI on PUSCH | Samsung | Samsung | Samsung | Samsung |
| PUCCH | format 0 | Ericsson | Ericsson | Ericsson | Ericsson |
|  | format 1 | ZTE | ZTE | ZTE | ZTE |
|  | Multi-slot PUCCH | ZTE | ZTE | ZTE | ZTE |
|  | format 2 | Samsung | Samsung | Samsung | Samsung |
|  | format 3 & 4 | Huawei | Huawei | Huawei | Huawei |
| PRACH |  | CATT | CATT | CATT | CATT |
| Annex | FRC | China Telecom | China Telecom | China Telecom | China Telecom |
|  | Propagation conditions | Huawei | Huawei | Huawei | Huawei |
|  | Measurement system set-up and TT | N.A. | N.A. | China Telecom | Huawei |

Summary of simulation results

- PUSCH, PUCCH and PRACH simulation results

| Tdoc | Title | Source | Proposal/Notes |
| --- | --- | --- | --- |
| R4-1910801 | Summary of ideal and impairment results for NR BS demodulation requirements | China Telecom |  |
| R4-1911374 | Updated simulation results for NR BS demodulation performance in Rel-15 | Samsung |  |

- UCI on PUSCH simulation results

| Tdoc | Title | Source | Proposal |
| --- | --- | --- | --- |
| R4-1911376 | Summary of ideal and impairment results for UCI on PUSCH | Samsung |  |
| R4-1911375 | Updated simulation results for NR UCI on PUSCH | Samsung |  |

- Multi-slot PUCCH simulation results

| Tdoc | Title | Source | Proposal |
| --- | --- | --- | --- |
| R4-1911180 | Simulation results collection of multi-slot PUCCH | ZTE Wistron Telecom AB |  |
| R4-1911111 | Simulation results for NR PUCCH multi-slot demodulation requirement | Huawei, HiSilicon |  |
| R4-1911181 | Simulation results on multi-slot PUCCH | ZTE Wistron Telecom AB |  |
| R4-1911377 | Ideal and impairment simulation results for NR PUCCH with multi-slot | Samsung |  |

NR Rel-16 Demodulation

Contributions list and summary of proposals

Contribution list for discussion

| Tdoc | Title | Source | Proposal |
| --- | --- | --- | --- |
| R4-1910797 | On PUSCH demodulation requirements | China Telecom | Proposal 1: Introduce new PUSCH performance tests at 30% throughput. Proposal 2: For 30% PUSCH throughput test, only cover MCS 2, 1 Tx and the minimal channel bandwidth per SCS. Proposal 3: For 30% PUSCH throughput test, configure 1 PRB if possible. Proposal 4: Discuss whether the FR2 PUSCH 2T2R MCS12 demodulation test can be applicable from Rel-15. |
| R4-1910985 | Discussion on PUSCH performance requirement with 30% throughput metric | CATT | Proposal 1: It is necessary to introduce PUSCH with 30% throughput metric to significantly reflect the HARQ processing capability. Proposal 2: RAN4 can adopt the test parameters for NR PUSCHwith 30% throughput metricin Table A5-1 and Table A5-2 in Annex. |
| R4-1911106 | Discussion on NR Rel-16 BS demodulation requirements | Huawei, HiSilicon | Observation: the existing test cases with test metric 70% maximum throughput for MCS2 and lower SNR points can be used to verify the HARQ combing feature. Proposal: Not introduce additional PUSCH test cases with test metric 30% maximum throughput. |
| R4-1911197 | On NR Rel-16 performance requirement enhancement BS demodulation | Nokia, Nokia Shanghai Bell | PUSCH 30%TPUT requirements: - Currently there are 330 test cases specified for CP-OFDM PUSCH alone: - CP-OFDM, FR1, conducted: 210 TC - CP-OFDM, FR1, radiated: 70 TC - CP-OFDM, FR2, radiated: 50 TC - RAN4 to introduce 20 or less new 30%TPUT requirements in Rel-16. Testing additional TPUT operating points only makes sense in cases where new operating points cannot be extrapolated from previous ones, i.e., in cases where the TPUT vs. SINR curve exhibits severe non-linear behaviour between the two knees. TPUT vs. SINR curves exhibit severe non-linear behaviour for CP-OFDM and high MCS (and tentatively lower SCS). DFT-s-OFDM does not exhibit non-linear curve behaviour. RAN4 to introduce no more than 20 new 30%TPUT requirements for CP-OFDM with MCS 16 and 15 kHz SCS. Low PRB number performance requirements should be prioritized in the previously mentioned constraints. PUSCH FR2 2T2R MCS 12 requirements: The 70% TPUT working point can be reached for all MCS 12 test cases in FR2 2T2R, with a worst-case margin of 7dB to the 20dB OTA test limit. Start result collection for PUSCH FR2 MCS 12 performance requirement simulation results and introduce the corresponding performance requirements in Rel-16 versions of TS 38.104 and TS 38.141-2. |
| R4-1911381 | View on BS performance requirements for NR performance enhancement | Samsung | Observation 1: 30%TP test point requirement is only specified for 1Tx requirements with small RB allocation and lower MCS level in order to guarantee cell edge UE with acceptable link budget performance. Observation 2: Considering the test purpose of 30% TP requirement, it is not realistic to duplicate the existing test cases for 30%TP, if requirements of 30% TP are introduced, how to down select of the existing test cases for requirement should be considered. Proposal 1: If 30% TP requirement is introduced, the 30% TP requirement is only considered for 1Tx requirements with small RB allocation and lower MCS level with limited test cases. The down selection test cases are preferred as Waveform: one of CP-OFDM and DFT-s-OFDM Antenna configuration: 1x2 SCS&BW: 15KHz, 5MHz 30KHz, 10MHz 60KHz, 50MHz 120KHz, 50MHz DMRS configuration: 1+1 for both FR1 and FR2 Time domain allocation: Type A MCS: MCS2 Proposal 2: The FRC parameters for FR2 PUSCH performance requirements for MCS 12 are follows: Table8: FRC parameters for FR2 PUSCH performance requirements, transform precoding disabled, Additional DM-RS position = pos0 and 2 transmission layer (16QAM, R=434/1024) Table9: FRC parameters for FR2 PUSCH performance requirements, transform precoding disabled, Additional DM-RS position = pos1 and 2 transmission layer (16QAM, R=434/1024) |
| R4-1912159 | Proposed new requirements for 30% throughput testing | Ericsson | This contribution proposes 12 new requirements for FR1 and 4 new requirements for FR2 with a 30% throughput test point. For both FR1 and FR2, after applicability rules are applied a single additional test will be added. |
| R4-1912160 | Draft CR to TS 38.104: Introduction of PUSCH requirements at 30% throughput point | Ericsson |  |
| R4-1912161 | Draft CR to TS 38.141-1: Introduction of PUSCH requirements at 30% throughput point | Ericsson |  |
| R4-1912162 | Draft CR to TS 38.141-2: Introduction of PUSCH requirements at 30% throughput point | Ericsson |  |
| R4-1912449 | Views on 30% TP test point for BS demodulation | NTT DOCOMO, INC. | Observation 1: The performance difference at 30%ile for QPSK and 16QAM between with HARQ and without HARQ is approximately 1.5dB and 4dB, respectively. Proposal 1: Introduce PUSCH requirements with 30%ile test metric for MCS2 (QPSK) and MCS16 (16QAM). Proposal 2: Define PUSCH requirements at 30%ile throughput metric for the following waveform, CBW and SCS combinations. - CP-OFDM - 15kHz SCS: 5/10/20MHz - 30kHz SCS: 10/20/40/100MHz - 60kHz SCS: 50/100MHz - 120kHz SCS: 50/100/200MHz - DFT-s-OFDM - 15kHz SCS: 5MHz - 30kHz SCS: 10MHz - 60kHz SCS: 50MHz - 120kHz SCS: 50MHz |

Contribution list for FR2 PUSCH 2T2R MCS12 simulations

| Tdoc | Title | Source | Proposal |
| --- | --- | --- | --- |
| R4-1910798 | Ideal and impairment results for FR2 PUSCH 2T2R MCS12 | China Telecom |  |
| R4-1910799 | Summary of ideal and impairment results for FR2 PUSCH 2T2R MCS12 | China Telecom |  |
| R4-1911107 | Simulation results for NR FR2 PUSCH with 2T2R and MCS12 performance requirements | Huawei, HiSilicon |  |
| R4-1911196 | NR Rel-16 performance requirement enhancement BS demodulation simulation results | Nokia, Nokia Shanghai Bell | - Simulation results for both 30% TP and MCS12 |
| R4-1912165 | Discussion on BS type 2-O PUSCH demodulation requirements with TBD and near 20dB SNR | Ericsson | - Observation - From Figure 2.1, only 60kHz SCS with DM-RS additional position 0 cases can’t reach 100% of maximum throughput, but they keep stable performance after reaching its highest point like other cases. There are a lot of margin between highest performance point and 70% of maximum performance point, so we can say the requirements based on these results are reliable. - In Table 2.2-3, we deliver all ideal results and implementation results (adding 3dB to ideal results). From implementation results, all SNR at 70% of maximum throughput are around 13~15dB which is still quite below 20dB. Then all these cases become testable and reproducible if we apply FRC definition in Table 2.2-1 and Table 2.2-2. |
| R4-1912166 | Draft CR to TS 38.104: BS 2-O PUSCH demodulation with transform precoding disabled | Ericsson |  |

Discussions

Issue 1: 30% TP test point

Agreements in the last meeting RAN#82 (RP-191587):

BS demodulation requirements:

- To study the tests coverage of PUSCH requirements for 30% TP test point. Limited test cases will be introduced if the existing test coverage is insufficient.

Open issues:

1: Whether to introduce PUSCH performance tests at 30% TP

- Option 1: Introduce PUSCH performance tests at 30% throughput (China Telecom, Samsung, CATT, DCM, Ericsson)

- Option 2: Not introduce PUSCH performance tests at 30% throughput (Huawei)

- Option 3: Introduce the PUSCH performance tests at 30% throughput with less than 21 number of requirements (Nokia)

2: If agreed to introduce, the related test parameters:

MCS:

- Option 1: MCS 2 (China Telecom, Samsung, Ericsson)

- Option 2: MCS 16 (Nokia, Ericsson)

- Option 2: MCS 2 and MCS 16 (CATT, DCM)

DCM: the motivations for this test: one is to test the functionality for HARQ, it is better to use MCS2; For soft combing performance, need to check the soft buffer, it is better to use MCS16.

Nokia: MCS16 is checking the receiver working for the HARQ combing and is more challenge， 30% TP is for test coverage constraint scenario

Ericsson：MCS2 is more realistic, agree with Nokia that MCS16 is more constraint for HARQ combing. Ok for either one.

Samsung：30% TP is for cell edge, MCS2 is more realistic. MCS 16 is high SNR and maybe in the middle of the coverage.

Ericsson: benefit of the test is to test the HARQ combing.

DCM: LTE define 30% TP for both QPSK and 16QAM. We can compromise to choose MCS2 considering the key motivation for 30% TP case is for cell edge coverage.

Samsung: most of LTE cases for 30% TP is QPSK.

CTC: Ok to choose MCS2 to reduce the workload.

MCS2 is agreed

Antenna:

- FR1:

  - Option 1: 1T2R (CATT, Samsung)

  - Option 2: 1T2R, 1T4R and 1T8R (Ericsson, CTC)

- FR2: 1T2R (China Telecom, CATT, Samsung, Ericsson)

Ericsson: different implementation. Requirements for different antenna. Propose to one test case, this is related to the simulation load.

Nokia: FR1, all BS should support 1T2R.

DCM: better to choose Option 2 and can discuss the test applicability rule for the number of test cases

Samsung: for cell coverage issue, for 4R and 8R, the requirements are almost -8dB or -9dB, no coverage issues. Multi-slot PUCH is only for 1T2R.

China Telecom: support Option2 that cover all our typical scenarios.

Ericsson：For HARQ combing functionality, we also set requirements for 4R and 8R antenna to ensure the HARQ requirements.

Samsung: For HARQ combing functionality, 1T2R is enough. No necessary to use 4R and 8R for HARQ combine

Ericsson: if the BS only support 8R

Nokia: only test one antenna, test applicability rule for one test. Agree to go with Ericsson.

Samsung: if the BS only support 8R, BS can choose any two of antenna from 8 Rx for the test, but the requirements are only defined for 2Rx. The similar issue is for BS with 64Rx. The same principle.

Ericsson: Ok to set requirements, it depend on the software. For BS with 64R, it still needs to choose requirements for 4R and 2R, just the simulation load is increased.

Samsung: The test point is for cell coverage and 2Rx is easy for TE.

Keysight: if you already buy test equipment for 8Rx for 70% TP test.

DCM: for test coverage, different SNR level for different number antenna, only one test case is for test

Nokia: For 30% TP test cases, BS only needs to test one of the test cases with the highest number of Rx antenna.

Samsung: For 8Rx requirements, how about for OTA test for FR1.

Ericsson: 3 set of requirements, but only one set of requirements for test. Strong views on the 3 set of requirements for 2R/4R/8R.  Combine other different parameters, total 16 cases.

Samsung: Firstly agree 1T2R for both FR1 and FR2. Then discuss 4R and 8R.

Only define one test case for one number of antennas.

Bandwidth/SCS:

- Option 1: the minimal channel bandwidth per SCS (5MHz CBW/15kHz SCS, 10MHz CBW/30kHz SCS, 50MHz CBW/60kHz SCS, 50MHz CBW/120kHz SCS) (China Telecom, CATT, Samsung, Ericsson)

- Option 2: Same CBW and SCS combinations as the existing 70%ile performance requirements defined (DCM):

  - CP-OFDM

    - 15kHz SCS: 5/10/20MHz

    - 30kHz SCS: 10/20/40/100MHz

    - 60kHz SCS: 50/100MHz

    - 120kHz SCS: 50/100/200MHz

  - DFT-s-OFDM

    - 15kHz SCS: 5MHz

    - 30kHz SCS: 10MHz

    - 60kHz SCS: 50MHz

    - 120kHz SCS: 50MHz

PRB number for PUSCH

- Option 1: 1 PRB (China Telecom)

- Option 2: Small RB allocation (Samsung)

- Option 3: The maximum number of PRB NRB for the agreed Bandwidth/SCS as defined in TS 38.101-1/2

Waveform:

- Option 1: CP-OFDM (Nokia, Samsung?, Ericsson)

- Option 2: Both CP-OFDM and DFT-s-OFDM (CATT, DCM)

PUSCH time domain resource allocation type

- FR1:

  - Option 1: Type A (Samsung)

  - Option 2: Type B (Huawei)

  - Option 3: Both Type A and Type B (Ericsson)

- FR2: Type B (Samsung, Ericsson, Huawei)

DM-RS configuration

- FR1: 1+1 (Samsung, Ericsson)

- FR2:

  - Option 1: 1+1 (Samsung, Huawei)

  - Option 2: 1+0 and 1+1 (Ericsson)

TDD UL-DL configuration: Same TDD patterns as the requirements defined for 70% throughput cases

Channel Model:

- FR1:

  - Option 1: TDLB100-400 Low (the one in the spec for MCS2)

  - Option 2: TDLA30-300 Low (Ericsson)

- FR2: TDLA30-300 Low (CATT, Ericsson, same as the existing requirements with MCS2)

Applicability rule:

- SCS: Only test the lowest supported SCS for each frequency range

- PUSCH mapping type: Only test a single PUSCH mapping type that BS declared to support

- DM-RS configuration: only test a single DM-RS pattern

Discussion:

| Parameter | value |  |
| --- | --- | --- |
|  | FR1 | FR2 |
| Transform precoding | Disabled | Disabled |
| Number of Tx | 1 | 1 |
| Number of Rx | 2 | 2 |
| Number of layers | 1 | 1 |
| Transmission scheme | Identity matrix (TPMI index 0) | Identity matrix (TPMI index 0) |
| TDD UL-DL configuration | 15 kHz SCS: - 3D1S1U, S=10D:2G:2U 30 kHz SCS: - 7D1S2U, S=6D:4G:4U | 60kHz SCS: - DDDSU, S=10D:2G:2U 120kHz SCS: - DDDSU, S=10D:2G:2U |
| DMRS type | type 1 | type 1 |
| Number of DMRS symbols | 1+1 | 1+1 |
| symbols length | 14 | 10 |
| start symbol index | 0 | 0 |
| Time domain resource allocation type | Option 1: type A Option 2: type B Option 3: type A and type B | type B |
| Frequency domain resource | Full applicable test bandwidth | Full applicable test bandwidth |
| MCS index | 2 | 2 |
| Carrier frequency (GHz) | 4 | 30 |
| Propagation condition | Option 1: TDLB100-400 Option 2: TDLA30-300 Low | TDL-A 30ns, 300Hz |
| SCS and BW | 15kHz: 5MHz; 30kHz: 10MHz; | 60kHz: 50MHz; 120kHz: 50MHz; |
| PTRS | Not configured | Not configured |
| Timing offset | 0 | 0 |
| Frequency offset | 0 | 0 |
| Code block group, Frequency hopping, Limited buffer rate matching | Disabled | Disabled |
| Number of HARQ transmissions | 4 | 4 |
| HARQ RV sequence | {0, 2, 3,1} | {0, 2, 3,1} |
| Testing metric | SNR @30% of maximum throughput | SNR @30% of maximum throughput |

Agreements:

Issue 2: Performance requirements for FR2 PUSCH 2T2R tests with MCS 12

Agreements in the last meeting RAN4#92 (R4-1910006):

Agree to replace all the existing FR2 2T2R requirements with MCS 16 by MCS12 in TS 38.104 and TS 38.141-2.

Reuse the agreed simulation assumptions for FR2 2T2R cases with MCS 16 by referring to Slide#6 in R4-1907241

Open issues:

1: The applicable release of the PUSCH FR2 requirements for MCS12:

- Option 1: Applicable from Rel-15 (CTC, Samsung, DCM)

- Option 2: Applicable from Rel-16 (Nokia, Ericsson, Huawei)

2: FRC

- R4-1911381 (Samsung):

Table 8: FRC parameters for FR2 PUSCH performance requirements, transform precoding disabled, Additional DM-RS position = pos0 and 2 transmission layer (16QAM, R=434/1024)

| Subcarrier spacing [kHz] | 60 | 60 | 120 | 120 | 120 |
| --- | --- | --- | --- | --- | --- |
| Allocated resource blocks | 66 | 132 | 32 | 66 | 132 |
| CP-OFDM Symbols per slot (Note 1) | 9 | 9 | 9 | 9 | 9 |
| Modulation | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM |
| Code rate (Note 2) | 434/1024 | 434/1024 | 434/1024 | 434/1024 | 434/1024 |
| Payload size (bits) | 24072 | 48168 | 11784 | 24072 | 48168 |
| Transport block CRC (bits) | 24 | 24 | 24 | 24 | 24 |
| Code block CRC size (bits) | 24 | 24 | 24 | 24 | 24 |
| Number of code blocks - C | 3 | 6 | 2 | 3 | 6 |
| Code block size including CRC (bits) (Note 2) | 8056 | 8056 | 5928 | 8056 | 8056 |
| Total number of bits per slot | 42768 | 85536 | 20736 | 42768 | 85536 |
| Total symbols per slot | 7128 | 14256 | 3456 | 7128 | 14256 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, Additional DM-RS position = pos0 with l0= 0 as per Table 6.4.1.1.3-3 of TS 38.211 [5]. NOTE 2: Code block size including CRC (bits) equals to K' in sub-clause 5.2.2 of TS 38.212 [15]. |  |  |  |  |  |

Table9: FRC parameters for FR2 PUSCH performance requirements, transform precoding disabled, Additional DM-RS position = pos1 and 2 transmission layer (16QAM, R=434/1024)

| Subcarrier spacing [kHz] | 60 | 60 | 120 | 120 | 120 |
| --- | --- | --- | --- | --- | --- |
| Allocated resource blocks | 66 | 132 | 32 | 66 | 132 |
| CP-OFDM Symbols per slot (Note 1) | 8 | 8 | 8 | 8 | 8 |
| Modulation | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM |
| Code rate (Note 2) | 434/1024 | 434/1024 | 434/1024 | 434/1024 | 434/1024 |
| Payload size (bits) | 21504 | 43032 | 10504 | 21504 | 43032 |
| Transport block CRC (bits) | 24 | 24 | 24 | 24 | 24 |
| Code block CRC size (bits) | 24 | 24 | 24 | 24 | 24 |
| Number of code blocks - C | 3 | 6 | 2 | 3 | 6 |
| Code block size including CRC (bits) (Note 2) | 7200 | 7200 | 5288 | 7200 | 7200 |
| Total number of bits per slot | 50688 | 101376 | 24576 | 50688 | 101376 |
| Total symbols per slot | 12672 | 25344 | 6144 | 12672 | 25344 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, Additional DM-RS position = pos0 with l0= 0 as per Table 6.4.1.1.3-3 of TS 38.211 [5]. NOTE 2: Code block size including CRC (bits) equals to K' in sub-clause 5.2.2 of TS 38.212 [15]. |  |  |  |  |  |

- R4-1912165(Ericsson)

| - Table 2.2-1: FRC parameters for FR2 PUSCH performance requirements, transform precoding disabled, Additional DM-RS position = pos0 and 2 transmission layers (16QAM, R=434/1024) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference channel | G-FR2-A4-6 | G-FR2-A4-7 | G-FR2-A4-8 | G-FR2-A4-9 | G-FR2-A4-10 |
| Subcarrier spacing [kHz] | 60 | 60 | 120 | 120 | 120 |
| Allocated resource blocks | 66 | 132 | 32 | 66 | 132 |
| CP-OFDM Symbols per slot (Note 1) | 9 | 9 | 9 | 9 | 9 |
| Modulation | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM |
| Code rate (Note 2) | 434/1024 | 434/1024 | 434/1024 | 434/1024 | 434/1024 |
| Payload size (bits) | 24072 | 48168 | 11784 | 24072 | 48168 |
| Transport block CRC (bits) | 24 | 24 | 24 | 24 | 24 |
| Code block CRC size (bits) | 24 | 24 | 24 | 24 | 24 |
| Number of code blocks - C | 3 | 6 | 2 | 3 | 6 |
| Code block size including CRC (bits) (Note 2) | 8056 | 8056 | 5928 | 8056 | 8056 |
| Total number of bits per slot | 57024 | 114048 | 27648 | 57024 | 114048 |
| Total symbols per slot | 14256 | 28512 | 6912 | 14256 | 28512 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, Additional DM-RS position = pos1 with l0= 0 and l =8 as per Table 6.4.1.1.3-3 of TS 38.211 [5]. NOTE 2: Code block size including CRC (bits) equals to K' in sub-clause 5.2.2 of TS 38.212 [15]. |  |  |  |  |  |

| Table 2.2-2: FRC parameters for FR2 PUSCH performance requirements, transform precoding disabled, Additional DM-RS position = pos1 and 2 transmission layers (16QAM, R=434/1024) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference channel | G-FR2-A4-16 | G-FR2-A4-17 | G-FR2-A4-18 | G-FR2-A4-19 | G-FR2-A4-20 |
| Subcarrier spacing [kHz] | 60 | 60 | 120 | 120 | 120 |
| Allocated resource blocks | 66 | 132 | 32 | 66 | 132 |
| CP-OFDM Symbols per slot (Note 1) | 8 | 8 | 8 | 8 | 8 |
| Modulation | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM |
| Code rate (Note 2) | 434/1024 | 434/1024 | 434/1024 | 434/1024 | 434/1024 |
| Payload size (bits) | 21504 | 43032 | 10504 | 21504 | 43032 |
| Transport block CRC (bits) | 24 | 24 | 24 | 24 | 24 |
| Code block CRC size (bits) | 24 | 24 | 24 | 24 | 24 |
| Number of code blocks - C | 3 | 6 | 2 | 3 | 6 |
| Code block size including CRC (bits) (Note 2) | 7200 | 7200 | 5288 | 7200 | 7200 |
| Total number of bits per slot | 50688 | 101376 | 24576 | 50688 | 101376 |
| Total symbols per slot | 12672 | 25344 | 6144 | 12672 | 25344 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, Additional DM-RS position = pos1 with l0= 0 and l =8 as per Table 6.4.1.1.3-3 of TS 38.211 [5]. |  |  |  |  |  |
| NOTE 2: Code block size including CRC (bits) equals to K' in sub-clause 5.2.2 of TS 38.212 [15]. |  |  |  |  |  |

Discussion:

Agreements: