3GPP TSG-RAN4 WG4 Meeting # 92Bis	R4-1912679

# Chongqing, China, 14 - 18 October, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.104 | CR | <CR#> | rev | - | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR: Updates to PF3 and PF4 performance requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-09-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Square brackets and TBD still exist for PUCCH format 3 and PUCCH format 4 performance requirements Updated simulation results from company are submitted. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated the PUCCH format 3 and PUCCH format 4 performance requirements as per the latest submitted simulation results. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification will still be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.5, 8.3.6, 11.3.2.5, 11.3.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.141-1 and TS 38.141-2 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

<<The Start of Updates>>

8.3.5	Performance requirements for PUCCH format 3

8.3.5.1	General

The performance is measured by the required SNR at UCI block error probability not exceeding 1%.

The UCI block error probability is defined as the conditional probability of incorrectly decoding the UCI information when the UCI information is sent. The UCI information does not contain CSI part 2.

The transient period as specified in TS 38.101-1 [17] subclause 6.3.3.1 is not taken into account for performance requirement testing, where the RB hopping is symmetric to the CC center, i.e. intra-slot frequency hopping is enabled.

Table 8.3.5.1-1: Test Parameters

| Parameter | Test 1 | Test 2 |
| --- | --- | --- |
| Modulation order | QPSK |  |
| First PRB prior to frequency hopping | 0 |  |
| Intra-slot frequency hopping | enabled |  |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs - 1) |  |
| Group and sequence hopping | neither |  |
| Hopping ID | 0 |  |
| Number of PRBs | 1 | 3 |
| Number of symbols | 14 | 4 |
| The number of UCI information bits | 16 | 16 |
| First symbol | 0 | 0 |

8.3.5.2	Minimum requirements

The UCI block error probability shall not exceed 1% at the SNR given in Table 8.3.5.2-1 and Table 8.3.5.2-2.

Table 8.3.5.2-1: Minimum requirements for PUCCH format 3 with 15 kHz SCS

| Test Number | Number of TX antennas | Number of RX antennas | Cyclic Prefix | Propagation conditions and correlation matrix (Annex G) | Additional DM-RS configuration | Channel bandwidth / SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 5 MHz | 10 MHz | 20 MHz |
| 1 | 1 | 2 | Normal | TDLC300-100 Low | No additional DM-RS | [0.2] | [1.1] | [0.3] |
|  |  |  |  |  | Additional DM-RS | [-0.1] | [0.5] | [-0.1] |
|  |  | 4 | Normal | TDLC300-100 Low | No additional DM-RS | [-3.8] | [-3.3] | [-3.8] |
|  |  |  |  |  | Additional DM-RS | [-4.3] | [-4.0] | [-4.0] |
|  |  | 8 | Normal | TDLC300-100 Low | No additional DM-RS | [-7.0] | [-6.7] | [-6.9] |
|  |  |  |  |  | Additional DM-RS | [-7.7] | [-7.5] | [-7.7] |
| 2 | 1 | 2 | Normal | TDLC300-100 Low | No additional DM-RS | [1.4] | [2.2] | [2.0] |
|  |  | 4 | Normal | TDLC300-100 Low | No additional DM-RS | [-3.1] | [-2.5] | [-2.5] |
|  |  | 8 | Normal | TDLC300-100 Low | No additional DM-RS | [-6.5] | [-6.0] | [-6.2] |

Table 8.3.5.2-2: Minimum requirements for PUCCH format 3 with 30 kHz SCS

| Test Number | Number of TX antennas | Number of RX antennas | Cyclic Prefix | Propagation conditions and correlation matrix (Annex G) | Additional DM-RS configuration | Channel bandwidth / SNR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 10 MHz | 20 MHz | 40 MHz | 100 MHz |
| 1 | 1 | 2 | Normal | TDLC300-100 Low | No additional DM-RS | [0.9] | [0.6] | [0.6] | [0.9] |
|  |  |  |  |  | Additional DM-RS | [0.5] | [0.3] | [0.0] | [0.1] |
|  |  | 4 | Normal | TDLC300-100 Low | No additional DM-RS | [-3.1] | [-3.4] | [-3.2] | [-3.5] |
|  |  |  |  |  | Additional DM-RS | [-3.7] | [-4.1] | [-4.0] | [-4.2] |
|  |  | 8 | Normal | TDLC300-100 Low | No additional DM-RS | [-6.6] | [-6.7] | [-6.8] | [-6.8] |
|  |  |  |  |  | Additional DM-RS | [-7.5] | [-7.6] | [-7.6] | [-7.7] |
| 2 | 1 | 2 | Normal | TDLC300-100 Low | No additional DM-RS | [1.8] | [2.0] | [2.0] | [1.5] |
|  |  | 4 | Normal | TDLC300-100 Low | No additional DM-RS | [-2.9] | [-3.0] | [-2.4] | [-3.0] |
|  |  | 8 | Normal | TDLC300-100 Low | No additional DM-RS | [-6.4] | [-6.0] | [-6.4] | [-6.2] |

8.3.6	Performance requirements for PUCCH format 4

8.3.6.1	General

The performance is measured by the required SNR at UCI block error probability not exceeding 1%.

The UCI block error probability is defined as the conditional probability of incorrectly decoding the UCI information when the UCI information is sent. The UCI information does not contain CSI part 2.

The transient period as specified in TS 38.101-1 [17] subclause 6.3.3.1 is not taken into account for performance requirement testing, where the RB hopping is symmetric to the CC center, i.e. intra-slot frequency hopping is enabled.

Table 8.3.6.1-1: Test parameters

| Parameter | Value |
| --- | --- |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs - 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of symbols | 14 |
| The number of UCI information bits | 22 |
| First symbol | 0 |
| Length of the orthogonal cover code | n2 |
| Index of the orthogonal cover code | n0 |

8.3.6.2	Minimum requirement

The UCI block error probability shall not exceed 1% at the SNR given in Table 8.3.6.2-1 and Table 8.3.6.2-2.

Table 8.3.6.2-1: Required SNR for PUCCH format 4 with 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic Prefix | Propagation conditions and correlation matrix (Annex G) | Additional DM-RS configuration | Channel bandwidth / SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 5 MHz | 10 MHz | 20 MHz |
| 1 | 2 | Normal | TDLC300-100 Low | No additional DM-RS | [1.8] | [2.6] | [2.2] |
|  |  |  |  | Additional DM-RS | [1.6] | [2.4] | [1.8] |
|  | 4 | Normal | TDLC300-100 Low | No additional DM-RS | [-2.3] | [-1.9] | [-2.2] |
|  |  |  |  | Additional DM-RS | [-2.9] | [-2.6] | [-2.7] |
|  | 8 | Normal | TDLC300-100 Low | No additional DM-RS | [-5.9] | [-5.7] | [-5.8] |
|  |  |  |  | Additional DM-RS | [-6.6] | [-6.4] | [-6.3] |

Table 8.3.6.2-2: Required SNR for PUCCH format 4 with 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic Prefix | Propagation conditions and correlation matrix (Annex G) | Additional DM-RS configuration | Channel bandwidth / SNR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 10 MHz | 20 MHz | 40 MHz | 100 MHz |
| 1 | 2 | Normal | TDLC300-100 Low | No additional DM-RS | [3.1] | [2.8] | [3.1] | [2.8] |
|  |  |  |  | Additional DM-RS | [2.8] | [2.3] | [3.1] | [2.2] |
|  | 4 | Normal | TDLC300-100 Low | No additional DM-RS | [-1.7] | [-1.9] | [-1.7] | [-2.1] |
|  |  |  |  | Additional DM-RS | [-2.0] | [-2.5] | [-2.5] | [-2.4] |
|  | 8 | Normal | TDLC300-100 Low | No additional DM-RS | [-5.6] | [-5.5] | [-5.5] | [-5.5] |
|  |  |  |  | Additional DM-RS | [-6.2] | [-6.1] | [-6.4] | [-6.2] |

<<The Second Updates>>

11.3.2.5	Performance requirements for PUCCH format 3

11.3.2.5.1	General

The performance is measured by the required SNR at UCI block error probability not exceeding 1%.

The UCI block error probability is defined as the conditional probability of incorrectly decoding the UCI information when the UCI information is sent. The UCI information does not contain CSI part 2.

The transient period as specified in TS 38.101-2 [18] subclause 6.3.3.1 is not taken into account for performance requirement testing, where the RB hopping is symmetric to the CC center, i.e. intra-slot frequency hopping is enabled.

Table 11.3.2.5.1-1: Test parameters

| Parameter | Test 1 | Test 2 |
| --- | --- | --- |
| Modulation order | QPSK |  |
| First PRB prior to frequency hopping | 0 |  |
| Intra-slot frequency hopping | enabled |  |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |  |
| Group and sequence hopping | neither |  |
| Hopping ID | 0 |  |
| Number of PRBs | 1 | 3 |
| Number of symbols | 14 | 4 |
| The number of UCI information bits | 16 | 16 |
| First symbol | 0 | 0 |

11.3.2.5.2	Minimum requirements

The UCI block error probability shall not exceed 1% at the SNR given in Table 11.3.2.5.2-1 and Table 11.3.2.5.2-2.

Table 11.3.2.5.2-1: Required SNR for PUCCH format 3 with 60 kHz SCS

| Test Number | Number of TX antennas | Number of demodulation branches | Cyclic Prefix | Propagation conditions and correlation matrix (Annex G) | Additional DM-RS configuration | Channel Bandwidth / SNR (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 50 MHz | 100 MHz |
| 1 | 1 | 2 | Normal | TDLA30-300 Low | No additional DM-RS | [1.6] | [0.7] |
|  |  |  |  |  | Additional DM-RS | [1.3] | [0.9] |
| 2 | 1 | 2 | Normal | TDLA30-300 Low | No additional DM-RS | [3.0] | [2.4] |

Table 8.3.2.5.2-2: Required SNR for PUCCH format 3 with 120 kHz SCS

| Test Number | Number of TX antennas | Number of demodulation branches | Cyclic Prefix | Propagation conditions and correlation matrix (Annex G) | Additional DM-RS configuration | Channel Bandwidth / SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 50 MHz | 100 MHz | 200 MHz |
| 1 | 1 | 2 | Normal | TDLA30-300 Low | No additional DM-RS | [1.4] | [0.7] | [0.7] |
|  |  |  |  |  | Additional DM-RS | [1.3] | [1.4] | [0.9] |
| 2 | 1 | 2 | Normal | TDLA30-300 Low | No additional DM-RS | [1.1] | [2.9] | [1.4] |

11.3.2.6	Performance requirements for PUCCH format 4

11.3.2.6.1	General

The performance is measured by the required SNR at UCI block error probability not exceeding 1%.

The UCI block error probability is defined as the conditional probability of incorrectly decoding the UCI information when the UCI information is sent. The UCI information does not contain CSI part 2.

The transient period as specified in TS 38.101-2 [18] subclause 6.3.3.1 is not taken into account for performance requirement testing, where the RB hopping is symmetric to the CC center, i.e. intra-slot frequency hopping is enabled.

Table 11.3.2.6.1-1: Test parameters

| Parameter | Value |
| --- | --- |
| Modulation | QPSK |
| First PRB prior to frequency hoppingstartingPRB | 0 |
| Number of PRBs | 1 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of symbols | 14 |
| The number of UCI information bits | 22 |
| First symbol | 0 |
| Length of the orthogonal cover code | n2 |
| Index of the orthogonal cover code | n0 |

11.3.2.6.2	Minimum requirements

The UCI block error probability shall not exceed 1% at the SNR given in Table 11.3.2.6.2-1 and Table 11.3.2.6.2-2.

Table 11.3.2.6.2-1: Required SNR for PUCCH format 4 with 60 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic Prefix | Propagation conditions and correlation matrix (Annex G) | Additional DM-RS configuration | Channel Bandwidth / SNR (dB) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 50 MHz | 100 MHz |
| 1 | 2 | Normal | TDLA30-300 Low | No additional DM-RS | [3.0] | [2.7] |
|  |  |  |  | Additional DM-RS | [3.1] | [3.5] |

Table 8.3.2.6.2-2: Required SNR for PUCCH format 4 with 120 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic Prefix | Propagation conditions and correlation matrix (Annex G) | Additional DM-RS configuration | Channel Bandwidth / SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 50 MHz | 100 MHz | 200MHz |
| 1 | 2 | Normal | TDLA30-300 Low | No additional DM-RS | [2.8] | [2.8] | [3.5] |
|  |  |  |  | Additional DM-RS | [3.6] | [3.8] | [3.2] |

<<End of Updates>>