3GPP TSG-RAN WG4 Meeting #92bis	R4-1912692

# Chongqing, China, 14th - 18th Oct,2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-1 | CR | - | rev | 0 | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on NR PUCCH format2 conducted performance requirement for TS 38.141-1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-10-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | PUCCH format2 performance requirements have been introduced for NR Rel-15. Update the requirments based on the latest simulation summary |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the requirements based on the latest simulation summary Update the initial conditions and procedure |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PUCCH format2 conducted performance requirement can not be verfied |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.141-2 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change>

### 8.3.3	Performance requirements for PUCCH format 2

#### 8.3.3.1	ACK missed detection

##### 8.3.3.1.1	Definition and applicability

The performance requirement of PUCCH format 2 for ACK missed detection is determined by the two parameters: probability of false detection of the ACK and the probability of detection of ACK. The performance is measured by the required SNR at probability of detection equal to 0.99. The probability of false detection of the ACK shall be 0.01 or less.

The probability of false detection of the ACK is defined as a probability of erroneous detection of the ACK when input is only noise.

The probability of detection of ACK is defined as probability of detection of the ACK when the signal is present.

Which specific test(s) are applicable to BS is based on the test applicability rules defined in subclause 8.1.2.

##### 8.3.3.1.2	Minimum requirements

The minimum requirements are in TS 38.104 [2] subclause 8.3.4.

##### 8.3.3.1.3	Test purpose

The test shall verify the receiver’s ability to detect ACK bits under multipath fading propagation conditions for a given SNR.

##### 8.3.3.1.4	Method of test

###### 8.3.3.1.4.1	Initial Condition

Test environment: Normal, see annex B.2.

RF channels to be tested for single carrier (SC): M; see subclause 4.9.1

###### 8.3.3.1.4.2	Procedure

1)	Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to all BS antenna connectors for diversity reception via a combining network as shown in annex D.5 and D.6 for BS type 1-C and type 1-H respectively.

2)	Adjust the AWGN generator, according to the channel bandwidth defined in table 8.3.3.1.4.2-1.

Table 8.3.3.1.4.2-1: AWGN power level at the BS input

| Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- |
| 15 | 5 | -83.5 dBm / 4.5 MHz |
|  | 10 | -80.3 dBm / 9.36 MHz |
|  | 20 | -77.2 dBm / 19.08MHz |
| 30 | 10 | -80.6 dBm / / 8.64 MHz |
|  | 20 | -77.4 dBm / 18.36 MHz |
|  | 40 | -74.2 dBm / 38.16 MHz |
|  | 100 | -70.1 dBm / 98.28 MHz |

3)	The characteristics of the wanted signal shall be configured according to TS 38.211 [17], and the specific test parameters are configured as blow:

Table 8.3.3.1.4.2-2: Test parameters

| Parameter | Value |
| --- | --- |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index - (Number of PRBs - 1) |
| Number of PRBs | 4 |
| Number of symbols | 1 |
| The number of UCI information bits | 4 |
| First symbol | 13 |
| DM-RS sequence generation | NID0=0 |

4)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex G.

5)	Adjust the equipment so that the SNR specified in table 8.3.3.1.5-1 and table 8.3.3.1.5-2 is achieved at the BS input during the UCI transmissions.

6)	The signal generator sends a test pattern with the pattern outlined in figure 8.3.3.1.4.2-1. The following statistics are kept: the number of ACKs detected in the idle periods and the number of missed ACKs.

Figure 8.3.3.1.4.2-1: Test signal pattern for PUCCH format 2 demodulation tests

##### 8.3.3.1.5	Test requirements

The fraction of falsely detected ACKs shall be less than 1% and the fraction of correctly detected ACKs shall be larger than 99% for the SNR listed in table 8.3.3.1.5-1 and table 8.3.3.1.5-2.

Table 8.3.3.1.5-1: Required SNR for PUCCH format 2 with 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic Prefix | Propagation conditions and correlation matrix (annex G) | Channel bandwidth / SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 5 MHz | 10 MHz | 20 MHz |
| 1 | 2 | Normal | TDLC300-100 Low | [6.4] | [6.2] | [6.5] |
|  | 4 | Normal | TDLC300-100 Low | [1.0] | [1.1] | [0.9] |
|  | 8 | Normal | TDLC300-100 Low | [-2.9] | [-2.9] | [-2.9] |

Table 8.3.3.1.5-2: Required SNR for PUCCH format 2 with 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic Prefix | Propagation conditions and correlation matrix (annex G) | Channel bandwidth/ SNR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 10MHz | 20MHz | 40MHz | 100MHz |
| 1 | 2 | Normal | TDLC300-100 Low | [6.1] | [6.2] | [6.1] | [6.3] |
|  | 4 | Normal | TDLC300-100 Low | [0.9] | [0.8] | [0.9] | [1.0] |
|  | 8 | Normal | TDLC300-100 Low | [-3.0] | [-3.0] | [-2.9] | [-2.7] |

#### 8.3.3.2	UCI BLER performance requirements

##### 8.3.3.2.1	Definition and applicability

The performance is measured by the required SNR at UCI block error probability not exceeding 1%.

The UCI block error probability is defined as the probability of incorrectly decoding the UCI information when the UCI information is sent. The UCI information does not contain CSI part 2.

Which specific test(s) are applicable to BS is based on the test applicability rules defined in subclause 8.1.2

The transient period as specified in TS 38.101-1 [21] subclause 6.3.3.1 is not taken into account for performance requirement testing, where the RB hopping is symmetric to the CC center, i.e. intra-slot frequency hopping is enabled.

##### 8.3.3.2.2	Minimum Requirement

The minimum requirement is TS 38.104 [2] subclause 8.3.4.

##### 8.3.3.2.3	Test purpose

The test shall verify the receiver’s ability to detect UCI under multipath fading propagation conditions for a given SNR.

##### 8.3.3.2.4	Method of test

###### 8.3.3.2.4.1	Initial Condition

Test environment: Normal, see annex B.2.

RF channels to be tested for single carrier (SC): M; see subclause 4.9.1

###### 8.3.3.2.4.2	Procedure

1)	Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to all BS antenna connectors for diversity reception via a combining network as shown in annex D.5 and D.6 for BS type 1-C and BS type 1-H respectively.

2)	Adjust the AWGN generator, according to the channel bandwidth defined in table 8.3.3.2.4.2-1.

Table 8.3.3.2.4.2-1: AWGN power level at the BS input

| Subcarrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- |
| 15 | 5 | -83.5 dBm / 4.5 MHz |
|  | 10 | -80.3 dBm / 9.36 MHz |
|  | 20 | -77.2 dBm / 19.08 MHz |
| 30 | 10 | -80.6 dBm / 8.64 MHz |
|  | 20 | -77.4 dBm / 18.36 MHz |
|  | 40 | -74.2 dBm / 38.16 MHz |
|  | 100 | -70.1 dBm / 98.28 MHz |

3)	The characteristics of the wanted signal shall be configured according to TS 38.211 [17], and the specific test parameters are configured as blow:

Table 8.3.3.2.4.2-2: Test parameters

| Parameter | Value |
| --- | --- |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs - 1) |
| Number of PRBs | 9 |
| Number of symbols | 2 |
| The number of UCI information bits | 22 |
| First symbol | 12 |
| DM-RS sequence generation | NID0=0 |

4)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex G.

5)	Adjust the equipment so that the SNR specified in table 8.3.3.2.5-1 or table 8.3.3.2.5-2 is achieved at the BS input during the UCI transmissions.

6)	The signal generator sends a test pattern with the pattern outlined in figure 8.3.3.2.4.2-1. The following statistics are kept: the number of incorrectly decoded UCI.

Figure 8.3.3.2.4.2-1: Test signal pattern for PUCCH format 2 demodulation tests

##### 8.3.3.2.5	Test requirements

The fraction of incorrectly decoded UCI shall be less than 1% for the SNR listed in table 8.3.3.2.5-1 and table 8.3.3.2.5-2.

Table 8.3.3.2.5-1: Required SNR for PUCCH format 2 with 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic Prefix | Propagation conditions and correlation matrix (annex G) | Channel bandwidth / SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 5 MHz | 10 MHz | 20 MHz |
| 1 | 2 | Normal | TDLC300-100 Low | [0.8] | [1.4] | [1.8] |
|  | 4 | Normal | TDLC300-100 Low | [-3.0] | [-2.6] | [-2.6] |
|  | 8 | Normal | TDLC300-100 Low | [-6.2] | [-6.1] | [-6.2] |

Table 8.3.3.2.5-2: Required SNR for PUCCH format 2 with 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic Prefix | Propagation conditions and correlation matrix (annex G) | Channel bandwidth/ SNR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 10MHz | 20MHz | 40MHz | 100MHz |
| 1 | 2 | Normal | TDLC300-100 Low | [1.1] | [1.7] | [1.0] | [0.9] |
|  | 4 | Normal | TDLC300-100 Low | [-2.7] | [-2.3] | [-2.7] | [-2.8] |
|  | 8 | Normal | TDLC300-100 Low | [-5.2] | [-5.2] | [-6.1] | [-5.3] |

<End of Change>