3GPP TSG-RAN4 Meeting #92bis 	R4-1912705

Chongqing , China, 14th – 18th Oct 2019

Title:	Reply LS on SFTD measurement

Reply To:	R4-1910716 (R2-1911700)

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	RAN WG 4

To:	RAN WG 2

Cc:			RAN WG 1

Contact Person:

Name:	Qiming Li

E-mail Address:	qiming.li@intel.com

Attachments:	N.A.

1. Overall Description:

RAN4 would like to thank RAN2 for the reply LS on SFTD measurements (R2-1911700). After discussion in RAN4, we have the following response:

Question: Is it necessary and feasible to introduce SFTD measurement between NR PCell and intra-frequency NR neighbour cells for NR SA in Rel-15?

[RAN4] From feasibility perspective, SFTD measurement between NR PCell and intra-frequency NR neighbour cells are already possible in Rel-15 based on the DRX-based SFTD measurements approach.

From necessity perspective, in RAN4’s view such measurement is not necessary for FR2 and FR1 TDD. Regarding FR1 FDD, the necessity was not justified in Rel-15 and RAN4 will not introduce corresponding requirements in Rel-15. Since only one or two SMTCs can be configured for a frequency layer, RAN4 understands that to enable UE to measure all cells on a frequency layer some network synchronization is needed to ensure the SSBs of all cells fall within the configured SMTC windows.

2. Actions:

To RAN WG2 group.

ACTION: 	RAN4 kindly asks RAN2 to take the above information into account in their future work.

3. Date of Next TSG-RAN WG4 Meetings:

TSG-RAN4 #93			18th – 22nd Nov 2019   Reno, Nevada, US

TSG-RAN4 #94			24th – 28th Feb 2020    Athens, GR