3GPP TSG-RAN WG4 Meeting #92bis	R4-1912737

Chongqing, China, 14 – 18 October 2019

Agenda Item:	7.13.3.3

Source:	Ericsson

Title:	Simulation results of PDSCH for HST single tap

Document for:	Discussion

# 1	Introduction

RAN4#92 agreed with the WF on UE demodulation requirements for LTE HST [1]. In WF, it was agreed that:

- Agreement - Introduce the HST demodulation performance requirement in HST single tap channel model with maximum Doppler shift 972Hz. - The applicability rule will be introduced. - Companies are encouraged to provide simulation results in next meeting - Use simulation settings from Rel-14 HST-SFN channel as a baseline but replace HST-SFN model by single tap model.

We provide our initial simulation result of HST single tap according to the way forward.

# 2	Simulation results

## 2.1	Simulation parameters

Table 1 list the one CC test cases used for CA scenario, and Table 2 shows the FRC used for the simulation. HST-SFN parameters are given in Table 3.

Table 1	Simulation list for PDSCH HST single tap with 500km/h

| Band-width | Reference channel | OCNG pattern | Propagation condition | Correlation matrix and antenna config. |
| --- | --- | --- | --- | --- |
| 10MHz | R.87 FDD/TDD | OP.1 FDD | HST-SFN | 2x2 |

Table 2	FRC used for simulation.

|  | R.87 |
| --- | --- |
| Channel BW | 10 |
| Allocated RBs | 50 |
| Modulation | 64QAM |
| TBS for subframes 1,2,3,4,6,7,8,9 (FDD) | 15264 |
| TBS for subframes 0 and 5 (FDD) | N/A |
| TBS for subframes 1 and 6 (TDD) | 6456 |
| TBS for subframes 4 and 9 (TDD) | 15264 |
| TBS for subframes 0,2,3,5,7,8 | N/A |

Table 3	HST single tap parameters.

| Parameter | Value |
| --- | --- |
|  | 300 m |
|  | 2 m |
|  | 500 km/h |
|  | 972 Hz |

## 2.2	Simulation results

Table 4 lists SNR values to achieve 70% of the maximum throughput based on the alignment simulation results for TDD/TDD with 2Rx/4Rx.

Table 4	Simulation results.

|  | Receive antennas | SNR (dB) to achieve 70% of the maximum throughput |
| --- | --- | --- |
| FDD | 2Rx | 8.2 |
|  | 4Rx | 5.7 |
| TDD | 2Rx | 8.2 |
|  | 4Rx | 5.6 |

# References

- R4-1910101, “Way forward on LTE UE HST performance requirements”, NTT DOCOMO.